3GPP TSG-RAN WG3 Meeting #108-e	R3-203904

Online, 1st - 10th June 2020

Agenda Item:	10.2.3.1

Source: 	CMCC

Title: 	(TP for TS 38.470) on F1 aspects for SON

Document for:	Approval

# Introduction

This contribution adds new functions and procedures for TS 38.470.

# Text proposal for TS 38.470

--------------------------------------------------Start of the change---------------------------------------------------------

5.2.6	Warning messages information transfer function

This function allows to cooperate with the warning message transmission procedures over NG interface. The gNB-CU is responsible for encoding the warning related SI message and sending it together with other warning related information for the gNB-DU to broadcast over the radio interface.

5.2.7	Remote Interference Management (RIM) message transfer function

This function enables the transfer of Remote Interference Management (RIM) backhaul messages between the gNB-CU and the gNB-DU. RIM messages are transferred over F1-C. The gNB-CU acts as a coordinator on behalf of its affiliated gNB-DUs, by merging the RIM information received from its gNB-DUs and forwarding the merged information to the target gNBs or gNB-CUs, transparently via the core network. Similarly, a gNB-CU distributes an incoming RIM backhaul message to all its concerned gNB-DUs.

5.2.8	Trace function

The Trace function provides means to control trace sessions for a UE over F1 interface.

5.2.x	Load management function

The load management function allows an gNB-CU to request the reporting of load measurements to gNB-DU and is used by gNB-DU to report the result of measurements admitted by gNB-DU.

5.2.y	Data exchange for self-optimisation function

This function allows the gNB-CU and the gNB-DU to exchange information in order to support self-optimization functionality.

-------------------------------------------Start of the second change------------------------------------------

6.1.7	Radio information transfer procedures

The F1 Radio information transfer procedures are listed below:

-	DU-CU Radio Information Transfer procedure

- 	CU-DU Radio Information Transfer procedure

6.1.8	UE Tracing procedures

The following procedures are used to trace the UE:

- Trace Start procedure

- Deactivate Trace procedure

6.1.x	Load management procedures

The load management procedures are listed as below:

- Resource Status Reporting Initiation

- Resource Status Reporting

6.2.y	Data exchange for self-optimisation procedures

The data exchange for self-optimisation procedures are used to transfer access, failure and mobility related information between the gNB-CU and the gNB-DU to enable self-optimisation

-	Access and Mobility Indication

--------------------------------------------End of the change-----------------------------------------------------