3GPP TSG-RAN WG3 Meeting #108-e	R3-203908

E-meeting, Jun 1st – Jun 12th, 2020

Agenda item:	8.1

Source:	Qualcomm Incorporated

Title:	(TP for TS 36.423) Support of Inter-RAT handover for Fast MCG Recovery

WID/SID:	 LTE_NR_DC_CA_enh-Core – Release 16

Document for:	Discussion and Decision

# Discussion

In the previous RAN2 #109-bis-e meeting, it was agreed to support inter-RAT HO via SRB3 for the fast MCG recovery procedure [1]. The following agreement was made:

Agreement:

Inter-RAT handover via SRB3 is supported upon MCG failure recovery, including the following scenarios:
	- (NG)EN-DC to NR;

- NR-DC to LTE/EPC;

- NR-DC to LTE/5GC;

- EN-DC to GERAN/UTRAN;

- NR-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G).

Actions:

To RAN3 group.

ACTION: RAN2 kindly ask RAN3 to take the above agreement into consideration and introduce the necessary signalling for supporting the sending of MobilityfromEutraCommand or MobilityfromNRCommand via SRB3 for fast MCG recovery.

RAN2 sent an LS to RAN3 [2] requesting RAN3 to make the required signalling changes to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure:

In the Annex, we indicate the modifications required to the RRC Transfer message definition in TS 36.423.

We have the following proposals.

Proposal 1. In order to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure, the following change is required to the RRC Transfer message definition:

Indicate in the RRC container that carries the network response RRC message from MeNB to SgNB that MobilityFromEUTRACommand message can also be included.

According to TS 36.331, MobilityFromEUTRACommand procedure can be used for inter-RAT handover from E-UTRAN to GERAN, UTRA, CDMA2000 systems, NR, or handover a UE to an E-UTRA cell connected to another type of CN. Thus, it covers the following Inter-RAT handover scenarios in the RAN2 agreement:

- EN-DC to NR;

- EN-DC to GERAN/UTRAN.

Proposal 2. RAN3 agrees on the changes proposed in the Annex.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following proposals:

Proposal 1. In order to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure, the following change is required to the RRC Transfer message definition:

Indicate in the RRC container that carries the network response RRC message from MeNB to SgNB that MobilityFromEUTRACommand message can also be included.

Proposal 2. RAN3 agrees on the changes proposed in the Annex.

# References

[1] Chair notes, RAN2 #109-bis-e meeting.

[2] R2-2004276, LS to RAN3 on Support of inter-RAT HO for fast MCG recovery.

# Annex: TP for TS 36.423

------------------------------ Start of change, Section 9.1.4.21 ---------------------------------------------------------------------

#### 9.1.4.21	RRC TRANSFER

This message is sent by the MeNB to the en-gNB or by the en-gNB to the MeNB to transfer an RRC message.

Direction: MeNB  en-gNB or en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Split SRB |  | 0..1 |  |  | YES | reject |
| >RRC Container | O |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 36.331 [9] and ciphered with the key of the MeNB | – |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type | – |  |
| >Delivery Status | O |  | 9.2.104 | DL RRC delivery status of split SRB | – |  |
| NR UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | Includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [31] containing the MeasurementReport message or FailureInformation message. | – |  |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Fast MCG Recovery via SRB3 from SgNB to MeNB |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | Includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [9] containing MCGFailureInformation message. | – |  |
| Fast MCG Recovery via SRB3 from MeNB to SgNB |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | Includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [9] containing the RRCConnectionReconfiguration or RRCConnectionRelease or MobilityFromEUTRACommand message. | – |  |

------------------------------ End of change, Section 9.1.4.21 ----------------------------------------------------------------------