3GPP TSG-RAN3 Meeting #108-e	R3-203909

E-meeting, 01 - 11 June 2020

Title: 	(TP to NGAP BL CR#0120 “Introduction of NB-IoT Paging and eDRX aspects”) Correction of asn.1

Source: 	Huawei

Agenda item:	14.1

Document Type:	other

- Introduction

As for a Sequence type extendable IE, we should always use extension container, this paper provides the TP to update the asn.1 part of the sequence type extendable IE, in the NGAP BL CR#0120 “Introduction of NB-IoT Paging and eDRX aspects”.

Note that there are still change on change needs to be fixed in this BL CR.

2. Text Proposal to 38.413 BL CR0120

9.4.5	Information Element Definitions

#### -- N

SKIPPED ASN.1 UNCHANGED

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

choice-Extensions		ProtocolIE-SingleContainer { {N3IWF-ID-ExtIEs} }

}

N3IWF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NAS-PDU ::= OCTET STRING

NASSecurityParametersFromNGRAN ::= OCTET STRING

NB-IoT-DefaultPagingDRX ::= ENUMERATED {rf128, rf256, rf512, rf1024, ... }

NB-IoT-PagingDRX ::= ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024, ... }

NB-IoT-Paging-eDRXCycle ::= ENUMERATED {hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, ...}

NB-IoT-Paging-TimeWindow ::= ENUMERATED {s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, ... }

NB-IoT-Paging-eDRXInfo ::= SEQUENCE {

nB-IoT-Paging-eDRXCycle NB-IoT-Paging-eDRXCycle,

nB-IoT-Paging-TimeWindow NB-IoT-Paging-TimeWindow OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { NB-IoT-Paging-eDRXInfo-ExtIEs} } OPTIONAL,

...

}

NB-IoT-Paging-eDRXInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NetworkInstance ::= INTEGER (1..256, ...)