3GPP TSG-RAN WG3 #108-e																	   R3-203917

Electronic meeting, 1st - 12th June, 2020

Agenda item:	31.2

Source:	NTT DOCOMO, INC.

Title:	Support beam level inter-RAT measurement report in E-CID measurement in LPPa

Document for:	Discussion and Decision

1	Introduction

In the last RAN3 meeting, we agreed that both cell-level and beam-level measurements for RSRP and RSRQ are supported in NR E-CID over NRPPa as following.  This paper we would like to further discuss the necessity of supporting beam level inter-RAT measurement in E-CID over LPPa.

-----------------------------------------------------Excerpt from [1] ---------------------------------------------

According to RAN3 agreements, the following information may be transferred from gNB to LMF:

-	SS Reference Signal Received Power (SS-RSRP)

-	SS Reference Signal Received Quality (SS-RSRQ)

-	CSI Reference Signal Received Power (CSI-RSRP)

-	CSI Reference Signal Received Quality (CSI-RSRQ)

-	NR Cell Global Identifier / Physical Cell ID

-	Cell Portion ID

Both cell-level and beam-level measurements for RSRP and RSRQ are supported.

---------------------------------------------------------------------------------------------------------------------

2	Discussion

2.1 Inter-RAT measurement report for E-CID in LPPa

As we could observe from the following excerption that in current LPPa spec [2], inter-RAT measurement report already includes NR measurement (i.e. NR ARFCN, NR PCI, NR SS-RSRP, NR SS-RSRQ). Regarding the SS-RSRP, SS-RSRQ measurement, it is not clear whether beam level measurement is supported, similar as NR E-CID measurement over NRPPa. We understand it is per resource measurement as defined in TS 38.215, so both cell-level and beam-level should be supported. In addition, as an operator we see big benefits of supporting NR beam level measurement reporting for EN-DC deployment, which could help enhance the accuracy of legacy E-CID positioning with relatively low extra development cost.

Observation1: In current LPPa spec, for inter-RAT measurement report, it is not clear whether NR beam level measurement is supported.

Proposal1: Support NR beam level measurement report in inter-RAT measurement report in E-CID measurement over LPPa.

------------------------------------------------------ Excerpt from [2] clause 9.1.1.2----------------------------------------------------

9.1.1.2	 E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by eNB to indicate that the requested E-CID measurement is successfully initiated.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Inter-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.15 |  | YES | ignore |

------------------------------------------------------------End of Excerption--------------------------------------------------------------

-----------------------------------------------------Excerpt from [2] clause 9.2.13-------------------------------------------------------

9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |
| >>Result NR | M | 1..<maxNRMeas> |  |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) |  |

----------------------------------------------------------End of Excerption------------------------------------------------------------

2.2 Contents of NR beam level measurement in LPPa

In current LTE RRC spec [3], as we could observe from the following  excerption, for inter-RAT measurement reporting, UE could report NR SSB measurement result, while CSI-RS measurement result is not included. Therefore, for contents of beam level measurement, SSB-Index, SSB-RSRP, SSB-RSRQ could be included in inter-RAT measurement in E-CID measurement over LPPa.

---------------------------------------------------Except from [3] clause 6.3.5-----------------------------------------------------------

MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility.

----unrelated parts are omitted-----

MeasResultCellNR-r15 ::=			SEQUENCE {

pci-r15								PhysCellIdNR-r15,

measResultCell-r15					MeasResultNR-r15,

measResultRS-IndexList-r15			MeasResultSSB-IndexList-r15				OPTIONAL,

...,

[[	cgi-Info-r15						CGI-InfoNR-r15				OPTIONAL

]]

}

MeasResultNR-r15 ::=				SEQUENCE {

rsrpResult-r15						RSRP-RangeNR-r15						OPTIONAL,

rsrqResult-r15						RSRQ-RangeNR-r15						OPTIONAL,

rs-sinr-Result-r15					RS-SINR-RangeNR-r15						OPTIONAL,

...

}

MeasResultSSB-IndexList-r15::=		SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF MeasResultSSB-Index-r15

MeasResultSSB-Index-r15 ::=		SEQUENCE {

ssb-Index-r15						RS-IndexNR-r15,

measResultSSB-Index-r15				MeasResultNR-r15					OPTIONAL,

...

}

----unrelated parts are omitted-----

----------------------------------------------------------End of Excerption------------------------------------------------------------

Observation2: In current LTE RRC Spec, UE could report inter-RAT measurement of NR SSB measurement results (i.e. SSB-Index, SSB-RSRP, SSB-RSRQ).

Proposal2: Include SSB-IndexList, SSB-Index, SS-RSRP beam value, SS-RSRQ beam value in inter-RAT measurement result in E-CID measurement over LPPa.

3	Conclusion

This contribution discussed the necessity of introducing beam level measurement in inter-RAT measurement in E-CID measurement over LPPa.

Following observations and proposals were obtained.

Observation1: In current LPPa spec, for inter-RAT measurement report, it is not clear whether NR beam level measurement is supported.

Observation2: In current LTE RRC Spec, UE could report inter-RAT measurement of NR SSB measurement results (i.e. SSB-Index, SSB-RSRP, SSB-RSRQ).

Proposal1: Support NR beam level measurement report in inter-RAT measurement report in E-CID measurement over LPPa.

Proposal2: Include SSB-Index, SSB-RSRP, SSB-RSRQ in inter-RAT measurement result in E-CID measurement over LPPa.

References

- R3-202646, LS on support for UL NR E-CID

- TS 36.455

- TS 36.331

ANNEX

TP for including NR beam level measurement in inter-RAT measurement in E-CID measurement over LPPa.

------------------------------------------------------------Start of TP-----------------------------------------------------------------------

UNCHANGED TEXT OMMITED

9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |
| >>Result NR | M | 1..<maxNRMeas> |  |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRP |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRQ |
| >>>NR SSB Index list | O | 1..<maxResultsPerSSBIndex> |  |  |
| >>>>SSB Index |  |  | INTEGER (0..63) |  |
| >>>>NR SSB-RSRP beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRP |
| >>>NR SSB-RSRQ beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRQ |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |
| maxResultsPerSSBIndex | Maximum no. of SSB indices that can be reported with in the message Value is 64. |

NEXT CHANGE

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxResultPerSSBIndex,

maxResultsPerSSBIndex,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

UNCHANGED TEXT OMMITED

ResultNR ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nRARFCN				NRARFCN,

nRPCI				NRPCI,

sS-NRRSRP			SS-NRRSRP	OPTIONAL,

sS-NRRSRQ			SS-NRRSRQ	OPTIONAL,

sSB-Index-List		SSB-Index-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} } OPTIONAL,

...

}

ResultNR-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SRSConfigurationForAllCells ::= SEQUENCE (SIZE (1.. maxServCell)) OF SRSConfigurationForOneCell

SRSConfigurationForOneCell ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

ul-bandwidth				ENUMERATED {n6, n15, n25, n50, n75, n100},

ul-cyclicPrefixLength		CPLength,

srs-BandwidthConfig			ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-Bandwidth				ENUMERATED {bw0, bw1, bw2, bw3},

srs-AntennaPort				ENUMERATED {an1, an2, an4, ...},

srs-HoppingBandwidth		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-cyclicShift				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7},

srs-ConfigIndex				INTEGER (0..1023),

maxUpPts					ENUMERATED {true}			OPTIONAL,	-- Cond ifTDD

transmissionComb			INTEGER (0..1),

freqDomainPosition			INTEGER (0..23),

groupHoppingEnabled			BOOLEAN,

deltaSS						INTEGER (0..29)				OPTIONAL,

sfnInitialisationTime		SFNInitialisationTime,

...

}

Subframeallocation ::= CHOICE {

oneFrame						BIT STRING (SIZE(6)),

fourFrames						BIT STRING (SIZE(24))

}

SS-NRRSRP ::= INTEGER (0..127)

SS-NRRSRQ ::= INTEGER (0..127)

SS-Index-List ::= SEQUENCE (SIZE(1..maxResultPerSSBIndex)) OF SEQUENCE {

sSB-Index					SSB-Index,

sS-NRRSRPBeamValue			SS-NRRSRP,

sS-NRRSRQBeamValue			SS-NRRSRQ,

...

}

----------------------------------------------------------End of TP-------------------------------------------------------------------------