3GPP TSG-RAN WG3 Meeting #108-e	R3-203946

# June 1st – 11th 2020, Online

Agenda Item:	13.3.2.2

Source:	Ericsson

Title:	Response to: R3-203525, R3-203844-5, R3-203617-8, R3-203155-6, R3-203916, R3-203343, R3-203612, R3-203151-2

Document for: 	Agreement

# Introduction

At the RAN3#107bis-e meeting, the following working assumptions (WAs) were taken:

- WA: For F1AP procedure to configure the DL mapping in the Donor-DU, and to configure the UL/DL mapping in the intermediate IAB, the UE-Associated F1AP procedure is used when the BH RLC CH is impacted (e.g. add a new BH RLC CH, or modify an existing BH RLC CH), and Non-UE-Associated F1AP procedure is used otherwise.

- WA: A new class-1 non-UE associated E1AP procedure is defined for donor-CU-CP to inform donor-CU-UP to update the DL UP TNL Information for multiple UEs and child IAB-MTs.

- WA: A new class-1 non-UE-associated F1AP procedure is defined for donor-CU-CP to inform IAB-DU to update UL FTEID, UL BH Info and DL FTEID for UP traffic

This paper discusses the consequences in case the above WAs are turned into agreements.

# Discussion

The common denominator of the above WAs is that NUA (F1AP and E1AP) signalling is used to (re)configure parameters that should be, or are even today, (re)configured by using UA signalling:

- Traffic mapping at donor DU and intermediate IAB nodes;

- UP TNL information update at donor CU-UP;

- UP TNL information update and BAP configuration info at IAB node.

In addition to previously expressed concerns about violating the UA-NUA signalling separation principle, some additional concerns are apparent.

The WA on traffic mapping at donor DU and intermediate IAB nodes states that the same IE would be used in both NUA and UA messages. However, it seems to have been forgotten that NUA and UA signalling are not in sync. In addition to the fact that the NUA and UA messages may use different TNLAs, it also holds that, at any given moment, there can be only one unacknowledged (i.e. in-flight) UA message for a given UE. On the other hand, this principle does not hold for NUA signalling, which allows multiple in-flight messages between a CU and DU at any given moment, thanks to the use of Transaction ID IE. In this case, there is a risk that two messages, one NUA and one UA, carrying different configurations pertaining to the same BH RLC channel, arrive at the destination in the order different from the order in which they were sent. This may cause serious issues, since the receiving node would be wrongly configured for traffic mapping. A similar problem was acknowledged a few meetings ago for the proposal to allow carrying different SRBs over different SCTP streams, that RAN3 fortunately rejected.

Observation 1: The consequence of WA for traffic mapping at donor DU and intermediate IAB nodes, allowing to carry the mapping configuration IE in both UA and NUA messages, is that two different configurations pertaining to the same BH RLC channel, one carried in a NUA and one carried in a UA message, may arrive at the destination in the order different from the order in which they are sent.

The main advantage claimed by the proponents of NUA signalling is signalling reduction – it is claimed that sending one NUA message for configuration of x BH RLC channels stretching between an IAB-DU towards x different children requires less signalling than sending x UA messages with the individual configurations, one per child. The claimed use case is IAB node migration, where, most often new BH RLC channels need to be established, and both the IAB-DU and IAB-MT side need to be reconfigured. If an existing BH RLC channel is used, at least new BAP routing ID(s) will need to be assigned to the migrating IAB-MTs.

However, since the IAB-MT side of a BH RLC channel is (re)configured via RRC signalling, it is still necessary to send x UA F1AP messages to IAB-MT’s parent IAB-DU, encapsulating the RRC reconfiguration of the IAB-MT. Here, the same UA F1AP message can carry both the configuration for the IAB-DU side of the BH RLC channel, and the RRC configuration for the IAB-MT side of the same channel. So, even if NUA signalling is used to configure the IAB-DU side of the channel, UA signalling carrying RRC messages must still be used to configure the IAB-MT side. Hence, the signalling reduction advantage claimed by the proponents of NUA signalling for traffic mapping at donor DU and intermediate IAB nodes is questionable.

Observation 2: The main benefit claimed by the proponents of NUA signalling for traffic mapping at donor DU and intermediate IAB nodes, i.e. signalling reduction, is questionable.

Finally, the papers R3-203845, R3-203617 and R3-203156 propose a new F1AP signalling procedures for updating the UP TNL information and UL backhaul information, which is *exactly* what the UE context management procedures in the IAB BL CR for TS 38.473 can achieve, as pointed in paper R3-203525. Hence, there is no reason to introduce new F1AP signalling for this purpose.

Observation 3: Papers R3-203845, R3-203617 and R3-203156 propose a new F1AP signalling procedure for updating the UP TNL information and UL backhaul information, which is exactly what the UE context management procedures in the IAB BL CR for TS 38.473 can achieve as of today.

Proposal: Change the abovementioned WAs so that UA signalling is used for the following purposes:

- Traffic mapping at donor DU and intermediate IAB nodes;

- UP TNL information update at donor CU-UP;

- UP TNL information update and BAP configuration info at IAB node.

# Conclusions

In this paper we discuss the consequences of WAs from RAN3#107bis-e on UP TNL information update at intra-donor migration and traffic mapping at IAB-donor-DU and intermediate IAB nodes. We observe the following:

Observation 1: The consequence of WA for traffic mapping at donor DU and intermediate IAB nodes, allowing to carry the mapping configuration IE in both UA and NUA messages, is that two different configurations pertaining to the same BH RLC channel, one carried in a NUA and one carried in a UA message, may arrive at the destination in the reverse order with respect to the order in which they are sent.

Observation 2: The main benefit claimed by the proponents of NUA signalling for traffic mapping at donor DU and intermediate IAB nodes, i.e. signalling reduction, is questionable.

Observation 3: Papers R3-203845, R3-203617 and R3-203156 propose a new F1AP signalling procedures for updating the UP TNL information and UL backhaul information, which is exactly what the UE context management procedures in the IAB BL CR for TS 38.473 can achieve as of today.

Based on the observations, the following is proposed:

Proposal: Change the abovementioned WAs so that UA signalling is used for the following purposes:

- Traffic mapping at donor DU and intermediate IAB nodes;

- UP TNL information update at donor CU-UP;

- UP TNL information update and BAP configuration info at IAB node.