3GPP TSG-RAN3 Meeting #108	R3-204170

01 - 12 June 2020 E-Meeting

Title: 	(TP for BL CR for TS 38.455/TS 38.473): TRP Geographical Coordinates

Source: 	Huawei, Deutsche Telekom, LGU+, BT, Orange, CMCC

Agenda item:	19.3

Document Type:	other

# 1. Introduction

Since Rel-15, the RAN node includes the RAN Access Point Position information to provide the location of the TPs (TRP) to the LMF (or E-SMLC) by using global coordinates. This coordinates are used in various positioning methods. In the last RAN3 meeting, it was agreed to include the “TRP Geographical Coordinates” in the TRP Information Exchange procedure [1] over NRPPa and F1 to allow the reporting from gNB to LMF.

This paper discusses “TRP Geographical Coordinates” accuracy with regards to future 5G small cells, IAB, indoor, enterprise and factory deployments.

The Rev1 provides:

- the high accuracy by using HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid. LPP like

- The unic choice of positioning location method for TRP

- E-CID legacy impact of the high Accuracy introduction

- Addition of different possible referential for the relative coordinates

# 2. Discussion

2.1 The precision of the TRP coordinate

The coordinate information of the TRP is important because the UE location is derived from the location of the TRPs. It is obvious to us that an accurate location information of the TRPs can directly improve the UE location calculation at LMF. In Rel-15, the TP (TRP) location information is described by the “NG-RAN Access Point Position”. This IE will be re-used to describe the TRP locations as “TRP Geographical Coordinates” Information Element.

In the “NG-RAN Access Point Position”, the coordinate has the following values using Point representation in Universal Geographical Area Description (GAD) TS 23.032 [2], where

- The latitude is coded with 24 bits: 1 bit of sign and a number between 0 and 223-1 coded in binary on 23 bits.

- The longitude, expressed in the range -180°, +180°, is coded as a number between -223 and 223-1, coded in 2's complement binary on 24 bits.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |

The “Degrees Of Longitude” is valued from -223..223-1 to represent the longitude range from -180 to 180 in degreee. It can be derived a positioning resolution about 2.4 meter. This precision of the positioning can be useful for some outdoor scenarios when the target accuracy is 10 meters.

However, in context of 5G verticals, it is far not enough for the indoor positioning. In typical indoor postioning scenario, such as positioning in factory, shopping mall, hospital, etc., the precise of the positoning is expected to be 1 meter/sub-meter level. Therefore, it is proposed to enhance the precision of the TRP coordianate. It is proposed to add higher accuracy representations of the degrees of latitude and longitude, which have the maximum value of degrees of latitude and longitude of 231-1.

Observation 1: The “NG-RAN Access Point Position” IE can be re-used to describe the location of TRP. But the precision is far not enough for the indoor postioning.

Proposal 1: Include the “NG-RAN Access Point Position” IE in the “TRP Geographical Coordinates” IE.

Proposal 2: Add higher accuracy representation of the degrees of latitude and longitude which have the maximum value of 231-1 in the “NG-RAN Access Point Position” IE.

2.2 The relative location with Cartesian coordinate

In some indoor scenarios, it might be difficult to obtain the accurate GPS coordinates of the TRPs, as the GPS signals could be blocked by the building walls, which requires additional and unnecessary work load of calibrating indoor location. Thus, the RAN node may not be able to provide the accurate or effective global coordinates to the LMF, which can lead to large error for the UE positioning.

An effective solution to this problem is to provide the relative location with Cartesian coordinate. The Cartesian coordinate can be described by XYZ values. With this information, the accuracy of the TRP location at LMF can be guaranteed for the scenario where accurate global coordinates are hard to obtain.

From engineering point of view, it seems to us, an easier and robust process to report the site location based on a reference point (antenna on roof top, well know reference point, etc … ) than to report the “GAD” coordinates” of the TRPs deployed as example belong a factory’s product line with upper administrative floor etc…

The signalling of Cartesian Coordinated associated with a reference point will also let know the LMF that the Geographical Coordinates are exact i.e. without the uncertainty related to GAD.

The relative location with Cartesian coordinate can be described as follows:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Referencial | M |  |  | Referencial point (0,0,0) from where the XYZ values are deduced from |
| >Relative Coordinate |  |  |  |  |
| >>Relative Coordinate ID | M |  | INTEGER(0.. 231-1,..) | Referential ID maped via OAM |
| XYZ unit | M |  | ENUMERATED (cm, dm,..) |  |
| X value | M |  | INTEGER (-231.. 231-1) |  |
| Y value | M |  | INTEGER (-231.. 231-1) |  |
| Z value | M |  | INTEGER (-215.. 215-1) |  |

The XYZ values describe the coordinate values on X, Y, Z axis respectively of the TRP. The “Referential” is used to uniquely identify a referential point (0, 0, 0) from where XYZ are derived. The Referential could be any particular global location where that exact location can be determined and serve as reference. We propose here to use a Relative Coordinate ID IE, where the exact coordinates could be mapped by OAM. The choice in the structure will allow the possibility to select other methods if needed e.g. Master TRP, GNSS Coordinates etc …

Observation 2: The global coordinate representation of the TPR location could lead to large error for indoor positioning, while relative Cartesian coordinate can guarantee the accuracy of the TRP location at LMF.

Proposal 3: Include the relative Cartesian coordinate as an option in “TRP Geographical Coordinates” IE.

A TP to 38.455 has been provided in this paper. We also provide a TP to 38.473, which can be find in [3].

The introduction of the higher accuracy will be able to be used in Rel-15 by backward compatible way and the Relative Coordinates will impact rel-16 only.

Proposal 4: Agree the TP for TS 38.455 and TS 38.473.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: The “NG-RAN Access Point Position” IE can be re-used to describe the location of TRP. But the precision of the TRP coordinates is far not enough for the indoor postioning.

Proposal 1: Include the “NG-RAN Access Point Position” IE in the “TRP Geographical Coordinates” IE.

Proposal 2: Add higher accuracy representation of the degrees of latitude and longitude which have the maximum value of 231-1 in the “NG-RAN Access Point Position” IE.

Observation 2: The global coordinate representation of the TPR location could lead to large error in indoor positioning, while relative Cartesian coordinate can guarantee the accuracy of the TRP location at LMF.

Proposal 3: Include the relative Cartesian coordinate as an option in “TRP Geographical Coordinates” IE.

The introduction of the higher accuracy will be able to be used in Rel-15 by backward compatible way and the Relative Coordinates will impact rel-16 only.

Proposal 4: Agree the TP for TS 38.455 and TS 38.473.

# 4. Reference

- RAN3#107bis Chairman notes.

- TS 23.032, Universal Geographical Area Description (GAD)

# Annex – TP to TS 38.455

Start of the first change

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

#### 8.2.3.x	Abnormal Conditions

If both the NG-RAN Access Point Position and the Access Point Geographical Coordinates IEs are included in the E-CID Measurement Result IE, the LMF shall consider the procedure as failed and initiate local error handling.

Next Change

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. This IE is mutualy exclusive with Access Point Geographical Coordinates |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] |
| >>Result RSRP EUTRA |  | 1 .. <maxCellReport> |  |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell |
| >>>EARFCN | M |  | INTEGER (0.. 262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] |
| >>> CGI EUTRA | O |  | 9.2.6 | Cell Global Identifier of the reported E-UTRA cell |
| >>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  |
| >>Result RSRQ EUTRA |  | 1 . <maxCellReport> |  |  |
| >>> PCI EUTRA | M |  | 9.2.7 | Physical Cell Identifier of the reported E-UTRA cell |
| >>>EARFCN | M |  | INTEGER (0..262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell |
| >>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  |
| Access Point Geographical Coordinates | O |  | 9.2.z10 | This IE is mutualy exclusive with NG-RAN Access Point Position |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

### 9.2.bb	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

[Editor’s Note: further details on the IEs are FFS]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.aa |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  |
| >CHOICE TRP Information Item | M |  |  |  |
| >>PRS ID | M |  | INTEGER (0..255) | DL PRS ID |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NG-RAN CGI | M |  | 9.2.6 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>Timing Information | M |  | 9.2.z6 |  |
| >>PRS Configurations | M |  | 9.2.z7 |  |
| >>SSB Configurations | M |  | 9.2.z8 |  |
| >>SFN Initialisation Time | M |  | BIT STRING (64) |  |
| >>Spatial Direction Information | M |  | 9.2.z9 |  |
| >>Geographical Coordinates | M |  | 9.2.z10 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

Next Change

### 9.2.z10	Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Access Point | M |  |  |  |
| >Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. |
| >High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | 9.2.xz | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Access Point Position Relative |  |  |  |  |
| >>NG-RAN Access Point Position Relative | M |  | 9.2.xx | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |

### 9.2.xz	 NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

### 9.2.xx	 NG-RAN Access Point Position Relative

The NG-RAN Access Point Position Relative IE is used to identify the geographical position of an NG-RAN Access Point using relative Cartesian coordinate, which is expressed as XYZ values.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Referencial | M |  |  | Reference point (0,0,0) from where the XYZ values are deduced from |
| >Relative Coordinate |  |  |  |  |
| >>Relative Coordinate ID | M |  | INTEGER(0.. 231-1,..) | Referential ID maped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>NG-RAN Access Point Position |  |  | 9.2.10 |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position |  |  | 9.2.xz |  |
| XYZ unit | M |  | ENUMERATED (cm, dm,..) |  |
| X value | M |  | INTEGER (-231.. 231-1) | Positive value represents northing from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-231.. 231-1) | Positive value represents easting from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-215.. 215-1) | Positive value represents height above reference point in units of XYZ Unit IE. |

Next Change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

accessPointGeogreaphicalCoordinates		GeographicalCoordinates		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

<Unchanged Text Omitted>

#### -- G

GeographicalCoordinates ::= CHOICE {

accessPointPosition							NG-RANAccessPointPosition,

highAccuracyAccessPointPosition				NG-RANHighAccuracyAccessPointPosition,

accessPointPositionRelative					NG-RANAccessPointPositionRelative,

choice-extension							ProtocolIE-SingleContainer { { GeographicalCoordinates-ExtIEs } }

}

GeographicalCoordinates-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

<Unchanged Text Omitted>

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NG-RANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100)

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NG-RANAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NG-RANHighAccuracyAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RANAccessPointPositionRelative ::= SEQUENCE {

referencial					Referencial,

xYZUnit						ENUMERATED {cm, dm, ...},

xValue						INTEGER (-2147483648..2147483647),

yValue						INTEGER (-2147483648..2147483647),

zValue						INTEGER (-32768..32767),

iE-Extensions				ProtocolExtensionContainer { { NG-RANAccessPointPositionRelative-ExtIEs} } OPTIONAL

}

NG-RANAccessPointPositionRelative-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Referencial ::= CHOICE {

relativeCoordinate				RelativeCoordinateID,

choice-extension				ProtocolIE-SingleContainer { { Referencial-ExtIEs } }

}

Referencial-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

RelativeCoordinateID ::= INTEGER (0..2147483647)

<Unchanged Text Omitted>

End of the last change

# Annex – TP to TS 38.473

Start of the first change

### 9.3.1.e	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | INTEGER(1..16384) |  |
| Cell Information | M |  | NR CGI 9.3.1.12 |  |
| PRS Configurations | O |  | 9.3.1.f |  |
| Geographical Coordinates | M |  | Access Point Position 9.3.1.z | The configured estimated geographical position of the antenna of the cell/TRP. |
| TRP Information Type Response List |  | 1 |  |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

Start of the next change

### 9.3.1.z	Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Access Point | M |  |  |  |
| >Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | 9.3.1.b | The configured estimated geographical position of the antenna of the cell/TRP. |
| >High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | 9.3.1.p | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Access Point Position Relative |  |  |  |  |
| >>NG-RAN Access Point Position Relative | M |  | 9.3.1.a | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |

Start of the next change

### 9.3.1.p	 NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [x].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

### 9.3.1.a	 NG-RAN Access Point Position Relative

The NG-RAN Access Point Position Relative IE is used to identify the geographical position of an NG-RAN Access Point using relative Cartesian coordinate, which is expressed as XYZ values.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Referencial | M |  |  | Reference point (0,0,0) from where the XYZ values are deduced from |
| >Relative Coordinate |  |  |  |  |
| >>Relative Coordinate ID | M |  | INTEGER(0.. 231-1,..) | Referential ID maped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>NG-RAN Access Point Position |  |  | 9.3.1.b |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position |  |  | 9.3.1.p |  |
| XYZ unit | M |  | ENUMERATED (cm, dm,..) |  |
| X value | M |  | INTEGER (-231.. 231-1) | Positive value represents northing from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-231.. 231-1) | Positive value represents easting from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-27.. 27-1) | Positive value represents height above reference point in units of XYZ Unit IE. |

Start of the next change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

GeographicalCoordinates ::= CHOICE {

accessPointPosition							NGRANAccessPointPosition,

highAccuracyAccessPointPosition				NGRANHighAccuracyAccessPointPosition,

accessPointPositionRelative					NGRANAccessPointPositionRelative,

choice-extension							ProtocolIE-SingleContainer { { GeographicalCoordinates-ExtIEs } }

}

GeographicalCoordinates-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

<Unchanged Text Omitted>

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100)

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NGRANHighAccuracyAccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NGRANAccessPointPositionRelative ::= SEQUENCE {

referencial					Referencial,

xYZUnit						ENUMERATED {cm, dm, ...},

xValue						INTEGER (-2147483648..2147483647),

yValue						INTEGER (-2147483648..2147483647),

zValue						INTEGER (-32768..32767),

iE-Extensions				ProtocolExtensionContainer { { NGRANAccessPointPositionRelative-ExtIEs} } OPTIONAL

}

NGRANAccessPointPositionRelative-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

Referencial ::= CHOICE {

relativeCoordinate				RelativeCoordinateID,

choice-extension				ProtocolIE-SingleContainer { { Referencial-ExtIEs } }

}

Referencial-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RelativeCoordinateID ::= INTEGER (0..2147483647)

<Unchanged Text Omitted>

TRPInformation ::= SEQUENCE {

tRPID							TRPID,

nRCGI							NRCGI,

pRSConfigurations				PRSConfigurations	OPTIONAL,

geographicalCoordinates		GeographicalCoordinates,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL

}

TRPInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of the last change