3GPP TSG-RAN3 Meeting #108e	R3-204172

Online, 1st – 11th June

Agenda Item:	19.3

Source: 	Ericsson, NTT Docomo, Huawei

Title:  	Reporting of SFTD measurements in NRPPa

Document for:	Discussion and approval

# TP to NRPPa: support of SFTD measurements transfer to LMF

START OF CHANGES

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] |
| >>Result RSRP EUTRA |  | 1 .. <maxCellReport> |  |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell |
| >>>EARFCN | M |  | INTEGER (0.. 262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] |
| >>> CGI EUTRA | O |  | 9.2.6 | Cell Global Identifier of the reported E-UTRA cell |
| >>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  |
| >>Result RSRQ EUTRA |  | 1 . <maxCellReport> |  |  |
| >>> PCI EUTRA | M |  | 9.2.7 | Physical Cell Identifier of the reported E-UTRA cell |
| >>>EARFCN | M |  | INTEGER (0..262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell |
| >>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  |
| >>Result SFTD |  | 1 . <maxCellReport> |  |  |
| >>>PCI EUTRA | M |  | 9.2.7 | Physical Cell Identifier of the reported E-UTRA cell |
| >>>EARFCN | M |  | INTEGER (0..262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported cell |
| >>>SFN-Offset | M |  | INTEGER (0..1023) | The observed SFN and frame timing difference that a UE measured between E-UTRA PCell and an measured Cell, |
| >>>MeanFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Average value of the frame boundary offset observed between E-UTRA PCell and the measured cell. |
| >>>MinFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Minimum value of the frame boundary offset observed between E-UTRA PCell and the measured cell. |
| >>>MaxFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Maximum value of the frame boundary offset |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

START OF ASN.1 CHANGE

NEXT CHANGE

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

...

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

resultSFTD								ResultSFTD,

measuredResultsValue-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

NEXT CHANGE

ResultRSRQ-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-EUTRA-Item

ResultRSRQ-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-UTRA			CGI-EUTRA OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRQ-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSFTD ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultSFTD-Item

ResultSFTD-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

SFN-Offset			INTEGER (0..1023),

FrameBoundary-Offset		INTEGER (-30720..30719),

MinFrameBoundary-Offset		INTEGER (-30720..30719),

MaxFrameBoundary-Offset		INTEGER (-30720..30719),

iE-Extensions	ProtocolExtensionContainer { { ResultSFTD-Item-ExtIEs} } OPTIONAL,

...

}

ResultSFTD-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

END OF CHANGE