3GPP TSG-RAN WG3 Meeting #108-e 	R3-204244

1-11 June 2020, Online

Agenda item:	13.2.1.2.

Source:	Nokia, Nokia Shanghai Bell, Huawei, Samsung

Title:	(TP for NR-IAB BL CR for TS 38.401) BH RLC channel mapping configuration in Donor-DU and intermediate IAB nodes

WID/SID:	NR_IAB-Core - Release 16

Document for:	Discussion and Decision

# Introduction

This contribution proposes new section in Stage-2 to clarify the BH RLC channel mapping, per e-mail discussion on CB#6.

# References

- R3-204099, Summary of Offline Discussion on CB: # 6_IAB_bearer_mapping

-------------------------------------------Start of Change-------------------------------------------

8.9. x	Traffic Mapping

8.9.x.1	Traffic Mapping from IP-layer to Layer-2

When forwarding IP packets, the IAB-donor-DU performs the traffic mapping from IP-layer to layer-2 as defined in TS 38.340 [xx]. The traffic mapping information is configured by the IAB-donor-CU, which contain the IP header information, and the BH information including the BAP routing ID and a list of egress link and BH RLC channel pairs.

Multiple traffic mappings can contain the same BAP routing ID and/or list of egress link and BH RLC channel pairs.

The traffic mappings can be configured as part of the F1AP UE Context Setup or F1AP UE Context Modification procedures. They may also be configured via the non-UE-associated BAP Mapping Configuration procedure.

NOTE: Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

8.9.x.2		BH RLC Channel Mapping on BAP Layer

When traffic is forwarded on BAP layer as described in TS 38.300 [2], the IAB-node performs the BH RLC channel mapping as defined in TS 38.340 [xx]. The BH RLC channel mapping information is configured by the IAB-donor-CU.

The BH RLC channel mappings can be configured as part of the F1AP UE Context Setup or F1AP UE Context Modification procedures. They may also be configured via the non-UE-associated BAP Configuration procedure.

NOTE: Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

-------------------------------------------End of changes ------------------------------------------