3GPP TSG-RAN WG3 meeting #108-e	R3-204252

Online, 1st – 11th June, 2020

Source: 	Huawei, Nokia, Nokia Shanghai Bell

Title:	(TP for NR_IAB BL CR for TS38.463) E1AP group signling for IAB topology change

Agenda Item:	13.3.2.2

Document for:	Agreement

- Introduction

In this paper, detail TP is provided for TS 38.463 to address the group signalling design for NUA E1AP messages used for IAB topology update.

# Appendix: Text Proposal for NR-IAB BL CR for TS 38.463

------------------------------------------------Start of Text Proposal-----------------------------------------------------

-------------------------------------------Change 1-------------------------------------------

# 8	E1AP procedures

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| gNB-CU-UP E1 Setup | GNB-CU-UP E1 SETUP REQUEST | GNB-CU-UP E1 SETUP RESPONSE | GNB-CU-UP E1 SETUP FAILURE |
| gNB-CU-CP E1 Setup | GNB-CU-CP E1 SETUP REQUEST | GNB-CU-CP E1 SETUP RESPONSE | GNB-CU-CP E1 SETUP FAILURE |
| gNB-CU-UP Configuration Update | GNB-CU-UP CONFIGURATION UPDATE | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-UP CONFIGURATION UPDATE FAILURE |
| gNB-CU-CP Configuration Update | GNB-CU-CP CONFIGURATION UPDATE | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-CP CONFIGURATION UPDATE FAILURE |
| E1 Release | E1 RELEASE REQUEST | E1 RELEASE RESPONSE |  |
| Bearer Context Setup | BEARER CONTEXT SETUP REQUEST | BEARER CONTEXT SETUP RESPONSE | BEARER CONTEXT SETUP FAILURE |
| Bearer Context Modification (gNB-CU-CP initiated) | BEARER CONTEXT MODIFICATION REQUEST | BEARER CONTEXT MODIFICATION RESPONSE | BEARER CONTEXT MODIFICATION FAILURE |
| Bearer Context Modification Required (gNB-CU-UP initiated) | BEARER CONTEXT MODIFICATION REQUIRED | BEARER CONTEXT MODIFICATION CONFIRM |  |
| Bearer Context Release (gNB-CU-CP initiated) | BEARER CONTEXT RELEASE COMMAND | BEARER CONTEXT RELEASE COMPLETE |  |
| IAB UP TNL Address Update | IAB UP TNL ADDRESS UPDATE | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE | IAB UP TNL ADDRESS UPDATE FAILURE |

-------------------------------------------Change 2-------------------------------------------

## 8.x	IAB Procedures

### 8.x.1	IAB UP TNL Address Update

#### 8.x.1.1	General

The purpose of the IAB UP TNL Address Update procedure is to allow the gNB-CU-CP to request the gNB-CU-UP to update the TNL Address(es) for all the DL F1-U GTP-U tunnels related to a TNL address, and allow the gNB-CU-UP to inform gNB-CU-CP the updated TNL Address(es) for all the UL F1-U GTP-U tunnels. The procedure uses non-UE associated signalling.

NOTE: 	This procedure is applicable for IAB-nodes, where the term “gNB-CU-CP” applies to IAB-donor-CU-CP, and the term “gNB-CU-UP” applies to IAB-donor-CU-UP.

NOTE:	Implementation must ensure the avoidance of potential race conditions, i.e. it must ensure that the UP configuration (e.g., UL/DL UP TNL address) update is not concurrently performed using the non-UE-associated IAB UP TNL Address Update procedure and the UE-associated procedures for Bearer Context Management.

#### 8.x.1.2	Successful Operation

IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE

IAB UP TNL ADDRESS UPDATE

gNB-CU-CP

gNB-CU-UP

Figure 8.x.1.2-1: IAB UP TNL Address Update procedure: Successful Operation.

Upon reception of the IAB UP TNL ADDRESS UPDATE message, if the DL UP TNL Address to Update List IE is contained, the gNB-CU-UP shall replace the old TNL Address(es) by the new TNL Address(es) for all the maintained DL F1-U GTP tunnels corresponding to the old TNL Address(es).

If the UL UP TNL Address to Update List IE is contained in the IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE message, the gNB-CU-CP shall take the new TNL address(es) as replacement for each corresponding old TNL address(es).

#### 8.x.1.3	Unsuccessful Operation

IAB UP TNL ADDRESS UPDATE FAILURE

IAB UP TNL ADDRESS UPDATE

gNB-CU-CP

gNB-CU-UP

Figure 8.x.1.3-1: IAB UP TNL Address Update procedure: Unsuccessful Operation.

If the gNB-CU-UP receives an IAB UP TNL ADDRESS UPDATE message, but cannot perform the update accordingly, it shall consider the update procedure as failed and respond with an IAB UP TNL ADDRESS UPDATE FAILURE and appropriate cause value.

If the IAB UP TNL ADDRESS UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the UP TNL address update towards the same gNB-CU-UP.

#### 8.x.1.4	Abnormal Conditions

Not Applicable.

-------------------------------------------Change 3-------------------------------------------

# 9	Elements for E1AP communication

## 9.1	General

>>>>>>>>unchanged parts are skipped<<<<<<<<

### 9.2.x	IAB Messages

#### 9.2.x.1	IAB UP TNL ADDRESS UPDATE

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to update the TNL address(es) of the DL F1-U GTP tunnel information.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| DL UP TNL Address To Update List |  | 0..1 |  |  | YES | reject |
| > DL UP TNL Address To Update Item IEs |  | 1..<maxnoofTNLAddresses> |  |  | - | - |
| >>Old TNL Address | M |  | 9.3.2.4 | The old transport Layer Address of IAB-DU for DL F1-U GTP tunnel. | - | - |
| >>New TNL Address | M |  | 9.3.2.4 | The new transport Layer Address of IAB-DU for DL F1-U GTP tunnel. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAddresses | Maximum no. of TNL addresses to be updated in one E1AP procedure. Value is 8. |

#### 9.2.x.2	IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE

This message is sent by the gNB-CU-UP to the gNB-CU-CP to acknowledge the update of TNL address in DL F1-U GTP tunnel information, or provide the updated TNL address(es) of the UL F1-U GTP tunnel information.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UL UP TNL Address to Update List |  | 0..1 |  |  | YES | ignore |
| > UL UP TNL Address Updated Item IEs |  | 1..<maxnoofTNLAddresses> |  |  | - | - |
| >>Old TNL Address | M |  | 9.3.2.4 | The old transport Layer Address of CU-UP for UL F1-U GTP tunnel. | - | - |
| >>New TNL Address | M |  | 9.3.2.4 | The new transport Layer Address of CU-UP for UL F1-U GTP tunnel. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAddresses | Maximum no. of TNL addresses updated in one E1AP procedure. Value is 8. |

#### 9.2.x.3	IAB UP TNL ADDRESS UPDATE FAILURE

This message is sent by the gNB-CU-UP to indicate IAB UP TNL address Update failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-------------------------------------------Change 4-------------------------------------------

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

>>>>>>>>unchanged parts are skipped<<<<<<<<

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the E1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Unknown or already allocated gNB-CU-CP UE E1AP ID, Unknown or already allocated gNB-CU-UP UE E1AP ID, Unknown or inconsistent pair of UE E1AP ID, Interaction with other procedure, PDCP Count Wrap Around, Not supported QCI value, Not supported 5QI value, Encryption algorithms not supported, Integrity protection algorithms not supported, UP integrity protection not possible, UP confidentiality protection not possible, Multiple PDU Session ID Instances, Unknown PDU Session ID, Multiple QoS Flow ID Instances, Unknown QoS Flow ID, Multiple DRB ID Instances, Unknown DRB ID, Invalid QoS combination, Procedure cancelled, Normal release, No radio resources available, Action desirable for radio reasons, Resources not available for the slice, PDCP configuration not supported, …, UE DL maximum integrity protected data rate reason, UP integrity protection failure, Release due to Pre-Emption) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, Unknown TNL address, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| Unknown or already allocated gNB-CU-CP UE E1AP ID | The action failed because the gNB-CU-CP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-CU-UP UE E1AP ID | The action failed because the gNB-CU-UP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU-UP) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE E1AP ID | The action failed because both UE E1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| PDCP COUNT wrap around | PDCP COUNT approaches the maximum value. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| Encryption algorithms not supported | The gNB-CU-UP is unable to support the selected encryption algorithm for the UE. |
| Integrity protection algorithms not supported | The gNB-CU-UP is unable to support the selected integrity protection algorithm for the UE. |
| UP integrity protection not possible | The PDU Session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU Session cannot be accepted according to the required user plane confidentiality protection policy |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknow. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Invalid QoS combination | The action was failed because of invalid QoS combination |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| No radio resources available | The requested node doesn’t have sufficient radio resources available. |
| Action desirable for radio reasons | The reason for requesting the action is radio related. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| PDCP configuration not supported, | The gNB-CU-UP is unable to support the selected PDCP configuration for the UE. |
| UE DL maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum downlink data rate for integrity protection supported by the UE. |
| UP integrity protection failure | The gNB-CU-UP detects an integrity protection failure in the UL PDU. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |
| Unknown TNL address | The action failed because the TNL address is unknown. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, NAS or Protocol. |

-------------------------------------------Change 5-------------------------------------------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MRDC-DataUsageReport,

DeactivateTrace,

TraceStart,

PrivateMessage,

IAB-UPTNLAddressUpdate,

IAB-UPTNLAddressUpdateAcknowledge,

IAB-UPTNLAddressUpdateFailure

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-DeactivateTrace,

id-TraceStart,

id-privateMessage,

id-iAB-UPTNLAddressUpdate

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired			|

bearerContextRelease						|

iAB-UPTNLAddressUpdate					,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication			|

mRDC-DataUsageReport					|

deactivateTrace							|

traceStart								|

privateMessage							,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

gNB-CU-UP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-UP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-UP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-UP-E1Setup

CRITICALITY				reject

}

gNB-CU-CP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-CP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-CP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-CP-E1Setup

CRITICALITY				reject

}

gNB-CU-UP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-UP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-UP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-UP-ConfigurationUpdate

CRITICALITY				reject

}

gNB-CU-CP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-CP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-CP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-CP-ConfigurationUpdate

CRITICALITY				reject

}

e1Release E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E1ReleaseRequest

SUCCESSFUL OUTCOME		E1ReleaseResponse

PROCEDURE CODE			id-e1Release

CRITICALITY				reject

}

bearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextSetupRequest

SUCCESSFUL OUTCOME		BearerContextSetupResponse

UNSUCCESSFUL OUTCOME	BearerContextSetupFailure

PROCEDURE CODE			id-bearerContextSetup

CRITICALITY				reject

}

bearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequest

SUCCESSFUL OUTCOME		BearerContextModificationResponse

UNSUCCESSFUL OUTCOME	BearerContextModificationFailure

PROCEDURE CODE			id-bearerContextModification

CRITICALITY				reject

}

bearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequired

SUCCESSFUL OUTCOME		BearerContextModificationConfirm

PROCEDURE CODE			id-bearerContextModificationRequired

CRITICALITY				reject

}

bearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseCommand

SUCCESSFUL OUTCOME		BearerContextReleaseComplete

PROCEDURE CODE			id-bearerContextRelease

CRITICALITY				reject

}

bearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseRequest

PROCEDURE CODE			id-bearerContextReleaseRequest

CRITICALITY				ignore

}

bearerContextInactivityNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextInactivityNotification

PROCEDURE CODE			id-bearerContextInactivityNotification

CRITICALITY				ignore

}

dLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLDataNotification

PROCEDURE CODE			id-dLDataNotification

CRITICALITY				ignore

}

uLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULDataNotification

PROCEDURE CODE			id-uLDataNotification

CRITICALITY				ignore

}

dataUsageReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataUsageReport

PROCEDURE CODE			id-dataUsageReport

CRITICALITY				ignore

}

gNB-CU-UP-CounterCheck E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-CounterCheckRequest

PROCEDURE CODE			id-gNB-CU-UP-CounterCheck

CRITICALITY				ignore

}

gNB-CU-UP-StatusIndication 	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-StatusIndication

PROCEDURE CODE			id-gNB-CU-UP-StatusIndication

CRITICALITY			ignore

}

privateMessage E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mRDC-DataUsageReport	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MRDC-DataUsageReport

PROCEDURE CODE			id-mRDC-DataUsageReport

CRITICALITY				ignore

}

deactivateTrace E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

traceStart E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

iAB-UPTNLAddressUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IAB-UPTNLAddressUpdate

SUCCESSFUL OUTCOME		IAB-UPTNLAddressUpdateAcknowledge

UNSUCCESSFUL OUTCOME		IAB-UPTNLAddressUpdateFailure

PROCEDURE CODE			id-iAB-UPTNLAddressUpdate

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info,

DLUPTNLAddressToUpdateItem,

DLUPTNLAddressUpdatedItem

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

id-DLUPTNLAddressToUpdateList,

id-ULUPTNLAddressToUpdateList,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset,

maxnoofTNLAddresses

FROM E1AP-Constants;

>>>>>>>>unchanged parts are skipped<<<<<<<<

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs E1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- IAB UP TNL ADDRESS UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- IAB UP TNL Address Update

--

-- **************************************************************

IAB-UPTNLAddressUpdate	::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { IAB-UPTNLAddressUpdateIEs} },

...

}

IAB-UPTNLAddressUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-DLUPTNLAddressToUpdateList			CRITICALITY ignore	TYPE DLUPTNLAddressToUpdateList			PRESENCE mandatory	},

...

}

DLUPTNLAddressToUpdateList       ::= SEQUENCE (SIZE(1.. maxnoofTNLAddresses))	OF DLUPTNLAddressToUpdateItem

-- **************************************************************

--

-- IAB UP TNL Address Update Acknowledge

--

-- **************************************************************

IAB-UPTNLAddressUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { IAB-UPTNLAddressUpdateAcknowledgeIEs} },

...

}

IAB-UPTNLAddressUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-ULUPTNLAddressToUpdateList			CRITICALITY ignore	TYPE ULUPTNLAddressToUpdateList			PRESENCE Optional	},

...

}

ULUPTNLAddressToUpdateList       ::= SEQUENCE (SIZE(1.. maxnoofTNLAddresses))	OF ULUPTNLAddressToUpdateItem

-- **************************************************************

--

-- IAB UP TNL Address Update Failure

--

-- **************************************************************

IAB-UPTNLAddressUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {IAB-UPTNLAddressUpdateFailureIEs} },

...

}

IAB-UPTNLAddressUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

>>>>>>>>unchanged parts are skipped<<<<<<<<

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer	{{Cause-ExtIEs}}

}

Cause-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

unknown-or-already-allocated-gnb-cu-cp-ue-e1ap-id,

unknown-or-already-allocated-gnb-cu-up-ue-e1ap-id,

unknown-or-inconsistent-pair-of-ue-e1ap-id,

interaction-with-other-procedure,

pPDCP-Count-wrap-around,

not-supported-QCI-value,

not-supported-5QI-value,

encryption-algorithms-not-supported,

integrity-protection-algorithms-not-supported,

uP-integrity-protection-not-possible,

uP-confidentiality-protection-not-possible,

multiple-PDU-Session-ID-Instances,

unknown-PDU-Session-ID,

multiple-QoS-Flow-ID-Instances,

unknown-QoS-Flow-ID,

multiple-DRB-ID-Instances,

unknown-DRB-ID,

invalid-QoS-combination,

procedure-cancelled,

normal-release,

no-radio-resources-available,

action-desirable-for-radio-reasons,

resources-not-available-for-the-slice,

pDCP-configuration-not-supported,

...,

ue-dl-max-IP-data-rate-reason,

uP-integrity-protection-failure,

release-due-to-pre-emption

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

unknown-TNL-address,

...

}

>>>>>>>>unchanged parts are skipped<<<<<<<<

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DLUPTNLAddressToUpdateItem	::= SEQUENCE {

oldTNLAdress						TransportLayerAddress,

newTNLAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLAddressToUpdateItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLAddressToUpdateItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-TX-Stop	::=	ENUMERATED	{

stop,

resume,

...

}

>>>>>>>>unchanged parts are skipped<<<<<<<<

#### -- U

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-Configuration	::=	ENUMERATED	{

no-data,

shared,

only,

...

}

ULUPTNLAddressToUpdateItem	::= SEQUENCE {

oldTNLAdress						TransportLayerAddress,

newTNLAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { ULUPTNLAddressToUpdateItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLAddressToUpdateItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

ULDataSplitThreshold	::=	ENUMERATED {b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, ...}

UP-Parameters ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF UP-Parameters-Item

UP-Parameters-Item ::= SEQUENCE {

uP-TNL-Information			UP-TNL-Information,

cell-Group-ID				Cell-Group-ID,

iE-Extensions				ProtocolExtensionContainer { { UP-Parameters-Item-ExtIEs } }	OPTIONAL,

...

}

UP-Parameters-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoS-Mapping-Information	CRITICALITY reject	EXTENSION QoS-Mapping-Information	PRESENCE optional},

...

}

UPSecuritykey	::= SEQUENCE {

encryptionKey				EncryptionKey,

integrityProtectionKey		IntegrityProtectionKey		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UPSecuritykey-ExtIEs } }	OPTIONAL,

...

}

UPSecuritykey-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-TNL-Information		::= 	CHOICE {

gTPTunnel		GTPTunnel,

choice-extension		ProtocolIE-SingleContainer	{{UP-TNL-Information-ExtIEs}}

}

UP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

>>>>>>>>unchanged parts are skipped<<<<<<<<

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

id-iAB-UPTNLAddressUpdate											ProcedureCode ::= xx

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAddresses							INTEGER ::= 8

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info								ProtocolIE-ID ::= 86

id-QoSMonitoringRequest										ProtocolIE-ID ::= 87

id-QoS-Mapping-Information								ProtocolIE-ID ::= xx

id-DLUPTNLAddressToUpdateList								ProtocolIE-ID ::= xx

id-ULUPTNLAddressToUpdateList								ProtocolIE-ID ::= xx

END

-- ASN1STOP