3GPP TSG-RAN WG3 Meeting#108-e	  R3-203961

1st – 12th June 2020

Agenda item:	10.3.2

Source:	Ericsson (moderator)

Title:	Summary of discussions on MDT for Inactive UEs

Document for:	Approval

# 1	Introduction

This paper provides summary of discussions at RAN#107bis-e on MDT for Inactive UEs:

CB: # 1010_Email_SONMDT_MDT_INACTIVE

-  Prioritization of Signaling MDT over management-based MDT

(E/// - moderator)

Summary of offline disc R3-203961

Revised TPs for agreeable issues

It is proposed to allocate related TPs to companies as follows:

- TBD

# 2	For the Chairman’s Notes

The majority of companies believe that the principles below can be agreed, provided that RAN2 confirms that “Management based MDT should not overwrite signaling based MDT” applies to all scenarios.

- For Inactive UEs: The solutions are based on signalling, from old Serving NG RAN node to new Serving NG RAN node, of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

- For Idle UEs:  The solutions are based on signalling from AMF to new Serving NG RAN node of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

RAN2 has discussed the LS from RAN3 in R3-202869 concerning whether “Management based MDT should not overwrite signaling based MDT” applies to all scenarios. The agreement taken by RAN2 is minuted in their Summary of SON/MDT discussion in R2-2005736 and is quoted below:

Agreements:

1	The management-based MDT configuration should not overwrite signaling based MDT configuration in all the single connection scenarios and EN-DC scenario. UE based solution is not supported in R16.

=>	Inform RAN3 that ”The propagation of signaling based immediate MDT configuration for the case of Xn inter-RAT intra-system handover can be supported.” has no impact on RAN2 stage3 specs and SA5 should be consulted.

With this agreement from RAN2 the following is proposed to agree on the following:

RAN3 shall provide network based solutions for Inactive and Idle UEs based on the following principles

- For Inactive UEs: The solutions are based on signalling, from old Serving NG RAN node to new Serving NG RAN node, of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

- For Idle UEs:  The solutions are based on signalling from AMF to new Serving NG RAN node of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

To the question of whether solutions following the principles above should be enhanced with information concerning the remaining logging duration of a signalling based logged MDT configuration:

- 3 companies answer that such enhancement is not needed

- 5 companies answer that such enhancement is needed

No consensus can be reached on this aspect.

To the question of whether a Logged MDT Status indication is also signalled for UEs in Connected Mode as part of the handover signalling over the Xn and NG interfaces and between MN and SN in DC configurations

- 3 companies answer that such enhancement is not needed

- 5 companies answer that such enhancement is needed

No consensus can be reached on this aspect.

It is proposed to converge on TPs for Xn and NG capturing the agreements above.

# 3	Discussion

At the last RAN3 meeting the following was captured:

A solution to enable the RAN to respect the agreement from RAN2 below is needed:

“Management based MDT should not overwrite signalling based MDT”

Network based solutions allowing enforcement of the RAN2 agreement in Idle mode are available (e.g. signalling of MDT Status flag from CN to RAN). RAN3 to decide if the discussion should be broadened to cover also Idle UEs

How to enforce RAN2 agreement for Idle UEs (e.g. signaling MDT status flag from CN to RAN? other solutions are not precluded): To be continued...

The above implies two points:

- RAN3 agreed that a solution needs to be found to ensure that the agreement from RAN2 can be enforced.

- RAN3 will analyse solutions also for Idle mode UEs

In this paper a summary of offline discussions is provided based on the papers received during RAN3-108-e

## 3.1	Solutions to enforce RAN2 agreements for Inactive and Idle UEs

The RAN2 agreement for which solutions need to be found is the following:

“Management based MDT should not overwrite signaling based MDT”

During RAN3-108-e solutions have been presented to enforce this agreement for Inactive and Idle UEs.

The basic building block of all proposed solutions is the following:

- For Inactive UEs: The solutions are based on signalling, from old Serving NG RAN node to new Serving NG RAN node, of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

- For Idle UEs:  The solutions are based on signalling from AMF to new Serving NG RAN node of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

Companies are invited to provide their views on the following question:

Should the basic principles in 1) and 2) be agreed for the solutions to be selected by RAN3?

| Company | Answer | Comment |
| --- | --- | --- |
| Ericsson | Yes | It is important that the new serving RAN receives an indication of whether the UE is configured with a Signalling Based Logged MDT configuration so to allow for a basic and simple mechanism for avoiding the possibility of overwriting signaling based configurations with management based configurations. This is the simplest level of solution. It is understood that this solution may not allow the newly serving RAN to know with accuracy when the logging duration of a signaling based logged MDT configuration expires. A smart implementation could determine whether a UE is still within the duration period by using timers and by monitoring the presence of the RRC logMeasAvailable IE. Despite the coarse accuracy in knowing when the logging duration expires, this mechanism fulfils the requirement from RAN2, i.e. it avoids that the UE signalling based MDT configuration is overwritten by a management based one. |
| ZTE | Need Clarify | First of all, RAN3 need receive confirm from RAN2 whether the issue is need to be cover for scenario other than EN-DC. To clarify the indication in the principals is Boolean IE or kind of “remaining timer”. It is not clear how “duration period by using timers” + “monitoring logMeasAvailable IE1”work. For example, when does the new node trigger the timer? |
| Samsung | Yes | We agree network based solution is needed in order to avoid overwrite problem. Prefer a complete solution can deal with all cases. |
| CATT | Yes |  |
| Huawei |  | Before going into the detailed solutions, we prefer to wait for the reply LS from RAN2. If RAN2 confirms that the agreement applies also to normal cases, we are fine to discuss any solutions. |
| CMCC | Fine to discuss any solutions, if scenarios are confirmed. | First, we sent LS to RAN2 asking whether the agreement is applied to all cases. RAN2 hasn’t confirmed yet. The requirement that the agreement can be applied to any UE states, if confirmed by RAN2, is new for MDT, we didn’t have this requirement in LTE. Although we had some discussion on RRC_inactive state in the past meetings, this is the first time in RAN3 to discuss solutions for all the scenarios. Basically the solutions can be classified into three categories, - Signalling based logged MDT “Flag“ or “indication” - Remaining timer - Absolute time + logged duration It is hard to say which one is a basic one. All candidate solutions have some pros and cons, so if the convergence cannot be easily achieved, first waiting for RAN2 confirmation is beneficial, if confirmed, it can also be discussed in Rel-17. |
| Nokia | Probably not needed in Rel-16 | Let's wait for the reply from RAN2, but the situation seems to be the following: The potential problem to be solved would be the case where MN and SN are connected to different OAM domains. That could be the case for EN-DC deployments, but it is also clear that "Only immediate MDT is supported for EN-DC scenario in R16 MDT" (LS from RAN2 received in R3-200096). Our understanding is therefore that the OAM domain for the SN (NR) will not configure logged MDT, hence there should be no conflicting logged MDT configurations coming from MN OAM and SN OAM. Also, it seems preferable to avoid using the CN for the purpose of coordination of MDT between OAM domains. So far, signalling based MDT configuration information has only been forwarded by the CN from OAM to the RAN for the selected UE. If any solution further impacting the CN is needed at all, RAN3 would need to check with SA2. |
| Qualcomm | Partially agree | In principle, we agree that some sort of indication from previous serving NG-RAN to current serving NG-RAN in case of INACTIVE and from CN to NG-RAN is needed. But in addition, we also recommend the following principles: - Instead of having a BOOLEAN indicator whether signalling MDT is configured or not, signalling the logged MDT remaining active time is a better solution, the presence of whose IE can also act as a BOOLEAN indicator as discussed in R3-203340 - An indicator from NG-RAN to CN is also needed when RRC connection is released (release could be NG-RAN initiated too). Signalling the remaining active time in the Release message to AMF also makes CN implementation simpler in that AMF need not compute remaining active time. Also please note that if RAN2 responds to the LS that this MDT prioritization is needed for IDLE mode as well, involvement of CN for IDLE mode in a network based solution is unavoidable. |

Conclusion: The majority of companies believe that the principles below can be agreed, provided that RAN2 confirms that “Management based MDT should not overwrite signaling based MDT” applies to all scenarios.

- For Inactive UEs: The solutions are based on signalling, from old Serving NG RAN node to new Serving NG RAN node, of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

- For Idle UEs:  The solutions are based on signalling from AMF to new Serving NG RAN node of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

RAN2 has discussed the LS from RAN3 concerning whether “Management based MDT should not overwrite signaling based MDT” applies to all scenarios, in R3-202869. The agreement taken by RAN2 is minuted in their Summary of SON/MDT discussion in R2-2005736 and is quoted below:

Agreements:

1	The management-based MDT configuration should not overwrite signaling based MDT configuration in all the single connection scenarios and EN-DC scenario. UE based soltuion is not supported in R16.

=>	Inform RAN3 that ”The propagation of signaling based immediate MDT configuration for the case of Xn inter-RAT intra-system handover can be supported.” has no impact on RAN2 stage3 specs and SA5 should be consulted.

With this agreement from RAN2 the following is proposed to agree on the following:

RAN3 shall provide network based solutions for Inactive and Idle UEs based on the following principles

- For Inactive UEs: The solutions are based on signalling, from old Serving NG RAN node to new Serving NG RAN node, of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

- For Idle UEs:  The solutions are based on signalling from AMF to new Serving NG RAN node of an indication of whether there is an active Signalling Based Logged MDT configuration at the UE

## 3.2	Enhancements to basic solution described in Section 3.1

A number of companies (see R3-203340, R3-203950, R3-203887) propose to enhance the basic solution described in Section 3.1 with notion of the time left for the Signalling Based Logged MDT configuration at the UE. This can be achieved in a number of ways, such as:

- To signal over Xn and NG interfaces to the new serving NG RAN node the remaining logging duration for the UE

- To signal over Xn and NG interfaces to the new serving NG RAN node the absolute time at which the UE was configured with the signalling based logged MDT configuration and the original logging duration for the configuration

Companies are invited to provide their views on the following question:

Should a solution based on the principles in 1) and 2) be agreed and if yes, which of the designs in 1) and 2) should be chosen?

| Company | Answer | Comment |
| --- | --- | --- |
| Ericsson | No | Although there is some value in enabling an accurate understanding at the new serving RAN node of when exactly the UE will become available for being configured with Management Based Logged MDT, this is not what RAN2 is requiring. RAN2 is requiring that “Management based MDT should not overwrite signaling based MDT” Hence the extra complexity for these solutions is not justified. It is worth noticing that solutions based on 1) above imply that the UE context remains “active” even when the UE is in suspended or idle mode, namely, timers will need to be kept running when the UE is suspended or idle. On the other side, solutions based on 2) above do not work in a very accurate way because it is not required that different RAN nodes share the same absolute time. Finally, it is possible for the serving RAN to understand if the logging duration of a Signalling Based Logged MDT configuration is still running by monitoring the signalling of the RRC logMeasAvailable IE. Unless the UE is in an area that is not within the MDT area scope, the UE will signal this IE as soon as it has a log ready. Not signalling this IE would mean the logging duration has expired. The RAN can also base its judgement of whether the logging duration has expired on timers. |
| ZTE | - Remaining timer as indicator | Solution Complex comparison The complex seems same for the two options on the table: Option A : “duration period by using timers in New node” + “monitoring logMeasAvailable IE” Option B1: only use “remaining timer” as indicator = remaining timer Option B2: only use “remaining timer” as indicator = absolute time (Not seen in the contributions) Context remains “active”Issue? Not sure fully understand the “issue”. It is clear from stage 2 description (blow and highlight in yellow) ,the timer will continues independent of UE RRC state. Therefor, it is true the timer is kept running when UE is in RRC_Inactive state. Otherwise, how does network know when the measurement end. And it is noting, Core network would not send logged MDT configuration to UE in RRC_IDLE. Therefore , the remaining time will not trigger for UE in RRC_IDLE. TS 37.320 - configuration of the logging duration. This configuration parameter defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. When the timer expires the logging is stopped and the configuration is cleared (except for the parameters that are required for further reporting e.g. network absolute time stamp, trace reference, trace recording session reference and TCE Id).. Solution of UE assistant with “quickly” signal RRC logMeasAvailable IE The solution does not work when the UE be configured with “event- triggerred logging” which newly introduced in NR. That means UE may not have any collections to Report. In addition, whether UE/Chip companies support this extra behavior that “signal RRC logMeasAvailable IE ASAP ” is also need further check. |
| Samsung | Yes | Yes, the purpose is to fulfil the RAN2 requirement. But without accurate information, overwrite problem will still happen. For example, MME sends a logged MDT configuration to gNB for a UE in T1, MME will assume T1 is the activation time for logged MDT. From the logging duration, MME can decide how to set the indication. i.e. when UE is connected to a new gNB, if MME think logging not expired, the MME will send “configured” indication to gNB. If MME think logging is expired, no sending of configured indicator. I think this is how basic network solution worked in Ng. However, MME can not know the correct activation time, since the old gNB may suspend the configuration if the UE is in inactive. So the activation time in MME calculation could be much early than real activation time. MME thought the logged MDT is expired (but in fact it is still running in UE) and sends no indication to gNB, the gNB therefore can configure a management based logged MDT, then overwrite problem is happen. |
| CATT | No | We think a simple solution which introduce an indication is enough. No need to over specify a complex solution while is still not accurate. |
| Huawei | Yes, in case that ran2 confirms the scenario | Agree with Samsung’s analysis above. And the MME/AMF impact should consult SA2. |
| CMCC | Yes, if scenarios are confirmed | The solution of absolute time + logged duration seems more accurate and has less CN impact. |
| Nokia | Probably not needed | If any solution impacting the CN is needed at all, RAN3 would need to check with SA2. |
| Qualcomm | Yes (Logged MDT Remaining Active Time Indicator) | We agree with ZTE’s comment as well. The complexity of the solution 1 (signalling the logged MDT remaining active time) vs. the complexity of using a flag to signal whether logged MDT is configured or not is pretty much the same. In response to the following E/// comment, “It is worth noticing that solutions based on 1) above imply that the UE context remains “active” even when the UE is in suspended or idle mode, namely, timers will need to be kept running when the UE is suspended or idle” Please note that T330 timer starts when logged MDT is configured in the UE and runs irrespective of UE RRC State. NG-RAN is already aware of when T330 timer starts and can easily compute the logged MDT remaining active time and propagate it over Xn to the new serving NG-RAN when UE resumes from INACTIVE. Similarly, there are only minor impacts in CN implementation. AMF is already aware when signalling based MDT is configured to NG-RAN and can compute the remaining active time when UE reconnects from IDLE as long it maintains the MDT context along with UE context. Also, in response to the other proposed solution of using logMeasAvailable as a coarse indicator, this will not work always either as it is an OPTIONAL IE and UE might not choose to signal it even if it has logged MDT results and T330 timer has not expired. We therefore recommend to use the remaining active solution for an optimized and more accurate network based only solution. |

Conclusion:

To the question of whether the main solution in section 3.1 should be enhanced with information concerning the remaining logging duration of a signalling based logged MDT configuration:

- 3 companies answer that such enhancement is not needed

- 5 companies answer that such enhancement is needed

No consensus can be reached on this aspect.

## 3.3	Should solutions also cover Connected mode UEs?

In R3-203950 it is proposed that a Logged MDT Status indication is also signalled for UEs in Connected Mode as part of the handover signalling over the Xn and NG interfaces and between MN and SN in DC configurations.

Companies are invited to provide their views on the following question:

Should a solution be also provided for Connected Mode UEs?

| Company | Answer | Comment |
| --- | --- | --- |
| Ericsson | No | The description in the running CR of TS 37.320 state that “MDT context is assumed to be released in the RAN nodes when the UE is in IDLE and INACTIVE state if corresponding MDT configuration has already been configured to UE” . Hence there is no requirement to release the context if the UE is in Connected mode. The RAN3 specifications allow transferring of the MDT configuration in HO signalling and DC management procedures for Ues in connected mode, hence it is not clear why the solutions should be extended to connected mode UEs |
| ZTE | No | Share the view with Ericsson. |
| Samsung | Yes | In TS32.422, it is said in section 4.5 of “Handling of MDT Trace sessions at handover for Logged MDT and Logged MBSFN MDT” that “In logged MDT and Logged MBSFN MDT mode, no propagation of the MDT configuration is performed.” In TS37.320 it is clearly said for idle or inactive, there is no logged MDT configuration propagation from source/old to target/new if this logged MDT is configured to the UE. Above is the current standard situation. In our understanding, in legacy, there is no need to propagate the logged MDT configuration in connected mode if the source has configured it into UE. If the source doesn’t send the configuration to UE, sure the source will send the MDT configuration to the target, then target can configure UE. It is why we have logged MDT configuration in HO REQUEST message. If the signaling based logged MDT configuration has been sent to the UE in source, there is no need to propagate the MDT configuration to the target, since target has no usage. UE will report the logging Availability/MDT record in the next Idle to Active transition. By then, the UE context is already released in NG-RAN. Even worse, propagation will lead to wrong behavior of the target, i.e. target don’t know whether source has configured UE and the target may configure the UE in a second time. Pls note there is no “overwrite” requirement in LTE. If we want to use the propagation to solve the “overwrite” problem in NR, this is new (may be conflict with SA5 specification), and even with the propagation, the overwrite problem still happen for connected mode. As said in above, the target is not sure whether source has configured UE or not if receiving logged MDT configuration. A simple and robust solution is to apply the same solution for connected mode. With the same reason, the solution for dual connectivity case is also necessary. |
| CATT | No | Similar view with E/// and ZTE |
| Huawei | Yes | We think if we want to solve the overriding issue of MDT configuration, all use cases should be covered. The precondition is to monitor the RAN2 progress on the LS that RAN3 sent out at last meeting. |
| CMCC | Yes, if scenarios are confirmed | We agree with Samsung’s analysis, the current stage 2 specifies, For Logged MDT in IDLE, CELL_PCH, URA_PCH states ,CELL_FACH state when second DRX cycle is used and INACTIVE, no need is identified to transfer an MDT context (any related configuration information about measurement and reporting) between (e/g)NBs/RNCs if corresponding MDT configuration has already been configured to UE. In addition, MDT context is assumed to be released in the RAN nodes when the UE is in IDLE and INACTIVE if corresponding MDT configuration has already been configured to UE. So if the requirements are confirmed, the solution for connected mode is also needed. |
| Nokia | Yes, if scenarios are confirmed | Agree with reasoning from Samsung, HW and CMCC, if RAN2 confirms that additional solution is needed in Rel-16. |
| Qualcomm | Yes | Agree with Samsung’s reasoning and also the fact if the enhanced solution discussed in 3.2 is agreed (signalling remaining active time), it is also useful to signal the remaining active time for logged MDT to target NG-RAN in addition to transferring the MDT configuration over Xn (already supported). This additional information can help the target NG-RAN be aware of the remaining active time for signalling based MDT after which it can accept management-based MDT configuration from OAM when UE continues to handover in RRC CONNECTED mode. |

Conclusion: to the question of whether a Logged MDT Status indication is also signalled for UEs in Connected Mode as part of the handover signalling over the Xn and NG interfaces and between MN and SN in DC configurations

- 3 companies answer that such enhancement is not needed

- 5 companies answer that such enhancement is needed

No consensus can be reached on this aspect.

# 4	Conclusion, Recommendations [if needed]

It is proposed to agree on the following:

# 5	References

[1]		R3-20xxxx, Title, Company