3GPP TSG-RAN3 Meeting #108-e	R3-204073

1st - 11th June 2020 E-Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.414 | CR | 0006 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rapporteur Clean-ups of TS38.414 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-06-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As rapporteur of TS38.414, clean-ups for each new release are expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clean-up various places. Impact analysis: Impact assessment towards the previous version of the specification (same release): -This CR has no impact from functional point of view. -The impact can be considered isolated because it only clean-ups various places. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial errors remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R3-203244, correct from Release16 to 15. |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

1	Scope

The present document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the NG interface.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[4]	IETF RFC 768 (1980-08): "User Datagram Protocol".

[5]	IETF RFC 8200 (2017-07): "Internet Protocol, Version 6 (IPv6) Specification".

[6]	IETF RFC 791 (1981-09): "Internet Protocol".

[7]	IETF RFC 2474 (1998-12): "Definition of the Differentiated Services Field (DS Field) in the Ipv4 and Ipv6 Headers".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description Stage 2"

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

gNB: as defined in TS 38.300 [8].

ng-eNB: as defined in TS 38.300 [8].

NG-RAN node: as defined in TS 38.300 [8].

NG-U: as defined in TS 38.300 [8].

PDU Session: as defined in TS 23.501[2].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

GTP	GPRS Tunnelling Protocol

IP	Internet Protocol

NGAP	NG Application Protocol

TEID	Tunnel Endpoint Identifier

UDP	User Datagram Protocol

UPF	User Plane Function

4	Data Link Layer

Any data link protocol that fulfills the requirements toward the upper layer may be used.

5	NG Interface user plane protocol

5.1	General

Both gNB and ng-eNB can be connected to the 5GC over the NG interface.

The transport layer for data streams over NG is an IP based Transport. The following figure shows the transport protocol stacks over NG.

Figure 5.1: Transport network layer for data streams over NG

The GTP-U (TS 29.281 [3]) protocol over UDP over IP shall be supported as the transport for data streams on the NG interface. The data link layer is as specified in clause 4.

The transport bearer is identified by the GTP-U TEID (TS 29.281 [3]) and the IP address (source TEID, destination TEID, source IP address, destination IP address).

5.2	GTP-U

The GTP-U (TS 29.281 [3]) protocol shall be used over the NG-U interface toward the 5GC.

5.3	UDP/IP

The path protocol used shall be UDP (IETF RFC 768 [4]).

The UDP port number for GTP-U shall be as defined in TS 29.281 [3].

The NG-RAN node and the 5GC shall support fragmentation and assembly of GTP packets at the IP layer.

The NG-RAN node and the 5GC shall support IPv6 (IETF RFC 8200 [5]) and/or IPv4 (IETF RFC 791 [6]).

There may be one or several IP addresses in the NG-RAN node and in the 5GC. The packet processing function in the 5GC shall send downstream packets of a given PDU Session to the NG-RAN node IP address (received in NGAP) associated to that particular PDU Session. The packet processing function in the NG-RAN node shall send upstream packets of a given PDU Session to the 5GC IP address (received in NGAP) associated to that particular PDU Session.

The Transport Layer Address signalled in NGAP messages is a bit string of

a)	32 bits in case of IPv4 address according to IETF RFC 791 [6]; or

b)	128 bits in case of IPv6 address according to IETF RFC 8200 [5]; or

c)	160 bits if both IPv4 and IPv6 addresses are signalled, in which case the IPv4 address is contained in the first 32 bits.

5.4	Diffserv code point marking

IP Differentiated Services code point marking (IETF RFC 2474 [7]) shall be supported. The mapping between traffic categories and Diffserv code points shall be configurable by O&M based on 5QI, the Priority Level (if explicitly signalled), and other NG-RAN traffic parameters (e.g. ARP). Traffic categories are implementation-specific and may be determined from the application parameters.

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////