3GPP TSG-RAN3 Meeting #108-e	R3-204074

1 June – 12 June 2020, Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1326 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR TS 25.413 on QoE measurement collection support for RANAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support attributes for QoE measurements and align to TS 28.405 as per SA5 LS S5-202305 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of attributes for QoE measurement purpose: “QMC ID” to identify the QoE measurement collection job, “Recording Session Indication” to identify the QoE recording session, “WithinArea” to propagate QoE measurement collection at handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support for QoE measurements and misalignment between Stage 3 specification and Stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 9.2.1.30, 9.2.1.129, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 – Submitted at TSG W3 Meeting #106 Rev. 2 – Submitted at TSG W3 Meeting #108-e in response to LS S5-202305 and LS S5-202304. Title changed from “On QoE measurement collection” to “CR TS 25.413 on QoE measurement collection support for RANAP”. Rebased on 25.413 v16.0.0 |  |  |  |  |  |  |  |  |

Start of the first change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 23.930 (version.4.0.0, 2001-04): "Iu Principles".

[2]	3GPP TS 25.410: "UTRAN Iu Interface: General Aspects and Principles".

[3]	3GPP TS 25.401: "UTRAN Overall Description".

[4]	3GPP TR 25.931: "UTRAN Functions, Examples on Signalling Procedures".

[5]	3GPP TS 25.412: "UTRAN Iu interface signalling transport".

[6]	3GPP TS 25.415: "UTRAN Iu interface user plane protocols".

[7]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[8]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[9]	3GPP TS 25.414: "UTRAN Iu interface data transport and transport signalling".

[10]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[11]	3GPP TS 48.008: "Mobile Switching Centre – Base Station System (MSC - BSS) interface; Layer 3 specification".

[12]	Void

[13]	ITU-T Recommendation X.691 (07/2002): "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[14]	ITU-T Recommendation X.680 (07/2002): "Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[15]	ITU-T Recommendation X.681 (07/2002): "Information technology - Abstract Syntax Notation One (ASN.1): Information object specification".

[16]	3GPP TS 23.110: "UMTS Access Stratum, Services and Functions".

[17]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[21]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[22]	3GPP TS 24.080: "Mobile radio Layer 3 supplementary services specification; Formats and coding".

[23]	3GPP TS 29.108: "Application of the Radio Access Network Application Part (RANAP) on the E-interface".

[24]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[25]	GSM TS 12.20: "Base Station System (BSS) management information".

[26]	3GPP TS 23.236: "Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes".

[27]	3GPP TS 43.051: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Overall description - Stage 2".

[28]	Void.

[29]	3GPP TS 43.059: "Functional stage 2 description of Location Services (LCS) in GERAN".

[30]	3GPP TS 22.071: "Location Services (LCS); Service description - Stage 1".

[31]	3GPP TR 25.994 (version.5.0.0): "Measures employed by the UMTS Radio Access Network (UTRAN) to overcome early User Equipment (UE) implementation faults".

[32]	3GPP TR 25.995 (version.5.0.0): "Measures employed by the UMTS Radio Access Network (UTRAN) to cater for legacy User Equipment (UE) which conforms to superseded versions of the RAN interface specification".

[33]	3GPP TS 23.195 (version.5.4.0): "Provision of UE Specific Behaviour Information to Network Entities".

[34]	3GPP TS 49.031: "Location Services (LCS) – Base Station System Application Part LCS Extension – (BSSAP-LE)".

[35]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[36]	3GPP TS 48.018: "General Packet Radio Service (GPRS); BSS GPRS Protocol (BSSGP)".

[37]	3GPP TS 32.421: "Subscriber and equipment trace: Trace concepts and requirements".

[38]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and Configuration Management".

[39]	3GPP TS 23.251: "Network sharing - Architecture and functional description".

[40]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service; Stage 1".

[41]	3GPP TS 23.246: "Multimedia Broadcast Multicast Service; Architecture and Functional Description".

[42]	3GPP TS 25.346: "Introduction of the Multimedia Broadcast Multicast Service (MBMS) in the Radio Access Network (RAN); Stage 2".

[43]	3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service UDI/RDI fallback and service modification; Stage 2".

[44]	3GPP TS 29.061 "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[45]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[46]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[47]	3GPP TS 43.055: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Dual Transfer Mode (DTM) - Stage 2".

[48]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[49]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[50]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception(FDD)".

[51]	3GPP TS 25.446: "MBMS Synchronisation Protocol(SYNC)".

[52]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; stage 2".

[53]	3GPP TS 23.007: "Restoration procedures"

[54]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2"

[55]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB) - Stage 2"

[56]	3GPP TS 22.220: "Service requirements for Home Node Bs and Home eNode Bs".

[57]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp Interface".

[58]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[59]	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[60]	3GPP TS 33.102: "3G Security; Security architecture".

[61]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[62]	3GPP TS 52.008: "Telecommunication management; GSM subscriber and equipment trace".

[63]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[64]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[65]	3GPP TS 23.139: "3GPP system – fixed broadband access network interworking".

[66]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0)", December 2013.

[67]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[68]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[69]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[70]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object".

[71]	IETF RFC 6848: "Specifying Civic Address Extensions in the Presence Information Data Format Location Object (PIDF-LO)".

[72]	3GPP TS 25.300: "Universal Terrestrial Radio Access Network (UTRAN); General description; Stage 2".

[xx]	3GPP TS 28.405: " Quality of Experience (QoE) measurement collection; Control and configuration".

Next change

## 3.3	Abbreviations

Applicable abbreviations can be found in TR 21.905 [35]. For the purposes of the present document, the following abbreviations apply:

AAL2	ATM Adaptation Layer type 2

ALCAP	Access Link Control Application Part

APN	Access Point Name

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

ATM	Asynchronous Transfer Mode

BDS	BeiDou Navigation Satellite System

BBF	Broadband Forum

BSC	Base Station Controller

CC	Call Control

CN	Core Network

CRNC	Controlling RNC

CS	Circuit Switched

CSG	Closed Subscriber Group

DCH	Dedicated Channel

DCN	Dedicated Core Network

DL	Downlink

DRNC	Drift RNC

DRNS	Drift RNS

DSCH	Downlink Shared Channel

eNB	E-UTRA NodeB

EP	Elementary Procedure

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

GANSS	Galileo and Additional Navigation Satellite Systems

GERAN	GSM/EDGE Radio Access Network

GPRS	General Packet Radio System

GSM	Global System for Mobile communications

GTP	GPRS Tunnelling Protocol

GWCN	GateWay Core Network

HNB	Home Node B

IE	Information Element

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IPv4	Internet Protocol (version 4)

IPv6	Internet Protocol (version 6)

IRAT	Inter-RAT

L-GW	Local GateWay

LIPA	Local IP Access

LHN	Local Home Network

LHN ID	Local Home Network ID

MBMS	Multimedia Broadcast Multicast Service

MBS	Metropolitan Beacon System

MDT	Minimization of Drive Tests

MM	Mobility Management

MOCN	Multi Operator Core Network

MSC	Mobile services Switching Center

MSISDN	MS International PSTN/ISDN Number

MTSI	Multimedia Telephony Service for IMS

NACC	Network Assisted Cell Change

NAS	Non Access Stratum

NNSF	NAS Node Selection Function

NRT	Non-Real Time

N-PDU	Network – Protocol Data Unit

OSP:IHOSS	Octet Stream Protocol: Internet-Hosted Octet Stream Service

P-TMSI	Packet TMSI

PDCP	Packet Data Convergence Protocol

PDP	Packet Data Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PPP	Point-to-Point Protocol

PS	Packet Switched

PSI	Packet System Information

PTP	Point To Point

PUESBINE	Provision of UE Specific Behaviour Information to Network Entities

QMC	QoE Measurement Collection

QoE	Quality of Experience

QoS	Quality of Service

RAB	Radio Access Bearer

RANAP	Radio Access Network Application Part

RAT	Radio Access Technology

RIM	RAN Information Management

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RRC	Radio Resource Control

rSRVCC	reverse Single Radio Voice Call Continuity

RT	Real Time

SAI	Service Area Identifier

SAP	Service Access Point

SDU	Service Data Unit

SGSN	Serving GPRS Support Node

S-GW	Serving GateWay

SI	System Information in GERAN

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SNA	Shared Network Area

SNAC	Shared Network Area Code

SRNC	Serving RNC

SRNS	Serving RNS

SRVCC	Single Radio Voice Call Continuity

TEID	Tunnel Endpoint Identifier

TMGI	Temporary Mobile Group Identity

TMSI	Temporary Mobile Subscriber Identity

UE	User Equipment

UEA	UMTS Encryption Algorithm

UESBI-Iu	UE Specific Behaviour Information - Iu

UIA	UMTS Integrity Algorithm

UL	Uplink

UMTS	Universal Mobile Telecommunications System

USCH	Uplink Shared Channel

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

Next change

#### 9.2.1.30	Target RNC to Source RNC Transparent Container

The Target RNC to Source RNC Transparent Container IE is an information element that is produced by the target RNC and is transmitted to the source RNC. In inter-system handovers to UTRAN, the IE is transmitted from the target RNC to the external relocation source.

This IE is transparent to CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING |  |  |  |
| d-RNTI | O |  | INTEGER (0..1048575) | May be included to allow the triggering of the Relocation Detect procedure from the Iur Interface. |  |  |
| UE Application Layer Measurement Support Indication | O |  | BITSTRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement. Bit 0 = QoE for streaming service Bit 1 = QoE for MTSI Value ‘1’ indicates “Supported” and value ‘0’ indicates “not Supported”. Unused bits are reserved for future use. | YES | Ignore |
| WithinArea | O |  | ENUMERATED(true, false) | This IE indicates if the target is within the QoE measurement scope. | YES | Ignore |

Next change

#### 9.2.1.129	UE Application layer measurement configuration for relocation

The purpose of the UE Application layer measurement configuration for relocation IE is to provide configuration information for the QoE function during relocation procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | YES | ignore |
| Trace Reference | M |  | 9.2.1.8 |  | YES | ignore |
| Trace Propagation Parameters | O |  | 9.2.1.68 | Optional for UTRAN. Not applicable to GERAN Iu Mode. | YES | ignore |
| Trace Collection Entity IP Address | O |  | 9.2.2.1 |  | YES | ignore |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MSTI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | Ignore |
| QMC ID | O |  | OCTET STRING (SIZE(3)) | This IE is used to specify the QoE reference defined as: MCC+MNC+QMC ID (ref. TS 28.405 [x]) | Yes | ignore |
| Recording Session Indication | O |  | ENUMERATED (on, off, ...) | This IE indicates if the recording session for the purpose of QoE measurement is ongoing. | Yes | ignore |

Start of ASN.1 changes

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

maxRAB-Subflows,

maxRAB-SubflowCombination,

maxNrOfLevels,

maxNrOfAltValues,

maxNrOfSNAs,

maxNrOfLAs,

maxNrOfPLMNsSN,

maxSet,

maxNrOfHSDSCHMACdFlows-1,

maxNrOfUEsToBeTraced,

maxNrOfInterfaces,

maxnoofMulticastServicesPerRNC,

maxMBMSSA,

maxMBMSRA,

maxnoofMulticastServicesPerUE,

maxNrOfEDCHMACdFlows-1,

maxGANSSSet,

maxNrOfEUTRAFreqs,

maxNrOfCellIds,

maxNrOfRAIs,

maxNrOfLAIs,

maxNrOfVol,

maxSizeOfIMSInfo,

maxnoofMDTPLMNs,

maxAddPosSet,

maxnoofPLMNs,

id-CN-DomainIndicator,

id-MessageStructure,

id-SRB-TrCH-Mapping,

id-TypeOfError,

id-hS-DSCH-MAC-d-Flow-ID,

id-SignallingIndication,

id-CellLoadInformationGroup,

id-TraceRecordingSessionInformation,

id-MBMSLinkingInformation,

id-AlternativeRABConfiguration,

id-AlternativeRABConfigurationRequest,

id-E-DCH-MAC-d-Flow-ID,

id-RAC,

id-Alt-RAB-Parameter-ExtendedGuaranteedBitrateInf,

id-Alt-RAB-Parameter-ExtendedMaxBitrateInf,

id-Ass-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-Ass-RAB-Parameter-ExtendedMaxBitrateList,

id-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-RAB-Parameter-ExtendedMaxBitrateList,

id-Requested-RAB-Parameter-ExtendedMaxBitrateList,

id-Requested-RAB-Parameter-ExtendedGuaranteedBitrateList,

id-LAofIdleModeUEs,

id-newLAListofIdleModeUEs,

id-LAListwithNoIdleModeUEsAnyMore,

id-ExtendedRNC-ID,

id-GANSS-PositioningDataSet,

id-d-RNTI-for-NoIuCSUP,

id-UE-History-Information,

id-UeApplicationLayerMeasurementSupportIndication,

id-SubscriberProfileIDforRFP,

id-Alt-RAB-Parameter-SupportedGuaranteedBitrateInf,

id-Alt-RAB-Parameter-SupportedMaxBitrateInf,

id-Ass-RAB-Parameter-SupportedGuaranteedBitrateList,

id-Ass-RAB-Parameter-SupportedMaxBitrateList,

id-RAB-Parameter-SupportedGuaranteedBitrateList,

id-RAB-Parameter-SupportedMaxBitrateList,

id-Requested-RAB-Parameter-SupportedMaxBitrateList,

id-Requested-RAB-Parameter-SupportedGuaranteedBitrateList,

id-PSRABtobeReplaced,

id-SRVCC-Information,

id-CSG-Id,

id-CSFB-Information,

id-IRAT-Measurement-Configuration,

id-Management-Based-MDT-Allowed,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-TraceRecordingSessionReference,

id-IMSI,

id-Management-Based-MDT-PLMN-List,

id-SignallingBasedMDTPLMNList,

id-M4Report,

id-M5Report,

id-M6Report,

id-M7Report,

id-TimingDifferenceULDL,

id-Trace-Collection-Entity-IP-Addess,

id-Serving-Cell-Identifier,

id-EARFCN-Extended,

id-LastE-UTRANPLMNIdentity,

id-RSRQ-Type,

id-RSRQ-Extension,

id-Additional-PositioningDataSet,

id-SRVCCSource,

id-WithinArea,

id-QMCID,

id-RecordingSessionIndication

FROM RANAP-Constants

Next change

#### -- A

UE-Application-Layer-Measurement-Configuration	::= SEQUENCE {

applicationLayerContainerForMeasurementConfiguration		OCTET STRING ( SIZE (1..1000)),

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

...,

serviceType													ServiceType

}

UE-Application-Layer-Measurement-Configuration-For-Relocation	::= SEQUENCE {

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

traceReference						TraceReference,

tracePropagationParameters			TracePropagationParameters OPTIONAL,

traceCollectionEntityIPAddress		TransportLayerAddress			OPTIONAL,

...,

serviceType							ServiceType,

iE-Extensions		ProtocolExtensionContainer { {UE-Application-Layer-Measurement-Configuration-For-Relocation-ExtIEs} } OPTIONAL

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MSTI-service,

...

}

UE-Application-Layer-Measurement-Configuration-For-Relocation-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

{ID id-QMCID						CRITICALITY ignore	EXTENSION QMC-ID						PRESENCE optional}|

{ID id-RecordingSessionIndication	CRITICALITY ignore	EXTENSION RecordingSessionIndication	PRESENCE optional},

...

}

QMC-ID ::= OCTET STRING (SIZE (3))

RecordingSessionIndication ::= ENUMERATED {

on,

off,

...

}

Next change

#### -- T

TargetRNC-ToSourceRNC-TransparentContainer ::= SEQUENCE {

rRC-Container			RRC-Container,

d-RNTI					D-RNTI 					OPTIONAL

-- May be included to allow the triggering of the Relocation Detect procedure from the Iur Interface --,

iE-Extensions			ProtocolExtensionContainer { {TargetRNC-ToSourceRNC-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetRNC-ToSourceRNC-TransparentContainer-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

{ ID id-UeApplicationLayerMeasurementSupportIndication	CRITICALITY ignore	EXTENSION  UeApplicationLayerMeasurementSupportIndication		PRESENCE optional}|

{ ID id-WithinArea	CRITICALITY ignore	EXTENSION  WithinArea		PRESENCE optional},

...

}

Next change

#### -- W

WithinArea ::= ENUMERATED {

true,

false

}

Next change

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

id-Additional-PositioningDataSet 						INTEGER ::= 284

id-CivicAddress											INTEGER ::= 285

id-SGSN-Group-Identity									INTEGER ::= 286

id-P-TMSI												INTEGER ::= 287

id-RANAP-Message										INTEGER ::= 288

id-PowerSavingIndicator									INTEGER ::= 289

id-UE-Usage-Type										INTEGER ::= 290

id-DCN-ID												INTEGER ::= 291

id-UE-Application-Layer-Measurement-Configuration		INTEGER ::= 292

id-UE-Application-Layer-Measurement-Configuration-For-Relocation				INTEGER ::= 293

id-UE-Application-Layer-Measurement-Capability			INTEGER ::= 294

id-UeApplicationLayerMeasurementSupportIndication		INTEGER ::= 295

id-SRVCCSource											INTEGER ::= 296

id-WithinArea											INTEGER ::= xx1

id-QMCID												INTEGER ::= xx2

id-RecordingSessionIndication							INTEGER ::= xx3

END

End of ASN.1 changes