3GPP TSG-RAN3 Meeting #108-e	R3-204104

E-meeting, 01 - 11 June 2020

Title: 	(TP for WWC BL CR for TS 38.413) Support for interfacing wireline and trusted non-3GPP access to the 5GC

Source: 	Huawei, Telecom Italia, BT, Broadcom, Nokia, Nokia Shanghai Bell

Agenda item:	21.2

Document for:	Other

Annex – TP

Change Begins

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | Ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 |  | YES | ignore |

#### /********Skip to next change**********/

#### 9.3.1.cc	W-AGF User Location Information

This IE indicates the location information via wireline access as specified in TS 23.316 [x].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE W-AGF User Location Information | M |  |  |  |
| >Global Line ID |  |  |  |  |
| >>Global Line ID | M |  | OCTET STRING | Encoded as defined in TS 23.003 [23]. |
| >>Line Type | O |  | ENUMERATED (DSL, PON, …) |  |
| >HFC Node ID |  |  |  |  |
| >>HFC Node ID | M |  | OCTET STRING | Indicates the identifier of the HFC node as specified in [m]. Encoded as defined in TS 23.003 [23]. |

#### /********Skip to next change**********/

9.4.4	PDU Definitions

<Unchanged Text Omitted>

WarningSecurityInfo,

WarningType,

RIMInformationTransfer,

RGLevelWirelineAccessCharacteristics,

AuthenticatedIndication,

PLMNIdentity

<Unchanged Text Omitted>

id-WarningSecurityInfo,

id-WarningType,

id-RIMInformationTransfer,

id-RGLevelWirelineAccessCharacteristics,

id-W-AGFIdentityInformation,

id-AuthenticatedIndication,

id-TNGFIdentityInformation,

id-TWIFIdentityInformation,

id-SelectedPLMNIdentity

<Unchanged Text Omitted>

InitialUEMessage ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialUEMessage-IEs} },

...

}

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU										PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY reject	TYPE UserLocationInformation						PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause					CRITICALITY ignore	TYPE RRCEstablishmentCause						PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI							CRITICALITY reject	TYPE FiveG-S-TMSI									PRESENCE optional		}|

{ ID id-AMFSetID								CRITICALITY ignore	TYPE AMFSetID										PRESENCE optional		}|

{ ID id-UEContextRequest						CRITICALITY ignore	TYPE UEContextRequest								PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional		}|

{ ID id-AuthenticatedIndication					CRITICALITY ignore	TYPE AuthenticatedIndication						PRESENCE optional		}|

{ ID id-SelectedPLMNIdentity					CRITICALITY ignore	TYPE PLMNIdentity									PRESENCE optional		},

...

}

<Unchanged Text Omitted>

GlobalLine-ID ::= SEQUENCE {

globalLineIdentity		GlobalLineIdentity,

lineType			LineType 							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GlobalLine-ID-ExtIEs} } 		OPTIONAL,

...

}

GlobalLine-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalLineIdentity ::= OCTET STRING

LineType ::= ENUMERATED {

dsl,

pon,

...

}

<Unchanged Text Omitted>

9.4.7	Constant Definitions

<Unchanged Text Omitted>

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-RGLevelWirelineAccessCharacteristics					ProtocolIE-ID ::= 183

id-W-AGFIdentityInformation								ProtocolIE-ID ::= 184

id-GlobalTNGF-ID										ProtocolIE-ID ::= 185

id-GlobalTWIF-ID										ProtocolIE-ID ::= 186

id-GlobalW-AGF-ID										ProtocolIE-ID ::= 187

id-UserLocationInformationW-AGF                   		ProtocolIE-ID ::= 188

id-UserLocationInformationTNGF							ProtocolIE-ID ::= 189

id-AuthenticatedIndication								ProtocolIE-ID ::= 190

id-TNGFIdentityInformation								ProtocolIE-ID ::= 191

id-TWIFIdentityInformation								ProtocolIE-ID ::= 192

id-UserLocationInformationTWIF							ProtocolIE-ID ::= 193

id-SelectedPLMNIdentity									ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<Unchanged Text Omitted>