3GPP TSG-RAN WG3 #108-e	R3-204190

# Online, 1-11 June 2020

Agenda Item:	19.5

Source: 	Ericsson

Title:  	(TP to TS 38.473 BL CR): Addition of SFN Initialization Time and DL Beam Information to F1AP

Document for:	Other

# Discussion

## SFN Initialization time

Following the RAN1 LS in R1- 2003054, RAN3 is tasked to introduce the SFN Initialization time in RAN3 NR positioning specs.

It was agreed during the e-mail discussion to align F1AP with NRPPa on this aspect:

| Company | Yes/No | Comments on procedure |
| --- | --- | --- |
| Ericsson | Yes | To mimic how it was defined in SLmAP, this new IE needs to be added to the F1 POSITIONING MEASUREMENT REQUEST and POSITIONING MEASUREMENT UPDATE messages |
| Huawei | yes |  |
| Qualcomm | Yes | SFN Initialization time should be supported in F1AP procedures corresponding to those agreed for NRPPa. |
| Intel | Yes |  |
| Nokia | Yes | Align with NRPPa. |

Therefore it is proposed to add the SFN Initialization time IE as part of the SRS Configuration IE (new IE in 9.3.1.x) and in the TRP Information IE (in 9.3.1.e)

Proposal 1: Add the SFN Initialization Time in the SRS Configuration IE and in the TRP Information IE.

## DL Beam information

Taking into consideration the comments received to include the DL Beam Information to NRPPa, it is proposed to:

- Sort the beam information under the angle, instead of PRS beams, so that PRS radiation gain under the same angle can be grouped together to facilitate DL-AoD angle calculation at LMF.

- Align the DL Beam Information in NRPPa to the LPP NR-DL-PRS-BeamInfo in LPP to add LCS to GCS angle parameters, to allow the translation from the local coordinate system (LCS) to the global coordinate system (GCS).

A tabular example is provided below to be added to the PRS Configuration IE in 9.3.1.f

### 9.3.1.xx1	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. <maxnoNRPRSBeamtypes> |  |  |
| >PRS Angle Item |  | 1..<maxnoofAngleInfo> |  |  |
| >>NR PRS Azimuth | M |  | INTEGER (0,..,359) |  |
| >>NR PRS Azimuth fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>NR PRS Elevation | O |  | INTEGER (0,..,180) |  |
| >>NR PRS Elevation fine | O |  | INTEGER (0,..,9) | Fine angles |
| LCS to GCS Translation |  | 1 .. <maxnolcs-gcs-translation> |  |  |
| >>Alpha | O |  | INTEGER (0,..,359) |  |
| >>Alpha-fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>Beta | O |  | INTEGER (0,..,359) |  |
| >>Beta-fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>Gamma | O |  | INTEGER (0,..,359) |  |
| >>Gamma-fine | O |  | INTEGER (0,..,9) | Fine angles |

| Range bound | Explanation |
| --- | --- |
| maxnoNRPRSBeamtypes | Maximum no. of NR PRS beam information types that can reported with one message. Value is 64. |
| maxnoofAngleInfo | Maximum no of PRS angle information that can be included within PRS configurations IE. Value is 65535. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 64. |

# TP to F1AP BL CR

START OF CHANGE

#### 9.2.x.10	TRP INFORMATION REQUEST

This message is sent by a gNB-CU to request information for TRPs hosted by a gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | - | - |
| TRP Information Type List |  | 1 |  |  | - | - |
| >TRP Information Type Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type | M |  | ENUMERATED (arfcn, sfn init time, FFS, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

NEXT CHANGE

### 9.3.1.x	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

NEXT CHANGE

### 9.3.1.e	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | INTEGER(1..16384) |  |
| Cell Information | M |  | NR CGI 9.3.1.12 |  |
| PRS Configurations | O |  | 9.3.1.f |  |
| NG-RAN Access Point Position | M |  | Access Point Position 9.3.1.b | The configured estimated geographical position of the antenna of the cell/TRP. |
| TRP Information Type Response List |  | 1 |  |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>>SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

NEXT CHANGE

### 9.3.1.f	PRS Configurations

The PRS Configurations IE contains configurations of PRS associated with a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information | O |  | 9.3.1.xx1 |  |

NEXT CHANGE

### 9.3.1.xx1	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. <maxnoNRPRSBeamtypes> |  |  |
| >PRS Angle Item |  | 1..<maxnoofAngleInfo> |  |  |
| >>NR PRS Azimuth | M |  | INTEGER (0,..,359) |  |
| >>NR PRS Azimuth fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>NR PRS Elevation | O |  | INTEGER (0,..,180) |  |
| >>NR PRS Elevation fine | O |  | INTEGER (0,..,9) | Fine angles |
| LCS to GCS Translation |  | 1 .. <maxnolcs-gcs-translation> |  |  |
| >>Alpha | O |  | INTEGER (0,..,359) |  |
| >>Alpha-fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>Beta | O |  | INTEGER (0,..,359) |  |
| >>Beta-fine | O |  | INTEGER (0,..,9) | Fine angles |
| >>Gamma | O |  | INTEGER (0,..,359) |  |
| >>Gamma-fine | O |  | INTEGER (0,..,9) | Fine angles |

| Range bound | Explanation |
| --- | --- |
| maxnoNRPRSBeamtypes | Maximum no. of NR PRS beam information types that can reported with one message. Value is 64. |
| maxnoofAngleInfo | Maximum no of PRS angle information that can be included within PRS configurations IE. Value is 65535. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 64. |

NEXT CHANGE

Asn.1 on the work.

END OF CHANGES