3GPP TSG-RAN3 Meeting #108	R3-204196

E-Meeting, 01 - 12 June 2020

Title: 	(TP to BL CR for TS 38.470) FFS Removal

Source: 	Huawei

Agenda item:	19.1

Document for:	Other

# Introduction

In RAN3#107bis meeting, the CR to 38.470 has been agreed [1]. The following editor’s notes was included:

Editor’s Note: Further details should be discussed e.g. whether the posSIB can be area specific, and whether the assistance data can be targeted at specific cells by the LMF.

This paper futher provide the TP to remove the FFS to TS 38.470, we could also remove the FFS without the TP.

Editor’s Note: the above text needs futher check and revision

The text could be refine however, it is basic and acceptable for this version of specification. It is propose to remove the Editor’s Note

Proposal 1: Agree the TP.

Rev1: Simplify the text proposal according the comments, remove the text proposal from first version, the text is only the on from alst endorsed BL CR.

# Reference

- R3-202777 (TP for BL CR for TS 38.470): F1 support for NR positioning

# Annex A – TP for TS 38.470

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of NR-MIB. In case broadcast of SIB1 and other SI messages is needed, the gNB-DU is responsible for the encoding of SIB1 and the gNB-CU is responsible for the encoding of other SI messages. The gNB-DU may re-encode SIB9.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g the positioning related SIBs. The gNB-CU notifies gNB-DU about the SIBs, and the gNB-DU signals them directly.

To support Msg3 based on-demand SI as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested other SIs.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

### 5.2.X	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows to transfer the positioning assistance data from gNB-CU to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

### 6.1.x	Positioning procedures

The F1 Positioning procedures are listed below:

- Positioning Measurement procedure;

- TRP Information Exchange procedure;

- Positioning Information Exchange procedure;

- Positioning Assistance Information Control procedure;

- Positioning Assistance Information Feedback procedure;

- Positioning Measurement Report procedure;

- Positioning Measurement Modification procedure;

- Positioning Measurement Termination procedure;

- Positioning Measurement Failure Indication Procedure.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>