3GPP TSG-RAN WG3 Meeting #108e                                 R3-204249

Online, June 1 – June 12, 2020					        Revision of R3-203344

Agenda item:    13.2.1.2

Source: 		Qualcomm Incorporated

Title: 	(TP for NR_IAB BL CR TS 38300) IAB stage-2 clean up

Document for:	Other

# Introduction

- This paper includes a TP for BL CR to TS 38300. This TP aims to align the BL CR with RAN2’s present TS 38.300 v16.0.0 and to capture recent RAN3 agreements.

The revision of this paper is based on offline discussion CB#7 from RAN3#108e.

# Discussion

- Proposal: Include the TP below into BL CR to TS 38300.

# 3	Conclusion

This paper made to following proposal:

- Proposal: Include the TP below into BL CR to TS 38300.

# 4 TP for NR_IAB BL CR TS 38300

-------------------------------------------Change 1-------------------------------------------

### 6.11.2	Traffic Mapping from Upper Layers to Layer-2

In upstream direction, the IAB-donor-CU configures the IAB-node with mappings between upstream F1- and non-F1-traffic originated at the IAB-node, and the appropriate BAP routing ID, next-hop BAP address and Backhaul RLC channel. A specific mapping is configured:

-  for each F1-U GTP-U tunnel,

-  for non-UE associated F1AP messages,

-  for UE-associated F1AP messages,

-  for non-F1 traffic.

Multiple mappings can contain the same Backhaul RLC channel and/or next-hop BAP address and/or BAP routing ID.

In case the IAB-MT is NR-dual-connected (SA-mode only), the mapping may include two separate BH RLC channels, where the two BH RLC channels are established toward different parent IAB-DUs.

In case the IAB-node is configured with multiple IP addresses for F1-C on the NR leg, multiple mappings can be configured for non-UE-associated F1AP messages or UE-associated F1AP messages. The appropriate mapping is selected based on the IAB node’s implementation.

-------------------------------------------End of changes ------------------------------------------