3GPP TSG-RAN WG3 #108-e																		  R3-204509

June 1st – 12th, 2020 E-Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1331 | rev | 15 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Baseline CR for introducing Rel-16 LTE further mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Intel Corporation, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-06-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for Rel-16 LTE further mobility enhancements: RAN3-104 r1&r2: - To capture the agreed TP in R3-192709 and some TPs merge from R3-192665, R3-192416, including: - Reuse existing HO Prep procedure for CHO prep - Reuse existing HO cancel for canceling CHO from source - The target shall be able to signal successful CHO to the source - Reuse existing HO prep for the MBB HO - WA: When to start data forwarding is up to implementation RAN3-105 r3: - To capture the agreed TP in R3-194767 about further descriptions on Make-Before-Break HO preparation - To capture the agreed TP in R3-194640 about further descriptions on CHO - To capture the agreed TP in R3-194644 about CHO cancel procedure RAN3-105 r4: - The term NR-CGI is not correct, should be ECGI RAN3-105bis r5 - The definition of DAPS has not been introduced - To capture the agreements on SN status continuation for data forwarding RAN3-106 r6 - Submitted to RAN3#106 RAN3-106-r7 - To capture all the agreed TPs during RAN3#106 RAN3-107e r8 - Submitted to RAN3#107e RAN3-107e r9 - ASN.1 for CHO CANCEL procedure was missing RAN3-107e r10 - To capture all the agreed TPs during RAN3#107e - Some small corrections to ASN.1 RAN3-107bis-e r11 - Submitted to RAN3#107bis-e RAN3-107bis-e r12 - To capture all the agreed TPs during RAN3#107bis-e - Editorial changes remain RAN3-108e r13 - Submitted to RAN3#108e RAN3-108e r14 - Editorial updates during RAN3#108e RAN3-108e r15 - To capture all the agreed TPs during RAN3#108-e |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3-104: 1. Agreed TPs in R3-192709 and some TPs merge from R3-192665, R3-192416, including - Handover Preparation procedure is updated to enable triggering CHO. - New class-2 Handover Success procedure is introduced for the target to signal the source that the UE has successfully attached to the target. RAN3-105: 1. Agreed TP in R3-194767, including: - Add “Enhanced Make-Before-Break Information” IE with necessary contexts and FFS for further discussions - Corresponding ASN.1 to this added IE 2. Agreed TP in R3-194640, including: - The contexts about handling of parallel transactions for HO Preparation were added - some FFSes were removed 3. Agreed TP in R3-194644 - CHO Cancel procedure from the target node shall not be used in legacy HO - Both source and target nodes are allowed to initiate per-target cell CHO cancel; target node is only allowed to cancel a prepared CHO RAN3-105bis 1. Agreed change in R3-194965 - To change “NR CGI” to “ECGI” 2. Agreed TP in R3-196131 - To capture the agreements on SN status continuation during DAPS handover process 3. To introduce the definition of DAPS, and replace the previous conception of enhanced MBB or eMBB, including the concerned contexts, tabular and ASN.1. RAN3-106 1. All changes are kept as one author. RAN3-106 1. Agreed TP in R3-196699 - Enabling identification of the tunnel for late data forwarding 2. Agreed TP in R3-197615 - Clarify that candidate cells to be Cancelled list should be related to UE signaling when previously set up 3. Agreed TP in R3-197613 - Enabling modification of CHO 4. Agreed TP in R3-197611 - Clarify that the parallel requests can be identified with the target cell ID in the case that the source UE AP IDs are the same 5. Agreed TP in R3-197791 - Data Forwarding and SN Status Transfer for DAPS HO RAN3-107e 1. Update with the latest template in R3-200043 RAN3-107e 1. To add ASN.1 for CHO CANCEL procedure in R3-201298 RAN3-107e 1. Agreed TP in R3-201312 - Introduce the DAPS HO per DRB indicator and DAPS response Info into Handover Preparation procedure over X2 interface. 2. Agreed TP in R3-201078 - To define the value of maxnoofCellsinCHO as 16 3. Agreed TP in R3-201304 - Some corrections to Conditional HO Cancel 4. Agreed TP in R3-201359 - To introduce additional Information on CHO triggering to mitigate the risk of unnecessary resource reservation 5. Agreed TP in R3-201387 - To introduce necessary changes to make the early forwarding work for DAPS/CHO 6. Agreed TP in R3-201306 - Correction to CHO Per Target Cell TEID - Add ASN.1 part for “COUNTvalueExtended” and “PDCP-SNlength18” - Update the defintion of “DAPS HO” to refer to 36.300 RAN3-107bis-e 1. All changes are kept as one author, based on the latest spec. RAN3-107bis-e 1. Agreed TP in R3-202295 - Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios 2. Agreed TP in R3-202613 - Further Clarification of Maximum Number of CHO Preparations 3. Agreed TP in R3-202671 - Implementation of the arrival probability 4. Agreed TP in R3-202715 - Do not introduce one shot DAPS HO proposal from source. DAPS HO proposal is per E-RAB/DRB. 5. Agreed TP in R3-202754 - Enabling keeping 2 configs at the target 6. Rapporteur clean-ups according to R3-202734 and some ASN.1 error corrections RAN3-108e - All changes are kept as one author, based on the latest spec, with a complet ASN.1. - Remove changes on changes RAN3-108e 1. Agreed TP in R3-204122 - Refine X2AP Abnormal Conditions etc 2. Agreed TP in R3-204150 - Further Clarification of CHO in MR-DC operation 3. Agreed TP in R3-204164 - Conditional HO Cancel procedure and HO Success procedure 4. Agreed TP in R3-204185 - CHO Modification 5. Agreed TP in R3-204231 - HO SUCCESS alignment with F1 ACCESS SUCCESS 6. Agreed TP in R3-204294 - Change of the name of the Early Forwarding Transfer procedure 7. Agreed TP in R3-204300 - DAPS HO handling during preparation 8. Editorial changes from BL CR rapporteur Impact analysis, same release: the CR has limited functional and protocol impact as it affects only the CHO and RUDI concernig mobility enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 LTE further mobility enhancements is missing in TS 36.423. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 7, 8.1, 8.2.1, 8.2.2, 8.2.4, 8.2.X(new), 8.2.Y(new), 8.2.Z (new), 8.7.5, 9.1.1.1, 9.1.1.2, 9.1.1.3, 9.1.1.4, 9.1.1.6, 9.1.1.X(new), 9.1.1.Y(new), 9.1.1.Z (new), 9.2.1.43, 9.2.6, 9.2.x, 9.2.y, 9.2.C, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR0016 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev# -: R3-192709 Rev# 1: R3-193164 Rev# 2: R3-193358 Rev# 3: R3-194798 Rev# 4: R3-194965 Rev# 5: R3-196303 Rev# 6: R3-196501 Rev# 7: R3-197805 Rev# 8: R3-200043 Rev# 9: R3-201298 Rev# 10: R3-201449 Rev# 11: R3-201585 Rev# 12: R3-202888 Rev# 13: R3-203059 Rev# 14: R3-204205 Rev# 15: R3-204509 |  |  |  |  |  |  |  |  |

/////////////////////////////irrelevant operations skipped///////////////////////////////////////////

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ACL functionality: A functionality controlling the access to network nodes. In case of Access Control Lists (ACL) functionality is applied in a network node the network node may only accept connections from other peer network nodes once the source addresses of the sending network node is already known in the target node.

Elementary Procedure: X2AP protocol consists of Elementary Procedures (EPs). An X2AP Elementary Procedure is a unit of interaction between two eNBs. An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success or failure),

-	Class 2: Elementary Procedures without response.

E-RAB: Defined in TS 36.401 [2].

CSG Cell: as defined in TS 36.300 [15].

Dual Connectivity: as defined in TS 36.300 [15].

E-UTRA-NR Dual Connectivity: as defined in TS 37.340 [32].

Hybrid cell: as defined in TS 36.300 [15].

Master eNB: as defined in TS 36.300 [15].

Secondary Cell Group: as defined in TS 36.300 [15].

Secondary eNB: as defined in TS 36.300 [15].

en-gNB: as defined in TS 37.340 [32].

Conditional Handover: As defined in TS 36.300 [15].

DAPS HO: As defined in TS 36.300 [15].

Conditional PSCell Change: As defined in TS 37.340 [32].

Immediate Handover: Used in the context of Conditional Handover, to refer to a handover that is executed immediately after the UE receives the Handover Command.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

ARPI	Additional RRM Policy Index

ACL	Access Control List

BBF	Broadband Forum

BL	Bandwidth reduced Low complexity

CCO	Cell Change Order

CE	Coverage Enhancement

CHO	Conditional Handover

CoMP	Coordinated Multi Point

DAPS	Dual Active Protocol Stacks

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 7	Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Dual Connectivity. This function allows the eNB to request another eNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another en-gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the eNB or en-gNB for setup the X2 interface and implicitly perform an X2 Reset.

-	eNB Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the eNB to coordinate adaptation of mobility parameter settings with a peer eNB.

-	Mobility Robustness Optimisation. This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

-	X2 Release. This function allows an eNB to be aware that the signalling connection to a peer eNB is unavailable.

-	Message Transfer. This function allows indirect transport of X2AP messages to a peer eNB.

-	Registration. This function allows registration of eNB in case indirect transport of X2AP messages is supported.

-	Removing the X2. This function allows removing the signalling connection between two eNBs or between eNB and en-gNB in a controlled manner.

-	Inter-eNB UE Context Retrieval. This function allows retrieval of a UE context in case of resumption or re-establishment of an RRC connection.

-	Secondary RAT Data Usage Report. This function allows eNB to get the uplink and downlink data volumes for the Secondary RAT on a per E-RAB basis.

-	E-UTRA - NR Spectrum Sharing. This function allows uplink and downlink spectrum sharing between a number of E - UTRA and a number of NR cells with overlapping coverage.

-	EN-DC Configuration Transfer. This function supports en-gNB X2 TNL address discovery.

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel e) Handover Success f) Conditional Handover Cancel |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change f) MeNB initiated SgNB Release g) SgNB initiated SgNB Release h) SgNB Counter Check i) RRC transfer j) EN-DC X2 Setup k) EN-DC Configuration Update l) EN-DC Cell Activation m) SgNB Activity Notification n) EN-DC X2 Removal o) gNB Status Indication |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Retrieval | a) Retrieve UE Context b) Data Forwarding Address Indication |
| Secondary RAT Data Usage Report | Secondary RAT Data Usage Report |
| E-UTRA – NR Spectrum Sharing | E-UTRA - NR Cell Resource Coordination |
| EN-DC Configuration Transfer | EN-DC Configuration Transfer |

////////////////////irrelevant operations skipped///////////////////////////////////////////////////

8	X2AP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST | SGNB RELEASE REQUEST ACKNOWLEDGE | SGNB RELEASE REQUEST REJECT |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |
| EN-DC X2 Setup | EN-DC X2 SETUP REQUEST | EN-DC X2 SETUP RESPONSE | EN-DC X2 SETUP FAILURE |
| EN-DC Configuration Update | EN-DC CONFIGURATION UPDATE | EN-DC CONFIGURATION UPDATE ACKNOWLEDGE | EN-DC CONFIGURATION UPDATE FAILURE |
| EN-DC Cell Activation | EN-DC CELL ACTIVATION REQUEST | EN-DC CELL ACTIVATION RESPONSE | EN-DC CELL ACTIVATION FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| EN-DC X2 Removal | EN-DC X2 REMOVAL REQUEST | EN-DC X2 REMOVAL RESPONSE | EN-DC X2 REMOVAL FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SgNB Activity Notification | SGNB ACTIVITY NOTIFICATION |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| gNB Status Indication | GNB STATUS INDICATION |
| EN-DC Configuration Transfer | EN-DC CONFIGURATION TRANSFER |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |

8.2	Basic mobility procedures

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an eNB for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source eNB initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. When the source eNB sends the HANDOVER REQUEST message, it shall start the timer TRELOCprep.

If the Conditional Handover Information IE is contained in the HANDOVER REQUEST message, the target eNB shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the New eNB UE X2AP ID IE is contained in the Conditional Handover Information IE included in the HANDOVER REQUEST message, then the target eNB shall remove the existing prepared conditional HO identified by the New eNB UE X2AP ID IE and the Target Cell ID IE. It is up to the implementation of the target eNB when to remove the HO information.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

The source eNB may include in the GUMMEI IE any GUMMEI corresponding to the source MME node.

If at least one of the requested non-GBR E-RABs is admitted to the cell indicated by the Target Cell ID IE, the target eNB shall reserve necessary resources, and send the HANDOVER REQUEST ACKNOWLEDGE message back to the source eNB. The target eNB shall include the E-RABs for which resources have been prepared at the target cell in the E-RABs Admitted List IE. The target eNB shall include the E-RABs that have not been admitted in the E-RABs Not Admitted List IE with an appropriate cause value.

At reception of the HANDOVER REQUEST message the target eNB shall:

-	prepare the configuration of the AS security relation between the UE and the target eNB by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE.

For each E-RAB for which the source eNB proposes to do forwarding of downlink data, the source eNB shall include the DL Forwarding IE within the E-RABs To be Setup Item IE of the HANDOVER REQUEST message. The source eNB shall include the DL Forwarding IE if it requests a DAPS handover for that E-RAB. For each E-RAB that it has decided to admit, the target eNB may include the DL GTP Tunnel Endpoint IE within the E-RABs Admitted Item IE of the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding GTP tunnel endpoint, i.e. the information contained in the Transport Layer address IE and GTP TEID IE in the E-RAB To Be Switched in Downlink List IE of the PATH SWITCH REQUEST message (see TS 36.413 [4]) depending on implementation choice.

For each bearer in the E-RABs Admitted List IE, the target eNB may include the UL GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the source eNB shall stop the timer TRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source eNB shall start the timer TX2RELOCoverall. The source eNB is then defined to have a Prepared Handover for that X2 UE-associated signalling.

If the Trace Activation IE is included in the HANDOVER REQUEST message then the target eNB shall, if supported, initiate the requested trace function as described in TS 32.422 [6]. In particular, the target eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [6];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT and Trace" initiate the requested trace session and MDT session as described in TS 32.422 [6];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT Only" initiate the requested MDT session as described in TS 32.422 [6] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in TS 37.320 [31];

-	if the Trace Activation IE includes the UE Application layer measurement configuration IE, initiate the requested trace session and QoE Measurement Collection function as described in TS 36.300 [15].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Management Based MDT Allowed IE only or the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [6].

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

The source eNB shall, if supported and available in the UE context, include the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE in the HANDOVER REQUEST message, except if the source eNB selects a serving PLMN in the target eNB which is not included in the Management Based MDT PLMN List. If the Management Based MDT PLMN List IE is not present, the source eNB shall, if supported, include the Management Based MDT Allowed IE, if this information is available in the UE context, in the HANDOVER REQUEST message, except if the source eNB selects a serving PLMN in the target eNB different from the serving PLMN in the source eNB.

If the Handover Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target eNB shall

-	store the information received in the Handover Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, except when one of the E-RABs has a particular ARP value (TS 23.401 [12]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	not contained in the HANDOVER REQUEST message, the target eNB shall consider that no roaming and no access restriction apply to the UE.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message then the target eNB should initiate the requested location reporting functionality as defined in TS 36.413 [4].

If the SRVCC Operation Possible IE is included in the HANDOVER REQUEST message, the target eNB shall store the content of such IE in the UE context and use it as defined in TS 23.216 [20].

If the UE Security Capabilities IE included in the HANDOVER REQUEST message only contains the EIA0 algorithm as defined in TS 33.401 [18] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [18]), the eNB shall take it into use and ignore the keys received in the AS Security Information IE.

The HANDOVER REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE, if available.

If the Subscriber Profile ID for RAT/Frequency priority IE is contained in the HANDOVER REQUEST message, the target eNB shall store this information and the target eNB should use the information as defined in TS 36.300 [15].

If the Additional RRM Policy Index IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and the target eNB should use the information as defined in TS 36.300 [15].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target eNB shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store the collected information to be used for future handover preparations.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and use it as defined in TS 36.300 [15]. The target eNB shall, if supported, store the C-RNTI of the source cell received in the HANDOVER REQUEST message.

If the Expected UE Behaviour IE is provided in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and may use it to determine the RRC connection time.

If the ProSe Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the eNB shall, if supported, consider that the UE is authorized for the relevant ProSe service(s).

If the V2X Services Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the eNB shall, if supported, consider that the UE is authorized for the relevant service(s).

If the UE Context Reference at the SeNB IE is contained in the HANDOVER REQUEST message the target eNB may use it as specified in TS 36.300 [15]. In this case, the source eNB may expect the target eNB to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 36.300 [15]. If the UE Context Reference at the WT IE is contained in the HANDOVER REQUEST message, the target eNB may use it as specified in TS 36.300 [15]. In this case, the source eNB may expect the target eNB to include the WT UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message; the source eNB shall use this information as specified in TS 36.300 [15].

If the UE Context Reference at the SgNB IE is contained in the HANDOVER REQUEST message the target eNB may use it as specified in TS 37.340 [32]. In this case, the source eNB may expect the target eNB to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [32].

If the Bearer Type IE is included in the HANDOVER REQUEST message and is set to "non IP", then the target eNB shall not perform header compression for the concerned E-RAB.If the UE Sidelink Aggregate Maximum Bit Rate IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for V2X services.

If the NR UE Security Capabilities IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and send it to the respective peer node during subsequent handover preparations and/or EN-DC operations for the UE as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [15].

If the Subscription Based UE Differentiation Information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [12].

If the DAPS Request Information IE is included for an E-RAB to be setup in the HANDOVER REQUEST message, the target eNB shall consider that the request concerns a DAPS handover for that E-RAB, as described in TS 36.300 [15]. Accordingly, the target eNB shall include the DAPS Response Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Maximum Number of CHO Preparations IE is included in Conditional Handover Information IE contained in the the HANDOVER REQUEST ACKNOWLEDGE message, then the source eNB should not initiate more Handover Preparation procedures for a CHO for the same UE towards the target eNB than the number indicated in the Maximum Number of CHO Preparations IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information IE included in the HANDOVER REQUEST message, then the target eNB may use the information to allocate necessary resources for the incoming CHO.

If the EPC Handover Restriction List Container IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and shall use it as specified in TS 36.300 [15].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the E-RABs transferred to MeNB IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, then the source eNB shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the SgNB in the SN Status Transfer procedure towards the target eNB, as specified in TS 37.340 [32].

8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target eNB does not admit at least one non-GBR E-RAB, or a failure occurs during the Handover Preparation, the target eNB shall send the HANDOVER PREPARATION FAILURE message to the source eNB. The message shall contain the Cause IE with an appropriate value.

If the target eNB receives a HANDOVER REQUEST message containing RRC Context IE that does not include required information as specified in TS 36.331 [9], the target eNB shall send the HANDOVER PREPARATION FAILURE message to the source eNB.

If the Conditional Handover Information IE is contained in the HANDOVER REQUEST message and the target eNB rejects the handover or a failure occurs during the Handover Preparation, the target eNB shall include the Requested Target Cell ID IE in the HANDOVER PREPARATION FAILURE message.

##### Interactions with Handover Cancel procedure:

If there is no response from the target eNB to the HANDOVER REQUEST message before timer TRELOCprep expires in the source eNB, the source eNB should cancel the Handover Preparation procedure towards the target eNB by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source eNB shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned X2 UE-associated signalling.

8.2.1.4	Abnormal Conditions

If the target eNB receives a HANDOVER REQUEST message containing multiple E-RAB ID IEs (in the E-RABs To Be Setup List IE) set to the same value, the target eNB shall not admit the corresponding E-RABs.

If the target eNB receives a HANDOVER REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the target eNB shall not admit the corresponding E-RAB.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the target eNB (TS 33.401 [18]), the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the supported algorithms for integrity defined in the Integrity Protection Algorithms IE in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EIA0 algorithm in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [18]), the eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the target eNB receives a HANDOVER REQUEST message which does not contain the Handover Restriction List IE, and the PLMN to be used cannot be determined otherwise, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the target eNB receives a HANDOVER REQUEST message containing the Handover Restriction List IE, and the serving PLMN is not supported by the target cell, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the target eNB receives a HANDOVER REQUEST message which does not contain the CSG Membership Status IE, and the target cell is a hybrid cell, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the target cell is a CSG cell and the target eNB has not received any CSG ID of the source cell, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the target cell is a CSG cell with a different CSG from the source cell, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the CHO trigger IE is set to “CHO-replace” in the HANDOVER REQUEST message, but there is no CHO prepared for the included New eNB UE X2AP ID IE, or the candidate cell in the Target Cell ID IE was not prepared using the same UE-associated signaling connection, the target eNB shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

//////////////////////irrelevant operations skipped///////////////////////////////////////////////////

8.2.2	SN Status Transfer

8.2.2.1	General

The purpose of the SN Status Transfer procedure is to transfer the uplink PDCP SN and HFN receiver status and the downlink PDCP SN and HFN transmitter status either, from the source to the target eNB during an X2 handover, between the eNBs involved in dual connectivity and/or LWA, or between MeNB and en-gNB involved in EN-DC, for each respective E-RAB for which PDCP SN and HFN status preservation applies.

In case that the X2 handover is a DAPS handover, the SN Status Transfer procedure may also be used to transfer the uplink PDCP SN and HFN receiver status, or the downlink PDCP SN and HFN transmitter status for an E-RAB associated with RLC-UM and configured with DAPS as described in TS 36.300 [15].

If the SN Status Transfer procedure is applied in the course of dual connectivity, LWA, RRC connection re-establishment or EN-DC, in the subsequent specification text

-	the behaviour of the eNB from which the E-RAB context is transferred, i.e., the eNB involved in dual connectivity, LWA, RRC connection re-establishment from which data forwarding, is specified by the behaviour of the "source eNB",

-	the behaviour of the eNB to which the E-RAB context is transferred, i.e., the eNB involved in dual connectivity, LWA, RRC connection re-establishment to which data is forwarded, is specified by the behaviour of the "target eNB".

-	in case of EN-DC, the behaviour of the node from which the E-RAB context is transferred, i.e., either the en-gNB or the MeNB from which data is forwarded, is specified by the behaviour of the "source eNB",

-	in case of EN-DC, the behaviour of the node to which the E-RAB context is transferred, i.e., either the en-gNB or the MeNB to which data is forwarded, is specified by the behaviour of the "target eNB".

The procedure uses UE-associated signalling.

8.2.2.2	Successful Operation

Figure 8.2.2.2-1: SN Status Transfer, successful operation

Figure 8.2.2.2-2: MeNB initiated SN Status Transfer for EN-DC, successful operation

Figure 8.2.2.2-3: en-gNB initiated SN Status Transfer for EN-DC, successful operation

The source eNB initiates the procedure by stop assigning PDCP SNs to downlink SDUs and stop delivering UL SDUs towards the EPC and sending the SN STATUS TRANSFER message to the target eNB at the time point when it considers the transmitter/receiver status to be frozen. The target eNB using Full Configuration for this handover as per TS 36.300 [15] or for the EN-DC operations as per TS 37.340 [32] shall ignore the information received in this message. In case of EN-DC, if the target eNB performs PDCP version change or PDCP SN length change or RLC mode change for an E-RAB as specified in TS 37.340 [32], it shall ignore the information received for that E-RAB in this message.

In case that the X2 handover is a DAPS handover, the source eNB may continue assigning PDCP SNs to downlink SDUs and delivering uplink SDUs toward the EPC when initiating this procedure for E-RABs not configured with DAPS as in TS 36.300 [15].

The E-RABs Subject To Status Transfer List IE included in the SN STATUS TRANSFER message contains the E-RAB ID(s) corresponding to the E-RAB(s) for which PDCP SN and HFN status preservation shall be applied. In case that the X2 handover is a DAPS handover, this IE may contain the E-RAB ID(s) corresponding to the E-RAB(s) associated with RLC-UM.

If the source eNB includes in the SN STATUS TRANSFER message, the information on the missing and received uplink SDUs in the Receive Status Of UL PDCP SDUs IE or Receive Status Of UL PDCP SDUs Extended IE or Receive Status Of UL PDCP SDUs for PDCP SN Length 18 IE for each E-RAB for which the source eNB has accepted the request from the target eNB for uplink forwarding, then the target eNB may use it in a Status Report message sent to the UE over the radio.

For each E-RAB for which the DL COUNT Value IE is received in the SN STATUS TRANSFER message, the target eNB shall use it to mark with the value contained in the PDCP-SN IE of this IE the first downlink packet for which there is no PDCP SN yet assigned. If the DL COUNT Value Extended IE or DL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject To Status Transfer Item IE, the target eNB shall, if supported, use the value contained in the PDCP-SN Extended IE of the DL COUNT Value Extended IE or PDCP-SN Length 18 IE of the DL COUNT Value for PDCP SN Length 18 IE instead of the value contained in the PDCP-SN IE of the DL COUNT Value IE.

For each E-RAB for which the UL COUNT Value IE is received in the SN STATUS TRANSFER message, the target eNB shall not deliver any uplink packet which has a PDCP SN lower than the value contained in the PDCP-SN IE of this IE. If the UL COUNT Value Extended IE or UL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject To Status Transfer Item IE, the target eNB shall, if supported, use the value contained in the PDCP-SN Extended IE of the UL COUNT Value Extended IE or PDCP-SN Length 18 IE of the UL COUNT Value for PDCP SN Length 18 IE instead of the value contained in the PDCP-SN IE of the UL COUNT Value IE.

EN-DC

If the en-gNB sends the message to the MeNB, then the SgNB UE X2AP ID IE shall be included in the SN STATUS TRANSFER message, while the Old eNB UE X2AP ID IE is ignored. The SgNB UE X2AP ID IE is used as the old UE ID.

If the MeNB sends the message to the en-gNB, then the SgNB UE X2AP ID IE shall be included in the SN STATUS TRANSFER message, while the New eNB UE X2AP ID IE is ignored. The SgNB UE X2AP ID IE is used as the new UE ID.

8.2.2.3	Abnormal Conditions

If the target eNB receives this message for a UE for which no prepared handover exists at the target eNB, the target eNB shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.2.4	Handover Cancel

8.2.4.1	General

The Handover Cancel procedure is used to enable a source eNB to cancel an ongoing handover preparation or an already prepared handover.

The procedure uses UE-associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Handover Cancel, successful operation

The source eNB initiates the procedure by sending the HANDOVER CANCEL message to the target eNB. The source eNB shall indicate the reason for cancelling the handover by means of an appropriate cause value.

At the reception of the HANDOVER CANCEL message, the target eNB shall remove any reference to, and release any resources previously reserved to the concerned UE context.

The New eNB UE X2AP ID IE and, if available, the New eNB UE X2AP ID Extension IE shall be included if it has been obtained from the target eNB.

If the Candidate Cells To Be Cancelled List IE is included in the HANDOVER CANCEL message, the target eNB shall consider that the source eNB is cancelling only the handover associated to the candidate cells identified by the included ECGI and associated to the UE-associated signaling connection identified by the Old eNB UE X2AP ID IE (or the Old eNB UE X2AP ID Extension IE if included) and, if included , also by the New eNB UE X2AP ID IE (or the New eNB UE X2AP ID Extension IE if included).

8.2.4.3	Unsuccessful Operation

Not applicable.

8.2.4.4	Abnormal Conditions

Should the HANDOVER CANCEL message refer to a context that does not exist, the target eNB shall ignore the message.

If the Candidate Cells To Be Cancelled List IE is included in the HANDOVER CANCEL message and the handover is not associated to a conditional handover, the target eNB shall ignore the Candidate Cells To Be Cancelled List IE.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the target eNB shall ignore those non-associated candidate cells.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.2.X	Handover Success

8.2.X.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover to enable a target eNB to inform the source eNB that the UE has successfully accessed the target eNB.

The procedure uses UE-associated signalling.

8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Handover Success, successful operation

The target eNB initiates the procedure by sending the HANDOVER SUCCESS message to the source eNB.

If late data forwarding was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source eNB receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE under the same UE-associated signalling connection in the target eNB as cancelled.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source eNB node shall consider that the UE successfully executed the handover. The source eNB may initiate Handover Cancel procedure towards the other signaling connections or other candidate target eNBs for this UE, if any.

8.2.X.3	Unsuccessful Operation

Not applicable.

8.2.X.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source eNB shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.2.Y	Conditional Handover Cancel

8.2.Y.1	General

The Conditional Handover Cancel procedure is used to enable a target eNB to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

8.2.Y.2	Successful Operation

Figure 8.2.X.2-1: Conditional Handover Cancel, successful operation

The target eNB initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source eNB. The target eNB shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

The New eNB UE X2AP ID IE and, if available, the New eNB UE X2AP ID Extension IE shall be included.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source eNB shall consider that the target eNB is about to remove any reference to, and release any resources previously reserved for candidate cells associated to the UE-associated signalling identified by the Old eNB UE X2AP ID IE (or the Old eNB UE X2AP ID Extension IE if included) and the New eNB UE X2AP ID IE (or the New eNB UE X2AP ID Extension IE if included).

If the Candidate Cells To Be Cancelled List IE is also included, the source eNB shall consider that only the resources reserved for the cells identified by the included ECGI are about to be released.

8.2.Y.3	Unsuccessful Operation

Not applicable.

8.2.Y.4	Abnormal Conditions

Should the CONDITIONAL HANDOVER CANCEL message refer to a context that does not exist, the source eNB shall ignore the message.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the CONDITIONAL HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the source eNB shall ignore those non-associated candidate cells.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8.2.Z	Early Status Transfer

8.2.Z.1	General

The purpose of the Early Status Transfer procedure is to transfer the COUNT of the first downlink SDU that the source eNB forwards to the target eNB or the COUNT for discarding already forwarded downlink SDUs for respective E-RAB during DAPS Handover or Conditional Handover.

The procedure uses UE-associated signalling.

8.2.Z.2	Successful Operation

Figure 8.2.Z.2-1: Early Status Transfer during DAPS Handover or Conditional Handover, successful operation

Figure 8.2.Z.2-2: Early Status Transfer during Conditional Handover in dual connectivity or EN-DC operation, successful operation

Between source eNB and target eNB

The E-RABs Subject To Early Status Transfe List IE included in the EARLY STATUS TRANSFER message contains the E-RAB ID(s) corresponding to the E-RAB(s) subject to be simultaneously served by the source and the target eNBs during DAPS Handover or the E-RAB(s) transferred during Conditional Handover.

For each E-RAB for which the FIRST DL COUNT Value IE is received in the EARLY STATUS TRANSFER message, the target eNB shall use it as the COUNT of the first downlink SDU that the source eNB forwards to the target eNB. If the FIRST DL COUNT Value Extended IE or FIRST DL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject To Early Status Transfer Item IE, the target eNB shall, if supported, use this value instead of the value contained in the FIRST DL COUNT Value IE.

For each E-RAB for which the DISCARD DL COUNT Value IE is received in the EARLY STATUS TRANSFER message, the target eNB does not transmit forwarded downlink SDUs to the UE whose COUNT is less than the provided and discards them if transmission has not been attempted. If the DISCARD DL COUNT Value Extended IE or DISCARD DL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject To Early Status Transfer Item IE, the target eNB shall, if supported, use this value instead of the value contained in the DISCARD DL COUNT Value IE.

Between source SN (respectively, source en-gNB) and source MN (respectively, source eNB)

The E-RABs Subject To Early Status Transfer List IE included in the EARLY STATUS TRANSFER message contains the E-RAB ID(s) corresponding to the E-RAB(s) transferred during Conditional Handover.

For each E-RAB in the E-RABs Subject To Early Status Transfer List IE, the source eNB shall forward to the target, the value of the received FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE. If the FIRST DL COUNT Value Extended IE or FIRST DL COUNT Value for PDCP SN Length 18 IE is included, if supported, this value is forwarded instead of the value contained in the FIRST DL COUNT Value IE. If the DISCARD DL COUNT Value Extended IE or DISCARD DL COUNT Value for PDCP SN Length 18 IE is included, if supported, this value is forwarded instead of the value contained in the DISCARD DL COUNT Value IE.

#### 8.2.Z.3	Abnormal Conditions

If the target eNB receives this message for a UE for which no prepared DAPS Handover or Conditional Handover exists at the target eNB, the target eNB shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.15	Data Forwarding Address Indication

#### 8.3.15.1	General

The purpose of the Data Forwarding Address Indication procedure is to allow the new eNB to provide data forwarding addresses to the old eNB in case the RRC connection has been re-established, as specified in TS 36.300 [15].

For Dual Connectivity or EN-DC, the Data Forwarding Address Indication procedure is used during a Conditional Handover to provide data forwarding addresses from the MeNB to the SeNB as specified in TS 36.300 [15], or from the eNB to the en-gNB as specified in TS 37.340 [32].

The procedure uses UE-associated signalling.

#### 8.3.15.2	Successful Operation

Figure 8.3.15.2-1: Data Forwarding Address Indication, successful operation

Figure 8.3.15.2-2: Data Forwarding Address Indication for Conditional Handover, successful operation

The new eNB initiates the procedure by sending a DATA FORWARDING ADDRESS INDICATION message to the old eNB.

For each E-RAB included in E-RABs Data Forwarding Address List IE, the new eNB indicates that it requests data forwarding of downlink packets to the GTP TEID indicated in the DL GTP Tunnel Endpoint IE.

If the DATA FORWARDING ADDRESS INDICATION message includes the CHO DC Indicator IE, the SeNB (respectively, the en-gNB for EN-DC) shall, if supported, consider that the DATA FORWARDING ADDRESS INDICATION message concerns a Conditional Handover, and act as specified in TS 36.300 [15] for dual connectivity (respectively, act as specified in TS 37.340 [32] for EN-DC).

If the DATA FORWARDING ADDRESS INDICATION message includes the CHO DC Indicator IE, the SeNB (respectively, the en-gNB for EN-DC) shall, if supported, consider that the DATA FORWARDING ADDRESS INDICATION message concerns a Conditional Handover, and act as specified in TS 36.300 [15] for dual connectivity (respectively, act as specified in TS 37.340 [32] for EN-DC).

#### 8.3.15.3	Unsuccessful Operation

Not applicable.

#### 8.3.15.4	Abnormal Conditions

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.7.5	SgNB Reconfiguration Completion

#### 8.7.5.1	General

The purpose of the SgNB Reconfiguration Completion procedure is to provide information to the en-gNB whether the requested configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: SgNB Reconfiguration Complete procedure, successful operation.

The MeNB initiates the procedure by sending the SGNB RECONFIGURATION COMPLETE message to the en-gNB.

The SGNB RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the en-gNB. The MeNB may also provide NR RRCReconfigurationComplete message in the MeNB to SgNB Container IE.

-	or the configuration requested by the en-gNB has been rejected. The MeNB shall provide information with sufficient precision in the included Cause IE to enable the en-gNB to know the reason for an unsuccessful reconfiguration.

Upon reception of the SGNB RECONFIGURATION COMPLETE message the en-gNB shall stop the timer TDCoverall. In case of conditional PSCell change, the en-gNB shall also consider the procedure successful even if the timer TDCoverall has not been initiated when receiving this message.

#### 8.7.5.3	Abnormal Conditions

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1	Message Functional Definition and Content

9.1.1	Messages for Basic Mobility Procedures

9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – | – |
| >UE Security Capabilities | M |  | 9.2.29 |  | – | – |
| >AS Security Information | M |  | 9.2.30 |  | – | – |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – | – |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – | – |
| >E-RABs To Be Setup List |  | 1 |  |  | – | – |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >>>DAPS Request Information | O |  | 9.2.x |  | YES | ignore |
| >RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the RRC HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – | – |
| >Handover Restriction List | O |  | 9.2.3 |  | – | – |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – | – |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | Ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  |  |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB |  |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB |  |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  |  |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  |  |  |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  |  |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – | – |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  |  |  |
| >New eNB UE X2AP ID | C-ifCHOmod |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB |  |  |
| >New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |  |  |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target eNB to inform the source eNB about the prepared resources at the target.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| E-RABs Admitted List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – |  |
| >>DAPS Response Information | O |  | 9.2.y |  | YES | reject |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| WT UE Context Kept Indicator | O |  | UE Context Kept Indicator 9.2.85 | Indicates that the WT has acknowledged to keep the UE context | YES | ignore |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | ECGI 9.2.14 | Target cell indicated in the corresponding HANDOVER REQUEST message | YES | reject |
| >Maximum Number of CHO Preparations | O |  | 9.2.C |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

9.1.1.3	HANDOVER PREPARATION FAILURE

This message is sent by the target eNB to inform the source eNB that the Handover Preparation has failed.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| Requested Target Cell ID | O |  | ECGI 9.2.14 | Target cell indicated in the corresponding HANDOVER REQUEST message | YES | reject |

9.1.1.4	SN STATUS TRANSFER

This message is sent by the source eNB to the target eNB to transfer the uplink/downlink PDCP SN and HFN status during a handover or for EN-DC.

Direction: source eNB  target eNB (handover), eNB from which the E-RAB context is transferred  eNB to which the E-RAB context is transferred (RRC connection re-establishment or dual connectivity), MeNB/en-gNB from which the E-RAB context is transferred  en-gNB/MeNB to which the E-RAB context is transferred (EN-DC).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB and for dual connectivity/EN-DC at the eNB from which the E-RAB context is transferred | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the eNB to which the E-RAB context is transferred | YES | reject |
| E-RABs Subject To Status Transfer List |  | 1 |  |  | YES | ignore |
| >E-RABs Subject To Status Transfer Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>Receive Status Of UL PDCP SDUs | O |  | BIT STRING (4096) | PDCP Sequence Number = (First Missing SDU Number + bit position) modulo 4096 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>UL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 12 bit long PDCP-SN | – |  |
| >>DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 12 bit long PDCP-SN. | – |  |
| >>Receive Status Of UL PDCP SDUs Extended | O |  | BIT STRING (1..16384) | The IE is used in case of 15 bit long PDCP-SN in this release. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |
| >>UL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 15 bit long PDCP-SN | YES | ignore |
| >>DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper Frame Number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 15 bit long PDCP-SN. | YES | ignore |
| >>Receive Status Of UL PDCP SDUs for PDCP SN Length 18 | O |  | BIT STRING (1..131072) | The IE is used in case of 18 bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |
| >>UL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 18 bit long PDCP-SN | YES | ignore |
| >>DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper Frame Number that the target eNB/en-gNB should assign for the next DL SDU not having an SN yet in case of 18 bit long PDCP-SN. | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity/EN-DC at the eNB from which the E-RAB context is transferred. | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the eNB to which the E-RAB context is transferred. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the SgNB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1.1.6	HANDOVER CANCEL

This message is sent by the source eNB to the target eNB to cancel an ongoing handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | ECGI 9.2.14 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 16. |

////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1.1.Y	HANDOVER SUCCESS

This message is sent by the target eNB to the source eNB to indicate the successful access of the UE toward the target eNB.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 | Target cell indicated in the corresponding Handover Preparation procedure | YES | reject |

////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1.1.X	CONDITIONAL HANDOVER CANCEL

This message is sent by the target eNB to the source eNB to cancel an ongoing conditional handover.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | ECGI 9.2.14 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 16. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.1.1.Z	EARLY STATUS TRANSFER

This message is sent by the source eNB to the target eNB to transfer the COUNT value related to the forwarded downlink SDUs during DAPS Handover or Conditional Handover.

Direction: source eNB  target eNB (DAPS Handover or Conditional Handover).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for DAPS handover or Conditional handover at the source eNB | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for DAPS handover or Conditional handover at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for DAPS handover or Conditional handover at the source eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for DAPS handover or Conditional handover at the target eNB | YES | reject |
| CHOICE Procedure Stage | M |  |  |  | YES | reject |
| >First DL COUNT |  |  |  |  |  |  |
| >>E-RABs Subject To Early Status Transfer List |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RABs Subject To Early Status Transfer Item |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>FIRST DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/MeNB forwards to the target eNB/en-gNB in case of 12 bit long PDCP-SN |  |  |
| >>>>FIRST DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/MeNB forwards to the target eNB/en-gNB in case of 15 bit long PDCP-SN | YES | ignore |
| >>>>FIRST DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper frame number of the first DL SDU that the source eNB/MeNB forwards to the target eNB/en-gNB in case of 18 bit long PDCP-SN | YES | ignore |
| >DL Discarding |  |  |  |  |  |  |
| >>E-RABs Subject To DL Discarding List | M | 1 |  |  | – |  |
| >>>E-RABs Subject To DL Discarding Item |  | 1 .. <maxnoofBearers> |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>>DISCARD DL COUNT Value | M |  | COUNT Value 9.2.15 | PDCP-SN and Hyper frame number for which the target eNB/en-gNB should discard forwarded DL SDUs associated with lower values in case of 12 bit long PDCP-SN |  |  |
| >>>>DISCARD DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.66 | PDCP-SN and Hyper frame number for which the target eNB/en-gNB should discard forwarded DL SDUs associated with lower values in case of 15 bit long PDCP-SN | YES | ignore |
| >>>>DISCARD DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.82 | PDCP-SN and Hyper frame number for which the target eNB/en-gNB should discard forwarded DL SDUs associated with lower values in case of 18 bit long PDCP-SN | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.2.43	DATA FORWARDING ADDRESS INDICATION

This message is sent by the new eNB to indicate to the old eNB forwarding addresses for each E-RAB for which it admits data forwarding.

Direction: new eNB  old eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the old eNB | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the old eNB | YES | ignore |
| E-RABs Data Forwarding Address List |  | 1 |  |  | YES | ignore |
| > E-RABs Data Forwarding Address Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| CHO DC Indicator | O |  | ENUMERATED (true, ...) | Indicating that the DATA FORWARDING ADDRESS INDICATION message is for a Conditional Handover. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.6	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Handover Desirable for Radio Reasons, Time Critical Handover, Resource Optimisation Handover, Reduce Load in Serving Cell, Partial Handover, Unknown New eNB UE X2AP ID, Unknown Old eNB UE X2AP ID, Unknown Pair of UE X2AP ID, HO Target not Allowed, TX2RELOCoverall Expiry, TRELOCprep Expiry, Cell not Available, No Radio Resources Available in Target Cell, Invalid MME Group ID, Unknown MME Code, Encryption And/Or Integrity Protection Algorithms Not Supported, ReportCharacteristicsEmpty, NoReportPeriodicity, ExistingMeasurementID, Unknown eNB Measurement ID, Measurement Temporarily not Available, Unspecified,...,Load Balancing, Handover Optimisation, Value out of allowed range, Multiple E-RAB ID instances, Switch Off Ongoing, Not supported QCI value, Measurement not supported for the object,TDCoverall Expiry, TDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve user bit rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, MCG Mobility, SCG Mobility, Count reaches max value, Unknown Old en-gNB UE X2AP ID, PDCP Overload, CHO-CPC resources to be changed) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified,...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error,Abstract Syntax Error (Reject),Abstract Syntax Error (Ignore and Notify),Message not Compatible with Receiver State,Semantic Error,Unspecified,Abstract Syntax Error (Falsely Constructed Message),...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure,O&M Intervention,Not enough User Plane Processing Resources,Unspecified,...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question |
| Invalid MME Group ID | The target eNB doesn’t belong to the same pool area of the source eNB i.e. S1 handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target eNB did not admit all E-RABs included in the HANDOVER REQUEST and the source eNB estimated service continuity for the UE would be better by not proceeding with handover towards this particular target eNB. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TX2RELOCoverall Expiry | The reason for the action is expiry of timer TX2RELOCoverall. |
| TRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TRELOCprep expires. |
| Unknown MME Code | The target eNB belongs to the same pool area of the source eNB and recognizes the MME Group ID. However, the MME Code is unknown to the target eNB. |
| Unknown New eNB UE X2AP ID | The action failed because the New eNB UE X2AP ID or the MeNB UE X2AP ID is unknown. |
| Unknown Old eNB UE X2AP ID | The action failed because the Old eNB UE X2AP ID or the SeNB UE X2AP ID is unknown. |
| Unknown Pair of UE X2AP ID | The action failed because the pair of UE X2 AP IDs is unknown. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target eNB is unable to support any of the encryption and/or integrity protection algorithms supported by the UE, or the en-gNB is unable to support any of the NR encryption and/or integrity protection algorithms supported by the UE for EN-DC operation. |
| ReportCharacteristicsEmpty | The action failed because there is no characteristic reported. |
| NoReportPeriodicity | The action failed because the periodicity is not defined. |
| ExistingMeasurementID | The action failed because measurement-ID is already used. |
| Unknown eNB Measurement ID | The action failed because some eNB Measurement-ID is unknown. |
| Measurement Temporarily not Available | The eNB can temporarily not provide the requested measurement object. |
| Load Balancing | The reason for mobility settings change is load balancing. |
| Handover Optimisation | The reason for mobility settings change is handover optimisation. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the eNB2 Proposed Mobility Parameters IE is too low or too high. |
| Multiple E-RAB ID Instances | The action failed because multiple instances of the same E-RAB had been provided to the eNB. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving eNB in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related. |
| Measurement not Supported For The Object | At least one of the concerned cell(s) does not support the requested measurement. |
| TDCoverall Expiry | The reason for the action is expiry of timer TDCoverall. |
| TDCprep Expiry | The reason for the action is expiry of timer TDCprep. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Encryption Algorithms Not Supported | The requested eNB is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| User Inactivity | The action is requested due to user inactivity on all E-RABs, e.g., S1 is requested to be released in order to optimise the radio resources; or SeNB/en-gNB didn’t see activity on the DRB recently. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity and EN-DC only. |
| MCG Mobility | The procedure is initiated due to mobility related at MCG radio resource. |
| SCG Mobility | The procedure is initiated due to mobility related at SCG radio resource. |
| Count reaches max value | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old en-gNB UE X2AP ID | The action failed because the Old en-gNB UE X2AP ID or the SeNB UE X2AP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.x	DAPS Request Information

The DAPS Indicator IE indicates that the source eNB requests a DAPS HO for the concered E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Indicator | M |  | ENUMERATED (DAPS HO required, …) | Indicates that DAPS HO is requested |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.y	DAPS Response Information

The DAPS Response Indicator IE indicates the response to a requested DAPS Handover for the concerned E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, DAPS HO not accepted, …) | Indicates that DAPS Handover is accepted or not. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.2.C	Maximum Number of CHO Preparations

This IE indicates the maximum number of CHO preparations for a UE towards a candidate target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Number of CHO Preparations | M |  | INTEGER (1..16, ...) |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

X2AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM X2AP-CommonDataTypes

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ENBConfigurationUpdate,

ENBConfigurationUpdateAcknowledge,

ENBConfigurationUpdateFailure,

ErrorIndication,

HandoverCancel,

HandoverReport,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

LoadInformation,

PrivateMessage,

ResetRequest,

ResetResponse,

ResourceStatusFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusUpdate,

RLFIndication,

SNStatusTransfer,

UEContextRelease,

X2SetupFailure,

X2SetupRequest,

X2SetupResponse,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

X2Release,

X2APMessageTransfer,

SeNBAdditionRequest,

SeNBAdditionRequestAcknowledge,

SeNBAdditionRequestReject,

SeNBReconfigurationComplete,

SeNBModificationRequest,

SeNBModificationRequestAcknowledge,

SeNBModificationRequestReject,

SeNBModificationRequired,

SeNBModificationConfirm,

SeNBModificationRefuse,

SeNBReleaseRequest,

SeNBReleaseRequired,

SeNBReleaseConfirm,

SeNBCounterCheckRequest,

X2RemovalFailure,

X2RemovalRequest,

X2RemovalResponse,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

SgNBAdditionRequest,

SgNBAdditionRequestAcknowledge,

SgNBAdditionRequestReject,

SgNBReconfigurationComplete,

SgNBModificationRequest,

SgNBModificationRequestAcknowledge,

SgNBModificationRequestReject,

SgNBModificationRequired,

SgNBModificationConfirm,

SgNBModificationRefuse,

SgNBReleaseRequest,

SgNBReleaseRequestAcknowledge,

SgNBReleaseRequestReject,

SgNBReleaseRequired,

SgNBReleaseConfirm,

SgNBCounterCheckRequest,

SgNBChangeRequired,

SgNBChangeConfirm,

SgNBChangeRefuse,

RRCTransfer,

ENDCX2SetupRequest,

ENDCX2SetupResponse,

ENDCX2SetupFailure,

ENDCConfigurationUpdate,

ENDCConfigurationUpdateAcknowledge,

ENDCConfigurationUpdateFailure,

SecondaryRATDataUsageReport,

ENDCCellActivationRequest,

ENDCCellActivationResponse,

ENDCCellActivationFailure,

ENDCPartialResetRequired,

ENDCPartialResetConfirm,

EUTRANRCellResourceCoordinationRequest,

EUTRANRCellResourceCoordinationResponse,

SgNBActivityNotification,

ENDCX2RemovalRequest,

ENDCX2RemovalResponse,

ENDCX2RemovalFailure,

DataForwardingAddressIndication,

GNBStatusIndication,

ENDCConfigurationTransfer,

DeactivateTrace,

TraceStart,

HandoverSuccess,

EarlyStatusTransfer,

ConditionalHandoverCancel

FROM X2AP-PDU-Contents

id-cellActivation,

id-eNBConfigurationUpdate,

id-errorIndication,

id-handoverCancel,

id-handoverReport,

id-handoverPreparation,

id-loadIndication,

id-privateMessage,

id-reset,

id-resourceStatusReporting,

id-resourceStatusReportingInitiation,

id-rLFIndication,

id-snStatusTransfer,

id-uEContextRelease,

id-x2Setup,

id-mobilitySettingsChange,

id-x2Release,

id-x2APMessageTransfer,

id-seNBAdditionPreparation,

id-seNBReconfigurationCompletion,

id-meNBinitiatedSeNBModificationPreparation,

id-seNBinitiatedSeNBModification,

id-meNBinitiatedSeNBRelease,

id-seNBinitiatedSeNBRelease,

id-seNBCounterCheck,

id-x2Removal,

id-retrieveUEContext,

id-sgNBAdditionPreparation,

id-sgNBReconfigurationCompletion,

id-meNBinitiatedSgNBModificationPreparation,

id-sgNBinitiatedSgNBModification,

id-meNBinitiatedSgNBRelease,

id-sgNBinitiatedSgNBRelease,

id-sgNBChange,

id-sgNBCounterCheck,

id-rRCTransfer,

id-endcX2Setup,

id-endcConfigurationUpdate,

id-secondaryRATDataUsageReport,

id-endcCellActivation,

id-endcPartialReset,

id-eUTRANRCellResourceCoordination,

id-SgNBActivityNotification,

id-endcX2Removal,

id-dataForwardingAddressIndication,

id-gNBStatusIndication,

id-endcConfigurationTransfer,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-earlyStatusTransfer,

id-conditionalHandoverCancel

FROM X2AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode 	UNIQUE,

&criticality			Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

X2AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURES X2AP-ELEMENTARY-PROCEDURE ::= {

X2AP-ELEMENTARY-PROCEDURES-CLASS-1			|

X2AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-1 X2AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

reset													|

x2Setup													|

resourceStatusReportingInitiation						|

eNBConfigurationUpdate									|

mobilitySettingsChange									|

cellActivation											|

seNBAdditionPreparation									|

meNBinitiatedSeNBModificationPreparation				|

seNBinitiatedSeNBModification							|

seNBinitiatedSeNBRelease								|

x2Removal												|

retrieveUEContext										|

sgNBAdditionPreparation									|

meNBinitiatedSgNBModificationPreparation					|

sgNBinitiatedSgNBModification							|

meNBinitiatedSgNBRelease									|

sgNBinitiatedSgNBRelease									|

sgNBChange												|

endcX2Setup												|

endcConfigurationUpdate									|

endcCellActivation										|

endcPartialReset											|

eUTRANRCellResourceCoordination							|

endcX2Removal											,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-2 X2AP-ELEMENTARY-PROCEDURE ::= {

snStatusTransfer						|

uEContextRelease						|

handoverCancel							|

errorIndication							|

resourceStatusReporting					|

loadIndication							|

privateMessage							|

rLFIndication							|

handoverReport							|

x2Release								|

x2APMessageTransfer						|

seNBReconfigurationCompletion			|

meNBinitiatedSeNBRelease				|

seNBCounterCheck						|

sgNBReconfigurationCompletion			|

sgNBCounterCheck							|

rRCTransfer								|

secondaryRATDataUsageReport				|

sgNBActivityNotification					|

dataForwardingAddressIndication			|

gNBStatusIndication						|

endcConfigurationTransfer				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

earlyStatusTransfer					|

conditionalHandoverCancel,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

snStatusTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-snStatusTransfer

CRITICALITY				ignore

}

uEContextRelease X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				ignore

}

handoverCancel X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

handoverReport X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

errorIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

reset	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

x2Setup	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2SetupRequest

SUCCESSFUL OUTCOME		X2SetupResponse

UNSUCCESSFUL OUTCOME	X2SetupFailure

PROCEDURE CODE			id-x2Setup

CRITICALITY				reject

}

loadIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LoadInformation

PROCEDURE CODE			id-loadIndication

CRITICALITY				ignore

}

eNBConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBConfigurationUpdate

SUCCESSFUL OUTCOME		ENBConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENBConfigurationUpdateFailure

PROCEDURE CODE			id-eNBConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

rLFIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RLFIndication

PROCEDURE CODE			id-rLFIndication

CRITICALITY				ignore

}

privateMessage			X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mobilitySettingsChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MobilityChangeRequest

SUCCESSFUL OUTCOME		MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME	MobilityChangeFailure

PROCEDURE CODE			id-mobilitySettingsChange

CRITICALITY				reject

}

cellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

x2Release X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2Release

PROCEDURE CODE			id-x2Release

CRITICALITY				reject

}

x2APMessageTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2APMessageTransfer

PROCEDURE CODE			id-x2APMessageTransfer

CRITICALITY				reject

}

seNBAdditionPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBAdditionRequest

SUCCESSFUL OUTCOME		SeNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBAdditionRequestReject

PROCEDURE CODE			id-seNBAdditionPreparation

CRITICALITY				reject

}

seNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReconfigurationComplete

PROCEDURE CODE			id-seNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSeNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequest

SUCCESSFUL OUTCOME		SeNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSeNBModificationPreparation

CRITICALITY				reject

}

seNBinitiatedSeNBModification	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequired

SUCCESSFUL OUTCOME		SeNBModificationConfirm

UNSUCCESSFUL OUTCOME	SeNBModificationRefuse

PROCEDURE CODE			id-seNBinitiatedSeNBModification

CRITICALITY				reject

}

meNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequest

PROCEDURE CODE			id-meNBinitiatedSeNBRelease

CRITICALITY				ignore

}

seNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequired

SUCCESSFUL OUTCOME		SeNBReleaseConfirm

PROCEDURE CODE			id-seNBinitiatedSeNBRelease

CRITICALITY				reject

}

seNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBCounterCheckRequest

PROCEDURE CODE			id-seNBCounterCheck

CRITICALITY				reject

}

x2Removal	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2RemovalRequest

SUCCESSFUL OUTCOME		X2RemovalResponse

UNSUCCESSFUL OUTCOME	X2RemovalFailure

PROCEDURE CODE			id-x2Removal

CRITICALITY				reject

}

retrieveUEContext	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

sgNBAdditionPreparation		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBAdditionRequest

SUCCESSFUL OUTCOME		SgNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBAdditionRequestReject

PROCEDURE CODE			id-sgNBAdditionPreparation

CRITICALITY				reject

}

sgNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReconfigurationComplete

PROCEDURE CODE			id-sgNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSgNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequest

SUCCESSFUL OUTCOME		SgNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBModificationPreparation

CRITICALITY				reject

}

sgNBinitiatedSgNBModification		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequired

SUCCESSFUL OUTCOME		SgNBModificationConfirm

UNSUCCESSFUL OUTCOME	SgNBModificationRefuse

PROCEDURE CODE			id-sgNBinitiatedSgNBModification

CRITICALITY				reject

}

meNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequest

SUCCESSFUL OUTCOME		SgNBReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBReleaseRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBRelease

CRITICALITY				ignore

}

sgNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequired

SUCCESSFUL OUTCOME		SgNBReleaseConfirm

PROCEDURE CODE			id-sgNBinitiatedSgNBRelease

CRITICALITY				reject

}

sgNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBCounterCheckRequest

PROCEDURE CODE			id-sgNBCounterCheck

CRITICALITY				reject

}

sgNBChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBChangeRequired

SUCCESSFUL OUTCOME		SgNBChangeConfirm

UNSUCCESSFUL OUTCOME	SgNBChangeRefuse

PROCEDURE CODE			id-sgNBChange

CRITICALITY				reject

}

rRCTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

endcX2Setup X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCX2SetupRequest

SUCCESSFUL OUTCOME		ENDCX2SetupResponse

UNSUCCESSFUL OUTCOME	ENDCX2SetupFailure

PROCEDURE CODE			id-endcX2Setup

CRITICALITY				reject

}

endcConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCConfigurationUpdate

SUCCESSFUL OUTCOME		ENDCConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENDCConfigurationUpdateFailure

PROCEDURE CODE			id-endcConfigurationUpdate

CRITICALITY				reject

}

secondaryRATDataUsageReport	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

endcCellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCCellActivationRequest

SUCCESSFUL OUTCOME		ENDCCellActivationResponse

UNSUCCESSFUL OUTCOME	ENDCCellActivationFailure

PROCEDURE CODE			id-endcCellActivation

CRITICALITY				reject

}

endcPartialReset	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCPartialResetRequired

SUCCESSFUL OUTCOME		ENDCPartialResetConfirm

PROCEDURE CODE			id-endcPartialReset

CRITICALITY				reject

}

eUTRANRCellResourceCoordination X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EUTRANRCellResourceCoordinationRequest

SUCCESSFUL OUTCOME		EUTRANRCellResourceCoordinationResponse

PROCEDURE CODE			id-eUTRANRCellResourceCoordination

CRITICALITY				reject

}

sgNBActivityNotification 	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBActivityNotification

PROCEDURE CODE			id-SgNBActivityNotification

CRITICALITY				reject

}

endcX2Removal	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCX2RemovalRequest

SUCCESSFUL OUTCOME		ENDCX2RemovalResponse

UNSUCCESSFUL OUTCOME	ENDCX2RemovalFailure

PROCEDURE CODE			id-endcX2Removal

CRITICALITY				reject

}

dataForwardingAddressIndication	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataForwardingAddressIndication

PROCEDURE CODE			id-dataForwardingAddressIndication

CRITICALITY				ignore

}

gNBStatusIndication		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBStatusIndication

PROCEDURE CODE			id-gNBStatusIndication

CRITICALITY				ignore

}

endcConfigurationTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCConfigurationTransfer

PROCEDURE CODE			id-endcConfigurationTransfer

CRITICALITY				ignore

}

deactivateTrace X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-deactivateTrace

CRITICALITY				ignore

}

traceStart X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-traceStart

CRITICALITY				ignore

}

handoverSuccess X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverSuccess

PROCEDURE CODE			id-handoverSuccess

CRITICALITY				ignore

}

earlyStatusTransfer		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EarlyStatusTransfer

PROCEDURE CODE			id-earlyStatusTransfer

CRITICALITY				ignore

}

conditionalHandoverCancel X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConditionalHandoverCancel

PROCEDURE CODE			id-conditionalHandoverCancel

CRITICALITY				ignore

}

END

-- ASN1STOP

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

AerialUEsubscriptionInformation,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

E-RABsSubjectToDLDiscarding-List,

E-RABsSubjectToEarlyStatusTransfer-List,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RLC-Status,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UEsToBeResetList,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

SplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

SplitSRB,

NRUeReport,

EN-DC-ResourceConfiguration,

TAC,

NRFreqInfo,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

AdditionalPLMNs-Item,

RLCMode,

GBR-QosInformation,

DRB-ID,

FiveGS-TAC,

SULInformation,

Packet-LossRate,

ResourceType,

DataTrafficResourceIndication,

SpectrumSharingGroupID,

RRC-Config-Ind,

SGNB-Addition-Trigger-Ind,

UserPlaneTrafficActivityReport,

ERABActivityNotifyItemList,

PDCPSnLength,

Subscription-Based-UE-DifferentiationInfo,

LCID,

DuplicationActivation,

GNBOverloadInformation,

NewDRBIDrequest,

DesiredActNotificationLevel,

LocationInformationSgNB,

LocationInformationSgNBReporting,

EndcSONConfigurationTransfer,

NRNeighbour-Information,

InterfaceInstanceIndication,

BPLMN-ID-Info-NR,

EPCHandoverRestrictionListContainer,

AdditionalRRMPriorityIndex,

RequestedFastMCGRecoveryViaSRB3,

AvailableFastMCGRecoveryViaSRB3,

RequestedFastMCGRecoveryViaSRB3Release,

ReleaseFastMCGRecoveryViaSRB3,

FastMCGRecovery,

PartialListIndicator,

MaximumCellListSize,

MessageOversizeNotification,

TNLConfigurationInfo,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

RAN-UE-NGAP-ID,

CHOinformation-REQ,

CHOinformation-ACK,

DAPSRequestInfo,

DAPSResponseInfo,

LowerLayerPresenceStatusChange,

CandidateCellsToBeCancelledList,

CHO-DC-Indicator

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-AerialUEsubscriptionInformation,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-RLC-Status,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-UE-ContextInformation,

id-UE-HistoryInformation,

id-UE-X2AP-ID,

id-Measurement-ID,

id-ReportCharacteristics,

id-ENB1-Measurement-ID,

id-ENB2-Measurement-ID,

id-ENB1-Cell-ID,

id-ENB2-Cell-ID,

id-ENB2-Proposed-Mobility-Parameters,

id-ENB1-Mobility-Parameters,

id-ENB2-Mobility-Parameters-Modification-Range,

id-FailureCellPCI,

id-Re-establishmentCellECGI,

id-FailureCellCRNTI,

id-ShortMAC-I,

id-SourceCellECGI,

id-FailureCellECGI,

id-HandoverReportType,

id-UE-RLF-Report-Container,

id-PartialSuccessIndicator,

id-MeasurementInitiationResult-List,

id-MeasurementInitiationResult-Item,

id-MeasurementFailureCause-Item,

id-CompleteFailureCauseInformation-List,

id-CompleteFailureCauseInformation-Item,

id-CSGMembershipStatus,

id-CSG-Id,

id-MDTConfiguration,

id-ManagementBasedMDTallowed,

id-ABS-Status,

id-RRCConnSetupIndicator,

id-RRCConnReestabIndicator,

id-TargetCellInUTRAN,

id-MobilityInformation,

id-SourceCellCRNTI,

id-ManagementBasedMDTPLMNList,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-IntendedULDLConfiguration,

id-ExtendedULInterferenceOverloadInfo,

id-RNL-Header,

id-x2APMessage,

id-UE-HistoryInformationFromTheUE,

id-ExpectedUEBehaviour,

id-MeNB-UE-X2AP-ID,

id-SeNB-UE-X2AP-ID,

id-UE-SecurityCapabilities,

id-SeNBSecurityKey,

id-SeNBUEAggregateMaximumBitRate,

id-ServingPLMN,

id-E-RABs-ToBeAdded-List,

id-E-RABs-ToBeAdded-Item,

id-MeNBtoSeNBContainer,

id-E-RABs-Admitted-ToBeAdded-List,

id-E-RABs-Admitted-ToBeAdded-Item,

id-SeNBtoMeNBContainer,

id-ResponseInformationSeNBReconfComp,

id-UE-ContextInformationSeNBModReq,

id-E-RABs-ToBeAdded-ModReqItem,

id-E-RABs-ToBeModified-ModReqItem,

id-E-RABs-ToBeReleased-ModReqItem,

id-E-RABs-Admitted-ToBeAdded-ModAckList,

id-E-RABs-Admitted-ToBeModified-ModAckList,

id-E-RABs-Admitted-ToBeReleased-ModAckList,

id-E-RABs-Admitted-ToBeAdded-ModAckItem,

id-E-RABs-Admitted-ToBeModified-ModAckItem,

id-E-RABs-Admitted-ToBeReleased-ModAckItem,

id-SCGChangeIndication,

id-E-RABs-ToBeReleased-ModReqd,

id-E-RABs-ToBeReleased-ModReqdItem,

id-E-RABs-ToBeReleased-List-RelReq,

id-E-RABs-ToBeReleased-RelReqItem,

id-E-RABs-ToBeReleased-List-RelConf,

id-E-RABs-ToBeReleased-RelConfItem,

id-E-RABs-SubjectToCounterCheck-List,

id-E-RABs-SubjectToCounterCheckItem,

id-CoMPInformation,

id-ReportingPeriodicityRSRPMR,

id-RSRPMRList,

id-UE-RLF-Report-Container-for-extended-bands,

id-ProSeAuthorized,

id-CoverageModificationList,

id-ReportingPeriodicityCSIR,

id-CSIReportList,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-LHN-ID,

id-Correlation-ID,

id-SIPTO-Correlation-ID,

id-UE-ContextReferenceAtSeNB,

id-UE-ContextReferenceAtWT,

id-UE-ContextKeptIndicator,

id-UEs-ToBeReset,

id-UEs-Admitted-ToBeReset,

id-WT-UE-ContextKeptIndicator,

id-New-eNB-UE-X2AP-ID-Extension,

id-Old-eNB-UE-X2AP-ID-Extension,

id-MeNB-UE-X2AP-ID-Extension,

id-SeNB-UE-X2AP-ID-Extension,

id-SIPTO-BearerDeactivationIndication,

id-Tunnel-Information-for-BBF,

id-SIPTO-L-GW-TransportLayerAddress,

id-GW-TransportLayerAddress,

id-X2RemovalThreshold,

id-CellReportingIndicator,

id-V2XServicesAuthorized,

id-resumeID,

id-UE-ContextInformationRetrieve,

id-E-RABs-ToBeSetupRetrieve-Item,

id-NewEUTRANCellIdentifier,

id-MakeBeforeBreakIndicator,

id-UESidelinkAggregateMaximumBitRate,

id-uL-GTPtunnelEndpoint,

id-SgNBSecurityKey,

id-SgNBUEAggregateMaximumBitRate,

id-E-RABs-ToBeAdded-SgNBAddReqList,

id-MeNBtoSgNBContainer,

id-SgNB-UE-X2AP-ID,

id-RequestedSplitSRBs,

id-E-RABs-ToBeAdded-SgNBAddReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList,

id-SgNBtoMeNBContainer,

id-AdmittedSplitSRBs,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item,

id-ResponseInformationSgNBReconfComp,

id-UE-ContextInformation-SgNBModReq,

id-E-RABs-ToBeAdded-SgNBModReq-Item,

id-E-RABs-ToBeModified-SgNBModReq-Item,

id-E-RABs-ToBeReleased-SgNBModReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList,

id-E-RABs-Admitted-ToBeModified-SgNBModAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList,

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item,

id-E-RABs-ToBeReleased-SgNBModReqdList,

id-E-RABs-ToBeModified-SgNBModReqdList,

id-E-RABs-ToBeReleased-SgNBModReqd-Item,

id-E-RABs-ToBeModified-SgNBModReqd-Item,

id-E-RABs-ToBeReleased-SgNBChaConfList,

id-E-RABs-ToBeReleased-SgNBChaConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqList,

id-E-RABs-ToBeReleased-SgNBRelReq-Item,

id-E-RABs-ToBeReleased-SgNBRelConfList,

id-E-RABs-ToBeReleased-SgNBRelConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqdList,

id-E-RABs-ToBeReleased-SgNBRelReqd-Item,

id-E-RABs-SubjectToSgNBCounterCheck-List,

id-E-RABs-SubjectToSgNBCounterCheck-Item,

id-Target-SgNB-ID,

id-RRCContainer,

id-SRBType,

id-HandoverRestrictionList,

id-SCGConfigurationQuery,

id-SplitSRB,

id-NRUeReport,

id-InitiatingNodeType-EndcX2Setup,

id-InitiatingNodeType-EndcConfigUpdate,

id-RespondingNodeType-EndcX2Setup,

id-RespondingNodeType-EndcConfigUpdate,

id-NRUESecurityCapabilities,

id-PDCPChangeIndication,

id-ServedEUTRAcellsENDCX2ManagementList,

id-ServedEUTRAcellsToModifyListENDCConfUpd,

id-ServedEUTRAcellsToDeleteListENDCConfUpd,

id-ServedNRcellsToModifyListENDCConfUpd,

id-ServedNRcellsToDeleteListENDCConfUpd,

id-CellAssistanceInformation,

id-Globalen-gNB-ID,

id-ServedNRcellsENDCX2ManagementList,

id-Old-SgNB-UE-X2AP-ID,

id-UE-ContextReferenceAtSgNB,

id-SecondaryRATUsageReportList,

id-ActivationID,

id-ServedNRCellsToActivate,

id-ActivatedNRCellList,

id-MeNBResourceCoordinationInformation,

id-SgNBResourceCoordinationInformation,

id-UEAppLayerMeasConfig,

id-SelectedPLMN,

id-SubscriberProfileIDforRFP,

id-InitiatingNodeType-EutranrCellResourceCoordination,

id-RespondingNodeType-EutranrCellResourceCoordination,

id-DataTrafficResourceIndication,

id-SpectrumSharingGroupID,

id-ListofEUTRACellsinEUTRACoordinationReq,

id-ListofEUTRACellsinEUTRACoordinationResp,

id-ListofEUTRACellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationResp,

id-RRCConfigIndication,

id-SGNB-Addition-Trigger-Ind,

id-RequestedSplitSRBsrelease,

id-AdmittedSplitSRBsrelease,

id-E-RABs-AdmittedToBeModified-SgNBModConfList,

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item,

id-UEContextLevelUserPlaneActivity,

id-ERABActivityNotifyItemList,

id-MeNBCell-ID,

id-InitiatingNodeType-EndcX2Removal,

id-RespondingNodeType-EndcX2Removal,

id-uLpDCPSnLength,

id-dL-Forwarding,

id-E-RABs-DataForwardingAddress-List,

id-E-RABs-DataForwardingAddress-Item,

id-Subscription-Based-UE-DifferentiationInfo,

id-RLCMode-transferred,

id-dLPDCPSnLength,

id-secondarysgNBDLGTPTEIDatPDCP,

id-secondarymeNBULGTPTEIDatPDCP,

id-lCID,

id-duplicationActivation,

id-GNBOverloadInformation,

id-new-drb-ID-req,

id-NRNeighbourInfoToModify,

id-DesiredActNotificationLevel,

id-LocationInformationSgNB,

id-LocationInformationSgNBReporting,

id-endcSONConfigurationTransfer,

id-EUTRANTraceID,

id-additionalPLMNs-Item,

id-InterfaceInstanceIndication,

id-BPLMN-ID-Info-NR,

id-EPCHandoverRestrictionListContainer,

id-ERABs-transferred-to-MeNB,

id-AdditionalRRMPriorityIndex,

id-LowerLayerPresenceStatusChange,

id-FastMCGRecovery-SN-to-MN,

id-FastMCGRecovery-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-PartialListIndicator,

id-MaximumCellListSize,

id-MessageOversizeNotification,

id-CellandCapacityAssistInfo,

id-TNLConfigurationInfo,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-UEContextReferenceatSourceNGRAN,

id-CHOinformation-REQ,

id-CHOinformation-ACK,

id-DAPSRequestInfo,

id-RequestedTargetCellID,

id-CandidateCellsToBeCancelledList,

id-DAPSResponseInfo,

id-ProcedureStage,

id-CHO-DC-Indicator,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands

FROM X2AP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause									PRESENCE mandatory}|

{ ID id-TargetCell-ID						CRITICALITY reject	TYPE ECGI									PRESENCE mandatory}|

{ ID id-GUMMEI-ID							CRITICALITY reject	TYPE GUMMEI									PRESENCE mandatory}|

{ ID id-UE-ContextInformation				CRITICALITY reject	TYPE UE-ContextInformation					PRESENCE mandatory}|

{ ID id-UE-HistoryInformation				CRITICALITY ignore	TYPE UE-HistoryInformation					PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation						PRESENCE optional}|

{ ID id-SRVCCOperationPossible				CRITICALITY ignore	TYPE SRVCCOperationPossible					PRESENCE optional}|

{ ID id-CSGMembershipStatus					CRITICALITY reject	TYPE CSGMembershipStatus					PRESENCE optional}|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation					PRESENCE optional}|

{ ID id-Masked-IMEISV						CRITICALITY ignore	TYPE Masked-IMEISV							PRESENCE optional}|

{ ID id-UE-HistoryInformationFromTheUE		CRITICALITY ignore	TYPE UE-HistoryInformationFromTheUE		PRESENCE optional}|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore	TYPE ExpectedUEBehaviour					PRESENCE optional}|

{ ID id-ProSeAuthorized						CRITICALITY ignore	TYPE ProSeAuthorized						PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSeNB			CRITICALITY ignore	TYPE UE-ContextReferenceAtSeNB			PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional}|

{ ID id-V2XServicesAuthorized				CRITICALITY ignore	TYPE V2XServicesAuthorized					PRESENCE optional}|

{ ID id-UE-ContextReferenceAtWT				CRITICALITY ignore	TYPE UE-ContextReferenceAtWT				PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities				PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSgNB			CRITICALITY ignore	TYPE UE-ContextReferenceAtSgNB			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation		CRITICALITY ignore	TYPE AerialUEsubscriptionInformation	PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo				PRESENCE optional}|

{ ID id-CHOinformation-REQ						CRITICALITY ignore	TYPE CHOinformation-REQ								PRESENCE optional},

...

}

UE-ContextInformation ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-List				E-RABs-ToBeSetup-List,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList		OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformation-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ManagementBasedMDTallowed			CRITICALITY ignore	EXTENSION ManagementBasedMDTallowed					PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore	EXTENSION MDTPLMNList									PRESENCE optional }|

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional }|

{ ID id-EPCHandoverRestrictionListContainer CRITICALITY ignore	EXTENSION EPCHandoverRestrictionListContainer		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional},

...

}

E-RABs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetup-ItemIEs} }

E-RABs-ToBeSetup-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeSetup-Item	 CRITICALITY ignore 	TYPE E-RABs-ToBeSetup-Item 	PRESENCE mandatory },

...

}

E-RABs-ToBeSetup-Item ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding													OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetup-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeSetup-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-BearerType		CRITICALITY reject	EXTENSION BearerType		PRESENCE optional}|

{ ID id-DAPSRequestInfo		CRITICALITY ignore	EXTENSION DAPSRequestInfo			PRESENCE optional},

...

}

MobilityInformation ::= BIT STRING (SIZE(32))

UE-ContextReferenceAtSeNB ::= SEQUENCE {

source-GlobalSeNB-ID		GlobalENB-ID,

seNB-UE-X2AP-ID				UE-X2AP-ID,

seNB-UE-X2AP-ID-Extension	UE-X2AP-ID-Extension,

iE-Extensions				ProtocolExtensionContainer { {UE-ContextReferenceAtSeNB-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtSeNB-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UE-ContextReferenceAtWT ::= SEQUENCE {

wTID					WTID,

wT-UE-XwAP-ID			WT-UE-XwAP-ID,

iE-Extensions			ProtocolExtensionContainer { {UE-ContextReferenceAtWT-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtWT-ItemExtIEs	X2AP-PROTOCOL-EXTENSION ::= {

...

}

UE-ContextReferenceAtSgNB ::= SEQUENCE {

source-GlobalSgNB-ID			GlobalGNB-ID,

sgNB-UE-X2AP-ID				SgNB-UE-X2AP-ID,

iE-Extensions				ProtocolExtensionContainer { {UE-ContextReferenceAtSgNB-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtSgNB-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-List						CRITICALITY ignore	TYPE E-RABs-Admitted-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List		PRESENCE optional}|

{ ID id-TargeteNBtoSource-eNBTransparentContainer	CRITICALITY ignore	TYPE TargeteNBtoSource-eNBTransparentContainer	PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator						CRITICALITY ignore	TYPE UE-ContextKeptIndicator	PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}-- The id-SeNB-UE-X2AP-ID-Extension shall not be sent and shall be ignored, if received.--|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-WT-UE-ContextKeptIndicator					CRITICALITY ignore	TYPE UE-ContextKeptIndicator	PRESENCE optional}|

{ ID id-ERABs-transferred-to-MeNB					CRITICALITY ignore	TYPE E-RAB-List			PRESENCE optional}|

{ ID id-CHOinformation-ACK							CRITICALITY ignore	TYPE CHOinformation-ACK									PRESENCE optional},

...

}

E-RABs-Admitted-List 		::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ItemIEs} }

E-RABs-Admitted-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-Item 	PRESENCE mandatory	}

}

E-RABs-Admitted-Item ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

uL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

dL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-Item-ExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSResponseInfo				CRITICALITY reject	EXTENSION DAPSResponseInfo			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-RequestedTargetCellID			CRITICALITY reject	TYPE ECGI						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER REPORT

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverReport-IEs}},

...

}

HandoverReport-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType							CRITICALITY ignore	TYPE HandoverReportType									PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-SourceCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-FailureCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-Re-establishmentCellECGI					CRITICALITY ignore	TYPE ECGI													PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to “HO to Wrong Cell” -- |

{ ID id-TargetCellInUTRAN							CRITICALITY ignore	TYPE TargetCellInUTRAN									PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to "InterRAT ping-pong" --|

{ ID id-SourceCellCRNTI								CRITICALITY ignore	TYPE CRNTI													PRESENCE optional}|

{ ID id-MobilityInformation							CRITICALITY ignore	TYPE MobilityInformation									PRESENCE optional}|

{ ID id-UE-RLF-Report-Container						CRITICALITY ignore	TYPE UE-RLF-Report-Container							PRESENCE optional}|

{ ID id-UE-RLF-Report-Container-for-extended-bands	CRITICALITY ignore	TYPE UE-RLF-Report-Container-for-extended-bands	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EARLY STATUS TRANSFER

--

-- **************************************************************

EarlyStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ EarlyStatusTransfer-IEs}},

...

}

EarlyStatusTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject		TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject		TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject		TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY reject		TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-ProcedureStage						CRITICALITY reject		TYPE ProcedureStageChoice						PRESENCE mandatory},

...

}

ProcedureStageChoice ::= CHOICE {

first-dl-count						FirstDLCount,

dl-discarding						DLDiscarding,

choice-extension					ProtocolIE-Single-Container { {ProcedureStageChoice-ExtIEs} }

}

ProcedureStageChoice-ExtIEs X2AP-PROTOCOL-IES ::= {

...

}

FirstDLCount ::= SEQUENCE {

e-RABsSubjectToEarlyStatusTransfer		E-RABsSubjectToEarlyStatusTransfer-List,

iE-Extension								ProtocolExtensionContainer { {FirstDLCount-ExtIEs} }	OPTIONAL,

...

}

FirstDLCount-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DLDiscarding ::= SEQUENCE {

e-RABsSubjectToDLDiscarding-List			E-RABsSubjectToDLDiscarding-List,

iE-Extension								ProtocolExtensionContainer { {DLDiscarding-ExtIEs} }	OPTIONAL,

...

}

DLDiscarding-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToStatusTransfer-List		CRITICALITY ignore	TYPE E-RABs-SubjectToStatusTransfer-List		PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE optional},

...

}

E-RABs-SubjectToStatusTransfer-List	::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToStatusTransfer-ItemIEs} }

E-RABs-SubjectToStatusTransfer-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToStatusTransfer-Item	CRITICALITY ignore	TYPE E-RABs-SubjectToStatusTransfer-Item 	PRESENCE mandatory	}

}

E-RABs-SubjectToStatusTransfer-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

receiveStatusofULPDCPSDUs				ReceiveStatusofULPDCPSDUs			OPTIONAL,

uL-COUNTvalue					COUNTvalue,

dL-COUNTvalue					COUNTvalue,

iE-Extensions							ProtocolExtensionContainer { {E-RABs-SubjectToStatusTransfer-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToStatusTransfer-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ReceiveStatusOfULPDCPSDUsExtended			CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsExtended			PRESENCE optional}|

{ ID id-ULCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended								PRESENCE optional}|

{ ID id-DLCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended								PRESENCE optional}|

{ ID id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	PRESENCE optional}|

{ ID id-ULCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18						PRESENCE optional}|

{ ID id-DLCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SIPTO-BearerDeactivationIndication	CRITICALITY ignore	TYPE SIPTOBearerDeactivationIndication		PRESENCE optional}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER SUCCESS

--

-- **************************************************************

HandoverSuccess ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverSuccess-IEs}},

...

}

HandoverSuccess-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY	reject TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-TargetCell-ID						CRITICALITY reject	TYPE ECGI						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER CANCEL

--

-- **************************************************************

ConditionalHandoverCancel ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ConditionalHandoverCancel-IEs}},

...

}

ConditionalHandoverCancel-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-CandidateCellsToBeCancelledList		CRITICALITY reject	TYPE CandidateCellsToBeCancelledList			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause								PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-Old-SgNB-UE-X2AP-ID					CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID					PRESENCE optional}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP REQUEST

--

-- **************************************************************

X2SetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupRequest-IEs}},

...

}

X2SetupRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID			CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-ServedCells				CRITICALITY reject	TYPE ServedCells			PRESENCE mandatory}|

{ ID id-GUGroupIDList			CRITICALITY reject	TYPE GUGroupIDList			PRESENCE optional}|

{ ID id-LHN-ID					CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP RESPONSE

--

-- **************************************************************

X2SetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupResponse-IEs}},

...

}

X2SetupResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-ServedCells					CRITICALITY reject	TYPE ServedCells			PRESENCE mandatory}|

{ ID id-GUGroupIDList				CRITICALITY reject	TYPE GUGroupIDList			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP FAILURE

--

-- **************************************************************

X2SetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupFailure-IEs}},

...

}

X2SetupFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause					CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-TimeToWait				CRITICALITY ignore		TYPE TimeToWait							PRESENCE optional} |

{ ID id-CriticalityDiagnostics	CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- LOAD INFORMATION

--

-- **************************************************************

LoadInformation ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{LoadInformation-IEs}},

...

}

LoadInformation-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellInformation				CRITICALITY ignore	TYPE CellInformation-List		PRESENCE mandatory} ,

...

}

CellInformation-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellInformation-ItemIEs} }

CellInformation-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellInformation-Item	CRITICALITY ignore	TYPE CellInformation-Item 	PRESENCE mandatory	}

}

CellInformation-Item ::= SEQUENCE {

cell-ID							ECGI,

ul-InterferenceOverloadIndication		UL-InterferenceOverloadIndication		OPTIONAL,

ul-HighInterferenceIndicationInfo		UL-HighInterferenceIndicationInfo		OPTIONAL,

relativeNarrowbandTxPower				RelativeNarrowbandTxPower				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {CellInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CellInformation-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ABSInformation						CRITICALITY ignore	EXTENSION ABSInformation							PRESENCE optional }|

{ ID id-InvokeIndication					CRITICALITY ignore	EXTENSION InvokeIndication							PRESENCE optional }|

{ ID id-IntendedULDLConfiguration			CRITICALITY ignore	EXTENSION SubframeAssignment						PRESENCE optional }|

{ ID id-ExtendedULInterferenceOverloadInfo	CRITICALITY ignore	EXTENSION ExtendedULInterferenceOverloadInfo	PRESENCE optional }|

{ ID id-CoMPInformation						CRITICALITY ignore	EXTENSION CoMPInformation							PRESENCE optional }|

{ ID id-DynamicDLTransmissionInformation	CRITICALITY ignore	EXTENSION DynamicDLTransmissionInformation		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE

--

-- **************************************************************

ENBConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdate-IEs}},

...

}

ENBConfigurationUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToAdd			CRITICALITY reject	TYPE ServedCells					PRESENCE optional}|

{ ID id-ServedCellsToModify			CRITICALITY reject	TYPE ServedCellsToModify			PRESENCE optional}|

{ ID id-ServedCellsToDelete			CRITICALITY reject	TYPE Old-ECGIs						PRESENCE optional}|

{ ID id-GUGroupIDToAddList			CRITICALITY reject	TYPE GUGroupIDList					PRESENCE optional}|

{ ID id-GUGroupIDToDeleteList		CRITICALITY reject	TYPE GUGroupIDList					PRESENCE optional}|

{ ID id-CoverageModificationList	CRITICALITY reject	TYPE CoverageModificationList		PRESENCE optional},

...

}

ServedCellsToModify::= SEQUENCE (SIZE (1..maxCellineNB)) OF ServedCellsToModify-Item

ServedCellsToModify-Item::= SEQUENCE {

old-ecgi						ECGI,

servedCellInfo					ServedCell-Information,

neighbour-Info					Neighbour-Information			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToModify-Item-ExtIEs} } OPTIONAL,

...

}

ServedCellsToModify-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-DeactivationIndication			CRITICALITY ignore	EXTENSION DeactivationIndication				PRESENCE optional }|

{ ID id-NRNeighbourInfoToModify			CRITICALITY ignore	EXTENSION NRNeighbour-Information				PRESENCE optional },

...

}

Old-ECGIs::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

ENBConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdateAcknowledge-IEs}},

...

}

ENBConfigurationUpdateAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE FAIURE

--

-- **************************************************************

ENBConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdateFailure-IEs}},

...

}

ENBConfigurationUpdateFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID			CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|-- The IE shall be present if the Registration Request IE is set to “Stop”, “Partial stop” or to “Add”--

{ ID id-Registration-Request		CRITICALITY reject	TYPE Registration-Request			PRESENCE mandatory}|

{ ID id-ReportCharacteristics		CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE optional}|

{ ID id-CellToReport				CRITICALITY ignore	TYPE CellToReport-List				PRESENCE mandatory}|

{ ID id-ReportingPeriodicity		CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional}|

{ ID id-PartialSuccessIndicator		CRITICALITY ignore	TYPE PartialSuccessIndicator		PRESENCE optional}|

{ ID id-ReportingPeriodicityRSRPMR	CRITICALITY ignore	TYPE ReportingPeriodicityRSRPMR		PRESENCE optional}|

{ ID id-ReportingPeriodicityCSIR	CRITICALITY ignore	TYPE ReportingPeriodicityCSIR		PRESENCE optional},

...

}

CellToReport-List 		::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellToReport-ItemIEs} }

CellToReport-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellToReport-Item	CRITICALITY ignore	TYPE CellToReport-Item 	PRESENCE mandatory}

}

CellToReport-Item ::= SEQUENCE {

cell-ID									ECGI,

iE-Extensions							ProtocolExtensionContainer { {CellToReport-Item-ExtIEs} } OPTIONAL,

...

}

CellToReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ReportingPeriodicity ::= ENUMERATED {

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

PartialSuccessIndicator ::= ENUMERATED {

partial-success-allowed,

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS RESPONSE

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusResponse-IEs}},

...

}

ResourceStatusResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional}|

{ ID id-MeasurementInitiationResult-List	CRITICALITY ignore	TYPE MeasurementInitiationResult-List	PRESENCE optional},

...

}

MeasurementInitiationResult-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {MeasurementInitiationResult-ItemIEs} }

MeasurementInitiationResult-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeasurementInitiationResult-Item	CRITICALITY ignore	TYPE MeasurementInitiationResult-Item	PRESENCE mandatory}

}

MeasurementInitiationResult-Item ::= SEQUENCE {

cell-ID											ECGI,

measurementFailureCause-List					MeasurementFailureCause-List	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {MeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

MeasurementInitiationResult-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementFailureCause-List ::= SEQUENCE (SIZE (1..maxFailedMeasObjects)) OF ProtocolIE-Single-Container { {MeasurementFailureCause-ItemIEs} }

MeasurementFailureCause-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeasurementFailureCause-Item	CRITICALITY ignore	TYPE MeasurementFailureCause-Item		PRESENCE mandatory}

}

MeasurementFailureCause-Item ::= SEQUENCE {

measurementFailedReportCharacteristics			ReportCharacteristics,

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { {MeasurementFailureCause-Item-ExtIEs} }	OPTIONAL,

...

}

MeasurementFailureCause-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS FAILURE

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusFailure-IEs}},

...

}

ResourceStatusFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID						CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID						CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-CompleteFailureCauseInformation-List	CRITICALITY ignore	TYPE CompleteFailureCauseInformation-List	PRESENCE optional},

...

}

CompleteFailureCauseInformation-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CompleteFailureCauseInformation-ItemIEs} }

CompleteFailureCauseInformation-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CompleteFailureCauseInformation-Item	CRITICALITY ignore	TYPE CompleteFailureCauseInformation-Item	PRESENCE mandatory}

}

CompleteFailureCauseInformation-Item ::= SEQUENCE {

cell-ID											ECGI,

measurementFailureCause-List					MeasurementFailureCause-List,

iE-Extensions									ProtocolExtensionContainer { {CompleteFailureCauseInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CompleteFailureCauseInformation-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-CellMeasurementResult	CRITICALITY ignore	TYPE CellMeasurementResult-List		PRESENCE mandatory},

...

}

CellMeasurementResult-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellMeasurementResult-ItemIEs} }

CellMeasurementResult-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellMeasurementResult-Item	CRITICALITY ignore	TYPE CellMeasurementResult-Item	PRESENCE mandatory}

}

CellMeasurementResult-Item ::= SEQUENCE {

cell-ID						ECGI,

hWLoadIndicator				HWLoadIndicator		OPTIONAL,

s1TNLLoadIndicator			S1TNLLoadIndicator	OPTIONAL,

radioResourceStatus			RadioResourceStatus	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacityGroup	CRITICALITY ignore	EXTENSION CompositeAvailableCapacityGroup		PRESENCE optional}|

{ ID id-ABS-Status						CRITICALITY ignore	EXTENSION ABS-Status								PRESENCE optional}|

{ ID id-RSRPMRList						CRITICALITY ignore	EXTENSION RSRPMRList								PRESENCE optional}|

{ ID id-CSIReportList					CRITICALITY ignore	EXTENSION CSIReportList								PRESENCE optional}|

{ ID id-CellReportingIndicator			CRITICALITY ignore	EXTENSION CellReportingIndicator					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs X2AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE REQUEST

--

-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeRequest-IEs}},

...

}

MobilityChangeRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID						CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID						CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-ENB1-Mobility-Parameters			CRITICALITY ignore	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-ENB2-Proposed-Mobility-Parameters	CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE ACKNOWLEDGE

--

-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeAcknowledge-IEs}},

...

}

MobilityChangeAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID			CRITICALITY reject	TYPE ECGI							PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID			CRITICALITY reject	TYPE ECGI							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE FAILURE

--

-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeFailure-IEs}},

...

}

MobilityChangeFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID									CRITICALITY ignore	TYPE ECGI												PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID									CRITICALITY ignore	TYPE ECGI												PRESENCE mandatory}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-ENB2-Mobility-Parameters-Modification-Range		CRITICALITY ignore	TYPE MobilityParametersModificationRange		PRESENCE optional}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RADIO LINK FAILURE INDICATION

--

-- **************************************************************

RLFIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{RLFIndication-IEs}},

...

}

RLFIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-FailureCellPCI								CRITICALITY ignore	TYPE PCI														PRESENCE mandatory}|

{ ID id-Re-establishmentCellECGI					CRITICALITY ignore	TYPE ECGI														PRESENCE mandatory}|

{ ID id-FailureCellCRNTI							CRITICALITY ignore	TYPE CRNTI														PRESENCE mandatory}|

{ ID id-ShortMAC-I									CRITICALITY ignore	TYPE ShortMAC-I													PRESENCE optional}|

{ ID id-UE-RLF-Report-Container						CRITICALITY ignore	TYPE UE-RLF-Report-Container								PRESENCE optional}|

{ ID id-RRCConnSetupIndicator						CRITICALITY reject	TYPE RRCConnSetupIndicator									PRESENCE optional}|

{ ID id-RRCConnReestabIndicator						CRITICALITY ignore	TYPE RRCConnReestabIndicator								PRESENCE optional}|

{ ID id-UE-RLF-Report-Container-for-extended-bands	CRITICALITY ignore	TYPE UE-RLF-Report-Container-for-extended-bands		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate 	CRITICALITY reject	TYPE ServedCellsToActivate		PRESENCE mandatory},

...

}

ServedCellsToActivate::= SEQUENCE (SIZE (1..maxCellineNB)) OF ServedCellsToActivate-Item

ServedCellsToActivate-Item::= SEQUENCE {

ecgi						ECGI,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToActivate-Item-ExtIEs} } OPTIONAL,

...

}

ServedCellsToActivate-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivatedCellList		CRITICALITY ignore	TYPE ActivatedCellList				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

ActivatedCellList ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ActivatedCellList-Item

ActivatedCellList-Item::= SEQUENCE {

ecgi								ECGI,

iE-Extensions						ProtocolExtensionContainer { {ActivatedCellList-Item-ExtIEs} } OPTIONAL,

...

}

ActivatedCellList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

--**************************************************************

--

#### -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- X2 RELEASE

--

-- **************************************************************

X2Release ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2Release-IEs}},

...

}

X2Release-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID			CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- X2AP MESSAGE TRANSFER

--

-- **************************************************************

X2APMessageTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2APMessageTransfer-IEs}},

...

}

X2APMessageTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RNL-Header	CRITICALITY reject	TYPE RNL-Header				PRESENCE mandatory}|

{ ID id-x2APMessage	CRITICALITY reject	TYPE X2AP-Message			PRESENCE optional},

...

}

RNL-Header ::= SEQUENCE {

source-GlobalENB-ID	GlobalENB-ID,

target-GlobalENB-ID	GlobalENB-ID	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {RNL-Header-Item-ExtIEs} } OPTIONAL,

...

}

RNL-Header-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

X2AP-Message ::= OCTET STRING

-- **************************************************************

--

#### -- SENB ADDITION REQUEST

--

-- **************************************************************

SeNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequest-IEs}},

...

}

SeNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-UE-SecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE conditional}|

-- This IE shall be present if the Bearer Option IE is set to the value “SCG bearer” --

{ ID id-SeNBSecurityKey					CRITICALITY reject	TYPE SeNBSecurityKey					PRESENCE conditional}|

-- This IE shall be present if the Bearer Option IE is set to the value “SCG bearer” --

{ ID id-SeNBUEAggregateMaximumBitRate	CRITICALITY reject	TYPE UEAggregateMaximumBitRate			PRESENCE mandatory}|

{ ID id-ServingPLMN						CRITICALITY ignore	TYPE PLMN-Identity						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-List			CRITICALITY reject	TYPE E-RABs-ToBeAdded-List				PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer				CRITICALITY reject	TYPE MeNBtoSeNBContainer				PRESENCE mandatory}|

{ ID id-CSGMembershipStatus				CRITICALITY reject	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional},

...

}

E-RABs-ToBeAdded-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-ItemIEs} }

E-RABs-ToBeAdded-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-Item	CRITICALITY reject	TYPE E-RABs-ToBeAdded-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-Item ::= CHOICE {

sCG-Bearer		E-RABs-ToBeAdded-Item-SCG-Bearer,

split-Bearer	E-RABs-ToBeAdded-Item-Split-Bearer,

...

}

E-RABs-ToBeAdded-Item-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding																	OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-Item-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-Item-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID				CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID		CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-BearerType					CRITICALITY ignore	EXTENSION BearerType			PRESENCE optional},

...

}

E-RABs-ToBeAdded-Item-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-Item-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-Item-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SeNBAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequestAcknowledge-IEs}},

...

}

SeNBAdditionRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-List		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List				CRITICALITY ignore	TYPE E-RAB-List							PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer					CRITICALITY reject	TYPE SeNBtoMeNBContainer				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-GW-TransportLayerAddress			CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-Tunnel-Information-for-BBF			CRITICALITY ignore	TYPE TunnelInformation					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-List ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-ItemIEs} }

E-RABs-Admitted-ToBeAdded-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-Item 		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-Item ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeAdded-Item-Split-Bearer,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB ADDITION REQUEST REJECT

--

-- **************************************************************

SeNBAdditionRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequestReject-IEs}},

...

}

SeNBAdditionRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RECONFIGURATION COMPLETE

--

-- **************************************************************

SeNBReconfigurationComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReconfigurationComplete-IEs}},

...

}

SeNBReconfigurationComplete-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-ResponseInformationSeNBReconfComp		CRITICALITY ignore	TYPE ResponseInformationSeNBReconfComp	PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

ResponseInformationSeNBReconfComp ::= CHOICE {

success				ResponseInformationSeNBReconfComp-SuccessItem,

reject-by-MeNB		ResponseInformationSeNBReconfComp-RejectByMeNBItem,

...

}

ResponseInformationSeNBReconfComp-SuccessItem ::= SEQUENCE {

meNBtoSeNBContainer				MeNBtoSeNBContainer OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSeNBReconfComp-SuccessItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSeNBReconfComp-SuccessItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResponseInformationSeNBReconfComp-RejectByMeNBItem ::= SEQUENCE {

cause							Cause,

meNBtoSeNBContainer				MeNBtoSeNBContainer																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSeNBReconfComp-RejectByMeNBItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSeNBReconfComp-RejectByMeNBItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST

--

-- **************************************************************

SeNBModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ SeNBModificationRequest-IEs}},

...

}

SeNBModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause									PRESENCE mandatory}|

{ ID id-SCGChangeIndication					CRITICALITY ignore	TYPE SCGChangeIndication					PRESENCE optional}|

{ ID id-ServingPLMN							CRITICALITY ignore	TYPE PLMN-Identity							PRESENCE optional}|

{ ID id-UE-ContextInformationSeNBModReq		CRITICALITY reject	TYPE UE-ContextInformationSeNBModReq	PRESENCE optional}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer					PRESENCE optional}|

{ ID id-CSGMembershipStatus					CRITICALITY reject	TYPE CSGMembershipStatus					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

UE-ContextInformationSeNBModReq ::= SEQUENCE {

uE-SecurityCapabilities			UESecurityCapabilities															OPTIONAL,

seNB-SecurityKey				SeNBSecurityKey 																OPTIONAL,

seNBUEAggregateMaximumBitRate	UEAggregateMaximumBitRate														OPTIONAL,

e-RABs-ToBeAdded				E-RABs-ToBeAdded-List-ModReq													OPTIONAL,

e-RABs-ToBeModified				E-RABs-ToBeModified-List-ModReq													OPTIONAL,

e-RABs-ToBeReleased				E-RABs-ToBeReleased-List-ModReq													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UE-ContextInformationSeNBModReqExtIEs} }		OPTIONAL,

...

}

UE-ContextInformationSeNBModReqExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-ModReqItemIEs} }

E-RABs-ToBeAdded-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeAdded-ModReqItem	PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeAdded-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeAdded-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeAdded-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding																	OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID				CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID		CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-BearerType					CRITICALITY ignore	EXTENSION BearerType			PRESENCE optional},

...

}

E-RABs-ToBeAdded-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-ModReqItemIEs} }

E-RABs-ToBeModified-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeModified-ModReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeModified-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeModified-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeModified-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeModified-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters														OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeModified-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters																OPTIONAL,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint																		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeModified-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-ModReqItemIEs} }

E-RABs-ToBeReleased-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeReleased-ModReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SeNBModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationRequestAcknowledge-IEs}},

...

}

SeNBModificationRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-ModAckList		PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeModified-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-ModAckList	PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeReleased-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-ModAckList	PRESENCE optional}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List											PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer							CRITICALITY ignore	TYPE SeNBtoMeNBContainer								PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-ModAckItemIEs} }

E-RABs-Admitted-ToBeAdded-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-ModAckItem	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-ModAckItem		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeAdded-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeModified-ModAckItemIEs} }

E-RABs-Admitted-ToBeModified-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeModified-ModAckItem		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-ModAckItem	PRESENCE mandatory}

}

E-RABs-Admitted-ToBeModified-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeModified-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeModified-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

s1-DL-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-ModAckItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-ModAckItemIEs} }

E-RABs-Admitted-ToBeReleased-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-ModAckItem		CRITICALITY ignore	TYPE E-RABs-Admitted-ToReleased-ModAckItem		PRESENCE mandatory}

}

E-RABs-Admitted-ToReleased-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeReleased-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST REJECT

--

-- **************************************************************

SeNBModificationRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationRequestReject-IEs}},

...

}

SeNBModificationRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUIRED

--

-- **************************************************************

SeNBModificationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBModificationRequired-IEs}},

...

}

SeNBModificationRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-SCGChangeIndication					CRITICALITY ignore	TYPE SCGChangeIndication				PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-ModReqd			CRITICALITY ignore	TYPE E-RABs-ToBeReleased-ModReqd		PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer					CRITICALITY ignore	TYPE SeNBtoMeNBContainer				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional},

...

}

E-RABs-ToBeReleased-ModReqd ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-ModReqdItemIEs} }

E-RABs-ToBeReleased-ModReqdItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-ModReqdItem	 CRITICALITY ignore		TYPE E-RABs-ToBeReleased-ModReqdItem	PRESENCE mandatory },

...

}

E-RABs-ToBeReleased-ModReqdItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqdItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqdItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION CONFIRM

--

-- **************************************************************

SeNBModificationConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationConfirm-IEs}},

...

}

SeNBModificationConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer			PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension			PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REFUSE

--

-- **************************************************************

SeNBModificationRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBModificationRefuse-IEs}},

...

}

SeNBModificationRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer		PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RELEASE REQUEST

--

-- **************************************************************

SeNBReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBReleaseRequest-IEs}},

...

}

SeNBReleaseRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-List-RelReq		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-List-RelReq		PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator				CRITICALITY ignore	TYPE UE-ContextKeptIndicator					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-MakeBeforeBreakIndicator			CRITICALITY ignore	TYPE MakeBeforeBreakIndicator					PRESENCE optional},

...

}

E-RABs-ToBeReleased-List-RelReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-RelReqItemIEs} }

E-RABs-ToBeReleased-RelReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-RelReqItem		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-RelReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-RelReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-RelReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-RelReqItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-RelReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-RelReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB RELEASE REQUIRED

--

-- **************************************************************

SeNBReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReleaseRequired-IEs}},

...

}

SeNBReleaseRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RELEASE CONFIRM

--

-- **************************************************************

SeNBReleaseConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReleaseConfirm-IEs}},

...

}

SeNBReleaseConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-List-RelConf		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-List-RelConf		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

E-RABs-ToBeReleased-List-RelConf ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-RelConfItemIEs} }

E-RABs-ToBeReleased-RelConfItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-RelConfItem		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-RelConfItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-RelConfItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-RelConfItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-RelConfItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-RelConfItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelConfItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelConfItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-RelConfItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelConfItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelConfItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB COUNTER CHECK REQUEST

--

-- **************************************************************

SeNBCounterCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBCounterCheckRequest-IEs}},

...

}

SeNBCounterCheckRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToCounterCheck-List	CRITICALITY ignore	TYPE E-RABs-SubjectToCounterCheck-List		PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

E-RABs-SubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToCounterCheckItemIEs} }

E-RABs-SubjectToCounterCheckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToCounterCheckItem	CRITICALITY ignore	TYPE E-RABs-SubjectToCounterCheckItem		PRESENCE mandatory},

...

}

E-RABs-SubjectToCounterCheckItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Count						INTEGER (0..4294967295),

dL-Count						INTEGER (0..4294967295),

iE-Extensions					ProtocolExtensionContainer { {E-RABs-SubjectToCounterCheckItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToCounterCheckItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- X2 REMOVAL REQUEST

--

-- **************************************************************

X2RemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalRequest-IEs}},

...

}

X2RemovalRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-X2RemovalThreshold			CRITICALITY reject	TYPE X2BenefitValue			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 REMOVAL RESPONSE

--

-- **************************************************************

X2RemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalResponse-IEs}},

...

}

X2RemovalResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 REMOVAL FAILURE

--

-- **************************************************************

X2RemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalFailure-IEs}},

...

}

X2RemovalFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-resumeID							CRITICALITY reject	TYPE ResumeID				PRESENCE mandatory}|

{ ID id-ShortMAC-I							CRITICALITY reject	TYPE ShortMAC-I				PRESENCE mandatory}|

{ ID id-NewEUTRANCellIdentifier				CRITICALITY reject	TYPE EUTRANCellIdentifier	PRESENCE mandatory}|

{ID id-FailureCellCRNTI						CRITICALITY reject	TYPE CRNTI					PRESENCE optional}|

{ID id-FailureCellPCI						CRITICALITY reject	TYPE PCI					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-GUMMEI-ID									CRITICALITY reject	TYPE GUMMEI													PRESENCE mandatory}|

{ ID id-UE-ContextInformationRetrieve				CRITICALITY reject	TYPE UE-ContextInformationRetrieve					PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible								PRESENCE optional}|

{ ID id-Masked-IMEISV								CRITICALITY ignore	TYPE Masked-IMEISV											PRESENCE optional}|

{ ID id-ExpectedUEBehaviour							CRITICALITY ignore	TYPE ExpectedUEBehaviour									PRESENCE optional}|

{ ID id-ProSeAuthorized								CRITICALITY ignore	TYPE ProSeAuthorized										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional}|

{ ID id-V2XServicesAuthorized						CRITICALITY ignore	TYPE V2XServicesAuthorized								PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation					PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo	CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo	PRESENCE optional},

...

}

UE-ContextInformationRetrieve ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-ListRetrieve		E-RABs-ToBeSetup-ListRetrieve,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList			OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

managBasedMDTallowed				ManagementBasedMDTallowed		OPTIONAL,

managBasedMDTPLMNList				MDTPLMNList						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformationRetrieve-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformationRetrieve-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional }|

{ ID id-EPCHandoverRestrictionListContainer CRITICALITY ignore	EXTENSION EPCHandoverRestrictionListContainer		PRESENCE optional },

...

}

E-RABs-ToBeSetup-ListRetrieve ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetupRetrieve-ItemIEs} }

E-RABs-ToBeSetupRetrieve-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeSetupRetrieve-Item	CRITICALITY ignore	TYPE E-RABs-ToBeSetupRetrieve-Item	PRESENCE mandatory},

...

}

E-RABs-ToBeSetupRetrieve-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

bearerType						BearerType	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetupRetrieve-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeSetupRetrieve-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uL-GTPtunnelEndpoint		CRITICALITY reject	EXTENSION GTPtunnelEndpoint		PRESENCE mandatory}|

{ ID id-dL-Forwarding				CRITICALITY ignore	EXTENSION DL-Forwarding			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST

--

-- **************************************************************

SgNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container {{SgNBAdditionRequest-IEs}},

...

}

SgNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-NRUESecurityCapabilities					CRITICALITY reject	TYPE NRUESecurityCapabilities					PRESENCE mandatory}|

{ ID id-SgNBSecurityKey							CRITICALITY reject	TYPE SgNBSecurityKey									PRESENCE mandatory}|

{ ID id-SgNBUEAggregateMaximumBitRate			CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-SelectedPLMN								CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-SgNBAddReqList			CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReqList			PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY reject	TYPE SplitSRBs										PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-SGNB-Addition-Trigger-Ind				CRITICALITY reject	TYPE SGNB-Addition-Trigger-Ind					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP				CRITICALITY ignore	TYPE SubscriberProfileIDforRFP					PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore	TYPE DesiredActNotificationLevel				PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional}|

{ ID id-LocationInformationSgNBReporting			CRITICALITY ignore	TYPE LocationInformationSgNBReporting			PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV									PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex					PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore	TYPE	 RequestedFastMCGRecoveryViaSRB3			PRESENCE optional}|

{ ID id-UEContextReferenceatSourceNGRAN			CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBAddReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBAddReq-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBAddReq-Item		CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID								DRB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters				E-RAB-Level-QoS-Parameters,

max-MCG-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE --

dL-Forwarding								DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG						GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint						GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional}|

{ ID id-BearerType							CRITICALITY ignore	EXTENSION BearerType				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint	OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration	OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs} }		 OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-duplicationActivation					CRITICALITY ignore	EXTENSION DuplicationActivation		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBAdditionRequestAcknowledge-IEs}},

...

}

SgNBAdditionRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY reject	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY reject	TYPE SgNB-UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList		PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List													PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer							CRITICALITY reject	TYPE SgNBtoMeNBContainer										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY reject	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-AdmittedSplitSRBs							CRITICALITY reject	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation			CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation				PRESENCE optional}|

{ ID id-RRCConfigIndication							CRITICALITY reject	TYPE RRC-Config-Ind												PRESENCE optional}|

{ ID id-LocationInformationSgNB						CRITICALITY ignore	TYPE LocationInformationSgNB								PRESENCE optional}|

{ ID id-AvailableFastMCGRecoveryViaSRB3				CRITICALITY ignore	TYPE AvailableFastMCGRecoveryViaSRB3						PRESENCE optional },

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-ItemIEs} }

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item 		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReqAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReqAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																						OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

rlc-Mode								RLCMode																									OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																						OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																						OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																				OPTIONAL,

-- This IE shall be present if MCG resource and SCG resource IEs in the EN-DC Resource Configuration IE are set to “present” and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE --

uL-Configuration							ULConfiguration																							OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions							ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresentExtIEs} }			OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG				GTPtunnelEndpoint,

secondary-sgNB-DL-GTP-TEIDatSCG		GTPtunnelEndpoint		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresentExtIEs} } 		OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-lCID				CRITICALITY ignore	EXTENSION LCID			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST REJECT

--

-- **************************************************************

SgNBAdditionRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBAdditionRequestReject-IEs}},

...

}

SgNBAdditionRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY reject	TYPE SgNB-UE-X2AP-ID			PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RECONFIGURATION COMPLETE

--

-- **************************************************************

SgNBReconfigurationComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReconfigurationComplete-IEs}},

...

}

SgNBReconfigurationComplete-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-ResponseInformationSgNBReconfComp		CRITICALITY ignore	TYPE ResponseInformationSgNBReconfComp			PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional},

...

}

ResponseInformationSgNBReconfComp ::= CHOICE {

success-SgNBReconfComp				ResponseInformationSgNBReconfComp-SuccessItem,

reject-by-MeNB-SgNBReconfComp		ResponseInformationSgNBReconfComp-RejectByMeNBItem,

...

}

ResponseInformationSgNBReconfComp-SuccessItem ::= SEQUENCE {

meNBtoSgNBContainer				MeNBtoSgNBContainer					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSgNBReconfComp-SuccessItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSgNBReconfComp-SuccessItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResponseInformationSgNBReconfComp-RejectByMeNBItem ::= SEQUENCE {

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSgNBReconfComp-RejectByMeNBItemExtIEs} }		OPTIONAL,

...

}

ResponseInformationSgNBReconfComp-RejectByMeNBItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST

--

-- **************************************************************

SgNBModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ SgNBModificationRequest-IEs}},

...

}

SgNBModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-SelectedPLMN							CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList							PRESENCE optional}|

{ ID	id-SCGConfigurationQuery				CRITICALITY ignore	TYPE SCGConfigurationQuery							PRESENCE optional}|

{ ID id-UE-ContextInformation-SgNBModReq		CRITICALITY reject	TYPE UE-ContextInformation-SgNBModReq				PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation			PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY ignore	TYPE SplitSRBs											PRESENCE optional}|

{ ID id-RequestedSplitSRBsrelease				CRITICALITY ignore	TYPE SplitSRBs											PRESENCE optional}|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore	TYPE DesiredActNotificationLevel					PRESENCE optional}|

{ ID id-LocationInformationSgNBReporting		CRITICALITY ignore	TYPE LocationInformationSgNBReporting				PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY ignore	TYPE ECGI												PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore	TYPE RequestedFastMCGRecoveryViaSRB3				PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore	TYPE RequestedFastMCGRecoveryViaSRB3Release		PRESENCE optional},

...

}

UE-ContextInformation-SgNBModReq ::= SEQUENCE {

nRUE-SecurityCapabilities		NRUESecurityCapabilities															OPTIONAL,

sgNB-SecurityKey				SgNBSecurityKey 																	OPTIONAL,

sgNBUEAggregateMaximumBitRate	UEAggregateMaximumBitRate															OPTIONAL,

e-RABs-ToBeAdded				E-RABs-ToBeAdded-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeModified				E-RABs-ToBeModified-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeReleased				E-RABs-ToBeReleased-SgNBModReq-List													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UE-ContextInformationSgNBModReqExtIEs} }			OPTIONAL,

...

}

UE-ContextInformationSgNBModReqExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{	ID id-SubscriberProfileIDforRFP					CRITICALITY ignore	EXTENSION SubscriberProfileIDforRFP									PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional}|

{ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	EXTENSION LowerLayerPresenceStatusChange				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBModReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBModReq-Item		CRITICALITY ignore	TYPE E-RABs-ToBeAdded-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID									DRB-ID,

en-DC-ResourceConfiguration				EN-DC-ResourceConfiguration,

resource-configuration					CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE --

dL-Forwarding							DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional}|

{ ID id-BearerType							CRITICALITY ignore	EXTENSION BearerType				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint		OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration																OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-duplicationActivation					CRITICALITY ignore	EXTENSION DuplicationActivation		PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-SgNBModReq-ItemIEs} }

E-RABs-ToBeModified-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeModified-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeModified-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																		OPTIONAL,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation																				OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																				OPTIONAL,

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional },

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters																			OPTIONAL,

meNB-UL-GTP-TEIDatPDCP						GTPtunnelEndpoint																					OPTIONAL,

uL-Configuration								ULConfiguration																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-secondarymeNBULGTPTEIDatPDCP			CRITICALITY ignore	EXTENSION GTPtunnelEndpoint			PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBModReq-ItemIEs} }

E-RABs-ToBeReleased-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationRequestAcknowledge-IEs}},

...

}

SgNBModificationRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBModAckList			PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeModified-SgNBModAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-SgNBModAckList		PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBModAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBModAckList		PRESENCE optional}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List													PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer							CRITICALITY ignore	TYPE SgNBtoMeNBContainer										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation			CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation				PRESENCE optional}|

{ ID id-AdmittedSplitSRBs							CRITICALITY ignore	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-AdmittedSplitSRBsrelease						CRITICALITY ignore	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-RRCConfigIndication							CRITICALITY reject	TYPE RRC-Config-Ind												PRESENCE optional}|

{ ID id-LocationInformationSgNB						CRITICALITY ignore	TYPE LocationInformationSgNB								PRESENCE optional}|

{ ID id-AvailableFastMCGRecoveryViaSRB3				CRITICALITY ignore	TYPE AvailableFastMCGRecoveryViaSRB3						PRESENCE optional}|

{ ID id-ReleaseFastMCGRecoveryViaSRB3		CRITICALITY ignore	TYPE ReleaseFastMCGRecoveryViaSRB3				PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item 	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																					OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE are set to “present” --

rlc-Mode									RLCMode																								OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE are set to “present” --

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																					OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																					OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																			OPTIONAL,

-- This IE shall be present if MCG resource and SCG resource IEs in the EN-DC Resource Configuration IE are set to “present” and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE --

uL-Configuration						ULConfiguration																							OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG					GTPtunnelEndpoint,

secondary-sgNB-DL-GTP-TEIDatSCG			GTPtunnelEndpoint			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-lCID				CRITICALITY ignore	EXTENSION LCID		PRESENCE optional},

...

}

E-RABs-Admitted-ToBeModified-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeModified-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeModified-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint																						OPTIONAL,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																						OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																				OPTIONAL,

uL-Configuration							ULConfiguration																							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG					GTPtunnelEndpoint																								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-secondarysgNBDLGTPTEIDatPDCP			CRITICALITY ignore	EXTENSION GTPtunnelEndpoint				PRESENCE optional}|

{ ID id-RLC-Status								CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional },

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeReleased-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item		CRITICALITY ignore	TYPE E-RABs-Admitted-ToReleased-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToReleased-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST REJECT

--

-- **************************************************************

SgNBModificationRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationRequestReject-IEs}},

...

}

SgNBModificationRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUIRED

--

-- **************************************************************

SgNBModificationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBModificationRequired-IEs}},

...

}

SgNBModificationRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication						CRITICALITY ignore	TYPE PDCPChangeIndication							PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-SgNBModReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBModReqdList		PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer						CRITICALITY ignore	TYPE SgNBtoMeNBContainer								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-E-RABs-ToBeModified-SgNBModReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeModified-SgNBModReqdList		PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation		CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-RRCConfigIndication						CRITICALITY reject	TYPE RRC-Config-Ind									PRESENCE optional}|

{ ID id-LocationInformationSgNB					CRITICALITY ignore	TYPE LocationInformationSgNB						PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBModReqdList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBModReqd-ItemIEs} }

E-RABs-ToBeReleased-SgNBModReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBModReqd-Item	 CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBModReqd-Item		PRESENCE mandatory },

...

}

E-RABs-ToBeReleased-SgNBModReqd-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReqd-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReqdList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-SgNBModReqd-ItemIEs} }

E-RABs-ToBeModified-SgNBModReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-SgNBModReqd-Item	 CRITICALITY ignore		TYPE E-RABs-ToBeModified-SgNBModReqd-Item		PRESENCE mandatory },

...

}

E-RABs-ToBeModified-SgNBModReqd-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresent ::= SEQUENCE {

requested-MCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters													OPTIONAL,

uL-Configuration								ULConfiguration																OPTIONAL,

sgNB-UL-GTP-TEIDatPDCP						GTPtunnelEndpoint															OPTIONAL,

s1-DL-GTP-TEIDatSgNB							GTPtunnelEndpoint															OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-new-drb-ID-req							CRITICALITY ignore	EXTENSION NewDRBIDrequest				PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG				GTPtunnelEndpoint				OPTIONAL,

secondary-sgNB-DL-GTP-TEIDatSCG		GTPtunnelEndpoint				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional}|

{ ID id-lCID				CRITICALITY ignore			EXTENSION LCID						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION CONFIRM

--

-- **************************************************************

SgNBModificationConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationConfirm-IEs}},

...

}

SgNBModificationConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-E-RABs-AdmittedToBeModified-SgNBModConfList	CRITICALITY ignore	TYPE E-RABs-AdmittedToBeModified-SgNBModConfList		PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer							CRITICALITY ignore	TYPE MeNBtoSgNBContainer										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation			CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation				PRESENCE optional},

...

}

E-RABs-AdmittedToBeModified-SgNBModConfList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container

{ {E-RABs-AdmittedToBeModified-SgNBModConf-ItemIEs} }

E-RABs-AdmittedToBeModified-SgNBModConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-AdmittedToBeModified-SgNBModConf-Item	 CRITICALITY ignore	TYPE E-RABs-AdmittedToBeModified-SgNBModConf-Item	 PRESENCE mandatory },

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REFUSE

--

-- **************************************************************

SgNBModificationRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBModificationRefuse-IEs}},

...

}

SgNBModificationRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer					CRITICALITY ignore	TYPE MeNBtoSgNBContainer			PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST

--

-- **************************************************************

SgNBReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequest-IEs}},

...

}

SgNBReleaseRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBRelReqList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqList		PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator					CRITICALITY ignore	TYPE UE-ContextKeptIndicator						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE optional}|

{ ID id-ERABs-transferred-to-MeNB				CRITICALITY ignore	TYPE E-RAB-List										PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBRelReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelReq-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelReq-Item		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequestAcknowledge-IEs}},

...

}

SgNBReleaseRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID									CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID									CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-MeNB-UE-X2AP-ID-Extension						CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional }|

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList	PRESENCE optional },

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF

ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemIEs} }

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item		PRESENCE mandatory},

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

rlc-Mode-transferred			RLCMode,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST REJECT

--

-- **************************************************************

SgNBReleaseRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequestReject-IEs}},

...

}

SgNBReleaseRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUIRED

--

-- **************************************************************

SgNBReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReleaseRequired-IEs}},

...

}

SgNBReleaseRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional }|

{ ID id-E-RABs-ToBeReleased-SgNBRelReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqdList			PRESENCE optional }|

{ ID id-SgNBtoMeNBContainer						CRITICALITY ignore	TYPE SgNBtoMeNBContainer									PRESENCE optional },

...

}

E-RABs-ToBeReleased-SgNBRelReqdList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelReqd-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelReqd-Item		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqd-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelReqd-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

rlc-Mode-transferred			RLCMode,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReqd-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE CONFIRM

--

-- **************************************************************

SgNBReleaseConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReleaseConfirm-IEs}},

...

}

SgNBReleaseConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBRelConfList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelConfList		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBRelConfList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelConf-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelConf-Item		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBRelConf-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresentExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB COUNTER CHECK REQUEST

--

-- **************************************************************

SgNBCounterCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBCounterCheckRequest-IEs}},

...

}

SgNBCounterCheckRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY reject	TYPE SgNB-UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToSgNBCounterCheck-List		CRITICALITY ignore	TYPE E-RABs-SubjectToSgNBCounterCheck-List		PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-SubjectToSgNBCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToSgNBCounterCheck-ItemIEs} }

E-RABs-SubjectToSgNBCounterCheck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToSgNBCounterCheck-Item		CRITICALITY ignore	TYPE E-RABs-SubjectToSgNBCounterCheck-Item		PRESENCE mandatory},

...

}

E-RABs-SubjectToSgNBCounterCheck-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Count						INTEGER (0..4294967295),

dL-Count						INTEGER (0..4294967295),

iE-Extensions					ProtocolExtensionContainer { {E-RABs-SubjectToSgNBCounterCheck-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToSgNBCounterCheck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB CHANGE REQUIRED

--

-- **************************************************************

SgNBChangeRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBChangeRequired-IEs}},

...

}

SgNBChangeRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Target-SgNB-ID						CRITICALITY reject	TYPE GlobalGNB-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-SgNBtoMeNBContainer					CRITICALITY reject	TYPE SgNBtoMeNBContainer			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB CHANGE CONFIRM

--

-- **************************************************************

SgNBChangeConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBChangeConfirm-IEs}},

...

}

SgNBChangeConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBChaConfList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBChaConfList		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBChaConfList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBChaConf-ItemIEs} }

E-RABs-ToBeReleased-SgNBChaConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBChaConf-Item		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBChaConf-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresentExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RRC TRANSFER

--

-- **************************************************************

RRCTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{RRCTransfer-IEs}},

...

}

RRCTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-SplitSRB								CRITICALITY reject	TYPE SplitSRB					PRESENCE optional}|

{ ID id-NRUeReport							CRITICALITY reject	TYPE NRUeReport					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-FastMCGRecovery-SN-to-MN				CRITICALITY ignore 	TYPE FastMCGRecovery				PRESENCE optional}|

{ ID id-FastMCGRecovery-MN-to-SN				CRITICALITY ignore 	TYPE FastMCGRecovery				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB CHANGE REFUSE

--

-- **************************************************************

SgNBChangeRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBChangeRefuse-IEs}},

...

}

SgNBChangeRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP REQUEST

--

-- **************************************************************

ENDCX2SetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupRequest-IEs}},

...

}

ENDCX2SetupRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcX2Setup			CRITICALITY reject	TYPE InitiatingNodeType-EndcX2Setup		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional}|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional},

...

}

InitiatingNodeType-EndcX2Setup ::= CHOICE {

init-eNB				ProtocolIE-Container	{{ENB-ENDCX2SetupReqIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCX2SetupReqIEs}},

...

}

ENB-ENDCX2SetupReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory}|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication					PRESENCE optional }|

-- NOTE: In the current version of this specification the Interface Instance Indication IE is not included in the Initiating NodeType IE --

{ ID id-CellandCapacityAssistInfo				CRITICALITY ignore	TYPE CellandCapacityAssistInfo						PRESENCE optional },

...

}

ServedEUTRAcellsENDCX2ManagementList ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

servedEUTRACellInfo					ServedCell-Information,

nrNeighbourInfo						NRNeighbour-Information		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ServedEUTRAcellsENDCX2Management-ExtIEs} } OPTIONAL,

...

}

ServedEUTRAcellsENDCX2Management-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

En-gNB-ENDCX2SetupReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory}|

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE mandatory}|

{ ID id-PartialListIndicator						CRITICALITY ignore	TYPE PartialListIndicator								PRESENCE optional },

...

}

ServedNRcellsENDCX2ManagementList ::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF SEQUENCE {

servedNRCellInfo						ServedNRCell-Information,

nRNeighbourInfo						NRNeighbour-Information OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {En-gNBServedCells-ExtIEs} } OPTIONAL,

...

}

En-gNBServedCells-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedNRCell-Information ::= SEQUENCE {

nrpCI				NRPCI,

nrCellID			NRCGI,

fiveGS-TAC			FiveGS-TAC	OPTIONAL,

configured-TAC		TAC			OPTIONAL,

broadcastPLMNs		BroadcastPLMNs-Item,

nrModeInfo			CHOICE {

fdd		FDD-InfoServedNRCell-Information,

tdd		TDD-InfoServedNRCell-Information,

...

},

measurementTimingConfiguration	OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { {ServedNRCell-Information-ExtIEs} } OPTIONAL,

...

}

ServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-additionalPLMNs-Item			CRITICALITY ignore	EXTENSION AdditionalPLMNs-Item			PRESENCE optional}|

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore 	EXTENSION BPLMN-ID-Info-NR				PRESENCE optional},

...

}

FDD-InfoServedNRCell-Information ::= SEQUENCE {

ul-NRFreqInfo				NRFreqInfo,

dl-NRFreqInfo				NRFreqInfo,

ul-NR-TxBW				NR-TxBW,

dl-NR-TxBW				NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {FDD-InfoServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-InfoServedNRCell-Information ::= SEQUENCE {

nRFreqInfo				NRFreqInfo,

nR-TxBW					NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {TDD-InfoServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellandCapacityAssistInfo::= SEQUENCE {

maximumCellListSize								MaximumCellListSize			OPTIONAL,

cellAssistanceInformation						CellAssistanceInformation	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {CellandCapacityAssistInfo-ExtIEs} } OPTIONAL,

...

}

CellandCapacityAssistInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellAssistanceInformation ::= CHOICE {

limited-list				Limited-list,

full-list				ENUMERATED {allServedNRcells, ...},

...

}

Limited-list 	::= SEQUENCE (SIZE (1..maxCellinengNB)) OF SEQUENCE {

nrCellID			NRCGI,

iE-Extensions						ProtocolExtensionContainer { {Limited-list-ExtIEs} } OPTIONAL,

...

}

Limited-list-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP RESPONSE

--

-- **************************************************************

ENDCX2SetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupResponse-IEs}},

...

}

ENDCX2SetupResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcX2Setup		CRITICALITY reject	TYPE RespondingNodeType-EndcX2Setup		PRESENCE mandatory},

...

}

RespondingNodeType-EndcX2Setup ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCX2SetupReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCX2SetupReqAckIEs}},

...

}

ENB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory}|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication					PRESENCE optional }|

{ ID id-CellandCapacityAssistInfo				CRITICALITY ignore	TYPE CellandCapacityAssistInfo						PRESENCE optional },

...

}

En-gNB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory}|

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE mandatory}|

{ ID id-PartialListIndicator						CRITICALITY ignore	TYPE PartialListIndicator								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP FAILURE

--

-- **************************************************************

ENDCX2SetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupFailure-IEs}},

...

}

ENDCX2SetupFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional } |

{ ID id-TimeToWait						CRITICALITY ignore		TYPE TimeToWait							PRESENCE optional	} |

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 		TYPE InterfaceInstanceIndication		PRESENCE optional }|

{ ID id-MessageOversizeNotification		CRITICALITY ignore 		TYPE MessageOversizeNotification		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE

--

-- **************************************************************

ENDCConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdate-IEs}},

...

}

ENDCConfigurationUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcConfigUpdate			CRITICALITY reject	TYPE InitiatingNodeType-EndcConfigUpdate			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication						PRESENCE optional}|

{ ID id-TNLConfigurationInfo						CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional	}|

{ ID id-TNLA-To-Add-List								CRITICALITY reject	TYPE TNLA-To-Add-List										PRESENCE optional }|

{ ID id-TNLA-To-Remove-List							CRITICALITY reject	TYPE TNLA-To-Remove-List									PRESENCE optional }|

{ ID id-TNLA-To-Update-List							CRITICALITY reject	TYPE TNLA-To-Update-List									PRESENCE optional },

...

}

InitiatingNodeType-EndcConfigUpdate::= CHOICE {

init-eNB			ProtocolIE-Container	{{ENB-ENDCConfigUpdateIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCConfigUpdateIEs}},

...

}

ENB-ENDCConfigUpdateIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellAssistanceInformation				CRITICALITY reject	TYPE CellAssistanceInformation							PRESENCE optional }|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList			PRESENCE optional }|

{ ID id-ServedEUTRAcellsToModifyListENDCConfUpd	CRITICALITY reject	TYPE ServedEUTRAcellsToModifyListENDCConfUpd		PRESENCE optional }|

{ ID id-ServedEUTRAcellsToDeleteListENDCConfUpd	CRITICALITY reject	TYPE ServedEUTRAcellsToDeleteListENDCConfUpd		PRESENCE optional },

...

}

ServedEUTRAcellsToModifyListENDCConfUpd ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

old-ECGI								ECGI,

servedEUTRACellInfo					ServedCell-Information,

nrNeighbourInfo						NRNeighbour-Information		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ServedEUTRAcellsToModifyListENDCConfUpd-ExtIEs} } OPTIONAL,

...

}

ServedEUTRAcellsToModifyListENDCConfUpd-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedEUTRAcellsToDeleteListENDCConfUpd ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

En-gNB-ENDCConfigUpdateIEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE optional }|

{ ID id-ServedNRcellsToModifyListENDCConfUpd		CRITICALITY reject	TYPE ServedNRcellsToModifyENDCConfUpdList		PRESENCE optional }|

{ ID id-ServedNRcellsToDeleteListENDCConfUpd		CRITICALITY reject	TYPE ServedNRcellsToDeleteENDCConfUpdList		PRESENCE optional },

...

}

ServedNRcellsToModifyENDCConfUpdList ::= SEQUENCE (SIZE (1..maxCellinengNB)) OF ServedNRCellsToModify-Item

ServedNRCellsToModify-Item::= SEQUENCE {

old-nrcgi						NRCGI,

servedNRCellInformation			ServedNRCell-Information,

nrNeighbourInformation			NRNeighbour-Information			OPTIONAL,

nrDeactivationIndication			DeactivationIndication			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedNRCellsToModify-Item-ExtIEs} } OPTIONAL,

...

}

ServedNRCellsToModify-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedNRcellsToDeleteENDCConfUpdList ::= SEQUENCE (SIZE (1..maxCellinengNB)) OF NRCGI

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

ENDCConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdateAcknowledge-IEs}},

...

}

ENDCConfigurationUpdateAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcConfigUpdate			CRITICALITY reject	TYPE RespondingNodeType-EndcConfigUpdate			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication						PRESENCE optional }|

{ ID id-TNLConfigurationInfo						CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional }|

{ ID id-TNLA-Setup-List								CRITICALITY ignore	TYPE TNLA-Setup-List											PRESENCE optional }|

{ ID id-TNLA-Failed-To-Setup-List					CRITICALITY ignore	TYPE TNLA-Failed-To-Setup-List							PRESENCE optional },

...

}

RespondingNodeType-EndcConfigUpdate::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCConfigUpdateAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCConfigUpdateAckIEs}},

...

}

ENB-ENDCConfigUpdateAckIEs X2AP-PROTOCOL-IES ::= {

...

}

En-gNB-ENDCConfigUpdateAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

ENDCConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdateFailure-IEs}},

...

}

ENDCConfigurationUpdateFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-TimeToWait						CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- EN-DC CELL ACTIVATION REQUEST

--

-- **************************************************************

ENDCCellActivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationRequest-IEs}},

...

}

ENDCCellActivationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRCellsToActivate 		CRITICALITY reject	TYPE ServedNRCellsToActivate				PRESENCE mandatory}|

{ ID id-ActivationID			 		CRITICALITY reject	TYPE ActivationID						PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ServedNRCellsToActivate::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF ServedNRCellsToActivate-Item

ServedNRCellsToActivate-Item::= SEQUENCE {

nrCellID						NRCGI,

iE-Extensions					ProtocolExtensionContainer { {ServedNRCellsToActivate-Item-ExtIEs} } OPTIONAL,

...

}

ServedNRCellsToActivate-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- EN-DC CELL ACTIVATION RESPONSE

--

-- **************************************************************

ENDCCellActivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationResponse-IEs}},

...

}

ENDCCellActivationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivatedNRCellList			CRITICALITY ignore	TYPE ActivatedNRCellList				PRESENCE mandatory}|

{ ID id-ActivationID					CRITICALITY reject	TYPE ActivationID						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

ActivatedNRCellList ::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF ActivatedNRCellList-Item

ActivatedNRCellList-Item::= SEQUENCE {

nrCellID						NRCGI,

iE-Extensions						ProtocolExtensionContainer { {ActivatedNRCellList-Item-ExtIEs} } OPTIONAL,

...

}

ActivatedNRCellList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

--**************************************************************

--

#### -- EN-DC CELL ACTIVATION FAILURE

--

-- **************************************************************

ENDCCellActivationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationFailure-IEs}},

...

}

ENDCCellActivationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivationID						CRITICALITY reject	TYPE ActivationID					PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SecondaryRATDataUsageReport-IEs}},

...

}

SecondaryRATDataUsageReport-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SecondaryRATUsageReportList			CRITICALITY reject	TYPE SecondaryRATUsageReportList			PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ACTIVITY NOTIFICATION

--

-- **************************************************************

SgNBActivityNotification ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBActivityNotification-IEs}},

...

}

SgNBActivityNotification-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-UEContextLevelUserPlaneActivity		CRITICALITY ignore	TYPE UserPlaneTrafficActivityReport		PRESENCE optional}|

{ ID id-ERABActivityNotifyItemList			CRITICALITY ignore	TYPE ERABActivityNotifyItemList			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET REQUIRED

--

-- **************************************************************

ENDCPartialResetRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetRequired-IEs}},

...

}

ENDCPartialResetRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET CONFIRM

--

-- **************************************************************

ENDCPartialResetConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetConfirm-IEs}},

...

}

ENDCPartialResetConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-Admitted-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-UTRA - NR CELL RESOURCE COORDINATION REQUEST

--

-- **************************************************************

EUTRANRCellResourceCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{EUTRANRCellResourceCoordinationRequest-IEs}},

...

}

EUTRANRCellResourceCoordinationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EutranrCellResourceCoordination	CRITICALITY reject	TYPE InitiatingNodeType-EutranrCellResourceCoordination			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication							CRITICALITY reject 	TYPE InterfaceInstanceIndication										PRESENCE optional },

...

}

InitiatingNodeType-EutranrCellResourceCoordination ::= CHOICE {

initiate-eNB			ProtocolIE-Container	{{ENB-EUTRA-NRCellResourceCoordinationReqIEs}},

initiate-en-gNB			ProtocolIE-Container	{{En-gNB-EUTRA-NRCellResourceCoordinationReqIEs}},

...

}

ENB-EUTRA-NRCellResourceCoordinationReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication			CRITICALITY reject	TYPE DataTrafficResourceIndication					PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID					CRITICALITY reject	TYPE SpectrumSharingGroupID							PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinEUTRACoordinationReq	CRITICALITY reject	TYPE ListofEUTRACellsinEUTRACoordinationReq		PRESENCE mandatory},

...

}

En-gNB-EUTRA-NRCellResourceCoordinationReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication			CRITICALITY reject	TYPE DataTrafficResourceIndication				PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinNRCoordinationReq		CRITICALITY reject	TYPE ListofEUTRACellsinNRCoordinationReq		PRESENCE mandatory }|

{ ID id-SpectrumSharingGroupID					CRITICALITY reject	TYPE SpectrumSharingGroupID						PRESENCE mandatory}|

{ ID id-ListofNRCellsinNRCoordinationReq		CRITICALITY reject	TYPE ListofNRCellsinNRCoordinationReq			PRESENCE mandatory},

...

}

ListofEUTRACellsinEUTRACoordinationReq ::= SEQUENCE (SIZE (0..maxCellineNB)) OF ECGI

ListofEUTRACellsinNRCoordinationReq ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

ListofNRCellsinNRCoordinationReq ::= SEQUENCE (SIZE (0..maxnoNRcellsSpectrumSharingWithE-UTRA)) OF NRCGI

-- **************************************************************

--

#### -- E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

EUTRANRCellResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{EUTRANRCellResourceCoordinationResponse-IEs}},

...

}

EUTRANRCellResourceCoordinationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EutranrCellResourceCoordination		CRITICALITY reject	TYPE RespondingNodeType-EutranrCellResourceCoordination			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

RespondingNodeType-EutranrCellResourceCoordination ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

...

}

ENB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication				CRITICALITY reject	TYPE DataTrafficResourceIndication				PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID						CRITICALITY reject	TYPE SpectrumSharingGroupID							PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinEUTRACoordinationResp		CRITICALITY reject	TYPE ListofEUTRACellsinEUTRACoordinationResp	PRESENCE mandatory},

...

}

En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication		CRITICALITY reject	TYPE DataTrafficResourceIndication			PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID				CRITICALITY reject	TYPE SpectrumSharingGroupID						PRESENCE mandatory}|

{ ID id-ListofNRCellsinNRCoordinationResp	CRITICALITY reject	TYPE ListofNRCellsinNRCoordinationResp		PRESENCE mandatory},

...

}

ListofEUTRACellsinEUTRACoordinationResp ::= SEQUENCE (SIZE (0..maxCellineNB)) OF ECGI

ListofNRCellsinNRCoordinationResp ::= SEQUENCE (SIZE (0..maxnoNRcellsSpectrumSharingWithE-UTRA)) OF NRCGI

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL REQUEST

--

-- **************************************************************

ENDCX2RemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalRequest-IEs}},

...

}

ENDCX2RemovalRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcX2Removal			CRITICALITY reject	TYPE InitiatingNodeType-EndcX2Removal		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

InitiatingNodeType-EndcX2Removal ::= CHOICE {

init-eNB				ProtocolIE-Container	{{ENB-ENDCX2RemovalReqIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCX2RemovalReqIEs}},

...

}

ENB-ENDCX2RemovalReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory},

...

}

En-gNB-ENDCX2RemovalReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL RESPONSE

--

-- **************************************************************

ENDCX2RemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalResponse-IEs}},

...

}

ENDCX2RemovalResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcX2Removal		CRITICALITY reject	TYPE RespondingNodeType-EndcX2Removal		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

RespondingNodeType-EndcX2Removal ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCX2RemovalReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCX2RemovalReqAckIEs}},

...

}

ENB-ENDCX2RemovalReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory},

...

}

En-gNB-ENDCX2RemovalReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL FAILURE

--

-- **************************************************************

ENDCX2RemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalFailure-IEs}},

...

}

ENDCX2RemovalFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 		TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- DATA FORWARDING ADDRESS INDICATION

--

-- **************************************************************

DataForwardingAddressIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataForwardingAddressIndication-IEs}},

...

}

DataForwardingAddressIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional }|

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional }|

{ ID id-E-RABs-DataForwardingAddress-List	CRITICALITY ignore	TYPE E-RABs-DataForwardingAddress-List		PRESENCE mandatory}|

{ ID id-CHO-DC-Indicator					CRITICALITY reject	TYPE CHO-DC-Indicator							PRESENCE optional},

...

}

E-RABs-DataForwardingAddress-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-DataForwardingAddress-ItemIEs} }

E-RABs-DataForwardingAddress-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-DataForwardingAddress-Item	CRITICALITY ignore	TYPE E-RABs-DataForwardingAddress-Item	PRESENCE mandatory},

...

}

E-RABs-DataForwardingAddress-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dl-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-DataForwardingAddress-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-DataForwardingAddress-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- GNB STATUS INDICATION

--

-- **************************************************************

GNBStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ { GNBStatusIndicationIEs} },

...

}

GNBStatusIndicationIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GNBOverloadInformation			CRITICALITY ignore	TYPE GNBOverloadInformation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

-- EN-DC CONFIGURATION TRANSFER

--

-- **************************************************************

ENDCConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationTransfer-IEs}},

...

}

ENDCConfigurationTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-endcSONConfigurationTransfer	CRITICALITY ignore	TYPE EndcSONConfigurationTransfer			PRESENCE optional}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

-- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID			CRITICALITY reject	TYPE UE-X2AP-ID						PRESENCE mandatory	}|

{ ID id-SgNB-UE-X2AP-ID			CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation			CRITICALITY ignore	TYPE TraceActivation				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID			CRITICALITY reject	TYPE UE-X2AP-ID						PRESENCE mandatory	}|

{ ID id-SgNB-UE-X2AP-ID			CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory	}|

{ ID id-EUTRANTraceID			CRITICALITY ignore	TYPE EUTRANTraceID					PRESENCE mandatory	},

...

}

END

-- ASN1STOP

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

id-E-RABUsageReport-Item,

id-SecondaryRATUsageReport-Item,

id-UEAppLayerMeasConfig,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-serviceType,

id-ProtectedEUTRAResourceIndication,

id-NRS-NSSS-PowerOffset,

id-NSSS-NumOccasionDifferentPrecoder,

id-CNTypeRestrictions,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-ECGI,

id-NRCGI,

id-MeNBCoordinationAssistanceInformation,

id-SgNBCoordinationAssistanceInformation,

id-NRNeighbourInfoToAdd,

id-LastNG-RANPLMNIdentity,

id-BPLMN-ID-Info-EUTRA,

id-NBIoT-UL-DL-AlignmentOffset,

id-UnlicensedSpectrumRestriction,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

maxnoofBands,

maxnoofBPLMNs,

maxnoofAdditionalPLMNs,

maxnoofCells,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofNeighbours,

maxnoofPRBs,

maxNrOfErrors,

maxPools,

maxnoofMBSFN,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMBMSServiceAreaIdentities,

maxnoofMDTPLMNs,

maxnoofCoMPHypothesisSet,

maxnoofCoMPCells,

maxUEReport,

maxCellReport,

maxnoofPA,

maxCSIProcess,

maxCSIReport,

maxSubband,

maxnooftimeperiods,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxUEsinengNBDU,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofBluetoothName,

maxnoofWLANName,

maxofNRNeighbours,

maxnoofextBPLMNs,

maxnoofextBPLMNsminus1,

maxnoofBPLMNsminus1,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofTNLAssociations,

maxnoofCellsinCHO

FROM X2AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM X2AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES

FROM X2AP-Containers;

#### -- A

ABSInformation ::= CHOICE {

fdd					ABSInformationFDD,

tdd					ABSInformationTDD,

abs-inactive		NULL,

...

}

ABSInformationFDD ::= SEQUENCE {

abs-pattern-info					BIT STRING (SIZE(40)),

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

measurement-subset					BIT STRING (SIZE(40)),

iE-Extensions						ProtocolExtensionContainer { {ABSInformationFDD-ExtIEs} } OPTIONAL,

...

}

ABSInformationFDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ABSInformationTDD ::= SEQUENCE {

abs-pattern-info					BIT STRING (SIZE(1..70, ...)),

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

measurement-subset					BIT STRING (SIZE(1..70, ...)),

iE-Extensions						ProtocolExtensionContainer { {ABSInformationTDD-ExtIEs} } OPTIONAL,

...

}

ABSInformationTDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ABS-Status ::= SEQUENCE {

dL-ABS-status								DL-ABS-status,

usableABSInformation						UsableABSInformation,

iE-Extensions								ProtocolExtensionContainer { {ABS-Status-ExtIEs} } OPTIONAL,

...

}

ABS-Status-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ActivationID ::= INTEGER (0..255)

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AdditionalSpecialSubframe-Info ::=		SEQUENCE {

additionalspecialSubframePatterns		AdditionalSpecialSubframePatterns,

cyclicPrefixDL							CyclicPrefixDL,

cyclicPrefixUL							CyclicPrefixUL,

iE-Extensions							ProtocolExtensionContainer { {AdditionalSpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

AdditionalSpecialSubframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalSpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

...

}

AdditionalSpecialSubframeExtension-Info ::=		SEQUENCE {

additionalspecialSubframePatternsExtension	AdditionalSpecialSubframePatternsExtension,

cyclicPrefixDL								CyclicPrefixDL,

cyclicPrefixUL								CyclicPrefixUL,

iE-Extensions								ProtocolExtensionContainer { {AdditionalSpecialSubframeExtension-Info-ExtIEs} } OPTIONAL,

...

}

AdditionalSpecialSubframeExtension-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalSpecialSubframePatternsExtension ::= ENUMERATED {

ssp10,

...

}

AvailableFastMCGRecoveryViaSRB3 ::= ENUMERATED {true,...}

ReleaseFastMCGRecoveryViaSRB3 ::= ENUMERATED {true,...}

AerialUEsubscriptionInformation ::= ENUMERATED {

allowed,

not-allowed,

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfMDT ::= CHOICE {

cellBased					CellBasedMDT,

tABased						TABasedMDT,

pLMNWide					NULL,

...,

tAIBased					TAIBasedMDT

}

AreaScopeOfQMC ::= CHOICE {

cellBased					CellBasedQMC,

tABased						TABasedQMC,

tAIBased					TAIBasedQMC,

pLMNAreaBased				PLMNAreaBasedQMC,

...

}

AS-SecurityInformation ::= SEQUENCE {

key-eNodeB-star		Key-eNodeB-Star,

nextHopChainingCount			NextHopChainingCount,

iE-Extensions						ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AS-SecurityInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPLMNs-Item ::= SEQUENCE (SIZE(1..maxnoofAdditionalPLMNs)) OF PLMN-Identity

#### -- B

BandwidthReducedSI::= ENUMERATED {

scheduled,

...

}

BearerType ::= ENUMERATED {

non-IP,

...

}

BenefitMetric ::= INTEGER (-101..100, ...)

BitRate ::= INTEGER (0..10000000000)

BroadcastPLMNs-Item ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList			OPTIONAL,

bt-rssi							ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BluetoothMeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofBPLMNsminus1)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastPLMNs-Item,

tac								TAC,

e-utraCI						EUTRANCellIdentifier,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-EUTRA-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofextBPLMNsminus1)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastextPLMNs,

fiveGS-TAC						FiveGS-TAC		OPTIONAL,

nr-CI							NRCellIdentifier,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-NR-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastextPLMNs ::= SEQUENCE (SIZE(1..maxnoofextBPLMNs)) OF PLMN-Identity

#### -- C

CapacityValue ::= INTEGER (0..100)

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

om-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

handover-desirable-for-radio-reasons,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

partial-handover,

unknown-new-eNB-UE-X2AP-ID,

unknown-old-eNB-UE-X2AP-ID,

unknown-pair-of-UE-X2AP-ID,

ho-target-not-allowed,

tx2relocoverall-expiry,

trelocprep-expiry,

cell-not-available,

no-radio-resources-available-in-target-cell,

invalid-MME-GroupID,

unknown-MME-Code,

encryption-and-or-integrity-protection-algorithms-not-supported,

reportCharacteristicsEmpty,

noReportPeriodicity,

existingMeasurementID,

unknown-eNB-Measurement-ID,

measurement-temporarily-not-available,

unspecified,

...,

load-balancing,

handover-optimisation,

value-out-of-allowed-range,

multiple-E-RAB-ID-instances,

switch-off-ongoing,

not-supported-QCI-value,

measurement-not-supported-for-the-object,

tDCoverall-expiry,

tDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

mCG-Mobility,

sCG-Mobility,

count-reaches-max-value,

unknown-old-en-gNB-UE-X2AP-ID,

pDCP-Overload,

cho-cpc-resources-tobechanged

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CellBasedMDT::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellBasedQMC::= SEQUENCE {

cellIdListforQMC		CellIdListforQMC,

iE-Extensions		ProtocolExtensionContainer { {CellBasedQMC-ExtIEs} } OPTIONAL,

...

}

CellBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellCapacityClassValue ::= INTEGER (1..100, ...)

CellDeploymentStatusIndicator ::= ENUMERATED {pre-change-notification, ...}

CellIdListforMDT ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF ECGI

CellIdListforQMC ::= SEQUENCE (SIZE(1..maxnoofCellIDforQMC)) OF ECGI

CellReplacingInfo ::= SEQUENCE {

replacingCellsList				ReplacingCellsList,

iE-Extensions					ProtocolExtensionContainer { {CellReplacingInfo-ExtIEs}}	OPTIONAL,

...

}

CellReplacingInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellReportingIndicator ::= ENUMERATED {stop-request, ... }

Cell-Size ::= ENUMERATED {verysmall, small, medium, large, ... }

CellType ::= SEQUENCE {

cell-Size						Cell-Size,

iE-Extensions					ProtocolExtensionContainer { {CellType-ExtIEs}}	OPTIONAL,

...

}

CellType-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CHO-DC-Indicator ::= ENUMERATED {true, ...}

CNTypeRestrictions ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF CNTypeRestrictionsItem

CNTypeRestrictionsItem ::= SEQUENCE {

plmn-Id				PLMN-Identity,

cn-type				ENUMERATED {fiveGC-forbidden, ... , epc-forbidden},

iE-Extensions		ProtocolExtensionContainer { {CNTypeRestrictionsItem-ExtIEs} } OPTIONAL,

...

}

CNTypeRestrictionsItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPHypothesisSet ::= SEQUENCE (SIZE(1..maxnoofCoMPCells)) OF CoMPHypothesisSetItem

CoMPHypothesisSetItem ::= SEQUENCE {

coMPCellID						ECGI,

coMPHypothesis					BIT STRING (SIZE(6..4400, ...)),

iE-Extensions					ProtocolExtensionContainer { {CoMPHypothesisSetItem-ExtIEs} } OPTIONAL,

...

}

CoMPHypothesisSetItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformation ::= SEQUENCE {

coMPInformationItem						CoMPInformationItem,

coMPInformationStartTime				CoMPInformationStartTime,

iE-Extensions							ProtocolExtensionContainer { {CoMPInformation-ExtIEs} } OPTIONAL,

...

}

CoMPInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformationItem ::= SEQUENCE (SIZE(1..maxnoofCoMPHypothesisSet)) OF

SEQUENCE {

coMPHypothesisSet					CoMPHypothesisSet,

benefitMetric						BenefitMetric,

iE-Extensions						ProtocolExtensionContainer { {CoMPInformationItem-ExtIEs} } OPTIONAL,

...

}

CoMPInformationItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformationStartTime ::= SEQUENCE (SIZE(0..1)) OF

SEQUENCE {

startSFN							INTEGER (0..1023, ...),

startSubframeNumber					INTEGER (0..9, ...),

iE-Extensions						ProtocolExtensionContainer { {CoMPInformationStartTime-ExtIEs} } OPTIONAL,

...

}

CoMPInformationStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue							CellCapacityClassValue				OPTIONAL,

capacityValue									CapacityValue,

iE-Extensions									ProtocolExtensionContainer { {CompositeAvailableCapacity-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacityGroup	::= SEQUENCE {

dL-CompositeAvailableCapacity					CompositeAvailableCapacity,

uL-CompositeAvailableCapacity					CompositeAvailableCapacity,

iE-Extensions									ProtocolExtensionContainer { {CompositeAvailableCapacityGroup-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacityGroup-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Correlation-ID ::= OCTET STRING (SIZE (4))

COUNTvalue ::= SEQUENCE {

pDCP-SN					PDCP-SN,

hFN						HFN,

iE-Extensions			ProtocolExtensionContainer { {COUNTvalue-ExtIEs} } OPTIONAL,

...

}

COUNTvalue-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueExtended ::= SEQUENCE {

pDCP-SNExtended			PDCP-SNExtended,

hFNModified				HFNModified,

iE-Extensions			ProtocolExtensionContainer { {COUNTValueExtended-ExtIEs} } OPTIONAL,

...

}

COUNTValueExtended-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTvaluePDCP-SNlength18 ::= SEQUENCE {

pDCP-SNlength18			PDCP-SNlength18,

hFNforPDCP-SNlength18	HFNforPDCP-SNlength18,

iE-Extensions			ProtocolExtensionContainer { {COUNTvaluePDCP-SNlength18-ExtIEs} } OPTIONAL,

...

}

COUNTvaluePDCP-SNlength18-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoverageModificationList ::= SEQUENCE (SIZE (1..maxCellineNB)) OF CoverageModification-Item

CoverageModification-Item ::= SEQUENCE {

eCGI							ECGI,

coverageState					INTEGER (0..15, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- Included in case the Cell Deployment Status Indicator IE is present

...

}

CPTransportLayerInformation		::= CHOICE {

endpointIPAddress						TransportLayerAddress,

endpointIPAddressAndPort					TransportLayerAddressAndPort,

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CRNTI ::= BIT STRING (SIZE (16))

CSGMembershipStatus ::= ENUMERATED {

member,

not-member

}

CSG-Id ::= BIT STRING (SIZE (27))

CSIReportList ::= SEQUENCE (SIZE(1..maxUEReport)) OF

SEQUENCE {

uEID							UEID,

cSIReportPerCSIProcess			CSIReportPerCSIProcess,

iE-Extensions					ProtocolExtensionContainer { {CSIReportList-ExtIEs} } OPTIONAL,

...

}

CSIReportList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CSIReportPerCSIProcess ::= SEQUENCE (SIZE(1.. maxCSIProcess)) OF

SEQUENCE {

cSIProcessConfigurationIndex	INTEGER (1..7, ...),

cSIReportPerCSIProcessItem		CSIReportPerCSIProcessItem,

iE-Extensions					ProtocolExtensionContainer { {CSIReportPerCSIProcess-ExtIEs} } OPTIONAL,

...

}

CSIReportPerCSIProcess-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CSIReportPerCSIProcessItem ::= SEQUENCE (SIZE(1.. maxCSIReport)) OF

SEQUENCE {

rI								INTEGER (1..8, ...),

widebandCQI						WidebandCQI,

subbandSize						SubbandSize,

subbandCQIList					SubbandCQIList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CSIReportPerCSIProcessItem-ExtIEs} } OPTIONAL,

...

}

CSIReportPerCSIProcessItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CyclicPrefixDL ::= ENUMERATED {

normal,

extended,

...

}

CyclicPrefixUL ::= ENUMERATED {

normal,

extended,

...

}

CHOtrigger ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOinformation-REQ ::= SEQUENCE {

cho-trigger						CHOtrigger,

new-eNB-UE-X2AP-ID				UE-X2AP-ID											OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "CHO-replace" --,

new-eNB-UE-X2AP-ID-Extension	UE-X2AP-ID-Extension								OPTIONAL,

cHO-EstimatedArrivalProbability	CHO-Probability										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CHOinformation-REQ-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-REQ-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-ACK ::= SEQUENCE {

requestedTargetCellID			ECGI,

maxCHOpreparations				MaxCHOpreparations											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-ACK-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-ACK-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

CandidateCellsToBeCancelledList ::= SEQUENCE (SIZE (1..maxnoofCellsinCHO)) OF ECGI

CHO-Probability ::= INTEGER (1..100)

#### -- D

DataTrafficResources ::= BIT STRING (SIZE(6..17600))

DataTrafficResourceIndication ::= SEQUENCE {

activationSFN					INTEGER (0..1023),

sharedResourceType				SharedResourceType,

reservedSubframePattern 		ReservedSubframePattern OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} } OPTIONAL,

...

}

DataTrafficResourceIndication-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DAPSRequestInfo ::= SEQUENCE {

dAPSIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInfo ::= SEQUENCE {

dAPSResponseIndicator					ENUMERATED { daps-HO-accepted, daps-HO-not-accepted,...},

iE-Extensions				ProtocolExtensionContainer { { DAPSResponseInfo-ExtIEs} } OPTIONAL,

...

}

DAPSResponseInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DeactivationIndication::= ENUMERATED {

deactivated,

...

}

DeliveryStatus ::= SEQUENCE {

highestSuccessDeliveredPDCPSN		INTEGER (0..4095),

iE-Extensions		ProtocolExtensionContainer { {DeliveryStatus-ExtIEs} } OPTIONAL,

...

}

DeliveryStatus-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DesiredActNotificationLevel	::= ENUMERATED {none, e-rab, ue-level, ...}

DL-ABS-status::= INTEGER (0..100)

DL-Forwarding ::= ENUMERATED {

dL-forwardingProposed,

...

}

DL-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DLResourceBitmapULandDLSharing ::= DataTrafficResources

DLResourcesULandDLSharing ::= CHOICE {

unchanged			NULL,

changed				DLResourceBitmapULandDLSharing,

...

}

DL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

DL-Total-PRB-usage::= INTEGER (0..100)

DRB-ID ::= INTEGER (1..32)

DuplicationActivation::= ENUMERATED {active, inactive, ...}

DynamicDLTransmissionInformation ::= CHOICE {

naics-active			DynamicNAICSInformation,

naics-inactive			NULL,

...

}

DynamicNAICSInformation ::= SEQUENCE {

transmissionModes					BIT STRING (SIZE(8))												OPTIONAL,

pB-information						INTEGER(0..3)														OPTIONAL,

pA-list								SEQUENCE (SIZE(0..maxnoofPA)) OF PA-Values,

iE-Extensions						ProtocolExtensionContainer { {DynamicNAICSInformation-ExtIEs} } OPTIONAL,

...

}

DynamicNAICSInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

EARFCN ::= INTEGER (0..maxEARFCN)

EARFCNExtension ::= INTEGER(maxEARFCNPlusOne..newmaxEARFCN, ...)

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EndcSONConfigurationTransfer ::= OCTET STRING

EnhancedRNTP ::= SEQUENCE {

enhancedRNTPBitmap			BIT STRING (SIZE(12..8800, ...)),

rNTP-High-Power-Threshold	RNTP-Threshold,

enhancedRNTPStartTime		EnhancedRNTPStartTime OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {EnhancedRNTP-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTP-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTPStartTime ::= SEQUENCE {

startSFN				INTEGER (0..1023, ...),

startSubframeNumber		INTEGER (0..9, ...),

iE-Extensions			ProtocolExtensionContainer { {EnhancedRNTPStartTime-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTPStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID ::= CHOICE {

macro-eNB-ID	BIT STRING (SIZE (20)),

home-eNB-ID		BIT STRING (SIZE (28)),

... ,

short-Macro-eNB-ID		BIT STRING (SIZE(18)),

long-Macro-eNB-ID		BIT STRING (SIZE(21))

}

EncryptionAlgorithms ::= BIT STRING (SIZE (16, ...))

EN-DC-ResourceConfiguration ::= SEQUENCE {

pDCPatSgNB		ENUMERATED {present, not-present, ...},

mCGresources	ENUMERATED {present, not-present, ...},

sCGresources	ENUMERATED {present, not-present, ...},

iE-Extensions					ProtocolExtensionContainer { {EN-DC-ResourceConfigurationExtIEs} }	OPTIONAL,

...

}

EN-DC-ResourceConfigurationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EPCHandoverRestrictionListContainer ::= OCTET STRING

-- This octets of the OCTET STRING contain the Handover Restriction List IE as specified in TS 36.413 [4]. --

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity

ERABActivityNotifyItemList ::= SEQUENCE (SIZE (0..maxnoofBearers)) OF ERABActivityNotifyItem

ERABActivityNotifyItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

activityReport					UserPlaneTrafficActivityReport,

iE-Extensions					ProtocolExtensionContainer { {ERABActivityNotifyItem-ExtIEs} }	OPTIONAL,

...

}

ERABActivityNotifyItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-ID ::= INTEGER (0..15, ...)

E-RAB-Level-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RAB-Level-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

E-RAB-Level-QoS-Parameters-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extended for introduction of downlink and uplink packet loss rate for enhanced Voice performance –

{ ID id-DownlinkPacketLossRate			CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional}|

{ ID id-UplinkPacketLossRate			CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional},

...

}

E-RAB-List ::= SEQUENCE (SIZE(1.. maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RAB-ItemIEs} }

E-RAB-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RAB-Item	 CRITICALITY ignore 	TYPE E-RAB-Item 	PRESENCE mandatory },

...

}

E-RAB-Item ::= SEQUENCE {

e-RAB-ID				E-RAB-ID,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

E-RAB-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABsSubjectToEarlyStatusTransfer-List ::= SEQUENCE (SIZE (1.. maxnoofBearers)) OF E-RABsSubjectToEarlyStatusTransfer-Item

E-RABsSubjectToEarlyStatusTransfer-Item ::= SEQUENCE {

e-RAB-ID											E-RAB-ID,

fIRST-DL-COUNTValue									COUNTvalue,

fIRST-DL-COUNTValueExtended							COUNTValueExtended													OPTIONAL,

fIRST-DL-COUNTValueforPDCPSNLength18				COUNTvaluePDCP-SNlength18													OPTIONAL,

iE-Extension		ProtocolExtensionContainer { { E-RABsSubjectToEarlyStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

E-RABsSubjectToEarlyStatusTransfer-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABsSubjectToDLDiscarding-List ::= SEQUENCE (SIZE (1.. maxnoofBearers)) OF E-RABsSubjectToDLDiscarding-Item

E-RABsSubjectToDLDiscarding-Item ::= SEQUENCE {

e-RAB-ID											E-RAB-ID,

dISCARD-DL-COUNTValue								COUNTvalue,

dISCARD-DL-COUNTValueExtended						COUNTValueExtended													OPTIONAL,

dISCARD-DL-COUNTValueforPDCPSNLength18				COUNTvaluePDCP-SNlength18													OPTIONAL,

iE-Extension		ProtocolExtensionContainer { { E-RABsSubjectToDLDiscarding-Item-ExtIEs} }					OPTIONAL,

...

}

E-RABsSubjectToDLDiscarding-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABUsageReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF ProtocolIE-Single-Container { {E-RABUsageReport-ItemIEs} }

E-RABUsageReport-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABUsageReport-Item	 CRITICALITY ignore 	TYPE E-RABUsageReport-Item 	PRESENCE mandatory },

...

}

E-RABUsageReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { {E-RABUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

E-RABUsageReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

...

}

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

EUTRANTraceID		::= OCTET STRING (SIZE (8))

EventType ::= ENUMERATED{

change-of-serving-cell,

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedActivity		ExpectedUEActivityBehaviour OPTIONAL,

expectedHOInterval		ExpectedHOInterval			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod					ExpectedActivityPeriod					OPTIONAL,

expectedIdlePeriod						ExpectedIdlePeriod						OPTIONAL,

sourceofUEActivityBehaviourInformation	SourceOfUEActivityBehaviourInformation	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExtendedULInterferenceOverloadInfo ::= SEQUENCE {

associatedSubframes							BIT STRING (SIZE (5)),

extended-ul-InterferenceOverloadIndication	UL-InterferenceOverloadIndication,

iE-Extensions								ProtocolExtensionContainer { {ExtendedULInterferenceOverloadInfo-ExtIEs} } OPTIONAL,

...

}

ExtendedULInterferenceOverloadInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedBitRate	::= INTEGER (10000000001..4000000000000,...)

#### -- F

FastMCGRecovery	::= SEQUENCE {

rrcContainer 				RRCContainer			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {FastMCGRecovery-ExtIEs} } OPTIONAL,

...

}

FastMCGRecovery-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

FDD-Info ::= SEQUENCE {

uL-EARFCN						EARFCN,

dL-EARFCN						EARFCN,

uL-Transmission-Bandwidth		Transmission-Bandwidth,

dL-Transmission-Bandwidth		Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {FDD-Info-ExtIEs} } OPTIONAL,

...

}

FDD-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UL-EARFCNExtension						CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-DL-EARFCNExtension						CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToUL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NRS-NSSS-PowerOffset					CRITICALITY ignore	EXTENSION NRS-NSSS-PowerOffset							PRESENCE optional}|

{ ID id-NSSS-NumOccasionDifferentPrecoder		CRITICALITY ignore	EXTENSION NSSS-NumOccasionDifferentPrecoder			PRESENCE optional},

...

}

FDD-InfoNeighbourServedNRCell-Information ::= SEQUENCE {

ul-NRFreqInfo			NRFreqInfo,

dl-NRFreqInfo			NRFreqInfo,

iE-Extensions			ProtocolExtensionContainer { {FDD-InfoNeighbourServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoNeighbourServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenInterRATs ::= ENUMERATED {

all,

geran,

utran,

cdma2000,

...,

geranandutran,

cdma2000andutran

}

ForbiddenTAs ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenTAs-Item

ForbiddenTAs-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenTAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenTAs-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

ForbiddenLAs ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF ForbiddenLAs-Item

ForbiddenLAs-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

forbiddenLACs		ForbiddenLACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenLAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenLAs-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenLACs ::= SEQUENCE (SIZE(1..maxnoofForbLACs)) OF LAC

Fourframes ::= BIT STRING (SIZE (24))

FreqBandIndicator ::= INTEGER (1..256, ...)

FreqBandIndicatorPriority ::= ENUMERATED {

not-broadcasted,

broadcasted,

...

}

FreqBandNrItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

supportedSULBandList	SEQUENCE (SIZE(0..maxnoofNrCellBands)) OF SupportedSULFreqBandItem,

iE-Extensions				ProtocolExtensionContainer { {FreqBandNrItem-ExtIEs} } OPTIONAL,

...

}

FreqBandNrItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for maximum bitrate > 10Gbps --

{ ID id-extended-e-RAB-MaximumBitrateDL		CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-MaximumBitrateUL		CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

GlobalENB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalENB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GlobalGNB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

gNB-ID					GNB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GNBOverloadInformation ::= ENUMERATED {overloaded, not-overloaded, ...}

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses			TransportLayerAddress,

iE-Extensions						ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } } OPTIONAL,

...

}

GTPTLA-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTPtunnelEndpoint ::= SEQUENCE {

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEI,

iE-Extensions					ProtocolExtensionContainer { {GTPtunnelEndpoint-ExtIEs} } OPTIONAL,

...

}

GTPtunnelEndpoint-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEI					::= OCTET STRING (SIZE (4))

GUGroupIDList		::= SEQUENCE (SIZE (1..maxPools)) OF GU-Group-ID

GU-Group-ID			::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

mME-Group-ID		MME-Group-ID,

iE-Extensions		ProtocolExtensionContainer { {GU-Group-ID-ExtIEs} } OPTIONAL,

...

}

GU-Group-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GUMMEI			::= SEQUENCE {

gU-Group-ID		GU-Group-ID,

mME-Code			MME-Code,

iE-Extensions					ProtocolExtensionContainer { {GUMMEI-ExtIEs} } OPTIONAL,

...

}

GUMMEI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID	BIT STRING (SIZE (22..32)),

...

}

#### -- H

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

...,

interRATpingpong

}

HandoverRestrictionList ::= SEQUENCE {

servingPLMN					PLMN-Identity,

equivalentPLMNs				EPLMNs					OPTIONAL,

forbiddenTAs				ForbiddenTAs			OPTIONAL,

forbiddenLAs				ForbiddenLAs			OPTIONAL,

forbiddenInterRATs			ForbiddenInterRATs		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {HandoverRestrictionList-ExtIEs} }	OPTIONAL,

...

}

HandoverRestrictionList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRrestrictioninEPSasSecondaryRAT			CRITICALITY ignore	EXTENSION NRrestrictioninEPSasSecondaryRAT			PRESENCE optional}|

{ ID id-CNTypeRestrictions						CRITICALITY ignore	EXTENSION CNTypeRestrictions								PRESENCE optional}|

{ ID id-NRrestrictionin5GS						CRITICALITY ignore	EXTENSION NRrestrictionin5GS								PRESENCE optional}|

{ ID id-LastNG-RANPLMNIdentity					CRITICALITY ignore	EXTENSION PLMN-Identity										PRESENCE optional}|

{ ID id-UnlicensedSpectrumRestriction			CRITICALITY ignore	EXTENSION UnlicensedSpectrumRestriction				PRESENCE optional},

...

}

HFN ::= INTEGER (0..1048575)

HFNModified ::= INTEGER (0..131071)

HFNforPDCP-SNlength18 ::= INTEGER (0..16383)

HWLoadIndicator ::= SEQUENCE {

dLHWLoadIndicator			LoadIndicator,

uLHWLoadIndicator			LoadIndicator,

iE-Extensions				ProtocolExtensionContainer { {HWLoadIndicator-ExtIEs} } OPTIONAL,

...

}

HWLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- I

IntegrityProtectionAlgorithms ::= BIT STRING (SIZE (16, ...))

InterfaceInstanceIndication ::= INTEGER (0..255, ...)

InterfacesToTrace ::= BIT STRING (SIZE (8))

InvokeIndication ::= ENUMERATED{

abs-information,

...,

naics-information-start,

naics-information-stop

}

#### -- J

#### -- K

Key-eNodeB-Star ::= BIT STRING (SIZE(256))

#### -- L

LAC					::= OCTET STRING (SIZE (2)) --(EXCEPT ('0000'H|'FFFE'H))

LastVisitedCell-Item ::= CHOICE {

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

...,

nG-RAN-Cell						LastVisitedNGRANCellInformation

}

LastVisitedEUTRANCellInformation ::= SEQUENCE {

global-Cell-ID					ECGI,

cellType						CellType,

time-UE-StayedInCell			Time-UE-StayedInCell,

iE-Extensions					ProtocolExtensionContainer { {LastVisitedEUTRANCellInformation-ExtIEs} } OPTIONAL,

...

}

LastVisitedEUTRANCellInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-11 to support enhanced granularity for time UE stayed in cell --

{ ID id-Time-UE-StayedInCell-EnhancedGranularity	CRITICALITY ignore	EXTENSION Time-UE-StayedInCell-EnhancedGranularity	PRESENCE optional}|

{ ID id-HO-cause									CRITICALITY ignore	EXTENSION Cause													PRESENCE optional},

...

}

LastVisitedGERANCellInformation ::= CHOICE {

undefined						NULL,

...

}

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LCID ::= INTEGER(1..32, ...)

LHN-ID ::= OCTET STRING(SIZE (32..256))

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

LoadIndicator ::= ENUMERATED {

lowLoad,

mediumLoad,

highLoad,

overLoad,

...

}

LocationInformationSgNB ::= SEQUENCE {

pSCell-id			NRCGI,

iE-Extensions		ProtocolExtensionContainer { {LocationInformationSgNB-ExtIEs} } OPTIONAL,

...

}

LocationInformationSgNB-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

LocationInformationSgNBReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType		EventType,

reportArea		ReportArea,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

suspend-lower-layers,

resume-lower-layers,

...

}

#### -- M

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { {M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M1ReportingTrigger::= ENUMERATED{

periodic,

a2eventtriggered,

...,

a2eventtriggered-periodic

}

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold		MeasurementThresholdA2,

iE-Extensions				ProtocolExtensionContainer { {M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M3Configuration ::= SEQUENCE {

m3period			M3period,

iE-Extensions		ProtocolExtensionContainer { {M3Configuration-ExtIEs} } OPTIONAL,

...

}

M3Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M3period ::= ENUMERATED {ms100, ms1000, ms10000, ... }

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-interval	M6report-interval,

m6delay-threshold	M6delay-threshold	OPTIONAL,

-- This IE shall be present if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink” --

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M6report-interval ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, ... }

M6delay-threshold ::= ENUMERATED { ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, ... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MakeBeforeBreakIndicator::= ENUMERATED {true, ...}

ManagementBasedMDTallowed ::= ENUMERATED {allowed, ...}

Masked-IMEISV ::= BIT STRING (SIZE (64))

MaxCHOpreparations ::= INTEGER(1..16, ...)

MDT-Activation		::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDT-Configuration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT				AreaScopeOfMDT,

measurementsToActivate		MeasurementsToActivate,

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2			OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting			OPTIONAL,

-- Included in case of periodic, or event-triggered periodic reporting for measurement M1

iE-Extensions				ProtocolExtensionContainer { {MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-M3Configuration							CRITICALITY ignore	EXTENSION M3Configuration								PRESENCE conditional}|

{ID id-M4Configuration							CRITICALITY ignore	EXTENSION M4Configuration								PRESENCE conditional}|

{ID id-M5Configuration							CRITICALITY ignore	EXTENSION M5Configuration								PRESENCE conditional}|

{ID id-MDT-Location-Info						CRITICALITY ignore	EXTENSION MDT-Location-Info							PRESENCE optional}|

{ID id-SignallingBasedMDTPLMNList				CRITICALITY ignore	EXTENSION MDTPLMNList									PRESENCE optional}|

{ID id-M6Configuration							CRITICALITY ignore	EXTENSION M6Configuration								PRESENCE conditional}|

{ID id-M7Configuration							CRITICALITY ignore	EXTENSION M7Configuration								PRESENCE conditional}|

{ ID id-BluetoothMeasurementConfiguration		CRITICALITY ignore	EXTENSION BluetoothMeasurementConfiguration		PRESENCE optional}|

{ ID id-WLANMeasurementConfiguration			CRITICALITY ignore	EXTENSION WLANMeasurementConfiguration			PRESENCE optional},

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDT-Location-Info ::= BIT STRING (SIZE (8))

Measurement-ID	::= INTEGER (1..4095, ...)

MeasurementsToActivate::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

MeNBCoordinationAssistanceInformation ::= ENUMERATED{

coordination-not-required,

...

}

MeNBResourceCoordinationInformation ::= SEQUENCE {

eUTRA-Cell-ID					ECGI,

uLCoordinationInformation		BIT STRING (SIZE(6..4400, ...)),

dLCoordinationInformation		BIT STRING (SIZE(6..4400, ...))		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MeNBResourceCoordinationInformationExtIEs} }		 OPTIONAL,

...

}

MeNBResourceCoordinationInformationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI									CRITICALITY ignore	EXTENSION NRCGI												PRESENCE optional}|

{ ID id-MeNBCoordinationAssistanceInformation		CRITICALITY reject	EXTENSION MeNBCoordinationAssistanceInformation	PRESENCE optional},

...

}

MeNBtoSeNBContainer ::= OCTET STRING

MME-Group-ID	::= OCTET STRING (SIZE (2))

MME-Code		::= OCTET STRING (SIZE (1))

MBMS-Service-Area-Identity-List ::= SEQUENCE (SIZE(1.. maxnoofMBMSServiceAreaIdentities)) OF MBMS-Service-Area-Identity

MBMS-Service-Area-Identity ::= OCTET STRING (SIZE (2))

MBSFN-Subframe-Infolist::= SEQUENCE (SIZE(1.. maxnoofMBSFN)) OF MBSFN-Subframe-Info

MBSFN-Subframe-Info	::= SEQUENCE {

radioframeAllocationPeriod		RadioframeAllocationPeriod,

radioframeAllocationOffset		RadioframeAllocationOffset,

subframeAllocation				SubframeAllocation,

iE-Extensions					ProtocolExtensionContainer { {MBSFN-Subframe-Info-ExtIEs} } 	OPTIONAL,

...

}

MBSFN-Subframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MobilityParametersModificationRange ::= SEQUENCE {

handoverTriggerChangeLowerLimit		INTEGER (-20..20),

handoverTriggerChangeUpperLimit		INTEGER (-20..20),

...

}

MobilityParametersInformation ::= SEQUENCE {

handoverTriggerChange			INTEGER (-20..20),

...

}

MultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofBands)) OF BandInfo

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extensions					ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs} } 	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

BandInfo	::= SEQUENCE {

freqBandIndicator		FreqBandIndicator,

iE-Extensions			ProtocolExtensionContainer { {BandInfo-ExtIEs} } 	OPTIONAL,

...

}

BandInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MeNBtoSgNBContainer ::= OCTET STRING

SplitSRBs ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SplitSRB ::= SEQUENCE {

rrcContainer 				RRCContainer			OPTIONAL,

srbType						SRBType,

deliveryStatus				DeliveryStatus			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SplitSRB-ExtIEs} } OPTIONAL,

...

}

SplitSRB-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

NBIoT-UL-DL-AlignmentOffset ::= ENUMERATED {

khz-7dot5,

khz0,

khz7dot5,

...

}

Neighbour-Information ::= SEQUENCE (SIZE (0..maxnoofNeighbours)) OF SEQUENCE {

eCGI						ECGI,

pCI							PCI,

eARFCN						EARFCN,

iE-Extensions				ProtocolExtensionContainer { {Neighbour-Information-ExtIEs} } OPTIONAL,

...

}

Neighbour-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NeighbourTAC		CRITICALITY ignore	EXTENSION TAC				PRESENCE optional}|

{ ID id-eARFCNExtension		CRITICALITY reject	EXTENSION EARFCNExtension	PRESENCE optional},

...

}

NextHopChainingCount ::= INTEGER (0..7)

NewDRBIDrequest::= ENUMERATED {true, ...}

Number-of-Antennaports ::= ENUMERATED {

an1,

an2,

an4,

...

}

NRFreqInfo ::= SEQUENCE{

nRARFCN			INTEGER (0.. 3279165),

freqBandListNr	SEQUENCE (SIZE(1..maxnoofNrCellBands)) OF FreqBandNrItem,

sULInformation	SULInformation		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { {NRFreqInfo-ExtIEs} } OPTIONAL,

...

}

NRFreqInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRCellIdentifier ::= BIT STRING (SIZE (36))

NRCGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier				NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {NRCGI-ExtIEs} } OPTIONAL,

...

}

NRCGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRNeighbour-Information ::= SEQUENCE (SIZE (1.. maxofNRNeighbours))OF SEQUENCE {

nrpCI							NRPCI,

nrCellID						NRCGI,

fiveGS-TAC						FiveGS-TAC	OPTIONAL,

configured-TAC					TAC			OPTIONAL,

measurementTimingConfiguration	OCTET STRING,

nRNeighbourModeInfo				CHOICE {

fdd		FDD-InfoNeighbourServedNRCell-Information,

tdd		TDD-InfoNeighbourServedNRCell-Information,

...

},

iE-Extensions						ProtocolExtensionContainer { {NRNeighbour-Information-ExtIEs} } OPTIONAL,

...

}

NRNeighbour-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRPCI ::= INTEGER (0..1007)

NRrestrictioninEPSasSecondaryRAT ::= ENUMERATED {

nRrestrictedinEPSasSecondaryRAT,

...

}

NRrestrictionin5GS ::= ENUMERATED {

nRrestrictedin5GS,

...

}

NRencryptionAlgorithms ::= BIT STRING (SIZE (16,...))

NRintegrityProtectionAlgorithms ::= BIT STRING (SIZE (16,...))

NR-TxBW	::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { {NR-TxBW-ExtIEs} } OPTIONAL,

...

}

NR-TxBW-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ...}

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ...}

NRS-NSSS-PowerOffset ::= ENUMERATED { minusThree, zero, three, ...}

FiveGS-TAC ::= OCTET STRING (SIZE (3))

NRUeReport ::= SEQUENCE {

uENRMeasurements			RRCContainer,

iE-Extensions			ProtocolExtensionContainer { { NRUeReport-ExtIEs} } OPTIONAL,

...

}

NRUeReport-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRUESecurityCapabilities ::= SEQUENCE {

nRencryptionAlgorithms				NRencryptionAlgorithms,

nRintegrityProtectionAlgorithms		NRintegrityProtectionAlgorithms,

iE-Extensions						ProtocolExtensionContainer { {NRUESecurityCapabilities-ExtIEs} }	OPTIONAL,

...

}

NRUESecurityCapabilities-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NSSS-NumOccasionDifferentPrecoder ::= ENUMERATED { two, four, eight, ...}

#### -- O

OffsetOfNbiotChannelNumberToEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

four,

five,

six,

seven,

eight,

nine,

...,

minusEightDotFive,

minusFourDotFive,

threeDotFive,

sevenDotFive

}

Oneframe ::= BIT STRING (SIZE (6))

#### -- P

Packet-LossRate	::= INTEGER(0..1000)

PA-Values ::= ENUMERATED {

dB-6,

dB-4dot77,

dB-3,

dB-1dot77,

dB0,

dB1,

dB2,

dB3,

...

}

PDCPChangeIndication ::= ENUMERATED {s-KgNB-update-required, pDCP-data-recovery-required,...}

PDCP-SN ::= INTEGER (0..4095)

PDCP-SNExtended ::= INTEGER (0..32767)

PDCP-SNlength18 ::= INTEGER (0..262143)

PDCPSnLength	::=	ENUMERATED {twelve-bits,eighteen-bits,...}

PCI ::= INTEGER (0..503, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

Port-Number ::= OCTET STRING (SIZE (2))

PRACH-Configuration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							BOOLEAN,

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)		OPTIONAL, -- present for TDD --

iE-Extensions							ProtocolExtensionContainer { {PRACH-Configuration-ExtIEs} }	OPTIONAL,

...

}

PLMNAreaBasedQMC ::= SEQUENCE {

plmnListforQMC		PLMNListforQMC,

iE-Extensions		ProtocolExtensionContainer { {PLMNAreaBasedQMC-ExtIEs} } OPTIONAL,

...

}

PLMNAreaBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

PLMNListforQMC ::= SEQUENCE (SIZE(1..maxnoofPLMNforQMC)) OF PLMN-Identity

PRACH-Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel				::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProSeAuthorized ::= SEQUENCE {

proSeDirectDiscovery		ProSeDirectDiscovery			OPTIONAL,

proSeDirectCommunication	ProSeDirectCommunication		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

ProSeAuthorized-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ProSeUEtoNetworkRelaying	CRITICALITY ignore	EXTENSION ProSeUEtoNetworkRelaying		PRESENCE optional},

...

}

ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeUEtoNetworkRelaying ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProtectedEUTRAResourceIndication::= SEQUENCE {

activationSFN							INTEGER (0..1023),

protectedResourceList					ProtectedResourceList,

mBSFNControlRegionLength				INTEGER (0..3) OPTIONAL,

pDCCHRegionLength						INTEGER (1..3) OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ProtectedEUTRAResourceIndication-ExtIEs} }	OPTIONAL,

...

}

ProtectedEUTRAResourceIndication-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

} -- Rapporteur: missing extension --

ProtectedFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimePeriodicity				INTEGER (1..320, ...),

protectedFootprintStartTime						INTEGER (1..20, ...),

iE-Extensions									ProtocolExtensionContainer { {ProtectedFootprintTimePattern-ExtIEs} }	OPTIONAL,

...

}

ProtectedFootprintTimePattern-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedResourceList ::= SEQUENCE (SIZE(1.. maxnoofProtectedResourcePatterns)) OF ProtectedResourceList-Item

ProtectedResourceList-Item ::= SEQUENCE {

resourceType					ResourceType,

intraPRBProtectedResourceFootprint 		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern 		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedFootprintTimePattern,

iE-Extensions							ProtocolExtensionContainer { {ProtectedResourceList-Item-ExtIEs} }	OPTIONAL,

...

}

ProtectedResourceList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

PartialListIndicator ::= ENUMERATED {partial, ...}

#### -- Q

QCI ::= INTEGER (0..255)

#### -- R

RadioframeAllocationOffset ::= INTEGER (0..7, ...)

RadioframeAllocationPeriod ::= ENUMERATED{

n1,

n2,

n4,

n8,

n16,

n32,

...

}

RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage							DL-Total-PRB-usage,

uL-Total-PRB-usage							UL-Total-PRB-usage,

iE-Extensions								ProtocolExtensionContainer { {RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

RadioResourceStatus-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage		PRESENCE optional}|

{ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage		PRESENCE optional},

...

}

RAN-UE-NGAP-ID ::= INTEGER (0..4294967295)

ReceiveStatusofULPDCPSDUs ::= BIT STRING (SIZE(4096))

ReceiveStatusOfULPDCPSDUsExtended ::= BIT STRING (SIZE(1..16384))

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18 ::= BIT STRING (SIZE(1..131072))

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

Registration-Request	::= ENUMERATED {

start,

stop,

...,

partial-stop,

add

}

RelativeNarrowbandTxPower ::= SEQUENCE {

rNTP-PerPRB							BIT STRING (SIZE(6..110, ...)),

rNTP-Threshold						RNTP-Threshold,

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

p-B									INTEGER (0..3,...),

pDCCH-InterferenceImpact			INTEGER (0..4,...),

iE-Extensions						ProtocolExtensionContainer { {RelativeNarrowbandTxPower-ExtIEs} } OPTIONAL,

...

}

RelativeNarrowbandTxPower-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-enhancedRNTP	CRITICALITY ignore	EXTENSION EnhancedRNTP		PRESENCE optional },

...

}

ReplacingCellsList ::= SEQUENCE (SIZE(0.. maxCellineNB)) OF ReplacingCellsList-Item

ReplacingCellsList-Item ::= SEQUENCE {

eCGI			ECGI,

...

}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, rinfinity}

ReportArea ::= ENUMERATED{

ecgi,

...

}

ReportCharacteristics	::= BIT STRING (SIZE (32))

ReportingPeriodicityCSIR ::= ENUMERATED {

ms5,

ms10,

ms20,

ms40,

ms80,

...

}

ReportingPeriodicityRSRPMR ::= ENUMERATED {

one-hundred-20-ms,

two-hundred-40-ms,

four-hundred-80-ms,

six-hundred-40-ms,

...

}

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true,...}

RequestedFastMCGRecoveryViaSRB3Release ::= ENUMERATED {true,...}

ReservedSubframePattern ::= SEQUENCE{

subframeType					SubframeType,

reservedSubframePattern			BIT STRING (SIZE(10..160)),

mBSFNControlRegionLength		INTEGER (0..3),

iE-Extensions					ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} }	OPTIONAL,

...

}

ReservedSubframePattern-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= ENUMERATED {

downlinknonCRS,

cRS,

uplink,

...

}

ResumeID	::= CHOICE {

non-truncated	BIT STRING(SIZE(40)),

truncated		BIT STRING(SIZE(24)),

...

}

RLCMode ::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions				ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,

...

}

RLC-Status-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

RNTP-Threshold ::= ENUMERATED {

minusInfinity,

minusEleven,

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

zero,

one,

two,

three,

...

}

RRC-Config-Ind ::= ENUMERATED {

full-config,

delta-config,

...

}

RRC-Context ::= OCTET STRING

RRCConnReestabIndicator ::= ENUMERATED {

reconfigurationFailure, handoverFailure, otherFailure, ...

}

-- The values correspond to the values of ReestablishmentCause reported from the UE in the RRCConnectionReestablishmentRequest, as defined in TS 36.331 [9]

RRCConnSetupIndicator::= ENUMERATED {

rrcConnSetup,

...

}

RSRPMeasurementResult ::= SEQUENCE (SIZE(1..maxCellReport)) OF

SEQUENCE {

rSRPCellID						ECGI,

rSRPMeasured					INTEGER (0..97, ...),

iE-Extensions					ProtocolExtensionContainer { {RSRPMeasurementResult-ExtIEs} } OPTIONAL,

...

}

RSRPMeasurementResult-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

RSRPMRList ::= SEQUENCE (SIZE(1..maxUEReport)) OF

SEQUENCE {

rSRPMeasurementResult			RSRPMeasurementResult,

iE-Extensions					ProtocolExtensionContainer { {RSRPMRList-ExtIEs} } OPTIONAL,

...

}

RSRPMRList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UEID	CRITICALITY ignore	EXTENSION UEID		PRESENCE optional},

...

}

RRCContainer ::= OCTET STRING

#### -- S

S1TNLLoadIndicator ::= SEQUENCE {

dLS1TNLLoadIndicator			LoadIndicator,

uLS1TNLLoadIndicator			LoadIndicator,

iE-Extensions					ProtocolExtensionContainer { {S1TNLLoadIndicator-ExtIEs} } OPTIONAL,

...

}

S1TNLLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SCGChangeIndication ::= ENUMERATED {pDCPCountWrapAround, pSCellChange, other, ...}

SecondaryRATUsageReportList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container {{SecondaryRATUsageReport-ItemIEs}}

SecondaryRATUsageReport-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-SecondaryRATUsageReport-Item		CRITICALITY reject	TYPE SecondaryRATUsageReport-Item		PRESENCE mandatory},

...

}

SecondaryRATUsageReport-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

secondaryRATType				ENUMERATED {nr, ..., nR-unlicensed },

e-RABUsageReportList			E-RABUsageReportList,

iE-Extensions				ProtocolExtensionContainer { {SecondaryRATUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

SecondaryRATUsageReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SeNBSecurityKey ::= BIT STRING (SIZE(256))

SeNBtoMeNBContainer ::= OCTET STRING

ServedCells ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

servedCellInfo					ServedCell-Information,

neighbour-Info					Neighbour-Information			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCell-ExtIEs} } OPTIONAL,

...

}

ServedCell-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRNeighbourInfoToAdd			CRITICALITY ignore	EXTENSION NRNeighbour-Information				PRESENCE optional },

...

}

ServedCell-Information ::= SEQUENCE {

pCI					PCI,

cellId				ECGI,

tAC					TAC,

broadcastPLMNs		BroadcastPLMNs-Item,

eUTRA-Mode-Info		EUTRA-Mode-Info,

iE-Extensions		ProtocolExtensionContainer { {ServedCell-Information-ExtIEs} } OPTIONAL,

...

}

ServedCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Number-of-Antennaports				CRITICALITY ignore	EXTENSION Number-of-Antennaports					PRESENCE optional}|

{ ID id-PRACH-Configuration					CRITICALITY ignore	EXTENSION PRACH-Configuration						PRESENCE optional}|

{ ID id-MBSFN-Subframe-Info					CRITICALITY ignore	EXTENSION MBSFN-Subframe-Infolist				PRESENCE optional}|

{ ID id-CSG-Id								CRITICALITY ignore	EXTENSION CSG-Id									PRESENCE optional}|

{ ID id-MBMS-Service-Area-List				CRITICALITY ignore	EXTENSION MBMS-Service-Area-Identity-List		PRESENCE optional}|

{ ID id-MultibandInfoList					CRITICALITY ignore	EXTENSION MultibandInfoList							PRESENCE optional}|

{ ID id-FreqBandIndicatorPriority			CRITICALITY ignore	EXTENSION FreqBandIndicatorPriority				PRESENCE optional}|

{ ID id-BandwidthReducedSI					CRITICALITY ignore	EXTENSION BandwidthReducedSI						PRESENCE optional}|

{ ID id-ProtectedEUTRAResourceIndication	CRITICALITY ignore	EXTENSION ProtectedEUTRAResourceIndication	PRESENCE optional}|

{ ID id-BPLMN-ID-Info-EUTRA					CRITICALITY ignore 	EXTENSION BPLMN-ID-Info-EUTRA						PRESENCE optional},

...

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

...

}

SgNBCoordinationAssistanceInformation ::= ENUMERATED{

coordination-not-required,

...

}

SgNBResourceCoordinationInformation ::= SEQUENCE {

nR-CGI							NRCGI,

uLCoordinationInformation		BIT STRING (SIZE(6..4400, ...)),

dLCoordinationInformation		BIT STRING (SIZE(6..4400, ...))	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SgNBResourceCoordinationInformationExtIEs} }		 OPTIONAL,

...

}

SgNBResourceCoordinationInformationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ECGI										CRITICALITY ignore	EXTENSION ECGI												PRESENCE optional}|

{ ID id-SgNBCoordinationAssistanceInformation		CRITICALITY reject	EXTENSION SgNBCoordinationAssistanceInformation	PRESENCE optional},

...

}

SgNB-UE-X2AP-ID ::= INTEGER (0..4294967295)

SIPTOBearerDeactivationIndication ::= ENUMERATED {

true,

...

}

SharedResourceType ::= CHOICE{

uLOnlySharing		ULOnlySharing,

uLandDLSharing		ULandDLSharing,

...

}

ShortMAC-I ::= BIT STRING (SIZE(16))

SGNB-Addition-Trigger-Ind	 ::= ENUMERATED {

sn-change,

inter-eNB-HO,

intra-eNB-HO,

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		SpecialSubframePatterns,

cyclicPrefixDL				CyclicPrefixDL,

cyclicPrefixUL				CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { {SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

SpecialSubframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxCellineNB)

SubbandCQI ::= SEQUENCE {

subbandCQICodeword0			SubbandCQICodeword0,

subbandCQICodeword1			SubbandCQICodeword1		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SubbandCQI-ExtIEs} } OPTIONAL,

...

}

Subscription-Based-UE-DifferentiationInfo ::= SEQUENCE {

periodicCommunicationIndicator	ENUMERATED {periodically, ondemand, ...}		OPTIONAL,

periodicTime					INTEGER (1..3600, ...) 							OPTIONAL,

scheduledCommunicationTime		ScheduledCommunicationTime 						OPTIONAL,

stationaryIndication			ENUMERATED {stationary, mobile, ...}			OPTIONAL,

trafficProfile 					ENUMERATED {single-packet, dual-packets, multiple-packets, ...} 			OPTIONAL,

batteryIndication				ENUMERATED {battery-powered, battery-powered-not-rechargeable-or-replaceable, not-battery-powered, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Subscription-Based-UE-DifferentiationInfo-ExtIEs} } OPTIONAL,

...

}

Subscription-Based-UE-DifferentiationInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ScheduledCommunicationTime ::= SEQUENCE {

dayofWeek				BIT STRING (SIZE(7))									OPTIONAL,

timeofDayStart			INTEGER (0..86399, ...) 								OPTIONAL,

timeofDayEnd			INTEGER (0..86399, ...) 								OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { ScheduledCommunicationTime-ExtIEs}}	OPTIONAL,

...

}

ScheduledCommunicationTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SRVCCOperationPossible ::= ENUMERATED {

possible,

...

}

SubbandCQI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SubbandCQICodeword0 ::= CHOICE {

four-bitCQI						INTEGER (0..15, ...),

two-bitSubbandDifferentialCQI	INTEGER (0..3, ...),

two-bitDifferentialCQI			INTEGER (0..3, ...),

...

}

SubbandCQICodeword1 ::= CHOICE {

four-bitCQI							INTEGER (0..15, ...),

three-bitSpatialDifferentialCQI		INTEGER (0..7, ...),

two-bitSubbandDifferentialCQI		INTEGER (0..3, ...),

two-bitDifferentialCQI				INTEGER (0..3, ...),

...

}

SubbandCQIList ::= SEQUENCE (SIZE(1.. maxSubband)) OF SubbandCQIItem

SubbandCQIItem ::= SEQUENCE {

subbandCQI			SubbandCQI,

subbandIndex		INTEGER (0..27,...),

iE-Extensions		ProtocolExtensionContainer { {SubbandCQIItem-ExtIEs} } OPTIONAL,

...

}

SubbandCQIItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SubbandSize ::= ENUMERATED {

size2,

size3,

size4,

size6,

size8,

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256)

SubframeAllocation ::= CHOICE {

oneframe						Oneframe,

fourframes						Fourframes,

...

}

SubframeAssignment ::= ENUMERATED {

sa0,

sa1,

sa2,

sa3,

sa4,

sa5,

sa6,

...

}

SubframeType ::= ENUMERATED{mbsfn,nonmbsfn,...}

SgNBSecurityKey ::= BIT STRING (SIZE(256))

SgNBtoMeNBContainer ::= OCTET STRING

SRBType ::= ENUMERATED {srb1, srb2, ...}

SCGConfigurationQuery ::= ENUMERATED {true,...}

SULInformation ::= SEQUENCE {

sUL-ARFCN				INTEGER (0.. 3279165),

sUL-TxBW				NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {SULInformation-ExtIEs} }		OPTIONAL,

...

}

SupportedSULFreqBandItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

iE-Extensions			ProtocolExtensionContainer { {SupportedSULFreqBandItem-ExtIEs} }		OPTIONAL,

...

}

SupportedSULFreqBandItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SULInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TABasedMDT::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (2))

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI-Item

TAI-Item ::= SEQUENCE {

tAC					TAC,

pLMN-Identity		PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TABasedQMC ::= SEQUENCE {

tAListforQMC		TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC		TAIListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item

TargetCellInUTRAN ::= OCTET STRING -- This IE is to be encoded according to the UTRAN Cell ID in the Last Visited UTRAN Cell Information IE in TS 25.413 [24]

TargeteNBtoSource-eNBTransparentContainer	::= OCTET STRING

TDD-Info ::= SEQUENCE {

eARFCN							EARFCN,

transmission-Bandwidth			Transmission-Bandwidth,

subframeAssignment				SubframeAssignment,

specialSubframe-Info			SpecialSubframe-Info,

iE-Extensions					ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalSpecialSubframe-Info			CRITICALITY ignore	EXTENSION AdditionalSpecialSubframe-Info				PRESENCE optional}|

{ ID id-eARFCNExtension							CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-AdditionalSpecialSubframeExtension-Info	CRITICALITY ignore	EXTENSION AdditionalSpecialSubframeExtension-Info	PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NBIoT-UL-DL-AlignmentOffset				CRITICALITY reject	EXTENSION NBIoT-UL-DL-AlignmentOffset					PRESENCE optional},

...

}

TDD-InfoNeighbourServedNRCell-Information ::= SEQUENCE {

nRFreqInfo				NRFreqInfo,

iE-Extensions			ProtocolExtensionContainer { {TDD-InfoNeighbourServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoNeighbourServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Threshold-RSRP ::= INTEGER(0..97)

Threshold-RSRQ ::= INTEGER(0..34)

TimeToWait ::= ENUMERATED {

v1s,

v2s,

v5s,

v10s,

v20s,

v60s,

...

}

Time-UE-StayedInCell ::= INTEGER (0..4095)

Time-UE-StayedInCell-EnhancedGranularity ::= INTEGER (0..40950)

TNLA-To-Add-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Add-Item

TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Add-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Update-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Update-Item

TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage 		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Update-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Remove-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Remove-Item

TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Remove-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Setup-Item

TNLA-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

TNLA-Setup-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Failed-To-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Failed-To-Setup-Item

TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

TNLA-Failed-To-Setup-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Add-Item

Transport-UP-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Remove-Item

Transport-UP-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLConfigurationInfo  ::= SEQUENCE {

transport-UP-Layer-Addresses-Info-To-Add-List		Transport-UP-Layer-Addresses-Info-To-Add-List						OPTIONAL,

transport-UP-Layer-Addresses-Info-To-Remove-List	Transport-UP-Layer-Addresses-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { TNLConfigurationInfo-ExtIEs } }											OPTIONAL,

...

}

TNLConfigurationInfo-ExtIEs 	X2AP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

eUTRANTraceID					EUTRANTraceID,

interfacesToTrace				InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TraceCollectionEntityIPAddress,

iE-Extensions					ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration			CRITICALITY ignore	EXTENSION MDT-Configuration			PRESENCE optional}|

{ ID id-UEAppLayerMeasConfig		CRITICALITY ignore	EXTENSION UEAppLayerMeasConfig		PRESENCE optional},

...

}

TraceCollectionEntityIPAddress ::= BIT STRING (SIZE(1..160, ...))

TraceDepth 		::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

Transmission-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...,

bw1

}

TransportLayerAddress			::= BIT STRING (SIZE(1..160, ...))

TransportLayerAddressAndPort		::= SEQUENCE {

endpointIPAddress				TransportLayerAddress,

portnumber						Port-Number

}

TunnelInformation ::= SEQUENCE {

transportLayerAddress	TransportLayerAddress,

uDP-Port-Number			Port-Number			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {Tunnel-Information-ExtIEs} } OPTIONAL,

...

}

Tunnel-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEaggregateMaximumBitRateDownlink	BitRate,

uEaggregateMaximumBitRateUplink		BitRate,

iE-Extensions						ProtocolExtensionContainer { {UEAggregate-MaximumBitrate-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-extended-uEaggregateMaximumBitRateDownlink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-uEaggregateMaximumBitRateUplink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

UEAppLayerMeasConfig ::= SEQUENCE {

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..1000)),

areaScopeOfQMC		AreaScopeOfQMC,

iE-Extensions		ProtocolExtensionContainer { {UEAppLayerMeasConfig-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasConfig-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-serviceType	CRITICALITY ignore	EXTENSION ServiceType	PRESENCE optional},

...

}

UE-ContextKeptIndicator ::= ENUMERATED {

true,

...

}

UEID ::= BIT STRING (SIZE (16))

UE-HistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCells)) OF LastVisitedCell-Item

UE-HistoryInformationFromTheUE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the VisitedCellInfoList field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-S1AP-ID ::= INTEGER (0.. 4294967295)

UE-X2AP-ID ::= INTEGER (0..4095)

UE-X2AP-ID-Extension ::= INTEGER (0..4095, ...)

UE-RLF-Report-Container::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-RLF-Report-Container-for-extended-bands ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-v9e0 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UESecurityCapabilities ::= SEQUENCE {

encryptionAlgorithms				EncryptionAlgorithms,

integrityProtectionAlgorithms		IntegrityProtectionAlgorithms,

iE-Extensions						ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} }		OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {UE-Sidelink-Aggregate-MaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UE-Sidelink-Aggregate-MaximumBitRate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UEsToBeResetList ::= SEQUENCE (SIZE (1.. maxUEsinengNBDU)) OF UEsToBeResetList-Item

UEsToBeResetList-Item::= SEQUENCE {

meNB-ID					UE-X2AP-ID,

meNB-ID-ext				UE-X2AP-ID-Extension														OPTIONAL,

sgNB-ID					SgNB-UE-X2AP-ID																OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {UEsToBeResetList-Item-ExtIEs} }				OPTIONAL,

...

}

UEsToBeResetList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULandDLSharing ::= SEQUENCE{

uLResourcesULandDLSharing				ULResourcesULandDLSharing,

dLResourcesULandDLSharing				DLResourcesULandDLSharing,

iE-Extensions			ProtocolExtensionContainer { {ULandDLSharing-ExtIEs} }				OPTIONAL,

...

}

ULandDLSharing-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULConfiguration::= SEQUENCE {

uL-PDCP			UL-UE-Configuration,

iE-Extensions					ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,

...

}

ULConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UL-UE-Configuration::= ENUMERATED { no-data, shared, only, ... }

UL-GBR-PRB-usage::= INTEGER (0..100)

UL-HighInterferenceIndicationInfo ::= SEQUENCE (SIZE(1..maxCellineNB)) OF UL-HighInterferenceIndicationInfo-Item

UL-HighInterferenceIndicationInfo-Item ::= SEQUENCE {

target-Cell-ID					ECGI,

ul-interferenceindication		UL-HighInterferenceIndication,

iE-Extensions					ProtocolExtensionContainer { {UL-HighInterferenceIndicationInfo-Item-ExtIEs} } OPTIONAL,

...

}

UL-HighInterferenceIndicationInfo-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UL-HighInterferenceIndication ::= BIT STRING (SIZE(1..110, ...))

UL-InterferenceOverloadIndication ::= SEQUENCE (SIZE(1..maxnoofPRBs)) OF UL-InterferenceOverloadIndication-Item

UL-InterferenceOverloadIndication-Item ::= ENUMERATED {

high-interference,

medium-interference,

low-interference,

...

}

UL-non-GBR-PRB-usage::= INTEGER (0..100)

ULOnlySharing ::= SEQUENCE{

uLResourceBitmapULOnlySharing	DataTrafficResources,

iE-Extensions					ProtocolExtensionContainer { {ULOnlySharing-ExtIEs} }				OPTIONAL,

...

}

ULOnlySharing-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULResourceBitmapULandDLSharing ::= DataTrafficResources

ULResourcesULandDLSharing ::= CHOICE {

unchanged			NULL,

changed				ULResourceBitmapULandDLSharing,

...

}

UL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)

UnlicensedSpectrumRestriction ::= ENUMERATED {

unlicensed-restricted,

...

}

UsableABSInformation ::= CHOICE {

fdd					UsableABSInformationFDD,

tdd					UsableABSInformationTDD,

...

}

UsableABSInformationFDD ::= SEQUENCE {

usable-abs-pattern-info				BIT STRING (SIZE(40)),

iE-Extensions						ProtocolExtensionContainer { {UsableABSInformationFDD-ExtIEs} } OPTIONAL,

...

}

UsableABSInformationFDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UsableABSInformationTDD ::= SEQUENCE {

usaable-abs-pattern-info			BIT STRING (SIZE(1..70, ...)),

iE-Extensions						ProtocolExtensionContainer { {UsableABSInformationTDD-ExtIEs} } OPTIONAL,

...

}

UsableABSInformationTDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

#### -- V

V2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 			PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {V2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

V2XServicesAuthorized-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

#### -- W

WidebandCQI ::= SEQUENCE {

widebandCQICodeword0		INTEGER (0..15, ...),

widebandCQICodeword1		WidebandCQICodeword1		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {WidebandCQI-ExtIEs} } OPTIONAL,

...

}

WidebandCQI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

WidebandCQICodeword1::= CHOICE {

four-bitCQI								INTEGER (0..15, ...),

three-bitSpatialDifferentialCQI			INTEGER (0..7, ...),

...

}

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig				WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList				OPTIONAL,

wlan-rssi					ENUMERATED {true, ...}				OPTIONAL,

wlan-rtt					ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {WLANMeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

WTID ::= CHOICE {

wTID-Type1			WTID-Type1,

wTID-Type2			WTID-Long-Type2,

...

}

WTID-Type1 ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

shortWTID						BIT STRING (SIZE(24)),

...

}

WTID-Long-Type2 ::= BIT STRING (SIZE(48))

WT-UE-XwAP-ID ::= OCTET STRING (SIZE (3))

#### -- X

X2BenefitValue ::= INTEGER (1..8, ...)

#### -- Y

#### -- Z

END

-- ASN1STOP

9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

X2AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-handoverCancel												ProcedureCode ::= 1

id-loadIndication												ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-snStatusTransfer												ProcedureCode ::= 4

id-uEContextRelease												ProcedureCode ::= 5

id-x2Setup														ProcedureCode ::= 6

id-reset														ProcedureCode ::= 7

id-eNBConfigurationUpdate										ProcedureCode ::= 8

id-resourceStatusReportingInitiation							ProcedureCode ::= 9

id-resourceStatusReporting										ProcedureCode ::= 10

id-privateMessage												ProcedureCode ::= 11

id-mobilitySettingsChange										ProcedureCode ::= 12

id-rLFIndication												ProcedureCode ::= 13

id-handoverReport												ProcedureCode ::= 14

id-cellActivation												ProcedureCode ::= 15

id-x2Release													ProcedureCode ::= 16

id-x2APMessageTransfer											ProcedureCode ::= 17

id-x2Removal													ProcedureCode ::= 18

id-seNBAdditionPreparation										ProcedureCode ::= 19

id-seNBReconfigurationCompletion								ProcedureCode ::= 20

id-meNBinitiatedSeNBModificationPreparation						ProcedureCode ::= 21

id-seNBinitiatedSeNBModification								ProcedureCode ::= 22

id-meNBinitiatedSeNBRelease										ProcedureCode ::= 23

id-seNBinitiatedSeNBRelease										ProcedureCode ::= 24

id-seNBCounterCheck												ProcedureCode ::= 25

id-retrieveUEContext											ProcedureCode ::= 26

id-sgNBAdditionPreparation										ProcedureCode ::= 27

id-sgNBReconfigurationCompletion									ProcedureCode ::= 28

id-meNBinitiatedSgNBModificationPreparation						ProcedureCode ::= 29

id-sgNBinitiatedSgNBModification									ProcedureCode ::= 30

id-meNBinitiatedSgNBRelease										ProcedureCode ::= 31

id-sgNBinitiatedSgNBRelease										ProcedureCode ::= 32

id-sgNBCounterCheck												ProcedureCode ::= 33

id-sgNBChange													ProcedureCode ::= 34

id-rRCTransfer													ProcedureCode ::= 35

id-endcX2Setup													ProcedureCode ::= 36

id-endcConfigurationUpdate										ProcedureCode ::= 37

id-secondaryRATDataUsageReport									ProcedureCode ::= 38

id-endcCellActivation											ProcedureCode ::= 39

id-endcPartialReset												ProcedureCode ::= 40

id-eUTRANRCellResourceCoordination								ProcedureCode ::= 41

id-SgNBActivityNotification										ProcedureCode ::= 42

id-endcX2Removal												ProcedureCode ::= 43

id-dataForwardingAddressIndication								ProcedureCode ::= 44

id-gNBStatusIndication											ProcedureCode ::= 45

id-deactivateTrace												ProcedureCode ::= 46

id-traceStart													ProcedureCode ::= 47

id-endcConfigurationTransfer										ProcedureCode ::= 48

id-handoverSuccess												ProcedureCode ::= XX

id-conditionalHandoverCancel										ProcedureCode ::= yy

id-earlyStatusTransfer										ProcedureCode ::= ZZ

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 65535

maxEARFCNPlusOne							INTEGER ::= 65536

newmaxEARFCN								INTEGER ::= 262143

maxInterfaces								INTEGER ::= 16

maxCellineNB								INTEGER ::= 256

maxnoofBands								INTEGER ::= 16

maxnoofBearers								INTEGER ::= 256

maxNrOfErrors								INTEGER ::= 256

maxnoofPDCP-SN								INTEGER ::= 16

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofForbLACs								INTEGER ::= 4096

maxnoofForbTACs								INTEGER ::= 4096

maxnoofBPLMNs								INTEGER ::= 6

maxnoofAdditionalPLMNs						INTEGER ::= 6

maxnoofNeighbours							INTEGER ::= 512

maxnoofPRBs									INTEGER ::= 110

maxPools									INTEGER ::= 16

maxnoofCells								INTEGER ::= 16

maxnoofMBSFN								INTEGER ::= 8

maxFailedMeasObjects						INTEGER ::= 32

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofTAforMDT								INTEGER ::= 8

maxnoofMBMSServiceAreaIdentities			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofCoMPHypothesisSet					INTEGER ::= 256

maxnoofCoMPCells							INTEGER ::= 32

maxUEReport									INTEGER ::= 128

maxCellReport								INTEGER ::= 9

maxnoofPA									INTEGER ::= 3

maxCSIProcess								INTEGER ::= 4

maxCSIReport								INTEGER ::= 2

maxSubband									INTEGER ::= 14

maxofNRNeighbours							INTEGER ::= 1024

maxCellinengNB								INTEGER ::= 16384

--	maxnoofNRCarriers						INTEGER ::= 32

maxnooftimeperiods							INTEGER ::= 2

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofTAforQMC								INTEGER ::= 8

maxnoofPLMNforQMC							INTEGER ::= 16

maxUEsinengNBDU								INTEGER ::= 8192

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoNRcellsSpectrumSharingWithE-UTRA		INTEGER ::= 64

maxnoofNrCellBands							INTEGER ::= 32

maxnoofBluetoothName						INTEGER ::= 4

maxnoofWLANName								INTEGER ::= 4

maxnoofextBPLMNs							INTEGER ::= 12

maxnoofextBPLMNsminus1						INTEGER ::= 11

maxnoofBPLMNsminus1							INTEGER ::= 11

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofCellsinCHO							INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

id-CellInformation															ProtocolIE-ID ::= 6

id-CellInformation-Item														ProtocolIE-ID ::= 7

id-New-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 9

id-Old-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 10

id-TargetCell-ID															ProtocolIE-ID ::= 11

id-TargeteNBtoSource-eNBTransparentContainer								ProtocolIE-ID ::= 12

id-TraceActivation															ProtocolIE-ID ::= 13

id-UE-ContextInformation													ProtocolIE-ID ::= 14

id-UE-HistoryInformation													ProtocolIE-ID ::= 15

id-UE-X2AP-ID																ProtocolIE-ID ::= 16

id-CriticalityDiagnostics													ProtocolIE-ID ::= 17

id-E-RABs-SubjectToStatusTransfer-List										ProtocolIE-ID ::= 18

id-E-RABs-SubjectToStatusTransfer-Item										ProtocolIE-ID ::= 19

id-ServedCells																ProtocolIE-ID ::= 20

id-GlobalENB-ID																ProtocolIE-ID ::= 21

id-TimeToWait																ProtocolIE-ID ::= 22

id-GUMMEI-ID																ProtocolIE-ID ::= 23

id-GUGroupIDList															ProtocolIE-ID ::= 24

id-ServedCellsToAdd															ProtocolIE-ID ::= 25

id-ServedCellsToModify														ProtocolIE-ID ::= 26

id-ServedCellsToDelete														ProtocolIE-ID ::= 27

id-Registration-Request														ProtocolIE-ID ::= 28

id-CellToReport																ProtocolIE-ID ::= 29

id-ReportingPeriodicity														ProtocolIE-ID ::= 30

id-CellToReport-Item														ProtocolIE-ID ::= 31

id-CellMeasurementResult													ProtocolIE-ID ::= 32

id-CellMeasurementResult-Item												ProtocolIE-ID ::= 33

id-GUGroupIDToAddList														ProtocolIE-ID ::= 34

id-GUGroupIDToDeleteList													ProtocolIE-ID ::= 35

id-SRVCCOperationPossible													ProtocolIE-ID ::= 36

id-Measurement-ID															ProtocolIE-ID ::= 37

id-ReportCharacteristics													ProtocolIE-ID ::= 38

id-ENB1-Measurement-ID														ProtocolIE-ID ::= 39

id-ENB2-Measurement-ID														ProtocolIE-ID ::= 40

id-Number-of-Antennaports													ProtocolIE-ID ::= 41

id-CompositeAvailableCapacityGroup											ProtocolIE-ID ::= 42

id-ENB1-Cell-ID																ProtocolIE-ID ::= 43

id-ENB2-Cell-ID																ProtocolIE-ID ::= 44

id-ENB2-Proposed-Mobility-Parameters										ProtocolIE-ID ::= 45

id-ENB1-Mobility-Parameters													ProtocolIE-ID ::= 46

id-ENB2-Mobility-Parameters-Modification-Range								ProtocolIE-ID ::= 47

id-FailureCellPCI															ProtocolIE-ID ::= 48

id-Re-establishmentCellECGI													ProtocolIE-ID ::= 49

id-FailureCellCRNTI															ProtocolIE-ID ::= 50

id-ShortMAC-I																ProtocolIE-ID ::= 51

id-SourceCellECGI															ProtocolIE-ID ::= 52

id-FailureCellECGI															ProtocolIE-ID ::= 53

id-HandoverReportType														ProtocolIE-ID ::= 54

id-PRACH-Configuration														ProtocolIE-ID ::= 55

id-MBSFN-Subframe-Info														ProtocolIE-ID ::= 56

id-ServedCellsToActivate													ProtocolIE-ID ::= 57

id-ActivatedCellList														ProtocolIE-ID ::= 58

id-DeactivationIndication													ProtocolIE-ID ::= 59

id-UE-RLF-Report-Container													ProtocolIE-ID ::= 60

id-ABSInformation															ProtocolIE-ID ::= 61

id-InvokeIndication															ProtocolIE-ID ::= 62

id-ABS-Status																ProtocolIE-ID ::= 63

id-PartialSuccessIndicator													ProtocolIE-ID ::= 64

id-MeasurementInitiationResult-List											ProtocolIE-ID ::= 65

id-MeasurementInitiationResult-Item											ProtocolIE-ID ::= 66

id-MeasurementFailureCause-Item												ProtocolIE-ID ::= 67

id-CompleteFailureCauseInformation-List										ProtocolIE-ID ::= 68

id-CompleteFailureCauseInformation-Item										ProtocolIE-ID ::= 69

id-CSG-Id																	ProtocolIE-ID ::= 70

id-CSGMembershipStatus														ProtocolIE-ID ::= 71

id-MDTConfiguration															ProtocolIE-ID ::= 72

id-ManagementBasedMDTallowed												ProtocolIE-ID ::= 74

id-RRCConnSetupIndicator													ProtocolIE-ID ::= 75

id-NeighbourTAC																ProtocolIE-ID ::= 76

id-Time-UE-StayedInCell-EnhancedGranularity									ProtocolIE-ID ::= 77

id-RRCConnReestabIndicator													ProtocolIE-ID ::= 78

id-MBMS-Service-Area-List													ProtocolIE-ID ::= 79

id-HO-cause																	ProtocolIE-ID ::= 80

id-TargetCellInUTRAN														ProtocolIE-ID ::= 81

id-MobilityInformation														ProtocolIE-ID ::= 82

id-SourceCellCRNTI															ProtocolIE-ID ::= 83

id-MultibandInfoList														ProtocolIE-ID ::= 84

id-M3Configuration															ProtocolIE-ID ::= 85

id-M4Configuration															ProtocolIE-ID ::= 86

id-M5Configuration															ProtocolIE-ID ::= 87

id-MDT-Location-Info														ProtocolIE-ID ::= 88

id-ManagementBasedMDTPLMNList												ProtocolIE-ID ::= 89

id-SignallingBasedMDTPLMNList												ProtocolIE-ID ::= 90

id-ReceiveStatusOfULPDCPSDUsExtended										ProtocolIE-ID ::= 91

id-ULCOUNTValueExtended														ProtocolIE-ID ::= 92

id-DLCOUNTValueExtended														ProtocolIE-ID ::= 93

id-eARFCNExtension															ProtocolIE-ID ::= 94

id-UL-EARFCNExtension														ProtocolIE-ID ::= 95

id-DL-EARFCNExtension														ProtocolIE-ID ::= 96

id-AdditionalSpecialSubframe-Info											ProtocolIE-ID ::= 97

id-Masked-IMEISV															ProtocolIE-ID ::= 98

id-IntendedULDLConfiguration												ProtocolIE-ID ::= 99

id-ExtendedULInterferenceOverloadInfo										ProtocolIE-ID ::= 100

id-RNL-Header																ProtocolIE-ID ::= 101

id-x2APMessage																ProtocolIE-ID ::= 102

id-ProSeAuthorized															ProtocolIE-ID ::= 103

id-ExpectedUEBehaviour														ProtocolIE-ID ::= 104

id-UE-HistoryInformationFromTheUE											ProtocolIE-ID ::= 105

id-DynamicDLTransmissionInformation											ProtocolIE-ID ::= 106

id-UE-RLF-Report-Container-for-extended-bands								ProtocolIE-ID ::= 107

id-CoMPInformation															ProtocolIE-ID ::= 108

id-ReportingPeriodicityRSRPMR												ProtocolIE-ID ::= 109

id-RSRPMRList																ProtocolIE-ID ::= 110

id-MeNB-UE-X2AP-ID															ProtocolIE-ID ::= 111

id-SeNB-UE-X2AP-ID															ProtocolIE-ID ::= 112

id-UE-SecurityCapabilities													ProtocolIE-ID ::= 113

id-SeNBSecurityKey															ProtocolIE-ID ::= 114

id-SeNBUEAggregateMaximumBitRate											ProtocolIE-ID ::= 115

id-ServingPLMN																ProtocolIE-ID ::= 116

id-E-RABs-ToBeAdded-List													ProtocolIE-ID ::= 117

id-E-RABs-ToBeAdded-Item													ProtocolIE-ID ::= 118

id-MeNBtoSeNBContainer														ProtocolIE-ID ::= 119

id-E-RABs-Admitted-ToBeAdded-List											ProtocolIE-ID ::= 120

id-E-RABs-Admitted-ToBeAdded-Item											ProtocolIE-ID ::= 121

id-SeNBtoMeNBContainer														ProtocolIE-ID ::= 122

id-ResponseInformationSeNBReconfComp										ProtocolIE-ID ::= 123

id-UE-ContextInformationSeNBModReq											ProtocolIE-ID ::= 124

id-E-RABs-ToBeAdded-ModReqItem												ProtocolIE-ID ::= 125

id-E-RABs-ToBeModified-ModReqItem											ProtocolIE-ID ::= 126

id-E-RABs-ToBeReleased-ModReqItem											ProtocolIE-ID ::= 127

id-E-RABs-Admitted-ToBeAdded-ModAckList										ProtocolIE-ID ::= 128

id-E-RABs-Admitted-ToBeModified-ModAckList									ProtocolIE-ID ::= 129

id-E-RABs-Admitted-ToBeReleased-ModAckList									ProtocolIE-ID ::= 130

id-E-RABs-Admitted-ToBeAdded-ModAckItem										ProtocolIE-ID ::= 131

id-E-RABs-Admitted-ToBeModified-ModAckItem									ProtocolIE-ID ::= 132

id-E-RABs-Admitted-ToBeReleased-ModAckItem									ProtocolIE-ID ::= 133

id-E-RABs-ToBeReleased-ModReqd												ProtocolIE-ID ::= 134

id-E-RABs-ToBeReleased-ModReqdItem											ProtocolIE-ID ::= 135

id-SCGChangeIndication														ProtocolIE-ID ::= 136

id-E-RABs-ToBeReleased-List-RelReq											ProtocolIE-ID ::= 137

id-E-RABs-ToBeReleased-RelReqItem											ProtocolIE-ID ::= 138

id-E-RABs-ToBeReleased-List-RelConf											ProtocolIE-ID ::= 139

id-E-RABs-ToBeReleased-RelConfItem											ProtocolIE-ID ::= 140

id-E-RABs-SubjectToCounterCheck-List										ProtocolIE-ID ::= 141

id-E-RABs-SubjectToCounterCheckItem											ProtocolIE-ID ::= 142

id-CoverageModificationList													ProtocolIE-ID ::= 143

id-ReportingPeriodicityCSIR													ProtocolIE-ID ::= 145

id-CSIReportList															ProtocolIE-ID ::= 146

id-UEID																		ProtocolIE-ID ::= 147

id-enhancedRNTP																ProtocolIE-ID ::= 148

id-ProSeUEtoNetworkRelaying													ProtocolIE-ID ::= 149

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18									ProtocolIE-ID ::= 150

id-ULCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 151

id-DLCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 152

id-UE-ContextReferenceAtSeNB												ProtocolIE-ID ::= 153

id-UE-ContextKeptIndicator													ProtocolIE-ID ::= 154

id-New-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 155

id-Old-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 156

id-MeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 157

id-SeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 158

id-LHN-ID																	ProtocolIE-ID ::= 159

id-FreqBandIndicatorPriority												ProtocolIE-ID ::= 160

id-M6Configuration															ProtocolIE-ID ::= 161

id-M7Configuration															ProtocolIE-ID ::= 162

id-Tunnel-Information-for-BBF												ProtocolIE-ID ::= 163

id-SIPTO-BearerDeactivationIndication										ProtocolIE-ID ::= 164

id-GW-TransportLayerAddress													ProtocolIE-ID ::= 165

id-Correlation-ID															ProtocolIE-ID ::= 166

id-SIPTO-Correlation-ID														ProtocolIE-ID ::= 167

id-SIPTO-L-GW-TransportLayerAddress											ProtocolIE-ID ::= 168

id-X2RemovalThreshold														ProtocolIE-ID ::= 169

id-CellReportingIndicator													ProtocolIE-ID ::= 170

id-BearerType																ProtocolIE-ID ::= 171

id-resumeID																	ProtocolIE-ID ::= 172

id-UE-ContextInformationRetrieve											ProtocolIE-ID ::= 173

id-E-RABs-ToBeSetupRetrieve-Item											ProtocolIE-ID ::= 174

id-NewEUTRANCellIdentifier													ProtocolIE-ID ::= 175

id-V2XServicesAuthorized													ProtocolIE-ID ::= 176

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-DL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 193

id-UL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 194

id-UEAppLayerMeasConfig														ProtocolIE-ID ::= 195

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 196

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 197

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 198

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 199

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 200

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 201

id-NRrestrictioninEPSasSecondaryRAT											ProtocolIE-ID ::= 202

id-SgNBSecurityKey															ProtocolIE-ID ::= 203

id-SgNBUEAggregateMaximumBitRate												ProtocolIE-ID ::= 204

id-E-RABs-ToBeAdded-SgNBAddReqList											ProtocolIE-ID ::= 205

id-MeNBtoSgNBContainer														ProtocolIE-ID ::= 206

id-SgNB-UE-X2AP-ID															ProtocolIE-ID ::= 207

id-RequestedSplitSRBs														ProtocolIE-ID ::= 208

id-E-RABs-ToBeAdded-SgNBAddReq-Item											ProtocolIE-ID ::= 209

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList								ProtocolIE-ID ::= 210

id-SgNBtoMeNBContainer														ProtocolIE-ID ::= 211

id-AdmittedSplitSRBs															ProtocolIE-ID ::= 212

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item								ProtocolIE-ID ::= 213

id-ResponseInformationSgNBReconfComp											ProtocolIE-ID ::= 214

id-UE-ContextInformation-SgNBModReq											ProtocolIE-ID ::= 215

id-E-RABs-ToBeAdded-SgNBModReq-Item											ProtocolIE-ID ::= 216

id-E-RABs-ToBeModified-SgNBModReq-Item										ProtocolIE-ID ::= 217

id-E-RABs-ToBeReleased-SgNBModReq-Item										ProtocolIE-ID ::= 218

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList									ProtocolIE-ID ::= 219

id-E-RABs-Admitted-ToBeModified-SgNBModAckList								ProtocolIE-ID ::= 220

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList								ProtocolIE-ID ::= 221

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item									ProtocolIE-ID ::= 222

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item								ProtocolIE-ID ::= 223

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item								ProtocolIE-ID ::= 224

id-E-RABs-ToBeReleased-SgNBModReqdList										ProtocolIE-ID ::= 225

id-E-RABs-ToBeModified-SgNBModReqdList										ProtocolIE-ID ::= 226

id-E-RABs-ToBeReleased-SgNBModReqd-Item										ProtocolIE-ID ::= 227

id-E-RABs-ToBeModified-SgNBModReqd-Item										ProtocolIE-ID ::= 228

id-E-RABs-ToBeReleased-SgNBChaConfList										ProtocolIE-ID ::= 229

id-E-RABs-ToBeReleased-SgNBChaConf-Item										ProtocolIE-ID ::= 230

id-E-RABs-ToBeReleased-SgNBRelReqList										ProtocolIE-ID ::= 231

id-E-RABs-ToBeReleased-SgNBRelReq-Item										ProtocolIE-ID ::= 232

id-E-RABs-ToBeReleased-SgNBRelConfList										ProtocolIE-ID ::= 233

id-E-RABs-ToBeReleased-SgNBRelConf-Item										ProtocolIE-ID ::= 234

id-E-RABs-SubjectToSgNBCounterCheck-List										ProtocolIE-ID ::= 235

id-E-RABs-SubjectToSgNBCounterCheck-Item										ProtocolIE-ID ::= 236

id-RRCContainer																ProtocolIE-ID ::= 237

id-SRBType																	ProtocolIE-ID ::= 238

id-Target-SgNB-ID															ProtocolIE-ID ::= 239

id-HandoverRestrictionList													ProtocolIE-ID ::= 240

id-SCGConfigurationQuery														ProtocolIE-ID ::= 241

id-SplitSRB																	ProtocolIE-ID ::= 242

id-NRUeReport																ProtocolIE-ID ::= 243

id-InitiatingNodeType-EndcX2Setup											ProtocolIE-ID ::= 244

id-InitiatingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 245

id-RespondingNodeType-EndcX2Setup											ProtocolIE-ID ::= 246

id-RespondingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 247

id-NRUESecurityCapabilities													ProtocolIE-ID ::= 248

id-PDCPChangeIndication														ProtocolIE-ID ::= 249

id-ServedEUTRAcellsENDCX2ManagementList										ProtocolIE-ID ::= 250

id-CellAssistanceInformation													ProtocolIE-ID ::= 251

id-Globalen-gNB-ID															ProtocolIE-ID ::= 252

id-ServedNRcellsENDCX2ManagementList											ProtocolIE-ID ::= 253

id-UE-ContextReferenceAtSgNB													ProtocolIE-ID ::= 254

id-SecondaryRATUsageReport													ProtocolIE-ID ::= 255

id-ActivationID																ProtocolIE-ID ::= 256

id-MeNBResourceCoordinationInformation										ProtocolIE-ID ::= 257

id-SgNBResourceCoordinationInformation										ProtocolIE-ID ::= 258

id-ServedEUTRAcellsToModifyListENDCConfUpd									ProtocolIE-ID ::= 259

id-ServedEUTRAcellsToDeleteListENDCConfUpd									ProtocolIE-ID ::= 260

id-ServedNRcellsToModifyListENDCConfUpd										ProtocolIE-ID ::= 261

id-ServedNRcellsToDeleteListENDCConfUpd										ProtocolIE-ID ::= 262

id-E-RABUsageReport-Item														ProtocolIE-ID ::= 263

id-Old-SgNB-UE-X2AP-ID														ProtocolIE-ID ::= 264

id-SecondaryRATUsageReportList												ProtocolIE-ID ::= 265

id-SecondaryRATUsageReport-Item												ProtocolIE-ID ::= 266

id-ServedNRCellsToActivate													ProtocolIE-ID ::= 267

id-ActivatedNRCellList														ProtocolIE-ID ::= 268

id-SelectedPLMN																ProtocolIE-ID ::= 269

id-UEs-ToBeReset															ProtocolIE-ID ::= 270

id-UEs-Admitted-ToBeReset													ProtocolIE-ID ::= 271

id-RRCConfigIndication														ProtocolIE-ID ::= 272

id-DownlinkPacketLossRate													ProtocolIE-ID ::= 273

id-UplinkPacketLossRate														ProtocolIE-ID ::= 274

id-SubscriberProfileIDforRFP												ProtocolIE-ID ::= 275

id-serviceType																ProtocolIE-ID ::= 276

id-AerialUEsubscriptionInformation											ProtocolIE-ID ::= 277

id-SGNB-Addition-Trigger-Ind												ProtocolIE-ID ::= 278

id-MeNBCell-ID																ProtocolIE-ID ::= 279

id-RequestedSplitSRBsrelease												ProtocolIE-ID ::= 280

id-AdmittedSplitSRBsrelease													ProtocolIE-ID ::= 281

id-NRS-NSSS-PowerOffset														ProtocolIE-ID ::= 282

id-NSSS-NumOccasionDifferentPrecoder										ProtocolIE-ID ::= 283

id-ProtectedEUTRAResourceIndication											ProtocolIE-ID ::= 284

id-InitiatingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 285

id-RespondingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 286

id-DataTrafficResourceIndication											ProtocolIE-ID ::= 287

id-SpectrumSharingGroupID													ProtocolIE-ID ::= 288

id-ListofEUTRACellsinEUTRACoordinationReq									ProtocolIE-ID ::= 289

id-ListofEUTRACellsinEUTRACoordinationResp									ProtocolIE-ID ::= 290

id-ListofEUTRACellsinNRCoordinationReq										ProtocolIE-ID ::= 291

id-ListofNRCellsinNRCoordinationReq											ProtocolIE-ID ::= 292

id-ListofNRCellsinNRCoordinationResp										ProtocolIE-ID ::= 293

id-E-RABs-AdmittedToBeModified-SgNBModConfList								ProtocolIE-ID ::= 294

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item								ProtocolIE-ID ::= 295

id-UEContextLevelUserPlaneActivity											ProtocolIE-ID ::= 296

id-ERABActivityNotifyItemList												ProtocolIE-ID ::= 297

id-InitiatingNodeType-EndcX2Removal											ProtocolIE-ID ::= 298

id-RespondingNodeType-EndcX2Removal											ProtocolIE-ID ::= 299

id-RLC-Status																ProtocolIE-ID ::= 300

id-CNTypeRestrictions														ProtocolIE-ID ::= 301

id-uLpDCPSnLength															ProtocolIE-ID ::= 302

id-BluetoothMeasurementConfiguration										ProtocolIE-ID ::= 303

id-WLANMeasurementConfiguration												ProtocolIE-ID ::= 304

id-NRrestrictionin5GS														ProtocolIE-ID ::= 305

id-dL-Forwarding															ProtocolIE-ID ::= 306

id-E-RABs-DataForwardingAddress-List										ProtocolIE-ID ::= 307

id-E-RABs-DataForwardingAddress-Item										ProtocolIE-ID ::= 308

id-Subscription-Based-UE-DifferentiationInfo								ProtocolIE-ID ::= 309

id-GNBOverloadInformation													ProtocolIE-ID ::= 310

id-dLPDCPSnLength															ProtocolIE-ID ::= 311

id-secondarysgNBDLGTPTEIDatPDCP												ProtocolIE-ID ::= 312

id-secondarymeNBULGTPTEIDatPDCP												ProtocolIE-ID ::= 313

id-lCID																		ProtocolIE-ID ::= 314

id-duplicationActivation													ProtocolIE-ID ::= 315

id-ECGI																		ProtocolIE-ID ::= 316

id-RLCMode-transferred														ProtocolIE-ID ::= 317

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList							ProtocolIE-ID ::= 318

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item							ProtocolIE-ID ::= 319

id-E-RABs-ToBeReleased-SgNBRelReqdList										ProtocolIE-ID ::= 320

id-E-RABs-ToBeReleased-SgNBRelReqd-Item										ProtocolIE-ID ::= 321

id-NRCGI																	ProtocolIE-ID ::= 322

id-MeNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 323

id-SgNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 324

id-new-drb-ID-req															ProtocolIE-ID ::= 325

id-endcSONConfigurationTransfer												ProtocolIE-ID ::= 326

id-NRNeighbourInfoToAdd														ProtocolIE-ID ::= 327

id-NRNeighbourInfoToModify													ProtocolIE-ID ::= 328

id-DesiredActNotificationLevel												ProtocolIE-ID ::= 329

id-LocationInformationSgNBReporting											ProtocolIE-ID ::= 330

id-LocationInformationSgNB													ProtocolIE-ID ::= 331

id-LastNG-RANPLMNIdentity													ProtocolIE-ID ::= 332

id-EUTRANTraceID															ProtocolIE-ID ::= 333

id-additionalPLMNs-Item														ProtocolIE-ID ::= 334

id-InterfaceInstanceIndication												ProtocolIE-ID ::= 335

id-BPLMN-ID-Info-EUTRA														ProtocolIE-ID ::= 336

id-BPLMN-ID-Info-NR															ProtocolIE-ID ::= 337

id-NBIoT-UL-DL-AlignmentOffset												ProtocolIE-ID ::= 338

id-ERABs-transferred-to-MeNB												ProtocolIE-ID ::= 339

id-AdditionalRRMPriorityIndex												ProtocolIE-ID ::= 340

id-LowerLayerPresenceStatusChange											ProtocolIE-ID ::= 341

id-FastMCGRecovery-SN-to-MN													ProtocolIE-ID ::= 342

id-RequestedFastMCGRecoveryViaSRB3											ProtocolIE-ID ::= 343

id-AvailableFastMCGRecoveryViaSRB3											ProtocolIE-ID ::= 344

id-RequestedFastMCGRecoveryViaSRB3Release									ProtocolIE-ID ::= 345

id-ReleaseFastMCGRecoveryViaSRB3									ProtocolIE-ID ::= 346

id-FastMCGRecovery-MN-to-SN													ProtocolIE-ID ::= 347

id-PartialListIndicator														ProtocolIE-ID ::= 348

id-MaximumCellListSize														ProtocolIE-ID ::= 349

id-MessageOversizeNotification												ProtocolIE-ID ::= 350

id-CellandCapacityAssistInfo												ProtocolIE-ID ::= 351

id-TNLConfigurationInfo														ProtocolIE-ID ::= 352

id-TNLA-To-Add-List															ProtocolIE-ID ::= 353

id-TNLA-To-Update-List														ProtocolIE-ID ::= 354

id-TNLA-To-Remove-List														ProtocolIE-ID ::= 355

id-TNLA-Setup-List															ProtocolIE-ID ::= 356

id-TNLA-Failed-To-Setup-List													ProtocolIE-ID ::= 357

id-UnlicensedSpectrumRestriction												ProtocolIE-ID ::= 358

id-UEContextReferenceatSourceNGRAN											ProtocolIE-ID ::= 359

id-EPCHandoverRestrictionListContainer										ProtocolIE-ID ::= 360

id-CHOinformation-REQ													ProtocolIE-ID ::= AAA

id-CHOinformation-ACK													ProtocolIE-ID ::= EEE

id-DAPSRequestInfo															ProtocolIE-ID ::= BBB

id-RequestedTargetCellID													ProtocolIE-ID ::= CCC

id-CandidateCellsToBeCancelledList											ProtocolIE-ID ::= DDD

id-DAPSResponseInfo															ProtocolIE-ID ::= XXX

id-ProcedureStage															ProtocolIE-ID ::= YYY

id-CHO-DC-Indicator															ProtocolIE-ID ::= ZZZ

END

-- ASN1STOP

9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

X2AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

X2AP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

X2AP-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID 		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

X2AP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID 		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

X2AP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {X2AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {X2AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {X2AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				X2AP-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		X2AP-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			X2AP-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {X2AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {X2AP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					X2AP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	X2AP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			X2AP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	X2AP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			X2AP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, X2AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, X2AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {X2AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {X2AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					X2AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			X2AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		X2AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {X2AP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {X2AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				X2AP-PRIVATE-IES.&id			({IEsSetParam}),

criticality		X2AP-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			X2AP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP