3GPP TSG-RAN WG3 #109	R3-204623

# Online, 17th - 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0008 | rev | 16 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Positioning in NRPPa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add support of NR positioning in NRPPa |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following functions are added: - Introduction of the Assistance Information Control and Assistance Information Feedback procedures (both Class 2) - Positioning information request/response/failure/update - UL-SRS Activation (Class 1) and UL-SRS Deactivation (Class2) procedures to extend the Positioning Information Transfer function - TRP information request/response/failure - Measurement request/response/failure/report/update/abort/failure indication - Support of UL NR E-CID Note: all the changes in highlight originate from non-agreed TPs, that resulted from the e-mail discussion #1. Further checking is required |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for NR positioning over NRPPa |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 7, 8.1, 8.2.3.2, 8.x, 8.x.2, 8.2.x, 8.2.y, 8.2.z, 8.z, 9.1.1.a, 9.1.1.b, 9.1.1.c, 9.1.1.d, 9.1.1.e, 9.1.1.f, 9.1.1.g, 9.1.a, 9.1.x, 9.2.5, 9.2.a, 9.2.b, 9.2.c, 9.2.d, 9.2.e, 9.2.aa, 9.2.bb, 9.2.x, 9.2.y, 9.2.ya, 9.2.yb, 9.2.y1, 9.2.y2, 9.2.y3, 9.2.y4, 9.2.y5, 9.2.y6, 9.2.z, 9.2.z1, 9.2.z2, 9.2.z3, 9.2.z4, 9.2.z5, 9.2.z6, 9.2.z7, 9.2.z8, 9.2.z9, 9.2.z10, 9.2.z11, 9.2.z12, 9.2.aa1, 9.2.bb1, 9.2.bb2, 9.2.xx1, 9.2.xx2, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0373 TS 38.305 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev6: added agreed TP in R3-197688 Rev9: added asn.1 with FFS related to editor’s notes Rev10: allocated to R3-200784, withdrawn Rev11: merged TP in R3-201290 Rev12: submitted to RAN3#107bis-e Rev13: captured R3-202872, R3-201694, R3-202755 and R3-202776 Rev14: submitted to RAN3#108-e Rev15: merged TP R3-204208, R3-204216, R3-204207. R3-204211, R3-204212, R3-204331, R3-204299, R3-204213 Rev16: rapporteur editorial corrections and addition of co-sourcing companies |  |  |  |  |  |  |  |  |

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[x]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[y]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

NEXT CHANGE

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

NG-RAN node: as defined in TS 38.300 [3].

ng-eNB: as defined in TS 38.300 [3].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CID	Cell-ID (positioning method)

E-CID	Enhanced Cell-ID (positioning method)

LMF	Location Management Function

OTDOA	Observed Time Difference of Arrival

NEXT CHANGE

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |

NEXT CHANGE

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |

NEXT CHANGE

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

NEXT CHANGE

## 8.x	Assistance Information Transfer Procedures

### 8.x.1	Assistance Information Control

#### 8.x.1.1	General

The purpose of the Assistance Information Control procedure is to allow the LMF to signal positioning assistance information to the NG-RAN Node for assistance information broadcasting.

#### 8.x.1.2	Successful Operation

# Figure 8.x.1.2-1: Assistance Information Control procedure

The LMF initiates the procedure by sending an ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall replace any previously stored assistance information and use the received information to configure assistance information broadcasting.

If the Broadcast Priority IE is included in the Assistance Information IE, the NG-RAN Node may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the NG-RAN Node or not broadcast).

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", the NG-RAN Node may start broadcasting the assistance information. If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "stop", the NG-RAN Node may stop broadcasting the assistance information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN shall, if supported, consider that the received assistance information is applicable to the cells in this IE.

#### 8.x.1.3	Abnormal Conditions

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", and no assistance information is available, the NG-RAN Node shall consider the procedure as failed.

If neither the Assistance Information IE nor the Broadcast IE are included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall consider the procedure as failed.

### 8.x.2	Assistance Information Feedback

#### 8.x.2.1	General

The purpose of the Assistance Information Feedback procedure is to allow the NG-RAN Node to give feedback to the LMF on assistance information broadcasting.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall consider that assistance information broadcasting could not be configured for the relevant information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall, if supported, consider that the feedback provided is applicable to the cells in this IE.

#### 8.x.2.3	Abnormal Conditions

Void.

NEXT CHANGE

### 8.2.x	Positioning Information Exchange

#### 8.2.x.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN NODE positioning information for the UE.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE in the POSITIONING INFORMATION RESPONSE message.

#### 8.2.x.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.x.4	Abnormal Conditions

Void.

### 8.2.y	Positioning Information Update

#### 8.2.y.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration.

#### 8.2.y.2	Successful Operation

Figure 8.2.y.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. This message shall include the SRS configuration information for all cells where the SRS configuration has changed.

#### 8.2.y.3	Unsuccessful Operation

Not Applicable.

#### 8.2.y.4	Abnormal Conditions

Void.

NEXT CHANGE

### 8.2.Z	TRP Information Exchange

#### 8.2.Z.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node.

#### 8.2.Z.2	Successful Operation

Figure 8.2.Z.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the LMF includes the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the LMF does not include the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

#### 8.2.Z.3	Unsuccessful Operation

Figure 8.2.Z.3-1: TRP Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any TRP, the NG-RAN node shall respond with a TRP INFORMATION FAILURE message.

NEXT CHANGE

### 8.2.q	Positioning Activation

#### 8.2.q.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN NODE to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE.

#### 8.2.q.2	Successful Operation

Figure 8.2.q.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

The message includes an indication of the UL SRS resource set to be activated. For semi-persistent UL SRS, the message also indicates the spatial relation for the semi-persistent UL SRS resource to be activated.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message.

#### 8.2.q.3	Unsuccessful Operation

Figure 8.2.q.3-1: Positioning Activation procedure, unsuccessful operation

If the NG-RAN node is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

#### 8.2.q.4	Abnormal Conditions

Void.

### 8.2.r	Positioning Deactivation

#### 8.2.r.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE.

#### 8.2.r.2	Successful Operation

Figure 8.2.y.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set to be deactivated.

#### 8.2.y.3	Unsuccessful Operation

Not Applicable.

#### 8.2.y.4	Abnormal Conditions

Void.

NEXT CHANGE

## 8.z	Measurement Information Transfer

### 8.z.1	Measurement

#### 8.z.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements.

#### 8.z.1.2	Successful Operation

Figure 8.z.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List  IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall report the Rx beam information, which is used for determining the gNB measurements.

#### 8.z.1.3	Unsuccessful Operation

Figure 8.z.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

#### 8.z.1.4	Abnormal Conditions

Not applicable.

### 8.z.2	Measurement Report

#### 8.z.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF.

#### 8.z.2.2	Successful Operation

Figure 8.z.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

### 8.z.3	Measurement Update

#### 8.z.3.1	General

The Measurement Update Procedure allows the LMF to notify the NG-RAN node of a change in a previously configured measurement.

#### 8.z.3.2	Successful Operation

# Figure 8.z.3.2.1: Measurement Update: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT UPDATE message. Upon receiving the message, the NG-RAN node shall overwrite the previously received measurement configuration.

#### 8.z.3.3	Unsuccessful Operation

Not applicable.

#### 8.z.3.4	Abnormal Conditions

If the NG-RAN node cannot identify the previously requested measurement to be modified, it shall consider the procedure as failed and initiate local error handling.

### 8.z.4	Measurement Abort

#### 8.z.4.1	General

The purpose of the Measurement Abort Procedure is to enable the LMF to abort an on-going measurement.

#### 8.z.4.2	Successful Operation

# Figure 8.z.4.2.1: Measurement Abort Procedure: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT ABORT message.

Upon receiving this message, the NG-RAN node shall terminate the on-going measurement identified by the LMF Measurement ID IE and may release any resources previously allocated for the same measurement.

#### 8.z.4.3	Unsuccessful Operation

Not applicable.

#### 8.z.4.4	Abnormal Conditions

If the NG-RAN node cannot identify the previously requested measurement to be aborted, it shall ignore the MEASUREMENT ABORT message.

### 8.z.5	Measurement Failure Indication

#### 8.z.2.1	General

The Measurement Failure Indication procedure allows the NG-RAN node to notify the LMF that the measurements previously requested with the Measurement procedure can no longer be reported.

#### 8.z.2.2	Successful Operation

Figure 8.z.2.2.1: Measurement Report procedure. Successful operation.

Upon reception of the MEASUREMENT FAILURE INDICATION message, the LMF shall consider that the indicated measurements have been terminated by the NG-RAN node.

NEXT CHANGE

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival) |  | - | - |
| Other-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Other-RAT Measurement Quantities Item | M |  | ENUMERATED (GERAN, UTRAN ,…, NR, EUTRA) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

#### 9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement is successfully initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Other-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.14 |  | YES | ignore |

#### 9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement cannot be initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by NG-RAN node to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

#### 9.1.1.5	E-CID MEASUREMENT REPORT

This message is sent by NG-RAN node to report the results of the requested E-CID measurement.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.5 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |

#### 9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the LMF to terminate the requested E-CID measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |

NEXT CHANGE

#### 9.1.1.a	POSITIONING INFORMATION REQUEST

This message is sent by LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.x |  | YES | ignore |

#### 9.1.1.b	POSITIONING INFORMATION RESPONSE

This message is sent by NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.c	POSITIONING INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the positioning information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.d	POSITIONING INFORMATION UPDATE

This message is sent by NG-RAN node to indicate that a change in the SRS configuration has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |

NEXT CHANGE

#### 9.1.1.e	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 .. <maxnoTRPs> |  |  | EACH | ignore |
| >TRP ID | M |  | 9.2.aa |  |  |  |
| TRP Information Type List |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >TRP Information Type Item | M |  | ENUMERATED (prs id, nr pci, ng-ran cgi, nr arfcn, timing info, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 16384 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

#### 9.1.1.f	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF. This message only applies when the NG-RAN node is a gNB.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List | M | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >TRP Information | M |  | 9.2.bb |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 16384. |

#### 9.1.1.g	TRP INFORMATION FAILURE

This message is sent by an NG-RAN node to indicate that the requested TRP information cannot be provided to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

#### 9.1.1.d1	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M (FFS) |  | 9.2.y1 |  | YES | reject |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.y2 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>SRS Resource Trigger | M (IE Encoding is FFS) |  | 9.2.y3 |  | YES | reject |
| Activation Time | O |  | 9.2.y4 |  | YES | ignore |

#### 9.1.1.d2	POSITIONING ACTIVATION RESPONSE

This message is sent by NG-RAN node to confirm successful UL SRS activation in the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.d3	POSITIONING ACTIVATION FAILURE

This message is sent by NG-RAN node to indicate that activation of UL SRS transmission in the UE was unsuccessful.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.d4	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| SRS Resource Set ID | M |  | 9.2.y1 |  | YES | Ignore |

NEXT CHANGE

### 9.1.a	Messages for Assistance Information Transfer Procedures

#### 9.1.a.1	ASSISTANCE INFORMATION CONTROL

This message is sent by the LMF to transfer assistance information.

Direction: LMF  NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information | O |  | 9.2.a |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, ...) |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.xx2 | The cell(s) that are requested to broadcast posSIB(s) according to the Assistance Information IE. | YES | reject |

#### 9.1.a.2	ASSISTANCE INFORMATION FEEDBACK

This message is sent by the NG-RAN Node to give feedback on assistance information broadcasting.

Direction: NG-RAN Node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information Failure List | O |  | 9.2.e |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.xx2 | The cells associated to the feedback provided in the Assistance Information Failure List IE. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

### 9.1.x	Messages for Measurement Information Transfer Procedures

#### 9.1.x.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 16 |

#### 9.1.x.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| >>Measurement Result | M |  | 9.2.z1 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 16 |

#### 9.1.x.3	MEASUREMENT FAILURE

This message is sent by the NG-RAN node to report measurement failure.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

#### 9.1.x.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Report List |  | 1 |  |  | YES | reject |
| >TRP Measurement Report Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| >>Measurement Result | M |  | 9.2.z1 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 16 |

#### 9.1.x.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |

#### 9.1.x.6	MEASUREMENT ABORT

This message is sent by the LMF to request the NG-RAN node to abort a measurement.

Direction: LMF   NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |

#### 9.1.x.7	MEASUREMENT FAILURE INDICATION

This message is sent by the NG-RAN node to indicate that the previously requested measurements can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

NEXT CHANGE

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Access Point Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0 .. <maxnoMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] | - |  |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] | - |  |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [9] | - |  |
| >>Result RSRP EUTRA |  | 1 .. <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0.. 262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.6 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  | - |  |
| >>Result RSRQ EUTRA |  | 1 . <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | 9.2.7 | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0..262143, …). | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  | - |  |
| >>Result SS-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>SS-RSRP per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>Result SS-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>SS-RSRQ per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>Result CSI-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value CSI-RSRP Cell | O |  | INTEGER (0..127) | CSI-RSRP measurement aggregated at cell level | - |  |
| >>>CSI-RSRP per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRP | M |  | INTEGER (0..127) | CSI-RSRP measurement per CSI-RS resource | - |  |
| >>Result CSI-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value CSI-RSRQ Cell | O |  | INTEGER (0..127) | CSI-RSRQ measurement aggregated at cell level | - |  |
| >>>CSI-RSRQ per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRQ | M |  | INTEGER (0..127) | CSI-RSRQ measurement per CSI-RS resource | - |  |
| >>Angle of Arrival NR | M |  | UL Angle of Arrival 9.2.z2 |  | YES | ignore |
| Access Point Geographical Coordinates | O |  | 9.2.z9 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |
| maxCellReportNR | Maximum no. of NR cells that can be reported with one message. Value is 9. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

NEXT CHANGE

### 9.2.13	Other-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Other-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Other-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER (0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER (-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER (0..49, ...) | This IE applies to FDD only. |
| >>Result NR |  | 1..<maxNRMeas> |  |  |
| >>>NR PCI | M |  | INTEGER (0..3279165) |  |
| >>>NR ARFCN | M |  | INTEGER (0..1007) |  |
| >>>SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level |
| >>>SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level |
| >>>SS-RSRP per SSB Resource |  | 0 .. <maxnoIndexesToReport> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource |
| >>>SS-RSRQ per SSB Resource |  | 0 .. <maxnoIndexesToReport> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource |
| >>Result EUTRA |  | 1..<maxEUTRAMeas> |  |  |
| >>>PCI EUTRA | M |  | 9.2.7 |  |
| >>>EARFCN | M |  | INTEGER (0..262143). |  |
| >>>RSRP EUTRA | O |  | INTEGER (0..97) |  |
| >>>RSRQ EUTRA | O |  | INTEGER (0..34) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 8. |
| maxEUTRAMeas | Maximum no. of EUTRA cells that can be reported with one message. Value is 8. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

NEXT CHANGE

### 9.2.a	Assistance Information

This IE contains the assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Broadcast Periodicity for the Pos SIBs, see TS 38.331 [x] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | 9.2.d |  |
| >>>PosSIB Segments | M |  | 9.2.b |  |
| >>>Assistance Information Meta Data | O |  | 9.2.c |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

NEXT CHANGE

### 9.2.b	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | TS 37.355 [y] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 37.355 [y]). Value is 64. |

NEXT CHANGE

### 9.2.c	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) | TS 38.331 [x] |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, ...) | TS 38.331 [x] |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) | TS 38.331 [x] |

NEXT CHANGE

### 9.2.d	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 37.355 [y].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType3-2, posSibType3-3, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ... ) |  |

NEXT CHANGE

### 9.2.e	Assistance Information Failure List

This parameter identifies the assistance information for which the NG-RAN Node failed to configure broadcasting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information Failure List |  | 1..<maxnoAssistInfoFailureListItems> |  |  |
| >PosSIB-Type | M |  | 9.2.d |  |
| >Outcome | M |  | ENUMERATED (failed, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoAssistInfoFailureListItems | Maximum no. of assistance information failure list items that can be signaled with one message. Value is 32. |

NEXT CHANGE

### 9.2.aa	TRP ID

The TRP ID IE is used to identify a TRP uniquely within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER (1..16384,…) | Identifies a TRP within an NG-RAN node |

### 9.2.bb	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.aa |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  |
| >CHOICE TRP Information Item | M |  |  |  |
| >>PRS ID | M |  | INTEGER (0..255) | DL PRS ID |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NG-RAN CGI | M |  | 9.2.6 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>Timing Information | M |  | 9.2.z5 |  |
| >>PRS Configuration | M |  | 9.2.z6 |  |
| >>SSB Configuration | M |  | 9.2.z7 |  |
| >>SFN Initialization Time | M |  | 9.2.y5 |  |
| >>Geographical Coordinates | M |  | 9.2.z9 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

### 9.2.x	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | O |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |
| Resource Type | O |  | ENUMERATED (semi-persistent, aperiodic, …) |  |
| CHOICE Bandwidth | M |  |  |  |
| >FR1 |  |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ...) |  |
| >FR2 |  |  | ENUMERATED (50, 100, 200, 400,…) |  |
| Number of SRS Resource Set | O |  | INTEGER (1..16,...) | The number of SRS Resource set for SRS transmission. Value 1 indicates low number of SRS resources whereas value 16 indicates the maximum number. |
| Number of SRS Resource Per Set | O |  | INTEGER (1..64,...) | The number of SRS Resources per resource set for SRS transmission. Value 1 indicates low number of SRS resources whereas value 64 indicates the maximum number. |
| Spatial Relation Information | O |  | 9.2.y2 |  |
| Pathloss Reference Information | O |  | 9.2.y6 |  |
| SSB Configuration | O |  | 9.2.z7 |  |

### 9.2.y	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

[Editor’s Note: further details on the IEs are FFS : IEs following  the “SRS Resource Set List” FFS ]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN Initialization Time | M |  | 9.2.y5 |  |
| SRS Resource Set List |  | 1 |  | [FFS … |
| >SRS Resource set item |  | 1..<maxnoSRS-ResourceSets> |  |  |
| >>Point A | M |  | INTEGER (0..3279165) NR ARFCN |  |
| >>Subcarrier Spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz) |  |
| >>CP Type | M |  | ENUMERATED(Normal, Extended) |  |
| >>Offset To Carrier | M |  | INTEGER(0..2199) | First usable RB to Point A in the number of PRBs |
| >>BWP Start | M |  | INTEGER(0..274) | Start PRB of the UL BWP to the first usable RB |
| >>SRS Resource Set ID | M |  | INTEGER(0.. 63) |  |
| >>SRS Resource | M | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>>CHOICE SRS Resource type |  |  |  |  |
| >>>> SRS Resource | M |  | 9.2.ya |  |
| >>>> Positioning SRS Resource | M |  | 9.2.yb |  |
| >>CHOICE Resource Type | M |  |  |  |
| >>>aperiodic |  |  |  |  |
| >>>> SRS Resource Trigger | M |  | 9.2.y3 |  |
| >>>semipersistent |  |  |  |  |
| >>>periodic |  |  |  |  |
| >>Pathloss Reference | O |  | 9.2.y6 | …FFS] |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets. Value is 16. |
| maxnoSRS-ResourcesPerSet | Maximum no of SRS resource per set. Value is 64. |

### 9.2.ya	SRS Resource [FFS]

This information element contains the SRS resource.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER(0..15) |  |
| Number of Ports | M |  | ENUMERATED(port1, ports2, ports4) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..12) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4) |  |
| Repetition Factor | M |  | ENUMERATED(1,2,4) |  |
| Frequency Domain Position | M |  | INTEGER(0..67) |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| B-SRS | M |  | INTEGER(0..3) |  |
| B-Hop | M |  | INTEGER(0..3) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| Periodicity | M |  | ENUMERATED(1,2,4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560) |  |
| Slot Offset | M |  | INTEGER(0..2559) |  |
| Sequence ID | M |  | INTEGER(0..1023) |  |

### 9.2.yb	Positioning SRS Resource [FFS]

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..12) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..6) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4 |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>Periodicity |  |  | ENUMERATED(1,2,4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >>Offset |  |  | INTEGER(0..81919,…) |  |
| >semi-persistent |  |  |  |  |
| >>Periodicity |  |  | ENUMERATED(1,2,4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >>Offset |  |  | INTEGER(0..81919,…) |  |
| >aperiodic |  |  |  |  |
| >>slot offset |  |  | INTEGER(1..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation | O |  |  |  |
| >SSB | M |  |  |  |
| >>PCI | O |  | NR PCI |  |
| >>SSB index | M |  | INTEGER(0..63) |  |
| >PRS | M |  |  |  |
| >>PRS ID | O |  | INTEGER(0..255) | Absent if it is included MEASUREMENT REQUEST. Optionally present if it is included in POSITIONING INFORMATION RESPONSE/UPDATE |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |

### 9.2.y1	SRS Resource Set ID

This information element indicates a resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER (0..15) | According to TS 38.331 [x] |

### 9.2.y2	Spatial Relation Information

This information element indicates a spatial relation for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation for Resource IDi |  | 1..<maxnoSpatialRelations> |  | According to TS 38.321 [y] |
| CHOICE Reference Signal | M |  |  |  |
| >NZP CSI-RS |  |  |  |  |
| >>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >SRS |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >Positioning SRS |  |  |  |  |
| >>SRS Pos Resource ID | M |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSpatialRelations | Maximum no. of Spatial Relations that can be configured. Value is 64. |

### 9.2.y3	SRS Resource Trigger

This information element indicates a DCI code point according to a SRS resource set configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Aperiodic SRS Resource Trigger List |  | 1..<maxnoSRS-TriggerStates> |  | According to TS 38.331 [x] |
| >Aperiodic SRS Resource Trigger |  |  | INTEGER (1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRSTriggerStates | Maximum no. of SRS trigger states. Value is 3. |

### 9.2.y4	Activation Time

This information element indicates the start time when the SRS activation is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN initialization time | M |  | 9.2.y5 |  |

### 9.2.y5	SFN Initialization Time

This information element indicates the SFN initialization time provided by LMF

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN initialization time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

### 9.2.y6	Pathloss Reference Information

This information element indicates a pathloss reference for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Pathloss Reference |  | 1..<maxnoPathlossReferences> |  | According to TS 38.331 [y] |
| CHOICE Reference Signal | M |  |  |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPathlossReferences | Maximum no. of Pathloss References that can be configured. Value is 16. |

### 9.2.z	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path List | M |  | 9.2.z11 |  |

### 9.2.z1	Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.z2 |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..127) |  |
| >>UL RTOA | M |  | 9.2.z |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.z10 |  |
| >Time Stamp | M |  | 9.2.z3 |  |
| >Measurement Quality | M |  | 9.2.z4 |  |
| >Measurement Beam Information | O |  | 9.2.aa1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

### 9.2.z2	UL Angle of Arrival [IEs detail description FFS]

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) |  |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) |  |
| Angle Coordinate System | O |  | ENUMERATED(LCS, GCS) |  |

### 9.2.z3	Time Stamp[IEs detail description FFS]

This information element contains the time stamp associated with the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| CHOICE Slot Index | M |  |  |  |
| >SCS-15 | M |  | INTEGER(0..9) |  |
| >SCS-30 | M, |  | INTEGER(0..19) |  |
| >SCS-60 | M |  | INTEGER(0..39) |  |
| >SCS-120 | M |  | INTEGER(0..79) |  |

### 9.2.z4	Measurement Quality[IEs detail description FFS]

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  | 1 |  |  |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m) |  |
| >Angle Measurement Quality |  | 1 |  |  |
| >> Azimuth Quality | M |  | INTEGER(0..255) |  |
| >> Zenith Quality | O |  | INTEGER(0..255) |  |

### 9.2.z5	Timing Information[IEs detail description FFS]

This information element contains timing information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  | 1 |  |  |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m) |  |
| >Angle Measurement Quality |  | 1 |  |  |
| >> Azimuth Quality | M |  | INTEGER(0..255) |  |
| >> Zenith Quality | O |  | INTEGER(0..255) |  |

### 9.2.z6	PRS Configuration

This information element contains the DL PRS configuration for the TRP.

[Editor’s Note: further details on the IEs are FFS : IEs following  the “NR-PRS Beam Information” FFS ]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information | O |  | 9.2.xx1 |  |
| PRS Resource Set List | M | 1..<maxnoofPRSresourceSet> |  | [FFS … |
| >PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >Subcarrier Spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >Point A | M |  | INTEGER (0..3279165) |  |
| NR ARFCN |  |  |  |  |
| >Comb Size | M |  | ENUMERATED(2, 4, 6, 12) |  |
| >CP Type | M |  | ENUMERATED(NCP, ECP) |  |
| >Resource Set Periodicity | M |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >Resource Set Slot Offset | M |  | INTEGER(0..81919,…) |  |
| >Resource Repetition Factor | M |  | ENUMERATED(1,2,4,6,8,16,32,…) |  |
| >Resource Time Gap | M |  | ENUMERATED(1,2,4,8,16,32,…) |  |
| >Resource Number of Symbols | M |  | ENUMERATED(2,4,6,12,…) |  |
| >PRS Muting |  |  |  |  |
| >>Option1 | O |  |  |  |
| >>>Muting Pattern | M |  | 9.2.z15 |  |
| >>>Occasion Group Length | M |  | ENUMERATED(1,2,4,8,…) |  |
| >>Option2 | O |  |  |  |
| >>>Muting Pattern | M |  | 9.2.z15 |  |
| >PRS Resource Transmit Power |  |  | INTEGER(-60..50) |  |
| >PRS Resource List | M | 1..<maxnoofPRSresource> |  |  |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |
| >>Sequence ID | M |  | INTEGER(0..4095,…) |  |
| >>RE Offset | M |  | INTEGER(0..11) |  |
| >>Resource Slot Offset | M |  | INTEGER(0..511,…) |  |
| >>Resource Symbol Offset | M |  | INTEGER(0..12,…) |  |
| >>QCL Info | O |  |  |  |
| >>>QCL Source SSB Index | O |  | INTEGER(0..63) |  |
| >>>QCL Source PRS Info | O |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) | If it is absent, the QCL source PRS resource ID is the same as the PRS resource ID |
| >PRS Angle Information List |  | 0..1 |  |  |
| >>PRS Angle Item |  | 1..<maxnoofAngleInfo> |  |  |
| >>>AoD | M |  | INTEGER(0..359) |  |
| >>>ZoD | O |  | INTEGER(0..180) |  |
| >>>Primary PRS ID | M |  | INTEGER(0..63) | [Editor’s note Note Primary come from removal of secondary] |
| >Angle Coordinate System | O |  | ENUMERATED(LCS, GCS) | …FFS] |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |
| maxnoofAngleInfo | Maximum no of PRS angle information that can be included within PRS configurations IE. Value is 65535. |

### 9.2.z7	SSB Configuration [IEs detail description FFS]

This information element contains the SSB information (e.g. time/frequency occupancy of SSBs) for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB Configuration | M |  | 9.2.z12 TF Configuration |  |

### 9.2.z8	Spatial Direction Information [FFS seems not needed]

This information element contains the spatial direction information of the DL PRS resources for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z9	Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Access Point | M |  |  |  |
| >Access Point Position |  |  |  |  |
| >>NG-RAN Access Point Position | M |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. |
| >High Accuracy Access Point Position |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position | M |  | 9.2.bb1 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Access Point Position Relative |  |  |  |  |
| >>NG-RAN Access Point Position Relative | M |  | 9.2.bb2 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |

### 9.2.z10	gNB Rx-Tx Time Difference[IEs detail description FFS]

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Rx-Tx Time Difference | M |  | INTEGER(0..FFS) | [FFS] |
| Additional Path List | M |  | 9.2.z11 |  |

### 9.2.z11	Additional Path List[IEs detail description FFS]

This information element contains the additional path results of time measurement.

[Editor’s first table seems not needed]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  |
| Relative Time of Path | M |  | FFS |  |
| Path Quality | O |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

[Editor’s following seems prefer]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  |
| >CHOICE Path Delay | M |  |  |  |
| >k0 | M |  | INTEGER(0..1970049,…) |  |
| >k1 | M |  | INTEGER(0..985025,…) |  |
| >k2 | M |  | INTEGER(0..492513,…) |  |
| >k3 | M |  | INTEGER(0..246257,…) |  |
| >k4 | M |  | INTEGER(0..123129,…) |  |
| >k5 | M |  | INTEGER(0..61565,…) |  |
| >Path Quality | O |  | 9.2.z4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

### 9.2.z12	TF Configuration

This information element contains the time and frequency configurations.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(15kHz, 30kHz, 120kHz, 240kHz,...) |  |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, 160ms, …) |  |
| SSB half frame offset | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| SFN initialization time | O |  | 9.2.y5 |  |

### 9.2.aa1	Measurement Beam Information

This information element contains the receiving beam information when measuring UL signals.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource ID | O |  | INTEGER(0..63) |  |
| PRS Resource Set ID | O |  | INTEGER(0..7) |  |
| SSB Index | O |  | INTEGER(0..63) |  |

### 9.2.bb1	 NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

### 9.2.bb2	 NG-RAN Access Point Position Relative

The NG-RAN Access Point Position Relative IE is used to identify the geographical position of an NG-RAN Access Point using relative Cartesian coordinate, which is expressed as XYZ values.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Referential | M |  |  | Reference point (0,0,0) from where the XYZ values are deduced from |
| >Relative Coordinate |  |  |  |  |
| >>Relative Coordinate ID | M |  | INTEGER(0.. 231-1,..) | Referential ID maped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>NG-RAN Access Point Position |  |  | 9.2.10 |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position |  |  | 9.2.xz |  |
| XYZ unit | M |  | ENUMERATED (cm, dm,..) |  |
| X value | M |  | INTEGER (-231.. 231-1) | Positive value represents northing from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-231.. 231-1) | Positive value represents easting from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-215.. 215-1) | Positive value represents height above reference point in units of XYZ Unit IE. |

NEXT CHANGE

### 9.2.xx1	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. < maxnoofResourcesPerSet > |  |  |
| >PRS resource ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. |
| >PRS Angle Item |  | 1..<maxnoofAngleInfo> |  |  |
| >>NR PRS Azimuth | M |  | INTEGER (0..359) |  |
| >>PRS resource angle ID | M |  | INTEGER (0..63) | The resource that radiates in that angle. |
| >>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles |
| >>NR PRS Elevation | O |  | INTEGER (0..180) |  |
| >>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles |
| LCS to GCS Translation |  | 1 .. <maxnolcs-gcs-translation> |  | Only the single value, 1, shall be used in this version of the specifications |
| >Alpha | O |  | INTEGER (0..359) |  |
| >Alpha-fine | O |  | INTEGER (0..9) | Fine angles |
| >Beta | O |  | INTEGER (0..359) |  |
| >Beta-fine | O |  | INTEGER (0..9) | Fine angles |
| >Gamma | O |  | INTEGER (0..359) |  |
| >Gamma-fine | O |  | INTEGER (0..9) | Fine angles |

| Range bound | Explanation |
| --- | --- |
| maxnoofResourcesPerSet | Maximum no. of NR PRS resource per set that can reported with one message. Value is 2. |
| maxnoofAngleInfo | Maximum no of PRS angle information that can be included within PRS configurations IE. Value is 65535. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. |

### 9.2.xx2	Positioning Broadcast Cells

This IE is used to indicate the cells that are requested to broadcast, or failed to broadcast, the associated posSIB(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Broadcast Cells |  | 1 .. <maxnoBcastCell> |  |  |
| >NG-RAN-CGI | M |  | 9.2.6 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoBcastCells | Maximum no. of cells broadcasting a posSIB in a NG-RAN node. Value is 16384. |

NEXT CHANGE

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

NRPPA-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

NRPPATransactionID

FROM NRPPA-CommonDataTypes

ErrorIndication,

PrivateMessage,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

OTDOAInformationRequest,

OTDOAInformationResponse,

OTDOAInformationFailure,

AssistanceInformationControl,

AssistanceInformationFeedback,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningInformationUpdate,

MeasurementRequest,

MeasurementResponse,

MeasurementFailure,

MeasurementReport,

MeasurementUpdate,

MeasurementAbort,

MeasurementFailureIndication,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation

FROM NRPPA-PDU-Contents

id-errorIndication,

id-privateMessage,

id-e-CIDMeasurementInitiation,

id-e-CIDMeasurementFailureIndication,

id-e-CIDMeasurementReport,

id-e-CIDMeasurementTermination,

id-oTDOAInformationExchange,

id-assistanceInformationControl,

id-assistanceInformationFeedback,

id-positioningInformationExchange,

id-positioningInformationUpdate,

id-Measurement,

id-MeasurementReport,

id-MeasurementUpdate,

id-MeasurementAbort,

id-MeasurementFailureIndication,

id-tRPInformationExchange,

id-positioningActivation,

id-positioningDeactivation

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NRPPA-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURES NRPPA-ELEMENTARY-PROCEDURE ::= {

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1			|

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementInitiation	|

oTDOAInformationExchange	|

positioningInformationExchange	|

measurement						|

tRPInformationExchange		|

positioningActivation,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementFailureIndication		|

e-CIDMeasurementReport					|

e-CIDMeasurementTermination				|

errorIndication							|

privateMessage							|

assistanceInformationControl			|

assistanceInformationFeedback 			|

positioningInformationUpdate			|

measurementReport						|

measurementUpdate						|

measurementAbort						|

measurementFailureIndication		|

positioningDeactivation,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

e-CIDMeasurementInitiation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-e-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-e-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-e-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-e-CIDMeasurementTermination

CRITICALITY				reject

}

oTDOAInformationExchange NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OTDOAInformationRequest

SUCCESSFUL OUTCOME		OTDOAInformationResponse

UNSUCCESSFUL OUTCOME	OTDOAInformationFailure

PROCEDURE CODE			id-oTDOAInformationExchange

CRITICALITY				reject

}

assistanceInformationControl NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationControl

PROCEDURE CODE			id-assistanceInformationControl

CRITICALITY				reject

}

assistanceInformationFeedback NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationFeedback

PROCEDURE CODE			id-assistanceInformationFeedback

CRITICALITY				reject

}

errorIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY			ignore

}

privateMessage			NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY			ignore

}

positioningInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-positioningInformationExchange

CRITICALITY				reject

}

positioningInformationUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-positioningInformationUpdate

CRITICALITY				ignore

}

measurement	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementRequest

SUCCESSFUL OUTCOME		MeasurementResponse

UNSUCCESSFUL OUTCOME	MeasurementFailure

PROCEDURE CODE			id-Measurement

CRITICALITY				reject

}

measurementReport	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementReport

PROCEDURE CODE			id-MeasurementReport

CRITICALITY				ignore

}

measurementUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementUpdate

PROCEDURE CODE			id-MeasurementUpdate

CRITICALITY				ignore

}

measurementAbort	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementAbort

PROCEDURE CODE			id-MeasurementAbort

CRITICALITY				ignore

}

measurementFailureIndication	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementFailureIndication

PROCEDURE CODE			id-MeasurementFailureIndication

CRITICALITY				ignore

}

tRPInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-tRPInformationExchange

CRITICALITY				reject

}

positioningActivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-positioningActivation

CRITICALITY				reject

}

positioningDeactivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-positioningDeactivation

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa.

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

UE-Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

MeasurementQuantities,

TrpMeasurementResult,

TRP-ID,

TRPInformationTypeList,

TRPInformationList,

TRP-MeasurementRequestList,

TRP-MeasurementResponseList,

MeasurementBeamInfoRequest,

PositioningBroadcastCells,

ActivationTime,

SRSResourceSetID,

SRSSpatialRelation,

SRSResourceTrigger,

TRPList

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-Measurement-ID,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-Measurement-ID,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-MeasurementQuantities,

id-MeasurementResult,

id-TRP-ID,

id-TRPInformationTypeList,

id-TRPInformationList,	id-TRP-MeasurementRequestList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION REQUEST

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID			CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics				PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity				PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” --

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities				PRESENCE mandatory}|

{ ID id-OtherRATMeasurementQuantities	CRITICALITY ignore	TYPE OtherRATMeasurementQuantities	PRESENCE optional}|

{ ID id-WLANMeasurementQuantities		CRITICALITY ignore	TYPE WLANMeasurementQuantities			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION RESPONSE

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID					PRESENCE optional}|

{ ID id-OtherRATMeasurementResult	CRITICALITY ignore	TYPE OtherRATMeasurementResult		PRESENCE optional}|

{ ID id-WLANMeasurementResult		CRITICALITY ignore	TYPE WLANMeasurementResult			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION FAILURE

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT TERMINATION

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION REQUEST

--

-- **************************************************************

OTDOAInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationRequest-IEs}},

...

}

OTDOAInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Group		CRITICALITY reject	TYPE OTDOA-Information-Type			PRESENCE mandatory},

...

}

OTDOA-Information-Type ::= SEQUENCE (SIZE(1..maxnoOTDOAtypes)) OF ProtocolIE-Single-Container { { OTDOA-Information-TypeIEs} }

OTDOA-Information-TypeIEs	NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Item		CRITICALITY reject	TYPE OTDOA-Information-Type-Item	PRESENCE mandatory},

...

}

OTDOA-Information-Type-Item ::= SEQUENCE {

oTDOA-Information-Type-Item		OTDOA-Information-Item,

iE-Extensions					ProtocolExtensionContainer { { OTDOA-Information-Type-ItemExtIEs} } OPTIONAL,

...

}

OTDOA-Information-Type-ItemExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION RESPONSE

--

-- **************************************************************

OTDOAInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationResponse-IEs}},

...

}

OTDOAInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOACells						CRITICALITY ignore	TYPE OTDOACells					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION FAILURE

--

-- **************************************************************

OTDOAInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{OTDOAInformationFailure-IEs}},

...

}

OTDOAInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION CONTROL

--

-- **************************************************************

AssistanceInformationControl ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationControl-IEs}},

...

}

AssistanceInformationControl-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Assistance-Information	CRITICALITY reject	TYPE Assistance-Information		PRESENCE optional}|

{ ID id-Broadcast				CRITICALITY reject	TYPE Broadcast					PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION FEEDBACK

--

-- **************************************************************

AssistanceInformationFeedback ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationFeedback-IEs}},

...

}

AssistanceInformationFeedback-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-AssistanceInformationFailureList	CRITICALITY reject	TYPE AssistanceInformationFailureList	PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs NRPPA-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION FAILURE

--

-- **************************************************************

PositioningInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PositioningInformationFailure-IEs}},

...

}

PositioningInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationUpdate-IEs}},

...

}

PositioningInformationUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REQUEST

--

-- **************************************************************

MeasurementRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementRequest-IEs}},

...

}

MeasurementRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList		CRITICALITY reject	TYPE TRP-MeasurementRequestList PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics		PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” –

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities		PRESENCE mandatory}|

{ ID id-SRSConfiguration				CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest		CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT RESPONSE

--

-- **************************************************************

MeasurementResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementResponse-IEs}},

...

}

MeasurementResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementResponseList	CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE

--

-- **************************************************************

MeasurementFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailure-IEs}},

...

}

MeasurementFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REPORT

--

-- **************************************************************

MeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementReport-IEs}},

...

}

MeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementReportList			CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT UPDATE

--

-- **************************************************************

MeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementUpdate-IEs}},

...

}

MeasurementUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

--

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT ABORT

--

-- **************************************************************

MeasurementAbort ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementAbort-IEs}},

...

}

MeasurementAbort-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

MeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailureIndication-IEs}},

...

}

MeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION REQUEST

--

-- **************************************************************

TRPInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationRequest-IEs}},

...

}

TRPInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPList						CRITICALITY reject	TYPE TRPList				PRESENCE mandatory}|

{ ID id-TRPInformationTypeList		CRITICALITY reject	TYPE TRPInformationTypeList	PRESENCE mandatory},	...

}

-- **************************************************************

--

#### -- TRP INFORMATION RESPONSE

--

-- **************************************************************

TRPInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationResponse-IEs}},

...

}

TRPInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPInformationList				CRITICALITY ignore	TYPE TRPInformationList			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION FAILURE

--

-- **************************************************************

TRPInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{TRPInformationFailure-IEs}},

...

}

TRPInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITONING ACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Activation Request

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	} |

{ ID id-ActivationTime			CRITICALITY ignore	TYPE ActivationTime				PRESENCE optional	},

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

sRSType-extension				ProtocolIE-Single-Container { { SRSType-ExtIEs} }

}

SRSType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelation	CRITICALITY ignore	EXTENSION SRSSpatialRelation PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

sRSResourceTrigger			SRSResourceTrigger,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- Positioning Activation Response

--

-- **************************************************************

PositioningActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationResponseIEs} },

...

}

PositioningActivationResponseIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Activation Failure

--

-- **************************************************************

PositioningActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationFailureIEs} },

...

}

PositioningActivationFailureIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITONING DEACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Deactivation

--

-- **************************************************************

PositioningDeactivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningDeactivationIEs} },

...

}

PositioningDeactivationIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSResourceSetID			CRITICALITY ignore	TYPE SRSResourceSetID		PRESENCE mandatory	} ,

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-SRSSpatialRelation,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofResourcesPerSet,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR

FROM NRPPA-Constants

Criticality,

NRPPATransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NRPPA-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers;

#### -- A

ActivationTime ::= BIT STRING (SIZE (64))

AccessPointItem ::= CHOICE {

nGRANAccessPointPosition							NG-RANAccessPointPosition,

nGRANhighAccuracyAccessPointPosition				NGRANHighAccuracyAccessPointPosition,

nGRANaccessPointPositionRelative					NGRANAccessPointPositionRelative,

choice-extension							ProtocolIE-Single-Container { { AccessPointItem-ExtIEs } }

}

AccessPointItem-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

AdditionalPathLoss ::= SEQUENCE (SIZE (1.. maxNoPath)) OF AdditionalPathLossItem

AdditionalPathLossItem ::= SEQUENCE {

relativeTimeOfPath	Assistance-Information , -- FFS dummy value for ASN compilation

pathQuality			Assistance-Information 	OPTIONAL, -- FFS dummy value for ASN compilation

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (0..3, ...)

Assistance-Information ::= SEQUENCE {

systemInformation				SystemInformation,

iE-Extensions					ProtocolExtensionContainer { { Assistance-Information-ExtIEs} } OPTIONAL,

...

}

Assistance-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationFailureList ::= SEQUENCE (SIZE (1..maxnoAssistInfoFailureListItems)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

outcome							Outcome,

iE-Extensions					ProtocolExtensionContainer { {AssistanceInformationFailureList-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationFailureList-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationMetaData ::= SEQUENCE {

encrypted			ENUMERATED {true, ...}	OPTIONAL,

gNSSID				ENUMERATED {gps, sbas, gzss, galileo, glonass, bds, ...}	OPTIONAL,

sBASID				ENUMERATED {waas, egnos, msas, gagan, ...}					OPTIONAL,	iE-Extensions		ProtocolExtensionContainer { { AssistanceInformationMetaData-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationMetaData-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BCCH ::= INTEGER (0..1023, ...)

Broadcast ::= ENUMERATED {

start,

stop,

...

}

-- Editor’s Note: Details of this IE are FFS pending RAN2 progress

BroadcastPeriodicity ::= ENUMERATED {

ms80,

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

...

}

PositioningBroadcastCells ::= SEQUENCE (SIZE (1..maxnoBcastCell)) OF NG-RAN-CGI

BSSID ::= OCTET STRING (SIZE(6))

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

protocol			CauseProtocol,

misc				CauseMisc,

cause-Extension	ProtocolIE-Single-Container {{ Cause-ExtensionIE }}

}

Cause-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

requested-item-not-supported,

requested-item-temporarily-not-available,

...

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CGI-EUTRA ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRAcellIdentifier			EUTRACellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-EUTRA-ExtIEs} } OPTIONAL,

...

}

CGI-EUTRA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CPLength-EUTRA ::= ENUMERATED {

normal,

extended,

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality					OPTIONAL,

nrppatransactionID				NRPPATransactionID				OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError			TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

DL-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

DL-PRS ::= SEQUENCE {

prsid 					INTEGER (0..255),

dl-PRSResourceSetID		INTEGER (0..7),

dl-PRSResourceID		INTEGER (0..63)		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DL-PRS-ExtIEs} }	OPTIONAL,

...

}

DL-PRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

E-CID-MeasurementResult-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-GeographicalCoordinates	CRITICALITY ignore	EXTENSION GeographicalCoordinates PRESENCE optional},

...

}

EUTRACellIdentifier ::= BIT STRING (SIZE (28))

EARFCN ::= INTEGER (0..262143, ...)

#### -- F

#### -- G

GeographicalCoordinates ::= SEQUENCE {

accessPointItem		AccessPointItem,

iE-Extensions					ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL,

...

}

GeographicalCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

GNB-RxTxTimeDiff ::= SEQUENCE {

-- Further details are FFS pending RAN2

rxTxTimeDiff		INTEGER (0..12), -- value FFS

additionalPathLoss	AdditionalPathLoss	OPTIONAL,

...

}

#### -- H

HESSID ::= OCTET STRING (SIZE(6))

#### -- I

#### -- J

#### -- K

#### -- L

#### -- M

Measurement-ID ::= INTEGER (1.. 65536)

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID,

pRS-Resource-Set-ID			PRS-Resource-Set-ID,

sSB-Index					SSB-Index,

iE-Extensions								ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL,

...

}

MeasurementBeamInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue					MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

... ,

sS-RSRP,

sS-RSRQ,

cSI-RSRP,

cSI-RSRQ,

angleOfArrivalNR

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

measuredResultsValue-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	},

...

}

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NG-RANAccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NG-RANAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NG-RANAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100),

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANHighAccuracyAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NGRANHighAccuracyAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NGRANAccessPointPositionRelative ::= SEQUENCE {

referential					NGRANAccessPointPositionRelativeReferential,

xYZunit						ENUMERATED {cm, dm,...},

xvalue						INTEGER (-2147483648..2147483647),

yvalue						INTEGER (-2147483648..2147483647),

zvalue						INTEGER (-32768..32767),

iE-Extensions				ProtocolExtensionContainer { { NGRANAccessPointPositionRelative-ExtIEs} } OPTIONAL,

...

}

NGRANAccessPointPositionRelative-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NGRANAccessPointPositionRelativeReferential::= CHOICE {

relativeCoordinate									RelativeCoordinateID,

referencePointCoordinates							NG-RANAccessPointPosition,

referencePointCoordinatesHighAccuracy				NGRANHighAccuracyAccessPointPosition,

choice-extension							ProtocolIE-Single-Container { { NGRANAccessPointPositionRelativeReferential-ExtIEs } }

}

NGRANAccessPointPositionRelativeReferential-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

NG-RAN-CGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nG-RANcell		NG-RANCell,

iE-Extensions				ProtocolExtensionContainer { {NG-RAN-CGI-ExtIEs} } OPTIONAL,

...

}

NG-RAN-CGI-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RANCell ::= CHOICE {

eUTRA-CellID	EUTRACellIdentifier,

nR-CellID		NRCellIdentifier,

nG-RANCell-Extension			ProtocolIE-Single-Container {{ NG-RANCell-ExtensionIE }}

}

NG-RANCell-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

NR-ARFCN ::= INTEGER (0..3279165)

NRCellIdentifier ::= BIT STRING (SIZE (36))

NR-PCI ::= INTEGER (0..1007)

NumberOfAntennaPorts-EUTRA ::= ENUMERATED {

n1-or-n2,

n4,

...

}

NumberOfDlFrames-EUTRA ::= ENUMERATED {

sf1,

sf2,

sf4,

sf6,

...

}

NumberOfDlFrames-Extended-EUTRA ::= INTEGER (1..160,...)

NumberOfFrequencyHoppingBands ::= ENUMERATED {

twobands,

fourbands,

...

}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191, ...)

#### -- O

OTDOACells ::= SEQUENCE (SIZE (1.. maxCellinRANnode)) OF SEQUENCE {

oTDOACellInfo					OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {OTDOACells-ExtIEs} } OPTIONAL,

...

}

OTDOACells-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

OTDOACell-Information-Item ::= CHOICE {

pCI-EUTRA									PCI-EUTRA,

cGI-EUTRA									CGI-EUTRA,

tAC											TAC,

eARFCN										EARFCN,

pRS-Bandwidth-EUTRA							PRS-Bandwidth-EUTRA,

pRS-ConfigurationIndex-EUTRA				PRS-ConfigurationIndex-EUTRA,

cPLength-EUTRA								CPLength-EUTRA,

numberOfDlFrames-EUTRA						NumberOfDlFrames-EUTRA,

numberOfAntennaPorts-EUTRA					NumberOfAntennaPorts-EUTRA,

sFNInitialisationTime-EUTRA					SFNInitialisationTime-EUTRA,

nG-RANAccessPointPosition					NG-RANAccessPointPosition,

pRSMutingConfiguration-EUTRA				PRSMutingConfiguration-EUTRA,

prsid-EUTRA									PRS-ID-EUTRA,

tpid-EUTRA									TP-ID-EUTRA,

tpType-EUTRA								TP-Type-EUTRA,

numberOfDlFrames-Extended-EUTRA				NumberOfDlFrames-Extended-EUTRA,

crsCPlength-EUTRA							CPLength-EUTRA,

dL-Bandwidth-EUTRA							DL-Bandwidth-EUTRA,

pRSOccasionGroup-EUTRA						PRSOccasionGroup-EUTRA,

pRSFrequencyHoppingConfiguration-EUTRA		PRSFrequencyHoppingConfiguration-EUTRA,

oTDOACell-Information-Item-Extension		ProtocolIE-Single-Container {{ OTDOACell-Information-Item-ExtensionIE }}

}

OTDOACell-Information-Item-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-TDD-Config-EUTRA-Item		CRITICALITY 	ignore	TYPE 	TDD-Config-EUTRA-Item		PRESENCE 	optional },

...

}

OTDOA-Information-Item ::= ENUMERATED {

pci,

cGI,

tac,

earfcn,

prsBandwidth,

prsConfigIndex,

cpLength,

noDlFrames,

noAntennaPorts,

sFNInitTime,

nG-RANAccessPointPosition,

prsmutingconfiguration,

prsid,

tpid,

tpType,

crsCPlength,

dlBandwidth,

multipleprsConfigurationsperCell,

prsOccasionGroup,

prsFrequencyHoppingConfiguration,

...,

tddConfig

}

OtherRATMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {OtherRATMeasurementQuantities-ItemIEs} }

OtherRATMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OtherRATMeasurementQuantities-Item	CRITICALITY reject	TYPE OtherRATMeasurementQuantities-Item PRESENCE mandatory}}

OtherRATMeasurementQuantities-Item ::= SEQUENCE {

otherRATMeasurementQuantitiesValue			OtherRATMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { OtherRATMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

OtherRATMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OtherRATMeasurementQuantitiesValue ::= ENUMERATED {

geran,

utran,

... ,

nR,

eUTRA

}

OtherRATMeasurementResult ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF OtherRATMeasuredResultsValue

OtherRATMeasuredResultsValue ::= CHOICE {

resultGERAN									ResultGERAN,

resultUTRAN									ResultUTRAN,

resultNR									ResultNR,

resultEUTRA									ResultEUTRA,

otherRATMeasuredResultsValue-Extension		ProtocolIE-Single-Container {{ OtherRATMeasuredResultsValue-ExtensionIE }}

}

OtherRATMeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

Outcome ::= ENUMERATED {

failed,

...

}

#### -- P

PCI-EUTRA ::= INTEGER (0..503, ...)

PhysCellIDGERAN ::= INTEGER (0..63, ...)

PhysCellIDUTRA-FDD ::= INTEGER (0..511, ...)

PhysCellIDUTRA-TDD ::= INTEGER (0..127, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

#### -- Editor’s Note: Details of the following IEs are FFS pending RAN2 progress

PosSIBs ::= SEQUENCE (SIZE (1.. maxNrOfPosSIBs)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

posSIB-Segments					PosSIB-Segments,

assistanceInformationMetaData	AssistanceInformationMetaData	OPTIONAL,

broadcastPriority				INTEGER (1..16,...)				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSIBs-ExtIEs} }	OPTIONAL,

...

}

PosSIBs-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Segments ::= SEQUENCE (SIZE (1.. maxNrOfSegments)) OF SEQUENCE {

assistanceDataSIBelement			OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { PosSIB-Segments-ExtIEs} }	OPTIONAL,

...

}

PosSIB-Segments-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Type ::= ENUMERATED {

posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType2-20,

posSibType2-21,

posSibType2-22,

posSibType2-23,

posSibType3-1,

posSibType3-2,

posSibType3-3,

posSibType4-1,

posSibType5-1,

...

}

PRS-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

PRSConfiguration ::= SEQUENCE {

-- IE contents are FFS pending RAN2

nR-PRS-Beam-Information NR-PRS-Beam-Information OPTIONAL,

...

}

NR-PRS-Beam-Information ::= SEQUENCE {

nR-PRS-Beam-InformationList SEQUENCE (SIZE(1.. maxnoofResourcesPerSet)) OF NR-PRS-Beam-InformationItem,

lCG-to-GCS-TranslationList SEQUENCE (SIZE(1..maxnolcs-gcs-translation)) OF LCG-to-GCS-TranslationItem,

iE-Extensions	ProtocolExtensionContainer { { NR-PRS-Beam-Information-IEs} } OPTIONAL,

...

}

NR-PRS-Beam-Information-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NR-PRS-Beam-InformationItem ::= SEQUENCE {

pRSresourceID INTEGER (0..7),

pRSAngleItem SEQUENCE (SIZE(1..maxnoofAngleInfo)) OF PRSAngleItem,

...

}

PRSAngleItem  ::= SEQUENCE {

pRSresourceAngleID		INTEGER (0..63),

nRPRSAzimuth		INTEGER (0..359),

nRPRSAzimuthFine	INTEGER (0..9) OPTIONAL,

nRPRSElevation		INTEGER (0..180) OPTIONAL,

nRPRSElevationFine	INTEGER (0..9) OPTIONAL,

...

}

LCG-to-GCS-TranslationItem::= SEQUENCE {

alpha				INTEGER (0..359) 	OPTIONAL,

alphaFine			INTEGER (0..9) 		OPTIONAL,

beta				INTEGER (0..359)	OPTIONAL,

betaFine			INTEGER (0..9) 		OPTIONAL,

gamma				INTEGER (0..359) 	OPTIONAL,

gammaFine			INTEGER (0..9) 		OPTIONAL,

...

}

PRS-ConfigurationIndex-EUTRA ::= INTEGER (0..4095, ...)

PRS-ID-EUTRA	::= INTEGER (0..4095, ...)

PRSMutingConfiguration-EUTRA ::= CHOICE {

two 												BIT STRING (SIZE (2)),

four 												BIT STRING (SIZE (4)),

eight 												BIT STRING (SIZE (8)),

sixteen 											BIT STRING (SIZE (16)),

thirty-two											BIT STRING (SIZE (32)),

sixty-four											BIT STRING (SIZE (64)),

one-hundred-and-twenty-eight						BIT STRING (SIZE (128)),

two-hundred-and-fifty-six							BIT STRING (SIZE (256)),

five-hundred-and-twelve								BIT STRING (SIZE (512)),

one-thousand-and-twenty-four						BIT STRING (SIZE (1024)),

pRSMutingConfiguration-EUTRA-Extension			ProtocolIE-Single-Container {{ PRSMutingConfiguration-EUTRA-ExtensionIE }}

}

PRSMutingConfiguration-EUTRA-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

PRSOccasionGroup-EUTRA ::= ENUMERATED {

og2,

og4,

og8,

og16,

og32,

og64,

og128,

...

}

PRSFrequencyHoppingConfiguration-EUTRA ::= SEQUENCE {

noOfFreqHoppingBands		NumberOfFrequencyHoppingBands,

bandPositions				SEQUENCE(SIZE (1..maxnoFreqHoppingBandsMinusOne)) OF NarrowBandIndex,

iE-Extensions				ProtocolExtensionContainer { { PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs} } OPTIONAL,

...

}

PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-ID ::= INTEGER (0..63)

PRS-Resource-Set-ID ::= INTEGER(0..7)

PRS-ID ::= INTEGER(0..255)

-- IE FFS

#### -- Q

#### -- R

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

referenceSignal-Extension				ProtocolIE-Single-Container {{ReferenceSignal-ExtensionIE }}

}

ReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

RelativeCoordinateID ::= INTEGER (0..2147483647)

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

numberOfSRSResourceSet		INTEGER (1..15, ...)		OPTIONAL,

numberOfSRSResourcePerSet	INTEGER (1..64, ...)		OPTIONAL,

spatialRelationInformation	SpatialRelationInformation	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedSRSTransmissionCharacteristics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRP-Item

ResultCSI-RSRP-Item ::= SEQUENCE {

nR-PCI					NR-PCI,

nR-ARFCN				NR-ARFCN,

nG-RAN-CGI				NG-RAN-CGI													OPTIONAL,

valueCSI-RSRP-Cell		ValueRSRP-NR												OPTIONAL,

cSI-RSRP-PerCSI-RS		ResultCSI-RSRP-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRP-PerCSI-RS-Item

ResultCSI-RSRP-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRQ-Item

ResultCSI-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI														OPTIONAL,

valueCSI-RSRQ-Cell	ValueRSRQ-NR													OPTIONAL,

cSI-RSRQ-PerCSI-RS		ResultCSI-RSRQ-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRQ-PerCSI-RS-Item

ResultCSI-RSRQ-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultEUTRA ::= SEQUENCE (SIZE (1.. maxEUTRAMeas)) OF ResultEUTRA-Item

ResultEUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

valueRSRP-EUTRA		ValueRSRP-EUTRA												OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultEUTRA-Item-ExtIEs} }	OPTIONAL,

...

}

ResultEUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRP-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-EUTRA-Item

ResultRSRP-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-EUTRA			CGI-EUTRA OPTIONAL,

valueRSRP-EUTRA		ValueRSRP-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRP-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-EUTRA-Item

ResultRSRQ-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-UTRA			CGI-EUTRA OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRQ-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRP-Item

ResultSS-RSRP-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI													OPTIONAL,

valueSS-RSRP-Cell	ValueRSRP-NR												OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRP-PerSSB-Item

ResultSS-RSRP-PerSSB-Item ::= SEQUENCE {

sSB-Index			INTEGER (0..63),

valueSS-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRQ-Item

ResultSS-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI													OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR												OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRQ-PerSSB-Item

ResultSS-RSRQ-PerSSB-Item ::= SEQUENCE {

sSB-Index			INTEGER (0..63),

valueSS-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRMeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

valueSS-RSRP-Cell	ValueRSRP-NR											OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR											OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB									OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} }	OPTIONAL,

...

}

ResultNR-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SRSConfiguration ::= SEQUENCE {

-- IE contents are FFS pending RAN2

sFNInitializationTime BIT STRING (SIZE(64)),

iE-Extensions					ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SFNInitialisationTime-EUTRA ::= BIT STRING (SIZE (64))

SpatialRelationInformation ::= SEQUENCE {

spatialInformation		SpatialInformation,

iE-Extensions					ProtocolExtensionContainer { { SpatialRelationInformation-ExtIEs } } OPTIONAL,

...

}

SpatialRelationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- IE FFS

SpatialInformation ::= CHOICE {

pRSInformation		PRSInformation,

sSBInformation		SSB,

choice-extension				ProtocolIE-Single-Container {{ SpatialInformation-ExtIEs }}

}

SpatialInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

-- IE FFS

PRSInformation  ::= SEQUENCE {

pRS-ID					PRS-ID,

pRS-Resource-Set-ID		PRS-Resource-Set-ID,

pRS-Resource-ID			PRS-Resource-ID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSInformation-ExtIEs} } OPTIONAL,

...

}

PRSInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- IE FFS

SpatialRelationforResourceID ::= SEQUENCE (SIZE(1..maxnoSpatialRelations)) OF ReferenceSignal

SRSPosResourceID ::= INTEGER (0..63, ...)

SRSResourceID ::= INTEGER (0..63, ...)

SRSResourceSetID ::= INTEGER (0..15, ...)

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL,

...

}

SRSResourceTrigger-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSSpatialRelation ::= SEQUENCE {

spatialRelationforResourceID				SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SRSSpatialRelation-ExtIEs} }	OPTIONAL,

...

}

SRSSpatialRelation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSB ::= SEQUENCE {

pCI-NR				INTEGER  (0..1007),

ssb-index			INTEGER  (0..63),

tF-Configuration	TF-Configuration	OPTIONAL, -- TF-Configuration is FFS se spatial relation

iE-Extensions		ProtocolExtensionContainer { {SSB-ExtIEs} }	OPTIONAL,

...

}

SSB-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz120, kHz240, ...},

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sFN-initialization-time		BIT STRING (SIZE(64))	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TF-Configuration-ExtIEs} }	OPTIONAL,

...

}

TF-Configuration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- TF Configuration is FFS see spatial relation

SSBConfiguration ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

SSB-Index ::= INTEGER(0..63)

SSID ::= OCTET STRING (SIZE(1..32))

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TAC ::= OCTET STRING (SIZE(3))

TDD-Config-EUTRA-Item ::= SEQUENCE {

subframeAssignment			ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6, ... },

iE-Extensions				ProtocolExtensionContainer { { TDD-Config-EUTRA-Item-Item-ExtIEs } } OPTIONAL,

...

}

TDD-Config-EUTRA-Item-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimingInformation ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

TimeStamp ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

TP-ID-EUTRA ::= INTEGER (0..4095, ...)

TP-Type-EUTRA ::= ENUMERATED { prs-only-tp, ... }

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp								OPTIONAL,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOAMeasurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension							ProtocolIE-Single-Container { { TrpMeasuredResultsValue-ExtIEs } }

}

TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TrpMeasurementQuality ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRP-ID							TRP-ID,

...

}

TRP-MeasurementResponseList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementResponseItem

TRP-MeasurementResponseItem ::= SEQUENCE {

tRP-ID							TRP-ID,

measurementResult				TrpMeasurementResult,

...

}

TRPInformationList ::= SEQUENCE (SIZE (1.. maxnoTRPs)) OF SEQUENCE {

tRP-ID							TRP-ID,

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { {TRPInformation-ExtIEs} } OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPInformation ::= SEQUENCE (SIZE (1..maxnoTRPInfoTypes)) OF TRPInformationItem

TRPInformationItem ::= CHOICE {

prsid										INTEGER  (0..255),

pCI-NR										INTEGER  (0..1007),

nG-RAN-CGI									NG-RAN-CGI,

aRFCN										INTEGER (0..3279165),

timingInformation							TimingInformation,

pRSConfiguration							PRSConfiguration,

sSBConfiguration							SSBConfiguration,

sFNInitializationTime						BIT STRING (SIZE(64)),

-- spatialDirectionInformation					SpatialDirectionInformation,

-- FFS seems not needed remove from ASN.1 for compilation

geographicalCoordinates						GeographicalCoordinates,

...

-- other IEs to be added later

}

TRPInformationTypeList ::= SEQUENCE (SIZE(1.. maxnoTRPInfoTypes)) OF TRPInformationTypeItem

TRPInformationTypeItem ::= ENUMERATED {

prsid,

nrPCI,

nG-RAN-CGI,

arfcn,

timingInfo,

pRSConfig,

sSBConfig,

sFNInitTime,

spatialDirectInfo,

geoCoord,

-- other items to be added here

...

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF TRPItem

TRPItem ::= SEQUENCE {

tRP-ID		TRP-ID,

...

}

TRP-ID ::= INTEGER (1.. maxnoTRPs, ...)

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UARFCN ::= INTEGER (0..16383, ...)

UE-Measurement-ID ::= INTEGER (1..15, ..., 256)

UTRA-EcN0 ::= INTEGER (0..49, ...)

UTRA-RSCP ::= INTEGER (-5..91, ...)

UL-AoA ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

UL-RTOAMeasurement ::= SEQUENCE {

-- IE contents are FFS pending RAN2

additionalPathLoss	AdditionalPathLoss	OPTIONAL,

...

}

UL-SRS-RSRP ::= SEQUENCE {

-- IE contents are FFS pending RAN2

...

}

#### -- V

ValueRSRP-EUTRA ::= INTEGER (0..97, ...)

ValueRSRQ-EUTRA ::= INTEGER (0..34, ...)

ValueRSRP-NR ::= INTEGER (0..127)

ValueRSRQ-NR ::= INTEGER (0..127)

#### -- W

WLANMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {WLANMeasurementQuantities-ItemIEs} }

WLANMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-WLANMeasurementQuantities-Item	CRITICALITY reject	TYPE WLANMeasurementQuantities-Item PRESENCE mandatory}}

WLANMeasurementQuantities-Item ::= SEQUENCE {

wLANMeasurementQuantitiesValue			WLANMeasurementQuantitiesValue,

iE-Extensions							ProtocolExtensionContainer { { WLANMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasurementQuantitiesValue ::= ENUMERATED {

wlan,

...

}

WLANMeasurementResult ::= SEQUENCE (SIZE (1..maxNoMeas)) OF WLANMeasurementResult-Item

WLANMeasurementResult-Item ::= SEQUENCE {

wLAN-RSSI			WLAN-RSSI,

sSID				SSID					OPTIONAL,

bSSID				BSSID					OPTIONAL,

hESSID				HESSID					OPTIONAL,

operatingClass		WLANOperatingClass		OPTIONAL,

countryCode			WLANCountryCode			OPTIONAL,

wLANChannelList		WLANChannelList			OPTIONAL,

wLANBand			WLANBand				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementResult-Item-ExtIEs } }	OPTIONAL,

...

}

WLANMeasurementResult-Item-ExtIEs	NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLAN-RSSI ::= INTEGER (0..141, ...)

WLANBand ::= ENUMERATED {band2dot4, band5, ...}

WLANChannelList ::= SEQUENCE (SIZE (1..maxWLANchannels)) OF WLANChannel

WLANChannel ::= INTEGER (0..255)

WLANCountryCode ::= ENUMERATED {

unitedStates,

europe,

japan,

global,

...

}

WLANOperatingClass ::= INTEGER (0..255)

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

NRPPA-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

NRPPATransactionID		::= INTEGER (0..32767)

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local			INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-assistanceInformationControl									ProcedureCode ::= 100

id-assistanceInformationFeedback								ProcedureCode ::= 101

id-positioningInformationExchange								ProcedureCode ::= 102

id-positioningInformationUpdate									ProcedureCode ::= 103

id-Measurement													ProcedureCode ::= 104

id-MeasurementReport											ProcedureCode ::= 105

id-MeasurementUpdate											ProcedureCode ::= 105

id-MeasurementAbort												ProcedureCode ::= 106

id-MeasurementFailureIndication									ProcedureCode ::=  107

id-tRPInformationExchange										ProcedureCode ::= 108

id-positioningActivation										ProcedureCode ::= 109

id-positioningDeactivation										ProcedureCode ::= 110

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxIndexesReport							INTEGER ::= 64

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxCellReportNR								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxEUTRAMeas								INTEGER ::= 8

maxGERANMeas								INTEGER ::= 8

maxNRMeas									INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNoPath									INTEGER ::= 2

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNoOfMeasTRPs								INTEGER ::= 16

maxnoTRPs									INTEGER ::= 16384

maxnoTRPInfoTypes							INTEGER ::= 64

maxnoofResourcesPerSet						INTEGER ::= 2

maxnoofAngleInfo							INTEGER ::= 65535

maxnolcs-gcs-translation					INTEGER ::= 3

maxnoBcastCell								INTEGER ::= 16384

maxnoSRSTriggerStates						INTEGER ::= 3

maxnoSpatialRelations						INTEGER ::= 64

maxnoMeas									INTEGER ::= 63

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 100

id-Broadcast													ProtocolIE-ID ::= 101

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 102

id-SRSConfiguration												ProtocolIE-ID ::= 103

id-MeasurementResult											ProtocolIE-ID ::= 105

id-TRP-ID														ProtocolIE-ID ::= 106

id-TRPInformationTypeList										ProtocolIE-ID ::= 107

id-TRPInformationList											ProtocolIE-ID ::= 108

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 109

id-ResultSS-RSRP												ProtocolIE-ID ::= 110

id-ResultSS-RSRQ												ProtocolIE-ID ::= 111

id-ResultCSI-RSRP												ProtocolIE-ID ::= 112

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 113

id-AngleOfArrivalNR												ProtocolIE-ID ::= 114

id-GeographicalCoordinates										ProtocolIE-ID ::= 115

id-PositioningBroadcastCells									ProtocolIE-ID ::= 118

id-LMF-Measurement-ID											ProtocolIE-ID ::= 119

id-RAN-Measurement-ID											ProtocolIE-ID ::= 120

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 121

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 122

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 123

id-SRSType														ProtocolIE-ID ::= 124

id-ActivationTime												ProtocolIE-ID ::= 125

id-SRSResourceSetID												ProtocolIE-ID ::= 126

id-TRPList														ProtocolIE-ID ::= 127

id-SRSSpatialRelation											ProtocolIE-ID ::= 128

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

NRPPA-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Containers (5)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NRPPA-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

NRPPA-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

NRPPA-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { NRPPA-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		NRPPA-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					NRPPA-PROTOCOL-EXTENSION.&id			({ExtensionSetParam}),

criticality			NRPPA-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		NRPPA-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { NRPPA-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { NRPPA-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PRIVATE-IES.&id				({IEsSetParam}),

criticality		NRPPA-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

END OF CHANGES