3GPP TSG-RAN WG3 #109-e     					                                                   R3-204636

- 28 Aug 2020

Online

Agenda item:	31.3.7.1

Source: 	ZTE

Title: 	Discussion on Left issues of  MLB in Release 16

Document for:	Discussion and Approval

Introduction

In Release 16, MLB has been discussed, and some agreements have been reached. However, there is some ambiguous description on some issues. In this contribution, we will clarify the ambiguous issues of MLB in Release 16.

Discussion

Reporting mechanism of MLB

Some agreements of reporting mechanism of MLB in Release 16 are shown as follows:

----------------------------------------------------TS 38.423--------------------------------------------------------------------------------

NG-RAN node1 initiates the procedure by sending the RESOURCE STATUS REQUEST message to NG-RAN node2 to start a measurement, stop a measurement or add cells to report for a measurement. Upon receipt, NG-RAN node2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

----------------------------------------------------TS 38.423--------------------------------------------------------------------------------

However, this above highlighted agreement seems not very considerate. For example, if the SSB index within the cell that is already initiated for reporting has been changed, the Resource Status Request message should not be ignored, because this measurement should be regarded as a new measurement. Furthermore, assuming the SSB indexes in the original measurement are SSB1 and SSB2, this scenario of adding a cell that is already initiated for reporting can be divided into three cases.

Case 1: The SSB indexes in the newly triggered measurement are still SSB1 and SSB2.

Case 2: The SSB indexes in the newly triggered measurement are SSB1, SSB2 and SSB3.

Case 3: The SSB indexes in the newly triggered measurement are SSB1, SSB3 and SSB4.

For Case 1, it is obvious that the Resource Status Request message shall be ignored as there is not any new measurement.

For Case 2 and Case 3, the new measurement objects have been added and the new Measurement ID should be provided. In this cases, the Resource Status Request message should not be ignored.

Proposal 1: The description of the above agreement should be changed to “If measurements are already initiated

for a cell indicated in the Cell To Report IE with the same Measurement ID, this information shall be ignored”

DL/UL scheduling PDCCH CCE usage over Xn and F1

After checking the related ASN.1 code of Radio Resource Status, the DL and UL scheduling PDCCH CCE usage over Xn are missing, while the two IEs over F1 exist.

----------------------------------------------------------TS 38.423------------------------------------------------------------------

GNB-RadioResourceStatus	::= SEQUENCE {

ssbAreaRadioResourceStatus-List				SSBAreaRadioResourceStatus-List,

iE-Extensions							ProtocolExtensionContainer { { GNB-RadioResourceStatus-ExtIEs} },

...

}

GNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

sSBIndex								INTEGER(0..63),

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

----------------------------------------------------------TS 38.423------------------------------------------------------------------

----------------------------------------------------------TS 38.473------------------------------------------------------------------

SSBAreaRadioResourceStatusList::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF	SSBAreaRadioResourceStatusItem

SSBAreaRadioResourceStatusItem::= SEQUENCE {

sSBIndex					INTEGER(0..63),

sSBAreaDLGBRPRBusage		INTEGER (0..100),

sSBAreaULGBRPRBusage		INTEGER (0..100),

sSBAreaDLnon-GBRPRBusage	INTEGER (0..100),

sSBAreaULnon-GBRPRBusage	INTEGER (0..100),

sSBAreaDLTotalPRBusage		INTEGER (0..100),

sSBAreaULTotalPRBusage		INTEGER (0..100),

dLschedulingPDCCHCCEusage	INTEGER (0..100)		OPTIONAL,

uLschedulingPDCCHCCEusage	INTEGER (0..100) 		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SSBAreaRadioResourceStatusItem-ExtIEs} } OPTIONAL

}

SSBAreaRadioResourceStatusItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

----------------------------------------------------------TS 38.473------------------------------------------------------------------

Observation: The DL and UL scheduling PDCCH CCE usage are missing over Xn.

In addition, the IE structure related to the two IEs are different over Xn and F1, and the corresponding IE tables are shown as below.

----------------------------------------------------------TS 38.423------------------------------------------------------------------

9.2.2.50	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Radio Resource Status Type | M |  |  |  |
| >ng-eNB |  |  |  |  |
| >>DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| >>UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| >>DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| >>UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| >>DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| >>UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| >gNB |  |  |  |  |
| >>SSB Area Radio Resource Status List |  | 1 |  |  |
| >>>SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| >>>>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| >>>>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| >>>>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| >>>>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| >>>>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

----------------------------------------------------------TS 38.423------------------------------------------------------------------

----------------------------------------------------------TS 38.473------------------------------------------------------------------

9.3.1.129	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

----------------------------------------------------------TS 38.473------------------------------------------------------------------

It is obvious that  the IEs DL/UL scheduling PDCCH CCE usage in F1 and Xn represent different granularity in current specifications. In Xn, the two IEs are represented as cell level,  while the two IEs are represented as SSB level over F1.

Proposal 2: The granularity of DL/UL scheduling PDCCH CCE usage within the Radio Resource Status IE should be aligned over F1 and Xn.

RRC Connections over Xn

In current TS38.423, the description of the RRC Connections IE is ambiguous. To be specific, “status” would cause confusion with the word “state”, bringing unnecessary consideration of other RRC states like RRC_INACTIVE, though the RRC Connections IE only focuses on UEs in RRC_CONNECTED state.

--------------------------------------------TS 38.423----------------------------------------------------

9.2.2.56	RRC Connections

The RRC Connections IE indicates the overall status of RRC connections per cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of RRC Connections | M |  | 9.2.2.57 |  |
| Available RRC Connection Capacity Value | M |  | 9.2.2.58 |  |

9.2.2.57	Number of RRC Connections

The Number of RRC Connections IE indicates the absolute number of UEs in RRC_CONNECTED mode.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of RRC Connections | M |  | INTEGER (1..65536,...) |  |

9.2.2.58	Available RRC Connection Capacity Value

The Available RRC Connection Capacity Value IE indicates the residual percentage of the number of RRC connections, relative to the maximum number of RRC connections supported by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available RRC Connection Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |

-----------------------------------------------TS 38.423---------------------------------------------------

To avoid the ambiguity, the description of the RRC Connections IE could be reworded as follows.

The RRC Connections IE indicates the information about RRC connections related to UEs in RRC_CONNECTED mode per cell.

Proposal 3: The description of the RRC Connections IE should be reworded to avoid the ambiguity.

Conclusion

Proposal 1: The description of the above agreement should be changed to “If measurements are already initiated

for a cell indicated in the Cell To Report IE with the same Measurement ID, this information shall be ignored”.

Observation: The DL and UL scheduling PDCCH CCE usage are missing over Xn.

Proposal 2: The granularity of DL/UL scheduling PDCCH CCE usage within the Radio Resource Status should be aligned over F1 and Xn.

Proposal 3: The description of the RRC Connections IE should be reworded to avoid the ambiguity.

The corresponding CRs for Xn and F1 are provided in [1]and[2]respectively.

References

- CR for TS38.423 on Remaining Issue of MLB  ZTE

- CR for TS38.473 on Remaining Issue of MLB  ZTE