3GPP TSG-RAN WG3 Meeting #109-e	R3-204745

Electronic Meeting, August 17th – 28th, 2020

Agenda item:		13.2.1

Source:	Intel Corporation

Title:	Inter-CU topology adaptation procedure

Document for:		Discussion and Decision

# Introduction

In RAN#86, WI on enhancement to IAB was approved in RP-193251. One of the objectives is topology adaptation enhancement, and it includes the following:

- Specification of procedures for inter-donor IAB-node migration to enhance robustness and load-balancing, including enhancements to reduce signalling load.

- Specification of enhancements to reduce service interruption due to IAB-node migration and BH RLF recovery.

- Specification of enhancements to topological redundancy, including support of CP/UP separation.

In this contribution inter-CU topology adaptation procedure is presented.

# Discussion

## baseline for Inter-CU topology adaptation procedure

In Release 16 an inter-donor IAB node with inter-DU intra-CU topology adaptation scenario is addressed in section 8.2.3.1 of TS 38.401 [1]. The following figure taken from section 9.7.5 TR 38.874 [2] illustrates the migration scenario:

In section 9.7.6 TR 38.874[2] inter-donor IAB-node migration scenario with inter-DU and inter-CU topology is described. The following diagram illustrate the scenario:

Figure 1

The following call flow illustrates the procedure:

Figure 2

Observation 1: In TR 38.874 [2] section 9.7.6 contains the migration procedure for an inter-donor IAB-node with inter-DU and inter-CU scenario.

Proposal 1: RAN3 to discuss and adopt the call flow in section 9.7.6 TR 38.874[2] with modification as necessary for TS 38.401 [1].

## Minimize congestion due to handover

In the above scenario, IAB-node 5 is acting as an access IAB node, when it receives the RRCReconfiguration message, it will stop granting UE1 uplink grant (See figure 3 below), hence stop the incoming data from UE1. However, IAB-node 7 doesn’t know a handover taking place and it will continue to allow UE2 to send data. Depends on the interruption time in step 10 of figure 2 and the number of UE accessing IAB-node 7, this will create a congestion and eventually a buffer overflow in the IAB-node, although hop by hop flow control via uplink grant might help, but it will be more effective for the donor-CU-CP to send a F1AP message to notify the access IAB node that there is handover happening in one of the parent IAB node.

Figure 3

Observation 2: Access IAB node connect through child node might takes longer to realize a handover is about to take place at the migrating parent IAB node.

Proposal 2: Donor-CU-CP sends a F1-AP message to notify the access IAB node about an imminent handover at one of the migrating parent’s IAB node.

An F1-AP message that can be used as a baseline for discussion is the Network Access Rate Reduction Message. The purpose of this message is to indicate to the gNB-DU that the rate at which UE are accessing the network need to be reduced from its current level. A flag can be added to this message to indicate all/certain percentage of uplink access should temporarily stop until handover is successful. This message should send by gNB-CU1 just before step 7 of Figure 2, and resend by gNB-CU2 to reset the access restriction after the handover is successful.

One possible realization for such an indication is provided in the appendix.

# Conclusion

In the present contribution we make the following observations:

Observation 1: In TR 38.874 [2] section 9.7.6 contains the migration procedure for an inter-donor IAB-node with inter-DU and inter-CU scenario.

Observation 2: Access IAB node connect through child node might takes longer to realize a handover is about to take place at the migrating parent IAB node.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 to discuss and adopt the call flow in section 9.7.6 TR 38.874[2] with modification as necessary for TS 38.401 [1].

Proposal 2: Donor-CU-CP sends a F1-AP message to notify the access IAB node about an imminent handover at one of the migrating parent’s IAB node.

# Appendix

#### 9.2.1.19	NETWORK ACCESS RATE REDUCTION

This message is sent by the gNB-CU to indicate to the gNB-DU a need to reduce the rate at which UEs access the network.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UAC Assistance Information | M |  | 9.3.1.83 |  | YES | reject |
| Imminent HO Indication | O |  | ENUMERATED(true, …) | Indicates that the gNB-DU is likely to be handed over. | YES | ignore |

New text show in highlighted yellow.

# Reference

[1] TS 38.401 “NG-RAN Architecture Description”

[2] TR 38.874 “Study on Integrated Access and Backhaul”