3GPP TSG-RAN3 Meeting #109-e	R3-204988

E-meeting, 17 - 28 August 2020

Title: 	Further discussion on NAS Non-Delivery

Source: 	Huawei

Agenda item:	9.3.1

Document for:	Discussion

1. Introduction

Last RAN3#108-e meeting discussed NAS non-delivery based on the reply LS from SA2 with the summary in [1].

- Discuss answer 2 of SA2 LS received in 4126 at next RAN3 meeting. Answer 2 to be continued...

This contribution intends to address the answer 2, and provide the corresponding update and further rely LS.

2. Discussion

- The reply LS is provided in [2], and copied as follows.

Q1/ RAN3 would like to ask SA2 to clarify the meaning of the above statement in TS 23.501 for the considered scenario and clarify whether option 1/ or option 2/ is the expected behaviour? Ans 1: N1N2Transfer failure notification from AMF to SMF is not needed. Thus, NG-RAN node trigger of the NAS Non delivery procedure for PDU Session Release is not required. For PDU session Resource Release Command, there should be a PDU Session Resource Release response from RAN before UE context release occurs. Q2/ In general, does SA2 see any other scenario for which the 5GC expects the NAS-non-delivery report in addition to those failed NAS-PDUs in the DL NAS Transport message? Ans 2: For "Initial Context Setup Request" & "PDU Session Resource Setup" which have non-PDU session related NAS messages, indication from RAN of failure of these non-PDU session related NAS messages may be needed. SA2 requests RAN3 to decide how to handle the failure notification to AMF of these non-PDU session related NAS messages. Typically, "PDU Session Resource Setup" containing non-PDU session related NAS message is sent in RRC CONNECTED state, but SA2 identified one possible case (Selective activation of UP connection ) where it could be sent by the AMF also when the gNB and UE are in RRC Inactive state.

- It can be observed that for answer 2, that SA2 think that for the non-PDU session related NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup", the NAS non-delivery may be needed. This is due to the fact there is no response message to indicate the cause for these non-PDU session NAS-PDU. Specifically

- the non-PDU session NAS-PDU in the PDU Session Resource Setup may include the NAS service setup message to indicate the selective activation of UP connection. When the inactive UE is unreachable, the AMF should be aware of the non-delivery result.

- So far for inactive UE, it is not much clear what the non-PDU session NAS-PDU will carry in the Initial Context Setup Request message (or even not needed at all given that the UE context has been already setup). But for idle UE, this NAS-PDU may carry the NAS registration accept message. In case this NAS-PDU can not be transferred to the UE, the NG-RAN should also notify the core network. Hence, for safety purpose, it is suggested to have the same principle to report the NAS non-delivery result to the core network.

- Hence the following proposal is made:

Report the NAS NON DELIVERY INDICATION for the UE level NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup" in case of failed delivery.

Reply to SA2 regarding RAN3 agreements.

3. Conclusion

This contribution continuously discuss the direct data forwarding for inter-syste handover with the following proposals.

- Report the NAS NON DELIVERY INDICATION for the UE level NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup" in case of failed delivery.

- Reply to SA2 regarding RAN3 agreements.

The CRs can be found in [3-4], and the reply LS is provided in [5].

4. Reference

- R3-204054 Summary of Offline Discussion on NAS PDU NON Delivery, Nokia, Nokia Shanghai Bell – Moderator

- R3-204239 Reply LS on NAS Non delivery for RRC Inactive state, Cisco

- R3-204989	Further discussion on NAS Non-Delivery, Huawei

- R3-204990	Further discussion on NAS Non-Delivery, Huawei

- R3-204991	[Draft] Further Reply LS on NAS Non delivery for RRC Inactive state, Huawei

- TS 38.413 v16.2.0.