3GPP TSG-RAN WG3 Meeting #109-e	R3-205200

# Online, August 17th – 27th 2020

Agenda Item:	15.2

Source:	Ericsson

Title:	pCR for TR 38.8xx: NR QoE Management Framework

Document for:	Decision

# Introduction

The SID for SI on NR QoE management (RP-193256) states the following objective:

- Study the potential RAN side solution for supporting a generic framework for triggering, configuring, measurement collection and reporting for various 5G use cases. [RAN3, RAN2]

  - Identify and study the potential solutions (e.g., LTE based solution, reusing MDT mechanism) for configuration and reporting of UE KPI information for certain services (e.g. latency).

In that respect, this paper discusses the NR QoE Management framework. The pCR for TR 38.8xx is given in the Annex.

# High-level solution

Since the experience at the UE application layer level (i.e. the QoE) may not reflect the achieved QoS on the radio layer, it is of high interest for the operators to monitor how the end user experiences the service provided. Based on the QoE measurement reports, the operators can optimize the use of network resources and provide better service to end users.

The QoE management has been defined for UTRAN and E-UTRAN. The E-UTRAN QoE management framework exists in two flavours:

- Signalling-based QoE, where the QoE measurement configuration (QMC) is delivered to the RAN node via the UE-associated NGAP TRACE START message, inside the UE Application layer measurement configuration IE, included in the Trace Activation IE. The QMC specifies the area scope for the measurement, where the area scope is defined via a list or cells/TAs/TAIs/PLMNs.

- Management-based QoE, where the OAM delivers the QMC to the RAN node.  Based on the measurement criteria indicates in the QMC, combined with the knowledge of UE capabilities, the RAN node selects the UEs that should execute the QoE measurements.

In our view, the LTE QoE management framework should be the baseline for the work on its NR counterpart. This effectively means that NR QoE management can be specified in both signalling-based and management-based flavours, whereas RAN2/3 should discuss the enhancements with respect to the LTE solution while taking the NR peculiarities into consideration.

Proposal 1: The LTE QoE management framework, including management- and signalling-based solutions is the baseline for NR QoE management work.

One aspect to consider is whether the signalling-based solution should be piggybacked on trace messages (as was the case in LTE), or it should be conveyed in dedicated, newly defined messages. In our view, 3GPP should specify a separate set of QoE procedures, for several reasons:

- Trace/MDT and QoE are distinctly different features with different purposes and should thus be supported in separate procedures.

- The NR trace messages are designed in a trace-centric way. For instance, in NGAP TRACE START message, the Interfaces to Trace and Trace Depth IEs are mandatory. Furthermore, either the Trace Collection Entity IP Address or the Trace Collection Entity URI IE should be considered by the recipient of the message. On the other hand, it seems reasonable to allow in the specifications that QoE and MDT reports are received by different collection entities, assigned with different IP addresses/URIs.

Proposal 2: In signalling-based NR QoE management, the QMC should conveyed to the RAN and between RAN nodes via dedicated, newly defined procedures, rather than re-using the NR Trace messages.

One prominent feature of LTE QoE framework is that the QoE reports are carried over a newly defined SRB type, introduced specifically for this purpose. The main reason for such a decision was the possibility to configure the SRB for QoE measurements with lower priority, in order to avoid that the QoE reports are completing for resources with the high-priority CP messages, such as RRC or NAS messages which need to be sent very quickly, whereas a large QoE report might congest the SRB. The above argument is valid in NR as well, and it is thus preferred to define a dedicated NR SRB type for carrying the NR QoE reports.

Proposal 3: The NR QoE reports are delivered over a newly defined dedicated SRB type.

Based on the above, the following is proposed:

Proposal 4: RAN3 to agree the pCR for TR 38.8xx, presented in the Annex.

# Conclusion

This paper discusses the NR QoE Management framework and proposes the following:

Proposal 1: The LTE QoE management framework, including management- and signalling-based solutions is the baseline for NR QoE management work.

Proposal 2: In signalling-based NR QoE management, the QMC should conveyed to the RAN and between RAN nodes via dedicated, newly defined procedures, rather than re-using the NR Trace messages.

Proposal 3: The NR QoE reports are delivered over a newly defined dedicated SRB type.

Proposal 4: RAN3 to agree the pCR for TR 38.8xx, presented in the Annex.

# Annex: pCR to TR 38.8xx

-------------------------------------------Change 1-------------------------------------------

### 6.x NR QoE management framework

The following recommendations are given for the normative work:

- LTE QoE management framework is recommended as the baseline for the normative work on NR QoE management,

- Both the management- and signalling-based options are supported for NR QoE management.

- RAN3 recommends defining dedicated procedures for the signalling-based solution.

- For carrying the QoE measurement reports, it is recommended to define a dedicated NR SRB type.

-------------------------------------------End of changes-------------------------------------------