3GPP TSG-RAN WG3 Meeting #109-e	R3-205206

# E-meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0451 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of RACH Conflict Resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of enhanced RACH conflict resolution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of RACH Conflict Resolution Info in Access and Mobility Indication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.25, 9.2.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| RACH Conflict Resolution Info | O |  |  |  |  |  |
| >DU-CU RACH Failure and Success Information | O |  | 9.2.3.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

#### 9.2.3.X	DU-CU RACH Failure and Success Information

The DU-CU RACH Failure and Success Information IE contains information about RACH successes and failures extracted at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | 9.3.1.12 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of RACH Failures on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Successes on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Failures on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |
| >>>Number of RACH Successes on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |