3GPP TSG-RAN WG3 #109	R3-205211

# Online, 17th - 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | xxxx | rev |  | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting of SFTD measurements in LPPa E-CID measurement results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of the SFN time differenciation measurements result in the E-CID measurement results to be reported in LPPa |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SFTD measurements with SFN offset and frame boundary values in the E-CID measurement result Impact Analysis: This CR has isolated impact with the previous version of the specification because this is additional information that may be used by receiving node This CR has no impact from functional point of view. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | insufficient measurements reports in E-CID. Insufficient information at E-SMLC for OTDOA assitance information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.1, 9.2.5, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by E-SMLC to initiate E-CID measurements.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SFTD) |  | - | - |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN,…) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value “Periodic”. |

NEXT CHANGE

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | ECGI 9.2.6 | E-UTRAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | OCTET STRING(2) | Tracking Area Code of the serving cell |
| E-UTRAN Access Point Position | O |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 1 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 2 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Result RSRP |  | 1 .. <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRP | M |  | INTEGER(0..97, …) |  |
| >>Result RSRQ |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRQ | M |  | INTEGER(0..34, …) |  |
| >>Result SFTD |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>SFN-Offset | M |  | INTEGER (0..1023) | The observed SFN and frame timing difference that a UE measured between an E-UTRA PCell and an NR Cell, |
| >>>MeanFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Average value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MinFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Minimum value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MaxFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Maximum value of the frame boundary offset |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

NEXT CHANGE

#### -- M

Measurement-ID ::= INTEGER (1..15, ...)

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue							MeasurementQuantitiesValue,

iE-Extensions										ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

...,

sFTD

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival				INTEGER (0..719),

valueTimingAdvanceType1			INTEGER (0..7690),

valueTimingAdvanceType2			INTEGER (0..7690),

resultRSRP						ResultRSRP,

resultRSRQ						ResultRSRQ,

...,

resultSFTD						ResultSFTD

}

MBSFNsubframeConfiguration ::= SEQUENCE (SIZE (1.. maxMBSFN-Allocations)) OF MBSFNsubframeConfigurationValue

MBSFNsubframeConfigurationValue ::=			SEQUENCE {

radioframeAllocationPeriod			ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset			INTEGER (0..7),

subframeAllocation					Subframeallocation

}

UNCHANGED TEXT OMITTED

#### -- R

RepetitionNumberofSIB1-NB ::= ENUMERATED {

r4,

r8,

r16,

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

...

}

ResultRSRP ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-Item

ResultRSRP-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRP		ValueRSRP,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRP-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-Item

ResultRSRQ-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRQ		ValueRSRQ,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRQ-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSFTD ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultSFTD-Item

ResultSFTD-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

sFN-Offset			INTEGER (0..1023),

frameBoundary-Offset		INTEGER (-30720..30719),

minFrameBoundary-Offset		INTEGER (-30720..30719),

maxFrameBoundary-Offset		INTEGER (-30720..30719),

iE-Extensions	ProtocolExtensionContainer { { ResultSFTD-Item-ExtIEs} } OPTIONAL,

...

}

ResultSFTD-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

END OF CHANGES