3GPP TSG-RAN WG3 #109-e							                                              R3-205322

17-28 August 2020

Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.423 Detection solution for SN change failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2020-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The detection solution for SN change failure needs to be introduced in NR. Since the MN can obtain all the information of the SN change-related failures, one-procedure method can be adopted for detecting the SN change failure in NR other than the two-procedure method used for detecting the connection failure due to intra-LTE mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add stage3 text to reflect the detection solution for SN change failure in NR, i.e. one-procedure method, the MN makes the determination and sends the SN CHANGE REPORT message to the source SN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Detection on SN change failure can not be supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.4.x, 9.1.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SN Change Report | SN CHANGE REPORT |

*** Second change, ommited text not changed ***

8.4.x	SN Change Report

8.4.x.1	General

The purpose of the SN Change Report procedure is to transfer mobility related information between eNBs.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SN Change Report, successful operation

An M-NG-RAN Node initiates the procedure by sending an SN CHANGE REPORT message to S-NG-RAN Node. By sending the message M-NG-RAN Node indicates to S-NG-RAN Node that a mobility-related problem was detected.

If the SN Change Report Type IE is set to "SN change too early", or "SN change to wrong cell", then the M-NG-RAN Node indicates to S-NG-RAN Node that, following a successful SN change from a cell of S-NG-RAN Node to a cell of new addition S-NG-RAN Node, a SCG failure occurred and UE is made to establish the radio link connection either at the original cell of S-NG-RAN Node (SN change Too early), or at another cell (SN change to Wrong Cell). The detection of SN change too early and SN change to wrong cell events is made according to TS 38.300 [9].

If the SN Change Report Type IE is set to "SN change too late", then the M-NG-RAN Node indicates to S-NG-RAN Node that, a SCG failure occurred in old S-NG-RAN node and UE is made to establish the radio link connection to a new S-NG-RAN Node (SN change too late). The detection of SN change too late is made according to TS 38.300 [9].

The M-NG-RAN Node may also include the SCG Failure Report received from the UE in the SN CHANGE REPORT as SCGFailureInformation defined in TS38.331[10]..

8.4.x.3	Unsuccessful Operation

Not applicable.

8.4.x.4	Abnormal Conditions

Void.

*** Third change, ommited text not changed ***

9.1.3.y	SN CHANGE REPORT

This message is sent by the M-NG-RAN Node to report a SN change failure event or other critical mobility problem.

Direction: M-NG-RAN Node  S-NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| SN Change Report Type | M |  | ENUMERATED (SN change too early, SN change too late，SN change to wrong cell, …) |  | YES | ignore |
| SN Change Cause | M |  | Cause 9.2.6 | Indicates SN change cause employed for SN change from S-NG-RAN Node | YES | ignore |
| Source PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 | PSCell ID in source SN (S-NG-RAN Node) | YES | ignore |
| Failure PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 | PSCell ID in failure SN or in target SN | YES | ignore |
| New addition PSCell ID | C- ifSNChangeReportType SNchangeToWrongCell |  | Global NG-RAN Cell Identity 9.2.2.27 | PSCell ID in new addition SN | YES | ignore |
| SCG Failure Report Container | O |  | OCTET STRING | FFS (TS 38.331 [10]) | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSNChangeReportType SNchangeToWrongCell | This IE shall be present if the SN Change Report Type IE is set to the value "SN change to wrong cell" |