3GPP TSG-RAN WG3 #108-e	R3-205491

Online, 1-11 June 2020

Agenda Item:	20.2.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # 34_NTN_nwID_handling

Document for:	Approval

# Introduction

CB: # 34_NTN_nwID_handling

- ZTE

The “On ground NTN gNB” shall know as little as possible about LEO satellite(s) flying in space, as NTN-GW(s) will do everything with NR-Uu signal in consequence as planned.

no need to introduce new NTN specific id or NTN specific handling of “network identities” for transparent LEO satellite.

- FH

Confirm that the list of network identities is complete with gNB ID, NCGI, NCI, NPCI, TAC, TAI, NID, and CAG ID.

Decide whether the previously introduced solutions for TAC/TAI of multiple TAIs and/or periodic registration area update are valid solutions or need further study in coordination with RAN2.

Decide whether gNB ID, NID, and CAG ID need further study in the scope of the WI.

Agree on paging solutions that do not require GNSS capability as baseline.

- HW

NG-RAN Network identifiers are reused without any restriction.

- CATT

Network Identifiers can be decoupled from strictly associated with geographical regions.

CGI for a given PLMN should keep constant, unless there is any topological shift on the link, e.g. switching to another satellite, another NTN gateway, or another gNB.

One cell broadcasts only one TAI per PLMN, and the boundary of TAs can float in Scenario C2.

- E///

It may be beneficial to differentiate a TA which contains NTN cells (fixed and/or moving) from a TA which does not.

Extend the TAC IE signaled over NGAP and XnAP with an optional TA Type IE, defined as e.g. ENUMERATED (NTN, NTN with moving cells, ...) so that the receiving node can identify that the cells associated with this TA are related to a NTN, and/or are moving with respect to the ground.

Introduce all necessary NTN-related IEs in RAN3 protocols as optional with criticality “reject”.

- Nok

current network identities handling can be reused for NTN

- SS

Both Earth Moving Cell ID and Earth Fixed Cell ID are supported

- CMCC

focus on Transparent satellite based NG-RAN architecture.

further evaluate the two solutions (fixed/moving identifiers on ground) and identify the scenarios which solution should be applied to.

need to support UE with and without location information schemes.

++ Suggested guidelines/topics for discussion from Chair (looking at possible consensus):

+ merge relevant discussion from 5166

+ Current NG-RAN Network identities are reused without any restriction (possible agreement)

+ whether to decouple network identifiers from strict association with geographical regions? (possible agreement/WA? Need to notify SA2?)

+ whether to introduce a flag for a TA containing NTN fixed or moving cells? (possible agreement/WA? If so, st3 TP/BL CR?)

+ whether to introduce any NTN-related IEs as optional, reject? (possible agreement?)

+ Paging, RAU, etc.: discussed in CB 36

+ st2 TP needed?

(Nok - moderator)

Summary of offline disc R3-205491

The discussion has two phases:

Phase 1: Enhancements to network identities handling

Phase 2: TBD

The deadline for Phase 1 is Thursday, August 20, 24:00 UTC. This allows us to discuss intermediate stage in Monday online session. We might be able to already achieve some agreements at this stage.

The deadline for Phase 2 is the same as for all email discussions, i.e., Tuesday, August 25, 1300 UTC.

# For the Chairman’s Notes

Propose the following:

Proposal 1: following network identities can be reused for NTN

- AMF Name

- NR Cell Global Identifier (NCGI)

- gNB Identifier (gNB ID)

- Global gNB ID

- Tracking Area identity (TAI)

- Single Network Slice Selection Assistance information (S-NSSAI)

Proposal 2: Tracking Area is coupled with geographical area.

Proposal 3-1: WA “no need to differentiate a TA that contains NTN cells (fixed and/or moving) from a TA which does not.”

Proposal 4: It is 2nd priority to support the UE without GNSS capability.

Proposal 5: There is no need to identify LEO satellite and NTN GW.

Proposal 7: current NG-RAN architecture is reused for NTN.

# Discussion

Background information:

TS 38.300 defines following network identities used in NG-RAN for identifying a specific network entity:

- AMF Name

- NR Cell Global Identifier (NCGI)

- gNB Identifier (gNB ID)

- Global gNB ID

- Tracking Area identity (TAI)

- Single Network Slice Selection Assistance information (S-NSSAI)

- Network Identifier (NID)

- Closed Access Group Identifier

NTN WID RP-201256 states:

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

•	FDD is assumed for core specification work for NR-NTN.

•	Earth fixed Tracking area is assumed with Earth fixed and moving cells

•	UEs with GNSS capabilities are assumed.

•	Transparent payload is assumed

## Network Identities handling

Contribution ([1][3][7]): no need to introduce new NTN specific id or NTN specific handling of “network identities” for transparent LEO satellite.

Q1: Please share your view on whether Rel-17 NTN can reuse current network identities handling.

| Company | Comment |
| --- | --- |
| Nokia | Yes. Rel-17 NTN can reuse current network identities handling. We do not see the need to introduce new NTN specific ID handling. |
| ZTE | Yes. No need to introduce new NTN specific ID handling. |
| Samsung | Yes. No see the big issue occur in case reuse current network identities handling |
| Ericsson | In general fine, but could we leave out NPN and other features out for a while and concentrate on basic functions? |
| CATT | Current network identities should be reused as possible, but we are afraid that we have to add some extra IDs, e.g. IDs of satellites—nevertheless the definition of these IDs seems to be outside 3GPP’s scope and thus can be defined as OCTET STRINGs if needed. |
| Thales | For Earth fixed cell scenarios, we share same view as Nokia => no need to introduce new NTN specific ID handling. |
| Huawei | Yes, there is no need for new network identities. |
| Fraunhofer IIS/HHI | Yes, no need to introduce NTN specific id or handling of “network identities for transparent LEO. |
| Qualcomm | Partly Yes – most of the above IDs can be kept as is. For NCGI, there are likely to be solutions which avoid change and solutions that may require a change. We believe, it is better to first consider the alternatives and then decide whether keeping NCGI without change is better. |
| Eutelsat | Yes in general. However, there may be value in identifying gateways (independent of gNB – they are unlikely to have a 1-to-1 mapping) for feeder link switchover (TR38.821 section 8.7) and management across international boundaries. The security value may also need to be studied.. Moderator: I assume you mean the NTN GW in TR38.821. NTN GW is a transport network node, which is not in the scope of NG-RAN architecture. If this is any issue, this may be discussed in future meeting. |

Summary:

- All 10 companies agreed the current network identities can be reused.

- 6 companies commented no need for new network identities.

- 1 company commented to concentrate basic functions, and leave out NPN

- 2 company commented some extra IDs may be needed, but this may be discussed in the future meeting if there is a need.

- 1 company commented the NCGI may need some discussion.

- It is proposed to agree the network identities (except NID and CAG ID) can be reused for NTN.

Proposal 1: following network identities can be reused for NTN

- AMF Name

- NR Cell Global Identifier (NCGI)

- gNB Identifier (gNB ID)

- Global gNB ID

- Tracking Area identity (TAI)

- Single Network Slice Selection Assistance information (S-NSSAI)

## Tracking Area decouple from geographical area

In current terrestrial network, the Tracking Area is fixed to a geographical area. Contribution ([4]) propose Network Identifiers can be decoupled from strictly associated with geographical regions, with the reason that geographical location information can be used for most cases that strictly associated with geographical location. The related network identifiers are CGI and TAI, and is mainly related to LEO transparent satellite.

- For Earth Fixed cell, the LEO satellite can adjust its antenna to serve a specific area. The coverage of a specific cell is fixed to a geographical area.

- For Earth Moving cell, contribution ([4]) proposes “CGI for a given PLMN should keep constant, unless there is any topological shift on the link, e.g. switching to another satellite, another NTN gateway, or another gNB.” The coverage of a specific cell is moving within an area.

So the question is mainly related to LEO Transparent satellite with Earth Moving cell.

Q2: Please share your view on whether need to decouple Tracking Area from geographical area, for LEO transparent satellite with Earth Moving cell.

| Company | Comment |
| --- | --- |
| Nokia | No. For Earth Moving cell, the coverage of a specific cell is still fixed to a geographical area, even the cell coverage is moving in this area (e.g. 10:00 covers west part of this area, 10:10 covers east part of this area). So there is no need to decouple TAI/CGI from geographical area. |
| ZTE | No. Same views as Nokia’s. |
| Samsung | Not sure if I understand the question. We assume fixed TAI is used. It is not decople with geographical area. For the CGI, in case of earth moving cell, if the conneted gNB is not changed, even the cell is moving, but the CGI is not changed. In this sense, it is not decouple with the geographical area. |
| Ericsson | No. Such approach would jeopardize basic TAI/Cell definition, definitions of identities which we aim at keeping for NTN purposes, see Q1. |
| CATT | For CGI, yes (should decouple). We prefer to use the CGI in a cycling manner within a geographical area, e.g.: For TAI, no (shouldn’t decouple). TAIs are used for paging (conventional paging at least), and should be associated with given geographical areas. Nevertheless the boundaries of TAs have to float slightly over time, as the footprints of NTN cells keep changing. What we propose is just “no need to associate strictly”, but they should be associated anyhow. |
| Thales | No. TAI and CGI should not be decoupled. However a mapping function between radio cell/beams and CGI may have to be introduced for Earth moving cell scenarios. |
| Huawei | No. We would like to agree this meeting and capture in stage 2 the recommendation from the TR that the “Tracking Area identity (TAI): used to identify tracking areas. A tracking area of a NTN is fixed on earth.” See R3-204963, on other CB. |
| Fraunhofer IIS/HHI | No. TAI/CGI should not be decoupled from geographical area, even if the borders of these geographical areas are not fixed. |
| Qualcomm | First we can assume for now (perhaps WA) that TA is earth-fixed and corresponds to a known geographical area. Now for decoupling we assume it means the relationship TA/cell can change. In general we think this is possible, but it depends whether we are thinking of UE-visible cell identifiers or CN visible cell identifiers. We might have an arrangement with moving UE visible cell identifiers (so decoupled at Uu) that is not apparent to the CN (coupled from CN perspective). In this case the CN visible cell ID (for example) points to a fixed area, either through configuration or through information encoded in the ID itself. This would require mapping work on the part of the satellite network. |
| Eutelsat | No. However, the more precise mapping is to geographic area at any given time may need to be defined for network management purposes, for example for emergency call localisation and call tracing. |

Summary:

- For Tracking Area, all 10 companies commented not to decouple Tracking Area from geographical area.

- This also applies to GEO, and LEO with Earth Fixed Cell.

Proposal 2: Tracking Area is coupled with geographical area.

## New Flag indicating TA for NTN fixed and/or moving cell

Contribution ([5]) proposed “It may be beneficial to differentiate a TA which contains NTN cells (fixed and/or moving) from a TA which does not.” Contribution ([5]) proposed to introduce a new TA Type IE for the TAC over NG interface and Xn interface.

Contribution ([5])  also proposed to use “Option/Reject” for all necessary NTN-related IEs in RAN3 protocols.

Q3-1: Please share your view on following aspects:

- whether need to differentiate a TA contains NTN cells (fixed and/or moving) from a TA which does not.

- If yes, whether need the new TA Type IE

| Company | Comment |
| --- | --- |
| Nokia | No. It is unclear why need to differentiate a TA contains NTN cells from a TA which does not. Please note SA2 decision that the CN does not require a new indicating from RAN, in order to determine the UE is accessing via a NTN cell. |
| ZTE | No. Not essential and the benefit is unclear. |
| Samsung | It seems too early to decide to differentiate a TA contains NTN cells (fixed and/or moving) from a TA which does not. It is possibly depend on SA2 decision. |
| Ericsson | [5] is our contribution and we say: yes, of course. However, if the cell and TA indication are able to indicate geographic locations, it seems that justification of such differentiation needs to come from somewhere else. |
| CATT | No. Same opinion as ZTE’s. |
| Thales | The rational for differentiating a TA containing NTN cells from a TA which does not, should be clarified. |
| Huawei | No The benefit to differentiate the TA of a NTN cell moving or not from other cell is not crystal clear based on fact that TA defines an earth fixed area. As stated in paper [5] this can be done per deployment if any benefit is discovered … |
| Fraunhofer IIS/HHI | No. No "special" TA type for NTN is necessary as long as we are still considering earth fixed identifier and the same procedures as in TN apply. |
| Qualcomm | It is unclear what the differentiation would be used for and by which NEs. Within the 5GCN, it may not be needed. However we also agree with the statements (e.g. from Samsung) that it may be too early to make a final decision, plus other groups may find a reason. |
| Eutelsat | No. Unless a benefit can be demonstrated. |

Summary:

- 8 companies commented no need to differentiate a TA contains NTN cells (fixed and/or moving) from a TA which does not, or questioned the benefit.

- 1 company commented to wait for SA2 decision.

- 1 company commented to differentiate a TA contains NTN cells (fixed and/or moving) from a TA which does not.

- Based on majority view, it is proposed to agree a WA “No need to differentiate a TA that contains NTN cells (fixed and/or moving) from a TA which does not”. This still allows the change in the future, e.g. if benefit is confirmed, or SA2 make a decision.

Proposal 3-1: WA “no need to differentiate a TA that contains NTN cells (fixed and/or moving) from a TA which does not.”

Q3-2: Please share your view on following aspects:

- use “Optional/reject” for all NTN-related IEs in RAN3 protocols.

| Company | Comment |
| --- | --- |
| Nokia | Yes. This may be taken as a principle, and use it in case a new NTN IE is introduced in future. |
| ZTE | For Rel-17, we do not see any new NTN-related IE, but the principle is ok! |
| Samsung | It is fine to take this principle for Rel-17. |
| Ericsson | we did this for other features as well, but let’s first discuss about stage-2-ish principles and then look into stage 3 details. |
| CATT | Yes. |
| Thales | NTN-related IEs may have to be defined. However, this should be further clarified once RAN1 and RAN2 decide how NR Uu parameterization will be performed. |
| Huawei | No. This statement is too early. We prefer handle IE case by case as usual e.g. if an NTN-related IE is introduced into handover procedure to notify an “emergency HO” to ground cell due to NTN-moving cell? The Handover will failed on legacy node… |
| Fraunhofer IIS/HHI | No. To early to state such decision. |
| Qualcomm | Would probably make sense, but no need to worry about this yet… |
| Eutelsat | Too early to decide. |

Summary:

- 4 yes, 2 no, 2 for later discussion.

- This can be discussed later when a stage-3 NTN IE is introduced.

## Support UE without GNSS capability

Contribution ([2][9]) propose to also consider support UE with/without GNSS capability. Considering the objective of the WID, the UE without GNSS may be treated with 2nd priority.

Q4: Please share your view on the scenario that the UE does not have GNSS capability.

| Company | Comment |
| --- | --- |
| Nokia | Ok to consider the UE does not have GNSS capability. |
| ZTE | Support. |
| Samsung | Supporting UE without GNSS capability is treated with 2nd priority |
| Ericsson | The question is: what does “using GNSS” actually mean? We would rather propose the opposite: Clarify use of GNSS first and take GNSS-less approach as “base-line”. |
| CATT | Suggest to follow the WID, that “UEs with GNSS capabilities are assumed”, at least in Rel-17. If a UE doesn’t have GNSS capability, doesn’t know where it is and thus doesn’t know where the satellites are over its sky, it seems challenging to catch the DL signalling and to get sync with the gNB. |
| Thales | UE with GNSS is assumed for the WID. However UE without GNSS may be considered with 2nd priority. |
| Huawei | We would prefer to consider the UE without GNSS capability as best effort and indeed 2nd priority |
| Fraunhofer IIS/HHI | UEs with GNSS capability are assumed for the WI. However, we think it should at least also be possible to connect UEs w/o GNSS capability to the network. |
| Qualcomm | We think that it will be a great deal easier to assume GNSS support and more realistic since higher end UEs which support NTN are very likely to have GNSS capability. So we agree with Thales and Huawei. But we don’t object to 3GPP making extra work by adding more complex support for the unlikely case of an NTN capable UE without GNSS. Note that other groups may have a view here. |
| Eutelsat | GNSS capability should be assumed in this release. |

Summary:

- 3 companies commented to support the UE does not have GNSS capability.

- 5 companies commented it is 2nd priority to support the UE without GNSS capability.

- 1 company commented to consider GNSS-less approach as baseline.

- It is proposed that it is the 2nd priority to support the UE without GNSS capability.

Proposal 4: It is 2nd priority to support the UE without GNSS capability.

## Any other issues

Description:

Question 5: Any need to identify LEO satellite and NTN-GW for management purposes?

| Company | Comment |
| --- | --- |
| ZTE | No. LEO Satellite and NTN-GW is out of RNL scope, and their management is TNL issue. (maybe out of 3GPP) |
| Samsung | Not in RAN3 scope. |
| CATT | Share the view with ZTE and SS. |
| Huawei | Same view as Samsung |
| Eutelsat | Possibly for management purposes, but also as stated in response to Q1, the identification of NTN-GW (or the set of associated parameter settings) may need to be made during Feeder link switchover, independently of gNB, see TR38.821section 8.7 This needs further study. Note that the NTN-GW is likely to have international connections to several gNBs |
| Nokia | Not sure what the management purpose is. Normally, there is a management center for satellite and NTN-GW, for example, the NTN Control Center in Thales R3-205173. If needed, this can be discussed in the future meeting. |

Summary:

- 5 companies proposed to not identify LEO satellite and NTN GW in RAN3 scope.

- 1 company proposed it may be further discussed.

Proposal 5: There is no need to identify LEO satellite and NTN GW.

Description:

Question 6: Should both earth moving cell ID and earth fixed cell ID be supported?

| Company | Comment |
| --- | --- |
| Samsung | Yes, both should be supported. Earth moving cell ID and earth fixed cell ID are applicable for different scenarios. |
| Ericsson | What is a “Moving Cell ID” as opposed to a “fixed cell ID”? |
| CATT | Further clarification on the terminologies seems needed. We understand that both of “Earth moving cell” and “Earth fixed cell” should be supported. The “Moving Cell ID” is a cell ID of a “Earth moving cell”? |
| Thales | In terms of Cell ID we may have to differentiate, in the case of Earth moving cells, the Cell Id seen by CN (Global Cell Id) from the Cell Id seen by the UE (Radio cell Id). |
| Huawei | I have other question for the fun is it possible that an “Earth moving cell” does not have a “Moving Cell ID” in the network?  If it helps the TA are fixed on earth but not the Cell ID |
| Samsung2 | Seems people have confusion on the moving cell Id and fixed cell Id. We discussed this issue in [8]. Here is some clarification. Two types for network identifies defined in TR - Stationary Identifiers on ground - Moving Identifiers on ground Fixed cell Id is refer to stationary Identify on ground. We don’t have detail discussion whether cell ID is earth fixed, or earth moving. Bu in SA2 conclusion on key issue 10, seems fixed cell ID is assumed. I copied it here for your information. “In the case of Earth-fixed cells (as core network sees them), whether and how the RAN node connected to 5GCN (e.g. CU or gNB) can map the radio component of the cell that can potentially have moving coverage (e.g. in the case of NGSO satellite) to earth-fixed cells represented by CGI would be up to RAN WGs to decide.” To Huawei’s question, it is possible earth moving cell have a fixed cell ID, but of course, it is not beneficial. For the earth fixed cell, it is beneficial to use fixed cell ID. E.g. no group mobility when feeder link switch. We haven’t discussed this issue in SI stage and just trigger the discussion. |
| Fraunhofer IIS/HHI | In case of transparent satellites, the gNB on ground is responsible for the cell id. As long as the satellite is within reach of the GW / gNB on ground, the cell id will be provided by the gNB and forwarded by the satellite to an area on ground. Here we can distinguish 2 cases: - Scenario C1: LEO with steerable beams: In this case the beam of the satellite is pointing to a fixed area on ground, hence the cell id can be considered as earth fixed - Scenario C2: LEO with beams move with the satellite: In this case the beam is moving with the satellite over the earth, hence the cell id is moving with the satellite over the earth and can be considered as earth moving. Note: The movement of the cell id is limited at least to the region on ground where the satellite is within reach of the GW / gNB on ground. Note that this cell id is not moving across the entire earth. Both options shall be supported, but the question is whether we need to distinguish both cases from a network point of view. We don’t think we need two different types of cell IDs. |
| Nokia | I do not understand earth moving cell ID and earth fixed cell ID. The Satellite just act as a repeater. Since this was not discussed before, this may be discussed in future meeting if needed. |
| Qualcomm | We need to fix the terminology. We made the same point as Thales above in another question (CN-visible vs UE-visible). Purely from radio perspective, we think both can be supported. |
| Eutelsat | Both moving cell and fixed should be defined. Id? |

Summary:

The question is unclear. This may be discussed in the future meeting.

Description:

Question 7: WA on fact that the NG-RAN Network identifier are re-use without any restriction?

| Company | Comment |
| --- | --- |
| Huawei | We would like to consider the following proposal for agreement [3] The NG-RAN Network identifier are re-use without any restriction. This complete the first question the current Network Identifier should be re-use as it is without any range split or partition etc … e.g. no dedicate Cell Id for NTN-cell, no PCI range for the NTN-cell identification etc … See some proposal in other CB, we propose a WA and if company would like to break it strong argument should be provided …. We can go for agreement. |
| Nokia | Yes. |

Proposal 7: current NG-RAN architecture is reused for NTN.

# Conclusion, Recommendations [if needed]

If needed

# References

R3-204662, Initial Thoughts on NTN Relevant ID Handling (ZTE)

R3-204687, NR-NTN: Network Identities Handling (Fraunhofer IIS, Fraunhofer HHI)

R3-204961, Discussion on Network IDs handling for NTN (Huawei)

R3-205118, Discussion on identifier handling in NTN (CATT)

R3-205155, Tracking Area Handling with NTN (Ericsson LM)

R3-205157, Tracking Area Handling with NTN - XnAP (Ericsson LM)

R3-205266, Discussion on network identities handling (Nokia, Nokia Shanghai Bell)

R3-205393, Network Identifier Handling (Samsung)

R3-205438, Discussion on key issues in NTN (CMCC)