3GPP TSG-RAN WG3 #109	R3-205645

# Online, 17th - 28th August 2020

Agenda Item:	19.4

Source: 	Ericsson

Title:  	TP to F1AP: correction of TRP geo-coordinates

Document for:	Others

# Discussion

This TP is a mirror of the related NRPPa TP to align the F1AP BL CR with the points raised in [1].

Proposal 1: Agree to the TP below to correct the TRP geographical coordinates

# References

[1]  R3-2052644, TP to NRPPa: correction of TRP geo-coordinates, Ericsson

# TP to F1AP: correction of TRP Geo-coordinates

START OF CHANGES

#### 9.3.1.k		Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  |
| >Direct |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  |
| >>>TRP Position | O |  | NG-RAN Access Point Position 9.3.1.b | The configured estimated geographical position of the antenna of the cell/TRP. |
| >>>TRP High Accuracy Access Position | O |  | NG-RAN High Accuracy Access Point Position 9.3.1.l | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Referenced |  |  |  |  |
| >>Reference Point | M |  | 9.3.1.k5 | The reference point is used to derive the TRP position |
| >>CHOICE Type | M |  |  |  |
| >>>TRP Position Relative Geodetic | O |  | Relative Geodetic Location 9.3.1.k2 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP |
| >>>TRP Position Relative Cartesian | O |  | Relative Cartesian Location 9.3.1.k3 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |
| DL-PRS Resource Coordinates | O |  | 9.3.1.k1 | DL-PRS Resource Coordinates relative to the TRP coordinate |

#### 9.3.1.k1	DL-PRS Resource Coordinates

This information element contains the geographical coordinates of the antenna reference points (ARP) for the DL-PRS Resources of a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL-PRS Resource Set ARP List | M | 1..<maxPRS-ResourceSets> |  |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >CHOICE DL-PRS Resource Set ARP Location | M |  |  | Relative to the geographical coordinates for the TRP. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource Set ID. |
| >>Relative Geodetic Location | O |  | Relative Geodetic Location 9.3.1.k2 |  |
| >>Relative Cartesian Location | O |  | Relative Cartesian Location 9.3.1.k3 |  |
| >DL-PRS Resource ARP List | M | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |
| >>CHOICE DL-PRS Resource ARP Location | M |  |  | Relative to the DL-PRS Resource Set ARP Location. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource ID. |
| >>>Relative Geodetic Location | O |  | Relative Geodetic Location 9.3.1.k2 |  |
| >>>Relative Cartesian Location | O |  | Relative Cartesian Location 9.3.1.k3 |  |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

#### 9.3.1.k2	Relative Geodetic Location

This information element provides a location relative to some known reference location in a relative geodetic coordinate system.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Milli-Arc-Second Units | M |  | ENUMERATED (0.03, 0.3, 3, ...) | Units and scale factor for the delta-latitude and delta-longitude fields. 0.03, 0.3, 3, milliarcseconds. TS 37.355 [y]. |
| Height Units | M |  | ENUMERATED (mm, cm, m, ...) | Units and scale factor for the delta-height field. 10-3 metre, 10-2 metre, TS 37.355 [y]. |
| Delta Latitude | M |  | INTEGER (-1024..1023) | Delta value in latitude in the unit provided in Milli-Arc-Second Units. TS 37.355 [y]. |
| Delta Longitude | M |  | INTEGER (-1024..1023) | Delta value in longitude in the unit provided in Milli-Arc-Second Units. TS 37.355 [y]. |
| Delta Height | M |  | INTEGER (-1024..1023) | Delta value in ellipsoidal height in the unit provided in Height Units. TS 37.355 [y]. |
| Location uncertainty | M |  | 9.3.1.k5 |  |

#### 9.3.1.k3 Relative Cartesian Location

This information element provides a location relative to some known reference location in a relative Cartesian coordinate system

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| XYZ unit | M |  | ENUMERATED (mm, cm, dm,..) |  |
| X value | M |  | INTEGER (-216.. 216-1) | Positive value represents northing from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-216.. 216-1) | Positive value represents easting from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-26.. 26-1) | Positive value represents height above reference point in units of XYZ Unit IE. |
| Location uncertainty | M |  | 9.3.1.k5 |  |

#### 9.3.1.k4	Reference Point Location

This information element provides a reference point location information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ReferencePoint | M |  |  | Reference point to which relative location information is related to |
| >Relative Coordinate ID |  |  |  |  |
| >>Relative Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped to WGS84 coordinates via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>NG-RAN Access Point Position |  |  | 9.3.1.b |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>NG-RAN High Accuracy Access Point Position |  |  | 9.3.1.l |  |

#### 9.3.1.k5	Location Uncertainty

This information element provides the location uncertainty information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| >Horizontal Uncertainty | M |  | INTEGER (0..255) | Horizontal uncertainty of the ARP latitude/longitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| >Horizontal Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |
| >Vertical Uncertainty | M |  | INTEGER (0..255) | Vertical uncertainty of the ARP altitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| >Vertical Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |

NEXT CHANGE

ASN.1 to be added during the meeting.

END OF CHANGES