3GPP TSG-RAN3 Meeting #109-E                                             	               R3-205658

E-meeting, 17 – 28 August 2020

Agenda Item:	10.2.1.2

Source:	Ericsson

Title:	SoD for SONMDT_EnergyEff

Document for:	Discussion, Decision

# Introduction

This is a summary of offline discussions for the topic of Energy Efficiency in AI 10.2.1.2.

        - 10.2.1.2. Energy Efficiency

OAM requirements

Including the postponed LSin from SA5 (R3-204288) received at RAN3 #108-e

The scope of the SoD is as follows:

CB: # 1002_SONMDT_EnergyEff

-  Topics to discuss:

- RLC level KPIs

- UL/DL PDCP SDU data volume measurement

- KPIs at specific network nodes

- Any other topic based on contributions submitted

- Reply LS to SA5

- If there are agreements, can proceed to draft reply LS

(E/// - moderator)

Summary of offline disc R3-205509

## Summary of First Round of Discussions

A proposal to rename the Energy Efficiency parameter into Energy Performance and to make this parameter unit-less could not be agreed.

On the ata volume metric to be used for EE measurements:

- 1 Company proposes to calculate DV at NR-U (PDCP) level

- 5 companies propose to calculate DV at RLC SDU level

- 1 company proposes to leave the choice to SA5

Given that it is RAN3 the group tasked to find a solution for EE measurements and given the majority view, the following is proposed:

- It is proposed to agree that Energy Efficiency measurements are calculated based on RLC SDU Data Volume measurements

- It is proposed to send an LS to SA5, checking feasibility of specification for RLC SDU Data Volume measurements and requesting specification of RLC SDU data volume measurements at the gNB-DU

There is no consensus on measurements of EE for the following nodes:

- gNB-CU-CP

- gNB-CU-UP

- gNB in split architecture

# For the Chairman’s Notes

Following agreements were proposed on the first round of offline discussion:

- It is proposed to agree that Energy Efficiency measurements are calculated based on RLC SDU Data Volume measurements

- It is proposed to send an LS to SA5, checking feasibility of specification for RLC SDU Data Volume measurements and requesting specification of RLC SDU data volume measurements at the gNB-DU

There is no consensus on measurements of EE for the following nodes:

- gNB-CU-CP

- gNB-CU-UP

- gNB in split architecture

# Discussion

## Metric Name

It was highlighted in R3-205538 that it seems erroneous to speak about Energy Efficiency when the metric under analysis in this discussion is [Data Volumes] / [Energy Consumption].

An “efficiency” metric is usually represented in a unit-less way, e.g. as a percentage.

Furthermore, it appears that ETSI EE and ITU-T are working on other metrics named “Energy Efficiency”, which are unit-less. Using the name Energy Efficiency for [Data Volumes] / [Energy Consumption] can only create confusion.

For this reason, it is proposed to name the metric under discussion ([Data Volumes] / [Energy Consumption]) as “Energy Performance”.

Proposal 1:  It is proposed to name the metric ([Data Volumes] / [Energy Consumption]) as “Energy Performance”

Companies are invited to provide their feedback on this proposal

| Company | Comments |
| --- | --- |
| Huawei | Sorry, but where is R3-205538? Where is proposal 1 coming from? In general, I think the main stuff of this email discussion should be how to reply the LS from SA5 on energy efficiency. Therefore, I propose to focus on the questions that SA5 asked in their LS in this email discussion. |
| ZTE | Renaming the energy efficiency as the energy performance is not necessary. |
| Nokia | R3-205538 seems to be a late revision of R3-205005, introducing this new proposal. Energy efficiency as specified in ETSI ES203228 is NOT unit-less, so we prefer to keep this term. |
| Qualcomm | As Nokia pointed out, Energy Efficiency in ETSI ES203228 has units of bits/J, so we probably don’t need to rename the metric. |
| Samsung | Energy Efficiency is expressed in bit/J in ETSI ES 203228, so it’s unnecessary to rename the metric. |
| CATT | Could not see strong argument to rename it. |

## Metric Definition

In the calculation of the ([Data Volumes] / [Energy Consumption]) metric, companies have expressed different views on how to calculate data volumes.

Two possible approaches are available:

Option 1 (R3-205538): Calculate UL and DL data volumes as volumes of data measured at RLC. This allows data volume to be calculated on a per gNB-DU basis

Option 2 (R3-204737): Calculate data volume as the UL and DL PDCP SDU data volume measurement defined in TS 28.552 [3], clause 5.1.3.6.2.1 and 5.1.3.6.2.2, provided that these measurements can be taken on a per F1-U interface basis (To be confirmed by SA5). This allows data volume to be calculated on a per gNB-DU basis

Companies are invited to provide comments on the above statement also taking into consideration the data duplication scenario in Section 3.3.

| Company | Comments |
| --- | --- |
| Huawei | We prefer to reuse the existing DV per interface defined in TS 28.552 as much as possible. And we think that it’s SA5 responsibility to make the decision on whether a RLC level DV is needed or not. And if needed, they can define it by them own without any impact on RAN3. |
| ZTE | We prefer option1 for per DU EE calculation, we can provide our understanding to SA5, then SA5 can take it into account for EE definition. |
| Nokia | We believe there is common understanding that EE should be assessed at network/cluster level based on energy consumption and performance collected at site level (gNB-DU) in order to be used for network energy efficiency assessment. Use of PDCP SDU (PDU?) at F1-U for this purpose is so far not confirmed and will therefore require further checking. Option 1 (RLC) seems OK. |
| Qualcomm | To calculate EE per DU, we need RLC SDU (or PDCP PDU) data volume measurements. Therefore, PDCP SDU DV measurements defined in “5.1.3.6.2 PDCP SDU data volume Measurement” should not be used for computing EE per DU. Based on RAN3 inputs, SA5 can either use: i) 5.1.3.6.1 PDCP PDU data volume Measurement, or ii) define a new metric |
| Samsung | It’s SA5 responsibility to make the decision on whether a RLC level DV is needed or not |
| CATT | It is OK to adopt option 1 |
| Ericsson | Option 1 (RLC SDU volume calculation) is clearly a feasible option for EE calculation. We would prefer Option 1 |

## Data Duplication

In R3-205307 the case of PDCP duplication is described. In this case the PDCP layer duplicates PDUs and sends them to two RLC layers.

It should be noted that PDCP SDUs entering RLC are numbered and will be signalled to the UE even if the RLC layer receives a command to flush the RLC buffer.

However, those PDCP SDUs that have not entered RLC at the time a command to flush the RLF buffer  or to discard some PDCP SDUs is received, will be removed by the gNB-DU.

This creates a case where a PDCP data volume metric could be affected by errors because while the PDCU PDU/SDU is signalled to the gNB-DU, the gNB-DU may not transmit it over the air and instead discard it.

Companies are invited to provide their comments to the case above and whether it affects the selection of the data volume metric in section 3.2.

| Company | Comments |
| --- | --- |
| Huawei | We don't see any impact on the data volume metric. |
| ZTE | A new EE KPI using “RLC Data Volumes”is needed for more accurate calculations. |
| Nokia | It seems that both options will introduce some error or bias, mainly due to the measurement being taken at the gNB-DU. The error seems negligible in most scenarios, and should be acceptable. |
| Qualcomm | Agree with Nokia. The error seems negligible. |
| Samsung | Agree with Nokia and QC. |
| CATT | Agree with Nokia |
| Ericsson | We believe that a calculation of data volumes at F1-U level might be subject to higher errors than a measure of RLC SDU data volumes, due to the possibility of PDCP PDUs not being transmitted over the air (e.g. due to buffer flushing) |

## Granularity of Data Volume / Energy Consumption

R3-205307 proposes to consider different granularities of the Data Volume / Energy Consumption metric. In the above sections the focus is on measurement of Data Volume / Energy Consumption per gNB-DU.

It is also clear that in a non split RAN architecture, it is already possible to achieve a measure of the Data Volume / Energy Consumption per gNB.

The remaining nodes for which Data Volume / Energy Consumption could be measured are

- gNB-CU-CP

- gNB-CU-UP

- gNB in split architecture

Companies are invited to provide their comments on whether such increase in granularity is required and is feasible.

| Company | Comments |
| --- | --- |
| Huawei | Again. We checked the granularity issue with our SA5 delegates, there is no any issue/problem found. Sa5 should be the responsibility group for EE. Therefore, I don't think the granularities of EE should be discussed here. In split architecture, whether per DU, per CU-CP, or per CU-UP EE is needed should be discussed and decided in SA5. What RAN3 could do is to provide feedback according to the questions in the LS from SA5. For example, we may notify SA5 in the reply LS that per gNB EE may be less accurate in split architecture, due to deployment reason. And let SA5 to make the final decision. |
| ZTE | We can ask SA5 if this is feasible to apply the defined EE for more granularity for a specific network element ,e.g, per gNB, per CU, per DU, and per UP. The different RLC volume and UP (PDCP) volume metric can be used for different network element. |
| Nokia | We believe that EE should be assessed at network/cluster level based on energy consumption and performance collected at site level (gNB-DU), however this aspect seems to be under the responsibility of SA5. |
| Samsung | It’s SA5 responsibility to make the decision |
| CATT | It’s SA5 responsibility to make the decision |
| Ericsson | As part of the RAN3 term of reference, RAN3 is responsible to provide requirements for OAM. It is therefore RAN3’s responsibility to guide SA5 on the granularity of the EE measurements. As mentioned by Nokia, the EE measurements defined by ETSI (i.e. the measurements that kickstarted discussions in RAN3) consist of [Data Volume]/[Energy Consumption per site]. Nodes like gNB-CU-UP and gNB-CU-CP are not located at a dedicated site unless a non split architecture is envisaged. Hence EE calculation for those nodes is not within the scope of the EE measurements. |

# Conclusion, Recommendations [if needed]

If needed