3GPP TSG-RAN3 Meeting #109-e	R3-205729

E-meeting, 17 - 28 August 2020

Title: 	Potential solutions to network slice service continuity

Source: 	Huawei, Nokia, Nokia Shanghai Bell, NEC, ZTE, Samsung, CMCC, CATT

Agenda item:	17.2

Document Type:	Discussion and Decision

Annex 1– TP for TR 38.832

6	Study necessity and mechanisms to support service continuity

6.1	Scenario and issue description

Editor Note: capture the description of scenario and issue.

6.2	Solutions

Editor Note: Capture the solutions for the scenario and issue.

6.2.Y Slice Re-mapping Message Sequence Charts

Editor’s note: Feasibility of this solution at system level requires further work including checking with SA2.

6.2.Y.1 Slice Remapping decision in target gNB at Xn based handover

Fig. 1: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB makes the slice re-mapping/fallback decision. The T-gNB may send the slice re-mapping/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the S-gNB.

- The T-gNB shall send the slice re-mapping/fallback decision to the AMF through the PATH SWITCH REQUEST message.

- The AMF responds the PATH SWITCH REQUEST ACKNOWLEDGE message. The AMF may reject the PDU sessions in the PDU Session Resource Released List IE.

6.2.Y.2 Slice Remapping decision in target gNB at NG based handover

Fig. 2: Slice re-mapping/fallback determined by the T-gNB

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- The AMF sends the HANDOVER REQUEST message to the T-gNB.

- If the UE’s ongoing slice(s) is rejected in the target gNB, based on the slice re-mapping policy described in section 6.2.X, the T-gNB shall include the re-mapped/fallback decision in the HANDOVER REQUEST ACKNOWLEDGE message to the AMF.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.

6.2.Y.3 Slice Remapping decision in 5GC at NG based handover

Fig. 3: Slice re-mapping/fallback determined by the AMF

- The S-gNB sends the HANDOVER REQUIRED message to the AMF.

- If the UE’s ongoing slice(s) is not supported by the T-gNB, the AMF may make the slice re-mapping/fallback decision and include the decision in the HANDOVER REQUEST message to the T-gNB.

- The T-gNB responds to the AMF through the HANDOVER REQUEST ACKNOWLEDGE message.

- The AMF may send the slice re-mapping/fallback decision to the S-gNB through the HANDOVER COMMAND message.