3GPP TSG-RAN WG3 Meeting #110-e	R3-205956

E-meeting, 2-12 Nov 2020

Agenda item:	10.2.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.463, NR_ENDC_SON_MDT_enh-Core) Adding slice to the load information

Document for:	Endorsement

# 1	Introduction

This document proposes text for a BL CR to E1AP. The proposed signaling allows to provide per-slice status indication, as described in [R3-205952].

# 2	Text proposal

Please note: ASN.1 to be added once the idea is agreed.

*** First change, omitted text not changed ***

#### 8.2.8.2	Successful Operation

Figure 8.3.7.2-1: DL Data Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the GNB-CU-UP STATUS INDICATION message to the gNB-CU-CP.

If the gNB-CU-UP Overload Information IE in the GNB-CU-UP STATUS INDICATION message indicates that the gNB-CU-UP is overloaded, the gNB-CU-CP shall apply overload reduction actions until informed, with a new GNB-CU-UP STATUS INDICATION message, that the overload situation has ceased.

If the Affected Slice IE is included in the GNB-CU-UP STATUS INDICATION message, the overload concerns resources dedicated to the indicated slice only.

The detailed overload reduction policy is up to gNB-CU-CP implementation.

*** Next change, omitted text not changed ***

#### 9.2.1.18	GNB-CU-UP STATUS INDICATION

This message is sent by the gNB-CU-UP to indicate to the gNB-CU-CP its status of overload.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |
| Affected Slice | O |  | S-NSSAI 9.3.1.9 |  | YES | ignore |

*** Next change, omitted text not changed ***

[ASN.1 to be added]

*** End of changes, remaining text not changed ***