3GPP TSG-RAN WG3 Meeting #110-e	R3-205958

E-meeting, 2-12 Nov 2020

Agenda item:	10.2.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.473, NR_ENDC_SON_MDT_enh-Core) RRC connection reject template

Document for:	Endorsement

# 1	Introduction

This document proposes text for a BL CR to F1AP. The proposed signaling allows the CU to provide the DU with a template of the RRCReject message, as described in [R3-205957].

# 2	Text proposal

First change, ommited text not changed

#### 8.2.9.2	Successful operation

gNB

-

DU

NETWORK ACCESS RATE REDUCTION

gNB

-

CU

Figure 8.2.9.2-1: Network Access Rate Reduction, Successful operation

The gNB-CU initiates the procedure by sending a NETWORK ACCESS RATE REDUCTION message to the gNB-DU. When receiving the NETWORK ACCESS RATE REDUCTION message the gNB-DU should take into account the information contained in the UAC assistance information to set the parameters for Unified Access Barring.

If the NID IE is contained in the NETWORK ACCESS RATE REDUCTION message, the gNB-DU should take it into account and combine the NID IE with the PLMN Identity IE to identify the SNPN.

Interaction with other procedures

If the RRC-Reject-Container IE is included in the NETWORK ACCESS RATE REDUCTION message, the DU shall, if supported, use the included RRC message to reject future access requests from the UE. In this case, the DU shall not initiate the Initial UL RRC Message Transfer procedure towards the gNB-CU.

Next change, ommited text not changed

#### 9.2.1.19	NETWORK ACCESS RATE REDUCTION

This message is sent by the gNB-CU to indicate to the gNB-DU a need to reduce the rate at which UEs access the network.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UAC Assistance Information | M |  | 9.3.1.83 |  | YES | reject |
| RRC-Reject-Container | O |  | RRC Container 9.3.1.6 | Includes the RRCReject IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |

Next change, ommited text not changed

-- **************************************************************

--

##### -- Network Access Rate Reduction

--

-- **************************************************************

NetworkAccessRateReduction ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NetworkAccessRateReductionIEs }},

...

}

NetworkAccessRateReductionIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID 					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-UAC-Assistance-Info				CRITICALITY reject	TYPE UAC-Assistance-Info			PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY ignore	TYPE RRCContainer					PRESENCE optional		},

...

}

Remaining text not changed