3GPP TSG-RAN WG3 Meeting #110-e                                             　    　　　　　　 	R3-205961

Online, November, 2020

Source: 	KDDI Corporation

Title:	Considerations on IAB-donor indicator

Tdoc Type:	Discussion

Agenda Item:	13.2.2. Reduction of Service Interruption

Document for:	Discussion and decision

Release:	Rel-17

Introduction

This paper discusses “Indication of donor-association” which was discussed in last RAN3 meeting CB: # 7_IAB_ServiceInterruptionReduction. The offline discussion is summarized in [1].

Discussion

With regard to “Indication of donor-association”, offline discussion CB#7 concluded the possible way forward as below [1]. But there is no formal agreement with this related proposal8 in the last RAN3 meeting.

Summary: - All companies think that cell selection during RLF recovery can be improved if the candidate cells broadcast a donor indicator. - All companies think that this should be handled by RAN2. Proposal 8: LS to be sent to RAN2 to consider broadcast of an IAB-donor indicator by IAB-DU for the support of cell selection during RLF recovery.

Above proposal8 only captures RLF recovery case, but also for the handover case where the IAB-DU is indicted to migrate other donor-CU, the IAB-DU needs an IAB-donor indication, so that the IAB-DU can understand whether it needs a new F1-C establishment or just maintain the existing F1-C association. So, we propose the following.

Proposal 1:

RAN3 agree a LS to ask RAN2 to introduce an IAB-donor indicator in RRC messages for RLF recovery case and Handover case which possibly leads to inter donor-CU change.

We also propose the draft LS contents as below.

1. Overall Description: RAN3 reaches consensus on the need of an IAB-donor indicator which is used by the IAB node to know whether it leads intra-Donor migration, or inter-Donor migration. RAN3 also acknowledges that this IAB-donor indicator should be provided in RRC messages for the cases, both RLF recovery case and Handover case. As to the priority of the indicated candidate donors, some companies propose to prioritize candidates that belong to the same donor over those that belong to a different donor, but no consensus in RAN3. Those aspects can be discussed in RAN2, if necessary. 2. Actions: To RAN WG2 ACTION: RAN3 kindly would like to ask RAN2 to consider the introduction of an IAB-donor indicator which is used by the IAB node to know whether it leads intra-Donor migration, or inter-Donor migration.

Proposal 2:

RAN3 develop a LS based on the above draft text.

Conclusion

In this contribution, we make the following proposals.

Proposal 1:

RAN3 agree a LS to ask RAN2 to introduce an IAB-donor indicator in RRC messages for RLF recovery and Handover which possibly leads to inter donor-CU change.

Proposal 2:

RAN3 develop a LS based on the above draft text.

References

[1] R3-205467 Summary of Offline Discussion on IAB service interruption	Qualcomm (moderator)