3GPP TSG-RAN WG3 #110-e																		   R3-206054

2-12 November 2020

Online

Title: 	Futher discussion on NR SCG release for UE power saving

Source: 	ZTE, NEC, China Telecom

Agenda item:	9.3.7.1

Document for:	Discussion and Decision

1. Introduction

In the last RAN3 #109-e meeting, the issue of NR SCG release for UE power saving raised from RAN2 (R3-202712, [1]) has been partly resolved, the related CRs were agreed as R3-2047300[2] and R3-204731[3] and reply LS was also agreed as R3-205764[4].

However, after the online and offline discussion, an issue on whether the MN can decide to reject SN initiated SN release request is not well handled.

This contribution continues working on a solution that takes into account the MN load condition before releasing the SN resources for UE power saving.

2. Discussion

Based on the latest progress, when the UE decides to save power, it can send UEAssistanceInformation (i.e., SCG specific UAI for power saving) to SN, then the SN is allowed to initiate SN release procedure, sending a new cause “UE Power Saving” included in the SN release required message, latter, the MN shall accept this required.

2.1 Remaining issue on NR SCG release for UE power saving

Currently, in both (NG) EN-DC and NR-DC, when SN decides to initiate SN release procedure, the MN is not allowed to reject the SN release required message, although the MN strongly prefers to keep SN.

So, it possibly introduces a problem in some cases. For instance, on the one hand, the SN initiates SN release procedure after receiving UE power saving preference information, but on the other hand, the MN has to accept to release SN although it sticks to keep SCG bearer due to load balance.

Observation 1: If the SN initiates SN release procedure because of UE power saving, the MN has to release SN although it wishes to keep SCG bearer due to network load balance.

2.2 Solutions on the remaining issue

In other word, the network operation from the MN to the SN is controversy, from SN’s point of view, it wishes to release SCG bearer due to UE power saving, but on the contrary, from MN’s point of view, it wishes to keep SCG bearer due to load balance.

In the summary of the offline discussion [5], at least two solutions focusing on this problem are on the table.

Solution 1: One possible implementation solution is after the SN release with the cause for UE power saving, the MN triggers SN addition immediately.

It is a reasonable scenario that MN is overloaded and do not want to release SCG for UE power saving.

In NR DC, as the SN is aware of the MN’s load by resource status reporting via Xn, a proper implementation by the SN is to not trigger SCG release for UE power saving.

In EN-DC, the SN is not aware of the MN’s load in current specification, but we do not think it is worth to introduce the resource status reporting mechanism, i.e. not worth to introduce such big spec impact to solve this special issue.

One possible implementation solution is after the SN release with the cause for UE power saving, the MN triggers SN addition immediately.

Solution 2: Modify current SN initiated SN release procedure, i.e., the MN is allowed to reject the SN release request message.

…since MN can acquire the new release cause value, can we modify current SN initiated SN release procedure, i.e., if it is possible that MN is allowed to reject the SN release request message? (Currently, MN cannot reject the SN release request).

Solution 1 has no impact on RAN3 spec, it is, in nature, a network implementation method. However, it seems low efficiency because the MN has to break then set up.

Solution 2 can satisfy RAN2’s requirement (i.e., UE power saving preference), moreover, it can also consider RAN3’s own requirement (i.e., network load balance).

Given that solution 2 is agreed, if the SN initiates SN release procedure because of UE power saving, the MN is allowed to reject the SN initiated SN release procedure, sending existing reject cause (Load Balancing) included in the new message (SN release reject).

Proposal 1: It is proposed that the MN can reject the SN initiated SN release procedure, sending the existing cause “Load Balancing” included in the SN release reject message.

2.3. Design of SN release reject message

In both TS36.423 and TS38.423, the SN release reject message is already included within MN initiated SN release procedure and is sent from SN to MN.

[TS 36.423]

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST REJECT

--

-- **************************************************************

SgNBReleaseRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequestReject-IEs}},

...

}

SgNBReleaseRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

[TS 38.423]

-- **************************************************************

--

#### -- S-NODE RELEASE REJECT

--

-- **************************************************************

SNodeReleaseReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseReject-IEs}},

...

}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

If the proposal 1 is agreed, we can extend the usage of the legacy SN release reject message, i.e., it can be sent from MN to SN used in SN initiated SN release procedure, including a bit ASN.1 impact as below.

[TS 36.423]

sgNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequired

SUCCESSFUL OUTCOME		SgNBReleaseConfirm

UNSUCCESSFUL OUTCOME	SgNBReleaseRequestReject

PROCEDURE CODE			id-sgNBinitiatedSgNBRelease

CRITICALITY				reject

}

[TS 38.423]

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

Observation 2: In both TS36.423 ande TS38.423, if the MN is allowed to reject SN initiated SN release procedure, the existing SN release reject message can be reused, including a minimal ASN.1 impact.

Proposal 1.1: It is proposed to introduce the existing SN release reject message in SN initiated SN release procedure in both TS36.423 and TS38.423.

2.4 Adding the corresponding text procedure for the new mechanism on NR SCG release for power saving

In the last meeting, the following agreement is achieved and has been captured in the agreed CRs [2] [3].

Existing container (the “UE Report”) in the RRC Transfer is reused

A new Cause specific for UE Power Saving is added

However, in both X2AP and XnAP protocols, the related text procedure is still missing, e.g.

[TS 36.423]

The procedure is also to enable transfer of the NR RRC message container with the UE Assistance information from the MeNB to the en-gNB, when received from the UE.

[TS 38.423]:

-	the NR RRC message container with the UE assistance information.

Proposal 2: It is proposed to add the corresponding text procedure for the new mechanism in X2AP and XnAP protocols.

3. Conclusion

After the above analysis, we provide the following observations and proposals

Observation 1: If the SN initiates SN release procedure because of UE power saving, the MN has to release SN although it wishes to keep SCG bearer due to network load balance.

Observation 2: In both TS36.423 ande TS38.423, if the MN is allowed to reject SN initiated SN release procedure, the existing SN release reject message can be reused, including a minimal ASN.1 impact..

Proposal 1: It is proposed that the MN can reject the SN initiated SN release procedure, sending the existing cause “Load Balancing” included in the SN release reject message.

Proposal 1.1: It is proposed to introduce the existing SN release reject message in SN initiated SN release procedure in both TS36.423 and TS38.423.

Proposal 2: It is proposed to add the corresponding text procedure for the new mechanism in X2AP and XnAP protocols.

Proposal 3: It is proposed to agree the corresponding CRs are R3-206055 for X2AP and R3-206056 for XnAP.

4. Reference

[1] R3-202712 LS on NR SCG release for power saving, RAN2

[2] R3-204730 CR36.423 for Introduction of NR SCG Release for Power Saving, CATT, ZTE, Huawei

[3] R3-204731 CR38.423 for Introduction of NR SCG Release for Power Saving, CATT, ZTE, Huawei

[4] R3-205764 Reply LS on NR SCG release for power saving, RAN3

[5] R3-205546 Summary of Offline Discussion on SCG release due to power saving, Nokia, Nokia Shanghai Bell

[6] R3-204880 SN release for power saving, NEC, BT