3GPP TSG-RAN WG3 Meeting #110-e	R3-206108

# E-meeting, 2 – 12 Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0412 | rev | 1 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AS-rekey and Emergency fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghao Bell, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | About AS-Rekey and Emergency Fallback, RAN3 sent LS to SA2 and SA3, and get the following feedback: SA3 S3-201484: It is acceptable to SA3, for the network to give up the AS re-keying procedure and only initiate the emergency fallback procedure when the two procedures collide. SA2 S2-2005909: When the AS re-keying procedure and the Emergency Fallback procedure collides, the AMF gives up the AS re-keying procedure and only initiates the emergency fallback procedure. If AMF includes both the Security Key IE and the Emergency Fallback Indicator IE within one NGAP UE CONTEXT MODIFICATION REQUEST message, NG-RAN takes it as abnormal case. Based on these inputs, it is needed to introduce corresponding Abnormal Conditions in NGAP UE Context Modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Abnormal Conditions in the NGAP UE Context Modification procedure about the case in which both Security Key IE and Emergency Fallback Indicator IE are received in the same message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only clarify the handling of simultaneously AS-rekey and Emergency fallback. This CR has an impact under functional point of view. The impact can be considered isolated because the change only affects the handling of simultaneously AS-rekey and Emergency fallback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lead to IOT issue and complexification of NG-RAN node and AMF implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: update reason for change to include SA2 reply LS inputs. And updated to based on latest version of specification. |  |  |  |  |  |  |  |  |

8.3.4	UE Context Modification

//skip the unchanged part

8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE context modification: unsuccessful operation

In case the UE context update cannot be performed successfully, the NG-RAN node shall respond with the UE CONTEXT MODIFICATION FAILURE message to the AMF with an appropriate cause value in the Cause IE.

If the New AMF UE NGAP ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node may use the received New AMF UE NGAP ID IE or Old AMF UE NGAP ID IE in the UE CONTEXT MODIFICATION FAILURE message.

8.3.4.4	Abnormal Conditions

If the UE CONTEXT MODIFICATION REQUEST message including the New AMF UE NGAP ID IE is received after the NG-RAN node has initiated another class 1 NGAP EP, the NG-RAN node shall be prepared to receive the response message containing an AMF UE NGAP ID with the value received in the New AMF UE NGAP ID IE.

If the NG-RAN node receives both the Emergency Fallback Indicator IE and one of the security IEs (either the Security Key IE or the UE Security Capabilities IE) in the UE Context Modification Request message, the NG-RAN node shall ignore both IEs and send back the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.