3GPP TSG-RAN3 Meeting #110-e	R3-206149

2 – 12 November 2020 E-Meeting

Title: 	(TP on MDT enhancement for 36.423) Support of MDT enhancement for EN-DC

Source: 	CATT

Agenda item:	10.3.2.2

Document Type:	other

# 1. Introduction

This contribution is the TP for TS36.423 v16.3.0 with addition of log MDT support in NGEN-DC and NE-DC.

# 2. Discussion

In Rel-16, only immediate MDT is supported for EN-DC scenario and it is agreed to support logged MDT in Rel-17.

This contribution remove the restriction that only immedaite MDT is supported in this version.

# 3. Annex – TP

////////////////////////////////////////////////////////////////////////Change Start ///////////////////////////////////////////////////////////////////////////

9.2.2	Trace Activation

Defines parameters related to trace activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRAN Trace ID | M |  | OCTET STRING (8) | The E-UTRAN Trace ID IE is composed of the following: Trace Reference defined in TS 32.422 [6] (leftmost 6 octets, with PLMN information coded as in 9.2.4), and Trace Recording Session Reference defined in TS 32.422 [6] (last 2 octets) | – |  |
| Interfaces To Trace | M |  | BIT STRING (8) | Each position in the bitmap represents a eNB interface: first bit =S1-MME, second bit =X2, third bit =Uu, fourth bit = F1-C, fifth bit = E1. Other bits reserved for future use. Value ‘1’ indicates ‘should be traced’. Value ‘0’ indicates ‘should not be traced’. | – |  |
| Trace Depth | M |  | ENUMERATED( minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, ...) | Defined in TS 32.421 [7] | – |  |
| Trace Collection Entity IP Address | M |  | BIT STRING (1..160,…) | For details on the Transport Layer Address, see TS 36.424 [8], TS 36.414 [19] | _ |  |
| MDT Configuration | O |  | 9.2.56 |  | YES | ignore |
| UE Application layer measurement configuration | O |  | 9.2.121 |  | YES | ignore |
| MDT Configuration NR | O |  | OCTET STRING | Defined in TS 38.413 [39]. | YES | ignore |

////////////////////////////////////////////////////////////////////////Change End///////////////////////////////////////////////////////////////////////////