3GPP TSG-RAN WG3 Meeting #110-e	R3-206190

E-meeting, 2 – 12 Nov 2020

Title: 	(TP for SON BLCR for 38.423) Load Balancing Enhancements

Source: 	Huawei

Agenda item:	10.2.1.5

Document Type:	Discussion and Decision

# 1. Introduction

The load balancing was discussed during RAN3 #109-e meeting. In the summary [1], SUL load was discussed but without agreement. In this document we would like to discuss SUL in load reporting, SUL and Slice in Mobility Settings Change.

# 2. Discussion

## 2.1 SUL

As in the current Radio Resource Status IE, there are only UL and DL PRB usage. While Supplementary Uplink Carrier (SUL) is not considered in MLB yet.

As we all know, the transition power of the gNB is always larger than it of the UE. And it will make the coverage of Downlink is larger than Uplink in NR cell which is shown in Figure 1.

Figure 1.The coverage of Uplink and Downlink in normal NR cell

Supplementary Uplink Carrier (SUL) is a new feature introduced to NR cell for wider uplink coverage with lower frequency carrier. For example, Figure 2 shows a larger coverage of Uplink with Sub3G (e.g. 1.8 GHZ) than with C-Band (e.g. 4 GHZ). Obviously, the UE with UL or SUL using different frequency which means different resource.

Figure 2. The coverage of Uplink and Downlink in SUL NR cell

As the loading of UL and SUL is different, the source gNB could know the loading distribution better if SUL loading is added in load reporting. What is more, SUL tends to be used at cell edges where many MLB-triggered HOs may take place. Certainly, we should pay more attention to SUL loading.

Add SUL associated IE to Radio Resource Status and Composite Available Capacity Group Xn and F1.

As described in [2], the target gNB shall only select SUL RACH configuration for UE when RSRP is lower than a threshold. So if the target gNB sends the source gNB the SUL threshold, the source gNB could easily know SUL or UL RACH resource will be configured for this UE by comparing the SUL threshold and measurements reports. In this way, the source gNB could make smarter HO decision based on the SUL/UL loading. The detailed example is as shown in Table 1:

Table 1. The disadvantage of the current PRB reporting mechanism

|  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| UL PRB usage | 90% | 90% | 40% | 40% |
| SUL PRB usage | 40% | 40% | 90% | 90% |
| Current reporting | 65% | 65% | 65% | 65% |
| RSRP | high | low | high | low |
| Current potential decision | Not handover | Not handover | Not handover | Not handover |
| Reasonable decision | Handover(SUL is available) | Handover(SUL is available) | Handover(UL is available) | Not handover |

Related to Table 2, Case 1, 2 and 3, Source gNB may select another cell as Target cell when it knows the PRB usage of the cell is already 65% which is obviously unreasonable. For example, in Case 1, the SUL PRB usage is 40% which is suitable to UE to access.

gNBs should exchange the threshold for only selecting SUL resource via Xn.

## 2.2 Slice

In the current load reporting mechanism, Slice is introduced as a load metrics. In this way, the source gNB could know the resource status of each Slice in the target gNB.

There was previously a study on grouping UEs so that UEs with special requirements would be handled in a special way from mobility perspective. But in the end it was concluded that it was difficult to introduce a standardised grouping in RAN. One reason for this was that it was difficult to specify a useful grouping on RAN level.

Slicing provides a standardised way to group UEs and it could be foreseen that different slices would have different requirements. Therefore, we think it would be beneficial to enable different mobility handling for different slices. This could be achieved by exchanging a delta between the cell specific handover trigger and the handover trigger used for this slice.

There is however a mapping problem related to slicing. One UE may use one or more slices, whereas mobility applies to the UE, meaning that it will impact all slices. One way would be to allow the exchange of delta for different combinations of slices so that the target cell could derive which handover trigger was used for the specific UE. Another way would be to include the slice ID causing the specific mobility behaviour in the HO preparation to the target.

Add Slice in Mobility Settings Change.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

- Add SUL associated IE to Radio Resource Status and Composite Available Capacity Group Xn and F1.

- gNBs should exchange the threshold for only selecting SUL resource via Xn.

- Add Slice in Mobility Settings Change.

# 4. Reference

- R3-205660, Summary of CB: # 1005_SONMDT_LoadBalance, CATT

- R3-204937, Load balancing enhancement, Huawei

# Annex – TP

TS 38.423

<<<<<<<<<<<<<<<<<<<< First Changes Begin >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.22	MOBILITY CHANGE REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate adaptation of mobility parameters.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node1 Mobility Parameters | O |  | Mobility Parameters Information 9.2.2.60 | Configuration change in NG-RAN node1 cell | YES | ignore |
| NG-RAN node2 Proposed Mobility Parameters | M |  | Mobility Parameters Information 9.2.2.60 | Proposed configuration change in NG-RAN node2 cell | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| NG-RAN node1 Slice specific offset list | 0..1 |  |  |  | YES | ignore |
| >Slice specific offset item | 1.. < maxnoofSliceItems> |  |  |  | YES | ignore |
| >>PLMN Identity |  |  | 9.2.2.4 |  | YES | ignore |
| >>S-NSSAI |  |  | 9.2.3.21 |  | YES | ignore |
| >>Mobility parameter offset |  |  | INTEGER(-6..6) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.23	MOBILITY CHANGE ACKNOWLEDGE

This message is sent by NG-RAN node2 to indicate to NG-RAN node1 that Proposed Mobility Parameters proposed by NG-RAN node1 were accepted.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.2 |  | YES | ignore |
| NG-RAN node2 Slice specific offset list | 0..1 |  |  |  | YES | ignore |
| >Slice specific offset item | 1.. < maxnoofSliceItems> |  |  |  | YES | ignore |
| >>PLMN Identity |  |  | 9.2.2.4 |  | YES | ignore |
| >>S-NSSAI |  |  | 9.2.3.21 |  | YES | ignore |
| >>Mobility parameter offset |  |  | INTEGER(-6..6) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.24	MOBILITY CHANGE FAILURE

This message is sent by the NG-RAN node2 to indicate to NG-RAN node1 that Proposed Mobility Parameters proposed by NG-RAN node1 were refused.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Mobility Parameters Modification Range | O |  | 9.2.2.61 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.2 |  | YES | ignore |

#### 9.2.2.50	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink, Uplink and Supplementary Uplink and the usage of PDCCH CCEs for Downlink,Uplink scheduling and Supplementary Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Radio Resource Status Type | M |  |  |  |
| >ng-eNB |  |  |  |  |
| >>DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage |
| >>UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage |
| >>DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage |
| >>UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage |
| >>DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage |
| >>UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage |
| >gNB |  |  |  |  |
| >>SSB Area Radio Resource Status List |  | 1 |  |  |
| >>>SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage |
| >>>>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage |
| >>>>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage |
| >>>>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage |
| >>>>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage |
| >>>>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage |
| >>>>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>>>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage |
| >>>>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage |
| >>>>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage |
| >>>>SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.2.2.51	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink, Uplink and Supplementary Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.2.2.52 | For the Downlink |
| Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.2.2.52 | For the Uplink |
| Composite Available Capacity Supplementary Uplink | M |  | Composite Available Capacity 9.2.2.52 | For the Supplementary Uplink |

<<<<<<<<<<<<<<<<<<<< First Changes End >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Sencond Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

<<<<<<<<<<<<<<<<<<<< Skip unchanged part >>>>>>>>>>>>>>>>>>>>

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-ssb-Area-SUL-GBR-PRB-usage,

id-ssb-Area-SUL-non-GBR-PRB-usage,

id-ssb-Area-SUL-Total-PRB-usage,

id-SUL-scheduling-PDCCH-CCE-usage,

id-compositeAvailableCapacitySupplementaryUplink,

id-NG-RAN-node1-Slice-specific-offset-list,

id-NG-RAN-node2-Slice-specific-offset-list,

<<<<<<<<<<<<<<<<<<<< Skip unchanged part >>>>>>>>>>>>>>>>>>>>

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink		CompositeAvailableCapacity,

compositeAvailableCapacityUplink	 	CompositeAvailableCapacity,

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} }	OPTIONAL,

...

}

CompositeAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-compositeAvailableCapacitySupplementaryUplink						CRITICALITY reject	TYPE CompositeAvailableCapacity							PRESENCE optional}

...

}

<<<<<<<<<<<<<<<<<<<< Skip unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- MOBILITY CHANGE REQUEST

--

-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeRequest-IEs}},

...

}

MobilityChangeRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode1MobilityParameters				CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-NG-RANnode2ProposedMobilityParameters	CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-NG-RAN-node1-Slice-specific-offset-list CRITICALITY ignore	TYPE NG-RAN-node1-Slice-specific-offset-list											PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE ACKNOWLEDGE

--

-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeAcknowledge-IEs}},

...

}

MobilityChangeAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional}|

{ ID id-NG-RAN-node1-Slice-specific-offset-list CRITICALITY ignore	TYPE NG-RAN-node1-Slice-specific-offset-list											PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE FAILURE

--

-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeFailure-IEs}},

...

}

MobilityChangeFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-MobilityParametersModificationRange 	CRITICALITY reject	TYPE MobilityParametersModificationRange		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Skip unchanged part >>>>>>>>>>>>>>>>>>>>

NG-RAN-node1-Slice-specific-offset-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF Slice-specific offset-item

Slice-specific-offset-item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAI						S-NSSAI,

Mobility-parameter-offset	INTEGER (-6..6)

iE-Extensions						ProtocolExtensionContainer { { Slice-specific-offset-item-ExtIEs} }	OPTIONAL,

...

}

Slice-specific-offset-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-node2-Slice-specific-offset-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF Slice-specific offset-item

Slice-specific-offset-item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAI						S-NSSAI,

Mobility-parameter-offset	INTEGER (-6..6)

iE-Extensions						ProtocolExtensionContainer { { Slice-specific-offset-item-ExtIEs} }	OPTIONAL,

...

}

Slice-specific-offset-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Skip unchanged part >>>>>>>>>>>>>>>>>>>>

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

sSBIndex								INTEGER(0..63),

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ssb-Area-SUL-GBR-PRB-usage	CRITICALITY ignore TYPE SUL-GBR-PRB-usage PRESENCE optional}|

{ ID id-ssb-Area-SUL-non-GBR-PRB-usage	CRITICALITY ignore TYPE SUL-non-GBR-PRB-usage PRESENCE optional}|

{ ID id-ssb-Area-SUL-Total-PRB-usage	CRITICALITY ignore TYPE SUL-Total-PRB-usage PRESENCE optional}|

{ ID id-SUL-scheduling-PDCCH-CCE-usage	CRITICALITY ignore TYPE SUL-scheduling-PDCCH-CCE-usage PRESENCE optional}

...

}

}

SUL-GBR-PRB-usage::= INTEGER (0..100)

SUL-non-GBR-PRB-usage::= INTEGER (0..100)

SUL-Total-PRB-usage::= INTEGER (0..100)

SUL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>