3GPP TSG-RAN WG3 Meeting #110-e	R3-206210

Electronic Meeting, Nov 2th – 13th, 2020

Agenda item:		13.3.1

Source:	Intel Corporation

Title:	Congestion Indication to CU-CP

Document for:		Discussion and Decision

# 1		Introduction

In RAN3#106, the following were agreed:

- Use current DDDS for e2e flow control in IAB; necessary enhancement to DDDS are not precluded (DDDS is defined in 38.425) - The existing flow control mechanism via DDDS is reused for IAB, i.e. the DDDS is sent from the access IAB node to the IAB-donor-CU-UP, or IAB-donor-CU (in case of no CP-UP split)

In RAN3#109-e, the following were agreed:

- UP-based and CP-based approaches for DL congestion mitigation in IAB networks are complementary. - In IAB DL end-to-end flow control, the access node sends feedback to the donor-CU-UP. - Discuss the improvements to DDDS for IAB UP-based congestion mitigation (e.g. packet marking, highest PDCP SN received from parent node, receiving data rate, received data volume). - The measures taken by the donor-CU-CP based on the CP-based approach are up to implementation. - End-to-end UL flow control is deprioritized in Rel17.

In this contribution, an enhancement to the existing flow control mechanism is presented.

# 2		Discussion

The agreement from RAN3#106 is to reuse the Downlink Data Delivery Status procedures as defined in TS 38.425 [1]. By using DDDS, the access IAB-Node DU can provide downlink status to the IAB-Donor-CU-UP.

In RAN3#109-e it was further agreed to improve DDDS for UP-based congestion mitigation by adding packet marking, highest PDCP SN received from parent node, receiving data rate, and received data volume.

In RAN3#109-e companies also agreed in addition to UP based congestion mitigation, a CP based method should also be considered as a complementary way for congestion mitigation, where the measures taken by the donor-CU-CP are up to implementation

The following figure illustrated a CP based congestion mitigation method where the congested IAB node sends a congestion indicator to CU-CP via F1-C.

Figure 1: Congested intermediate IAB node indicates congestion

By having the congested intermediate node to send a congestion indication to the donor-CU-CP allows the donor-CU-CP to take the appropriated action in the control plane to alleviate the congestion, this can be, but not limited to reconfiguration of the routing and mapping information.

Observation 1: By having the congested intermediate node to send a congestion indication to the donor-CU-CP allows the donor-CU-CP to take the appropriated action in the control plane to alleviate the congestion, this can be, but not limited to reconfiguration of the routing and mapping information.

Proposal 1: When DL congestion is detected in an IAB-node, a congestion indicator is sent to the donor-CU-CP via F1-C

The triggering mechanism can be based on the BAP flow control mechanism. For example, an IAB node should send a congestion indicator after a configurable number of BAP control feedback is triggered. Furthermore, the congested indicator can carry the Control PDU for flow control feedback as payload.

Observation 2: The BAP flow control mechanism can be used as a base to trigger off the congestion indicator.

Proposal 2: The triggering mechanism and payload for the congestion indicator can be based on the BAP flow control mechanism as defined in section 5.3 and section 6.2.3.1of TS 38.340 [2]

Proposal 3: When the number of times the BAP flow control feedback triggered has reached N (configurable), a congestion indicator is sent to the donor-CU-CP via F1-C

The following shows the control PDU for flow control feedback from TS 38.340 [2]

BAP control PDU format for flow control feedback per BH RLC channel

BAP control PDU format for flow control feedback per routing ID

# 3		Conclusion

In the present contribution we make the following observations:

Observation 1: By having the congested intermediate node to send a congestion indication to the donor-CU-CP allows the donor-CU-CP to take the appropriated action in the control plane to alleviate the congestion, this can be, but not limited to reconfiguration of the routing and mapping information.

Observation 2: The BAP flow control mechanism can be used as a base to trigger off the congestion indicator.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: When DL congestion is detected in an IAB-node, a congestion indicator is sent to the donor-CU-CP via F1-C

Proposal 2: The triggering mechanism and payload for the congestion indicator can be based on the BAP flow control mechanism as defined in section 5.3 and section 6.2.3.1of TS 38.340 [2]

Proposal 3: When the number of times the BAP flow control feedback triggered has reached N (configurable), a congestion indicator is sent to the donor-CU-CP via F1-C

# 4		Reference

[1] TS 38.425, “NR user plane protocol”

[2] TS 38.340, “Backhaul Adaptation Protocol (BAP) specification”