3GPP TSG-RAN WG3 Meeting #110-e	R3-206249

# Online, November 2nd – 12th 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | 4 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Local NG-RAN Node Identifier. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Verizon, Vodafone, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For UEs in RRC_INACTIVE, there is no standardized means to reference the NG-RAN node hosting the UE Context (old NG-RAN node) from the I-RNTI. In case of Resume in a different (new) NG-RAN node, some configuration is required in the old and in the new NG-RAN nodes, so that the new NG-RAN node can resolve the old NG-RAN ID from the I-RNTI. Different proprietary solutions can be used, likely expensive and/or complex to maintain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of a new “Local NG-RAN Node Identifier”. Add a normative section to describe the use of the “Local NG-RAN Node Identifier” to resolve the old NG-RAN node from the I-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only proprietary solutions, expensive and/or complex to maintain, to enable the “new NG-RAN node” to resolve the “old NG-RAN ID” from the I-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, Annex xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 CR R3-206246 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: rebased on 38.423 v16.1.0 Rev 2: resubmitted, rebased on 38.300 v16.1.0 Rev 3: resubmitted, rebased on 38.300 v16.2.0 as NR_newRAT-Core WI Rev 4: rebased on 38.300 v16.3.0 |  |  |  |  |  |  |  |  |

Start of the first change

## 8.2	Network Identities

The following identities are used in NG-RAN for identifying a specific network entity:

-	AMF Name: used to identify an AMF.

-	NR Cell Global Identifier (NCGI): used to identify NR cells globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell. The PLMN ID included in the NCGI should be the first PLMN ID within the set of PLMN IDs associated to the NR Cell Identity in SIB1, following the order of broadcast.

NOTE 1:	How to manage the scenario where a different PLMN ID has been allocated by the operator for an NCGI is left to OAM and/or implementation.

-	gNB Identifier (gNB ID): used to identify gNBs within a PLMN. The gNB ID is contained within the NCI of its cells.

-	Global gNB ID: used to identify gNBs globally. The Global gNB ID is constructed from the PLMN identity the gNB belongs to and the gNB ID. The MCC and MNC are the same as included in the NCGI.

NOTE 2:	It is not precluded that a cell served by a gNB does not broadcast the PLMN ID included in the Global gNB ID.

-	Tracking Area identity (TAI): used to identify tracking areas. The TAI is constructed from the PLMN identity the tracking area belongs to and the TAC (Tracking Area Code) of the Tracking Area.

-	Single Network Slice Selection Assistance information (S-NSSAI): identifies a network slice.

-	Network Identifier (NID): identifies an SNPN in combination with a PLMN ID.

-	Closed Access Group Identifier: identifies a CAG within a PLMN.

-	Local NG-RAN Node Identifier: used to identify an NG-RAN node within a set of NG-RAN nodes that can interoperate over the Xn-C interface.

Next change

Annex xx (normative):

Resolve old NG-RAN node from I-RNTI

The I-RNTI provides the new NG-RAN node a reference to the UE context in the old NG-RAN node.

To support the new NG-RAN node to resolve the old NG-RAN node from the I-RNTI, the new NG-RAN node should include in the first leftmost bits of the I-RNTI a Local NG-RAN Node Identifier as specified in TS 38.423.

End of changes