3GPP TSG-RAN3 Meeting #110-e	R3-206270

E-meeting, 2-12 Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0493 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on collision of AS-rekey and Emergency fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | About AS-Rekay and Emergency Fallback, SA3 replied RAN3 LS (R3-202833) in S3-201484: It is acceptable to SA3, for the network to give up the AS re-keying procedure and only initiate the emergency fallback procedure when the two procedures collide. SA2 replied RAN3 LS (R3-202833) in S2-2005909: When the AS re-keying procedure and the Emergency Fallback procedure collides, the AMF gives up the AS re-keying procedure and only initiates the emergency fallback procedure. If AMF includes both the Security Key IE and the Emergency Fallback Indicator IE within one NGAP UE CONTEXT MODIFICATION REQUEST message, NG-RAN takes it as abnormal case. SA2 approved attached CR to clarify this scenario. It is therefore needed to introduce similar Abnormal Conditions in NGAP UE Context Modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Abnormal Conditions in in NGAP UE Context Modification procedure about the case in which both Security Key IE and Emergency Fallback Indicator IE are received. - The NG-RAN proceeds with Emergency fallback, if success, indicates AMF the AS re-keying procedure is failed due to the collision of the procedures in UE CONTEXT MODIFICATION RESPONSE message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only clarify the handling of simultaneously AS-rekey and Emergency fallback. This CR has an impact under functional point of view. The impact can be considered isolated because the change affects the handling of simultaneously AS-rekey and Emergency fallback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause the IOT issue between NG-RAN node and AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.4, 9.2.2.8, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.3.4	UE Context Modification

<<<<<<<<< Unchanged texts are omitted >>>>>>>>>>>

8.3.4.4	Abnormal Conditions

If the UE CONTEXT MODIFICATION REQUEST message including the New AMF UE NGAP ID IE is received after the NG-RAN node has initiated another class 1 NGAP EP, the NG-RAN node shall be prepared to receive the response message containing an AMF UE NGAP ID with the value received in the New AMF UE NGAP ID IE.

If the NG-RAN node receives both the Security Key IE and the Emergency Fallback Indicator IE in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall proceed with emergency fallback and indicate AMF the failure of the AS re-keying due to the collision in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< the next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the NG-RAN node to confirm the performed UE context updates.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| RRC State | O |  | 9.3.1.92 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| AS re-key is failed | C-ifCollisionwithEmergencyFallback |  | ENUMERATED (true, …) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCollisionwithEmergencyFallback | This IE shall be present if both the Security Key IE and the Emergency Fallback Indicator IE are both included in the UE CONTEXT MODIFICATION REQUEST message. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< the next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

<<<<<<<<< Unchanged texts are omitted >>>>>>>>>>>

id-UERadioCapabilityForPaging,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-ASRekeyIsFailed

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< the next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RRCState					CRITICALITY ignore	TYPE RRCState					PRESENCE optional		}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...,

{ ID id-ASRekeyIsFailed				CRITICALITY ignore	TYPE ASRekeyIsFailed			PRESENCE conditional	}

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< the next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

<<<<<<<<< Unchanged texts are omitted >>>>>>>>>>>

#### -- A

ASRekeyIsFailed  ::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< the next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

<<<<<<<<< Unchanged texts are omitted >>>>>>>>>>>

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-SelectedPLMNIdentity									ProtocolIE-ID ::= 174

id-ASRekeyIsFailed										ProtocolIE-ID ::= xxx

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>