3GPP TSG-RAN WG3 #110-e	R3-20xxxx

E-meeting, 2nd – 12th November 2020	Revise of R3-205121

Source:	CATT

Title:	Discussion on dynamic change between PTM and PTP

Agenda Item:	22.2.3

Document for:	Discussion and decision

- Introduction

During the last RAN3 meeting, it seems consensus that it should be the gNB to decide whether to use PTM or PTP to deliver MBS packets toward a UE. In this contribution, we analyse how to handle such dynamic change between PTM and PTP if the gNB is a split one.

- Discussion

According to TR 23.757, when MBS service is delivered by a shared tunnel over the (MB-)N3 interface, the gNB can decide on its own that whether the data should be delivered in a PTM mode toward the UEs, or delivered in PTP mode separately. When and how the decision is made is obviously out of SA2’s scope (nevertheless the core network may still provide some assistant information toward the gNB if needed), and seemingly out of RAN2’s scope as well since it looks like “network implementation” from the perspective of UEs.

However, in RAN3 it does have some affect toward specifications, considering that one gNB can be further split into multiple parts. When and how the decision to switch the Uu delivery mode will affect whether and what signalling needs to be exchanged over E1AP, F1AP or F1-U maybe. Therefore it seems reasonable to discuss this issue in RAN3.

In our understanding, what Uu mode is used mainly depends on:

- a) whether the QoS demand for the given UE can be met if PTM is used, and

- b) if the QoS demand for the given UE can be met, what choice costs less radio resource, taking all the UEs within the cell into consideration.

Both factors depend on further information, e.g.

- the QoS demand,

- the radio conditions with the given UE (if too low, the UE may not receive anything by PTM effectively and thus PTP has to be used; while if too high, making all UEs receive by PTP may cost less radio resource if the UEs are few),

- observed packet loss rate from the given UE (if there is some kind of UE-specific UL feedback),

- the number of the UEs within the given cell, the load status around the frequency of PTM scheduling (which may make the gNB prefer to use a BWP not overlapping with the frequency of PTM scheduling and thus the UE cannot receive by PTM),

- applicability of space division multiplexing (more SDM often means PTP costs less),

- etc.

Generally speaking, these types of information are easier to acquire within the gNB-DU rather than within the gNB-CU. Therefore, we propose that at least the gNB-DU can trigger the Uu mode switch procedure between PTP and PTM.

Proposal 1: It should be supported that the gNB-DU triggers the Uu mode switch procedure between PTP and PTM for a given UE, if the PTM (MB-)N3 tunnel already exists.

Nevertheless, it is not precluded that gNB-CU (either gNB-CU-CP or gNB-CU-UP) may also trigger the Uu mode switch procedure for a given UE.

- Conclusion

Proposal: It should be supported that the gNB-DU triggers the Uu mode switch procedure between PTP and PTM for a given UE, if the PTM (MB-)N3 tunnel already exists.