3GPP TSG-RAN WG3 #110						                                               	R3-206331

2-12 November 2020

Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0159 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of inter-node handover with a shared split (S)gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,China Telecom,Nokia, Nokia Shanghai Bell,ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Direct_data_fw_NR-Core |  |  |  |  |  | Date: |  |  | 2020-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Considering various types of dual connectivity, it is possible that one gNB entity serves both as the source side and as the target side during an inter-node handover, e.g. inter-MN handover without SN change, handover bewteen NR SA and EN-DC with a shared SgNB/gNB. This very (S)gNB entity may be a CU/DU split or CU-CP/UP one. For this case, it remains unclear whether the existing F1AP/E1AP UE association used before the handover can be reused after the handover. We think UE context modification procedure may be used over F1AP and Bearer context modification procedure may be used over E1AP during the handover preparation phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that existing F1AP/E1AP UE association may be reused after handover if the gNB entity continues to serve for the UE, even if this handover is a inter-node one or even inter-system one. Impact assessment towards the previous version of the specification (same release): This CR is a clarification and has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on dual connectivity. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The signalling flow remains unspecified for inter-node handover with a shared split (S)gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR 38.463CR 0491 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR 38.460CR 0033 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////skip irrelevant text////////////////////////////////////////////////////////////////////////

8.9.x	Inter-node handover with a shared split (S)gNB

For inter-node handover involving dual connectivity, it is possible for one (S)gNB to serve both as the (M)gNB or SgNB before the handover, and as the (M)gNB or SgNB after the handover. If this (S)gNB is a split (S)gNB, the signalling flow may be as follows:

- During the handover preparation phase, a UE Context Modification procedure may be used over F1AP, and a Bearer Context Modification procedure may be used over E1AP. The UE Context Modification procedure over F1AP may be used to setup new radio bearers, modify radio bearers, and/or release old radio bearers. The Bearer Context Modification procedure over E1AP may be used to setup resources for new PDU sessions or E-RABs, and/or modify resources for PDU sessions or E-RABs.

- During the execution phase, context supension may not be invoked, except for the case that the Source (S)gNB-CU-CP may deliver the data forwarding address(es) toward the Source (S)gNB-CU-UP if needed.

- During the completion phase, the (S)gNB-CU-CP may invoke a Bearer Context Modification procedure with the (S)gNB-CU-UP in order to release the resources for old PDU sessions or E-RABs if needed.

////////////////////////////////////////////////////////////////////////end////////////////////////////////////////////////////////////////////////