3GPP TSG-RAN3 Meeting #110e	R3-206336

# Online, November 2nd - 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0501 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR38413 for clarification on UE-associated signalling in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In some NGAP procedures, there are not description on whether they use UE-associated signalling or non UE-associated signalling. Thus, there are ambiguity on whether thy use UE-associated signalling or non UE-associated signalling, which may lead un-alignment between AMF and ng-eNB/gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarify that the following procedures use UE-associated signalling: Handover Preparation Handover Resource Allocation Handover Notification Path Switch Request Secondary RAT Data Usage Report - To clarify that the following procedures use non UE-associated signalling: RAN Configuration Update AMF Configuration Update AMF Status Indication Impact Analysis Impacted functionality: The change only impacts the understanding of whether the respective procedures use UE-associated signalling or non UE-associated signalling. Inter-operability: No inter-operability issue is found.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are ambiguity on whether the related procedures use UE-associated signalling or non UE-associated signalling, which may lead un-alignment between AMFand ng-eNB/gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1, 8.4.2, 8.4.3, 8.4.4, 8.7.2, 8.7.3, 8.7.6, 8.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the 1st modified section>

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

The procedure uses UE-associated signalling.

<Skip unchange part>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

The procedure uses UE-associated signalling.

<Skip unchange part>

### 8.4.3	Handover Notification

#### 8.4.3.1	General

The purpose of the Handover Notification procedure is to indicate to the AMF that the UE has arrived to the target cell and the NG-based handover has been successfully completed.

The procedure uses UE-associated signalling.

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Handover notification

<Skip unchange part>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<Skip unchange part>

<End of the 1st modified section>

<Start of the 2nd modified section>

8.7.2	RAN Configuration Update

8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any.

The procedure uses non UE-associated signalling.

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

<Skip unchange part>

8.7.3	AMF Configuration Update

8.7.3.1	General

The purpose of the AMF Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any.

The procedure uses non UE-associated signalling.

8.7.3.2	Successful Operation

Figure 8.7.3.2-1: AMF configuration update: successful operation

<Skip unchange part>

<End of the 2nd modified section>

<Start of the 3rd modified section>

### 8.7.6	AMF Status Indication

#### 8.7.6.1	General

The purpose of the AMF Status Indication procedure is to support AMF management functions.

The procedure uses non UE-associated signalling.

#### 8.7.6.2	Successful Operation

Figure 8.7.6.2-1: AMF status indication

The AMF initiates the procedure by sending an AMF STATUS INDICATION message to the NG-RAN node.

Upon receipt of the AMF STATUS INDICATION message, the NG-RAN node shall consider the indicated GUAMI(s) will be unavailable and perform AMF reselection as defined in TS 23.501 [9].

The NG-RAN node shall, if supported, act accordingly as specified in TS 23.501 [9], based on the presence or absence of the Timer Approach for GUAMI Removal IE.

If the Backup AMF Name IE is included in the AMF STATUS INDICATION message, the NG-RAN node shall, if supported, perform AMF reselection considering the AMF as indicated by the Backup AMF Name IE as specified in TS 23.501 [9].

#### 8.7.6.3	Abnormal Conditions

Void.

<End of the 3rd modified section>

<Start of the 4th modified section>

8.15.1	Secondary RAT Data Usage Report

8.15.1.1	General

The purpose of the Secondary RAT Data Usage Report procedure is to provide information on the used resources of the secondary RAT (e.g. NR resources during MR-DC operation) as specified in TS 23.501 [9].

The procedure uses UE-associated signalling.

8.15.1.2	Successful Operation

Figure 8.15.1.2-1: Secondary RAT data usage report

The NG-RAN node initiates the procedure by sending the SECONDARY RAT DATA USAGE REPORT message to the AMF.

If the Handover Flag IE is included in the SECONDARY RAT DATA USAGE REPORT message, it indicates that for each PDU session the AMF should buffer the Secondary RAT Data Usage Report Transfer IE since the secondary RAT data usage report is sent due to handover as defined in TS 23.502 [10].

For each PDU session for which the Secondary RAT Usage Information List IE is included in the the Secondary RAT Data Usage Transfer IE, the SMF shall handle this information as specified in TS 23.502 [10].

The NG-RAN node shall, if supported, report in the SECONDARY RAT DATA USAGE REPORT message location information of the UE in the User Location Information IE.

8.15.1.3	Abnormal Conditions

Void.

<End of the 4th modified section>