3GPP TSG-RAN WG3 Meeting #110-e	R3-206342

# E-meeting, 2 – 12 Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | DraftCR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Activation and de-activation of SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN3 has agreed the procedures of deactivation/re-activation of SCG in SN addition procedure and in the SN modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of deactivation/re-activation of SCG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not support the deactivation/re-activation of SCG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.xx(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR 0160 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.xx	Deactivation and re-activation of SCG

In MR-DC with NR SCG, the network can deactivate the SCG in the SN addition procedure and in the SN modification procedure.

Figure 10.xx-1: Support of deactivation/re-activation of SCG in the SN addition procedure

1.	The MN decides to deactivate the NR SCG before adding the NR SCG.

2/3.	The MN triggers the SN addition procedure and informs the SN to deactivate the SCG.

4.	The MN informs the UE to deactivate the SCG.

5/6.	The MN receives the assistance information from the SN and the UE on the user traffic activity of the SCG.

7.	The MN decides to re-activate the SCG.

8/9.	The MN triggers the SN modification procedure and informs the SN to re-activate the SCG.

10.	The MN informs the UE to re-activate the SCG.

Figure 10.xx-2: Support of deactivation/re-activation of SCG in the SN modification procedure

0. The SN notifies the MN about user data inactivity.

1.	The MN decides to deactivate the NR SCG.

2/3.	The MN triggers the SN modification procedure and informs the SN to deactivate the SCG.

4.	The MN informs the UE to deactivate the SCG.

5/6.	The MN receives the assistance information from the SN and the UE on the user traffic activity of the SCG.

7.	The MN decides to re-activate the SCG.

8/9.	The MN triggers the SN modification procedure and informs the SN to re-activate the SCG.

10.	The MN informs the UE to re-activate the SCG.