3GPP TSG-RAN WG3 Meeting #110-e	R3-206343

# E-meeting, 2 – 12 Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0160 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Activation and de-activation of SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN3 has agreed the procedures of deactivation/re-activation of SCG in SN addition procedure and in the SN modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of deactivation/re-activation of SCG in CU/DU architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not support the deactivation/re-activation of SCG in CU/DU architecture |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS37.340 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.4	Multi-Connectivity operation

### 8.4.x	SCG deactivation/re-activation

This clause gives the NR SCG deactivation/re-activation in MR-DC given that the SgNB consists of a gNB-CU and gNB-DU(s).

SN addition procedure

Figure 8.4.x-1: SCG deactivation/re-activation in SN addition procedure

1~2. refer to TS 37.340 [12].

3~4.The CU-CP of SN sends the UE CONTEXT SETUP REQUEST message to the DU of SN to deactivate the SCG of the UE.

5~6.The CU-CP of SN sends the BEARER CONTEXT SETUP REQUEST message to the CU-UP of SN to deactivate the SCG of the UE.

7~8. refer to TS 37.340 [12].

9. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane activity.

10~13. refer to TS 37.340 [12].

14~15.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to re-activate the SCG of the UE.

16~17.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to re-activate the SCG of the UE.

18~19. refer to TS 37.340 [12].

SN modification procedure

Figure 8.4.x-2: SCG deactivation/re-activation in SN modification procedure

0-1. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane inactivity.

0-2~2. refer to TS 37.340 [12].

3~4.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to deactivate the SCG of the UE.

5~6.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to deactivate the SCG of the UE.

7~8. refer to TS 37.340 [12]

9. The CU-UP of SN sends the UE INACTIVITY NOTIFICATION to the CU-CP of SN to notifiy the use plane activity.

10~13. refer to TS 37.340 [12]

14~15.The CU-CP of SN sends the UE CONTEXT MODIFICATION REQUEST message to the DU of SN to re-activate the SCG of the UE.

16~17.The CU-CP of SN sends the BEARER CONTEXT MODIFICATION REQUEST message to the CU-UP of SN to re-activate the SCG of the UE.

18~19. refer to TS 37.340 [12].