3GPP TSG-RAN WG3 Meeting #110-e	R3-206374

E-meeting, 2 – 12 November, 2020

Agenda item:	10.2.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.473) Enhancement of RACH Conflict Resolution

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to TS 38.473, by introducing support for the transfer of RACH Assistance Information for RACH Conflict Resolution on F1 interface according to the discussion in [1]. RACH assistance information comprises RACH Failure and RACH Success information exchange from the gNB-DU to the gNB-CU to enable calculation of the RACH Failure Rate. The RACH Failure Rate information is also sent from the gNB-CU to the gNB-DU.

# References

[1]		R3-206372, RACH Remaining Aspects, Nokia, Nokia Shanghai Bell

# Annex	- TP for TR 38.473

<<< start of changes >>>

### 8.9.1	DU-CU Radio Information Transfer

#### 8.9.1.1	General

The purpose of the DU-CU Radio Information Transfer procedure is to transfer radio-related information from the gNB-DU to the gNB-CU. The procedure uses non-UE-associated signalling.

#### 8.9.1.2	Successful operation

Figure 8.9.1.2-1: DU-CU Radio Information Transfer procedure.

The gNB-DU initiates the procedure by sending the DU-CU RADIO INFORMATION TRANSFER message to the gNB-CU.

If the DU-CU RADIO INFORMATION TRANSFER message contains the DU-CU RIM Information  IE, the gNB-CU considers that the RIM-RS Detection Status IE indicates the RIM-RS detection status of the cell identified by Aggressor Cell ID IE.

If the DU-CU RADIO INFORMATION TRANSFER message contains the DU-CU RIM Information  IE, the gNB-CU shall take it into account for optimisation of RACH access procedures.

#### 8.9.1.3	Abnormal Conditions

Not applicable.

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE or the RACH Conflict Resolution Info IE, the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

### 9.2.8	Radio Information Transfer messages

#### 9.2.8.1	DU-CU RADIO INFORMATION TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE DU-CU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>DU-CU RIM Information | M |  | 9.3.1.91 |  | - | - |
| >RACH Conflict Resolution Info |  |  |  |  |  |  |
| >>DU-CU RACH Failure and Success Information | O |  | 9.3.1.X |  | YES | ignore |

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RACH Conflict Resolution Info | O |  |  |  |  |  |
| > CU-DU RACH Failure and Success Information | O |  | 9.3.1.Y |  | YES | ignore |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |

#### 9.3.1.X	DU-CU RACH Failure and Success Information

The DU-CU RACH Failure and Success Information IE contains information about RACH successes and failures extracted at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | 9.3.1.12 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of RACH Failures on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Successes on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Failures on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |
| >>>Number of RACH Successes on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

#### 9.3.1.Y	CU-DU RACH Failure and Success Information

The CU-DU RACH Failure and Success Information IE contains information about RACH successes and failures calculated at cells that are neighbor to cells served by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Reporting cell list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Reporting Cell ID | M |  | NR CGI 9.3.1.12 | The cell for which the RACH failure rate is calculated. |  |  |
| >>Neighbour Cell List |  | 1 |  | List of cells in the gNB-DU that are neighbours to the reporting cell |  |  |
| >>>Neighbour Cell Item |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Neighbour Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>SSB List |  |  |  | List of SSBs in the cell for which the RACH failure rate is calculated |  |  |
| >>>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of RACH Failures on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Successes on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Failures on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |
| >>>Number of RACH Successes on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<<< end of changes >>>