3GPP TSG-RAN WG3 Meeting #110-e	R3-206414

E-meeting, 2 - 12 Nov 2020

Title: 	(TP to TS 38.470 BL CR) Bearer management over F1 interface

Source: 	Huawei

Agenda item:	22.2.4

Document Type:	other

1. Introduction

In this contribution, we give a corresponding TP 38.470 on bearer management over F1 interface. The details for discussion can be referred to [1].

2. Text proposal to TS 38.470

--------------------------------Start of the Next Change-----------------------------

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BH	Backhaul

DRB	Data Radio Bearers

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhauling

IP	Internet Protocol

MB                      Multicast and Broadcast

NR-MIB	NR-Master Information Block

O&M	Operation and Maintenance

PA	Paging Area

PF	Paging Frame

PO	Paging Occasion

QoS	Quality of Service

RIM	Remote Interference Management

RLC	Radio Link Control

RRC	Radio Resource Control

SCTP	Stream Control Transmission Protocol

SRB	Signalling Radio Bearers

SIB1	System Information Block 1

SIB10	System Information Block 10

SL	Sidelink

TNL	Transport Network Layer

V2X	Vehicle-to-Everything

--------------------------------Start of the Next Change-----------------------------

5	Functions of the F1 interface

//skip unchanged part

5.X	Multicast Distribution Management function

The Multicast distribution management function is responsible for establishing and releasing the F1-U Multicast Distribution resources.

--------------------------------Start of the Next Change-----------------------------

6	Procedures of the F1 interface

//skip unchanged part

6.1.X	Multicast Distribution Management procedures

The following list of Multicast distribution management procedures are used to establish or release the F1-U Multicast Distribution resources. The procedures providing this functionality are:

-	Multicast Distribution Setup

-	Multicast Distribution Release

--------------------------------End of the Changes-----------------------------

3. Reference

- R3-206413 Bearer Management over F1 and E1, Huawei