3GPP TSG-RAN WG3 Meeting #110-e	R3-206415

E-meeting, 2 - 12 Nov 2020

Title: 	(TP to TS 38.460 BL CR) Bearer management over E1 interface

Source: 	Huawei

Agenda item:	22.2.4

Document Type:	other

1. Introduction

In this contribution, we give a corresponding TP 38.460 on bearer management over E1 interface. The details for discussion can be referred to [1].

2. Text proposal to TS 38.460

--------------------------------Start of the Next Change-----------------------------

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DL	Downlink

DRB	Data Radio Bearer

E1AP	E1 Application Protocol

IP	Internet Protocol

MB                      Multicast and Broadcast

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

--------------------------------Start of the Next Change-----------------------------

5	Functions of the E1 interface

//skip the unchanged part

5.2	TEIDs allocation

The gNB-CU-UP is responsible for the allocation of the F1-U UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the S1-U DL GTP TEID for each E-RAB and the NG-U DL GTP TEID for each PDU Session.

The gNB-CU-UP is responsible for the allocation of the X2-U DL/UL GTP TEID or the Xn-U DL/UL GTP TEID for each data radio bearer.

The gNB-CU-UP is responsible for the allocation of the NG-U DL GTP TEID for each MBS Session.

5.1.3	Load management function

The load management function allows an gNB-CU-CP to request the reporting of load measurements to gNB-DU and is used by gNB-CU-UP to report the result of measurements admitted by gNB-CU-UP.

5.1.x	Multicast Distribution Management function

The Multicast distribution management function is responsible for establishing or releasing the F1-U Multicast Distribution resources.

--------------------------------Start of the Next Change-----------------------------

6	Procedures of the E1 interface

//skip unchanged part

6.X	Multicast Distribution Management procedures

The following list of Multicast distribution management procedures are used to establish or release F1-U Multicast Distribution resources. The procedures providing this functionality are:

-	Multicast Distribution Setup

-	Multicast Distribution Release

--------------------------------End of the Changes-----------------------------

3. Reference

- R3-206413 Bearer Management over F1 and E1, Huawei