3GPP TSG-RAN WG3 Meeting #110-e	R3-206417

E-meeting, 2 – 12 Nov 2020

Title: 	(TP to BL CR of TS38.401) Mobility between MBS supporting nodes

Source: 	Huawei

Agenda item:	22.3.1

Document Type:	other

1. Introduction

This contribution provides the TP to BL CR of TS38.401, to support Mobility between MBS supporting nodes.

2. Text Proposal

--------------------------------Start of the Next Change-----------------------------

8.xx	Overall procedures for NR MBS

The following clauses describe the overall procedures for NR MBS involving E1 and F1.

8.xx.x	Inter-gNB-DU Mobility with MBS operation

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU with MBS operation. Figure 8.xx.x-1 shows the inter-gNB-DU mobility procedure with MBS operation.

Figure 8.xx.x-1: Inter-gNB-DU Mobility with MBS operation

1.	The UE sends a MeasurementReport message to the source gNB-DU.

2.	The source gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received MeasurementReport message.

3.	The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the target gNB-DU to create an UE context and provide MBS information including the TMGI.

4.	The target gNB-DU responds to the gNB-CU with an UE CONTEXT SETUP RESPONSE message, and provide the MBS configuration of the MBS session within the RRC container.

5. In case the F1-U multicast distribution needs to be setup, the target gNB-DU triggers the Multicast Distribution Setup procedure by including the DL F1-U GTP-U Tunnel info. Otherwise the step 5 and 5a to step 8a are skipped.

6.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU, which includes a generated RRCReconfiguration message with MBS configuration.

7.	The source gNB-DU forwards the received RRCReconfiguration message with MBS configuration to the UE.

8.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

9.	The UE access to the cell in the target gNB-DU.

10.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU, the source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

11.	In case the F1-U multicast distribution needs to be released, the multicast distribution release procedure will be triggered, otherwise the step 11 is skipped.

--------------------------------End of the Changes-----------------------------