3GPP TSG-RAN WG3 Meeting #110e	R3-206483

Online, 2-12 Nov 2020

Agenda item:	  22.2.1

Source:		Lenovo, Motorola Mobility

Title:		 Provisioning of Broadcast Services in RAN

Document for:		Discussion and Decision

1	Introduction

In this paper, we discuss provisioning of broadcast services in RAN.

2	Discussion

At last RAN#89-e meeting discussion took place on the support of broadcast services in Rel-17 based on the incoming SA2 LS [1]. Following discussion RAN agreed to keep the support of broadcast services in the scope of the Rel-17 NR MBS WI and sent corresponding reply LS [2] to SA2.

Referring to the SA2 study and the RAN2 email discussion [3] the MBS Broadcast services can be characterized as follows:

- The network is not aware about UEs receiving broadcast contents.

- Only PTM transmission applies.

- Any UE located in a Broadcast service area can receive Broadcast contents.

In view of above characteristics, we suggest the following in order to support reception of Broadcast services by the UEs incl. service continuity:

- Broadcast services can be received by the UEs in any RRC state, i.e. RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

- The LTE SC-PTM solutions wrt PTM configuration and operation shall be reused as much as possible.

- No PTP transmission of Broadcast services is supported.

- Use existing NG-RAN architecture and No MCE entity/node in RAN architecture for broadcast.

- No SYNC protocol for this release for broadcast.

- It is FFS on whether MBS counting function is supported for broadcast.

- It is FFS on whether MBS suspension and resumption function are supported for broadcast.

- A UE in RRC_IDLE and RRC_INACTIVE that is receiving or interested in receiving a Broadcast service can prioritize a frequency for reselection that provides the concerned service, i.e. the UE may consider the frequency supporting the broadcast service as highest priority.

- A UE in RRC_CONNECTED that is receiving or interested in receiving a Broadcast service can indicate its MBS interest to the gNB for service continuity.

Proposal: RAN3 is asked to agree on the proposals to support reception of Broadcast services by the UEs incl. service continuity.

3	Conclusion

In this contribution, we discuss provisioning of broadcast services in RAN. The following proposals are made:

Proposal: RAN3 is asked to agree on the proposals to support reception of Broadcast services by the UEs incl. service continuity.

- Broadcast services can be received by the UEs in any RRC state, i.e. RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

- The LTE SC-PTM solutions wrt PTM configuration and operation shall be reused as much as possible.

- No PTP transmission of Broadcast services is supported.

- Use existing NG-RAN architecture and No MCE entity/node in RAN architecture for broadcast.

- No SYNC protocol for this release for broadcast.

- It is FFS on whether MBS counting function is supported for broadcast.

- It is FFS on whether MBS suspension and resumption function are supported for broadcast.

- A UE in RRC_IDLE and RRC_INACTIVE that is receiving or interested in receiving a Broadcast service can prioritize a frequency for reselection that provides the concerned service, i.e. the UE may consider the frequency supporting the broadcast service as highest priority.

- A UE in RRC_CONNECTED that is receiving or interested in receiving a Broadcast service can indicate its MBS interest to the gNB for service continuity.

Reference

[1]	RP-201437, LS on RAN impact of FS_5MBS Study (S2-2006044, contact: Huawei), RAN#89e

[2]	RP-202086, Reply LS on RAN impact of FS_5MBS Study (contact: Huawei), RAN#89e

[3]	R2-20xxxxx, Summary of Email Discussion [Post111-e][906][MBS] Idle mode support, CATT