3GPP TSG-RAN3 #110-e	R3-206509

2nd November – 12th November 2020

Online

Agenda Item:	10.2.1.3

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.413): Inter RAT Energy Saving and Successful Handover support

Document for:	Discussion, TP

# Introduction

In the New WID on enhancement of data collection for SON/MDT in NR that was approved in RAN #86, in the objectives of the WID, support of Successful Handover Reports is included, as well as a need for Inter-RAT Inter-system energy saving solution.

To fulfil these tasks signalling of the successful HO report is needed. In the case that a direct Xn interface does not exist between the target and the source NG-RAN node, then there is need for the Successful Handover Report to be signaled over NG.

Therefore, we propose that the Uplink RAN Configuration Transfer and Downlink RAN Configuration Transfer messages are used to transmit the Successful Handover Report to the source NG-RAN node over NG in a container “Successful HO Report” that needs to be defined.

In order to fulfil the agreements taken by RAN3 on Inter RAT Energy Saving, the following is proposed

• The NG-RAN node sends Inter-System Cell State Indication to the eNB containing notification information for inter-RAT Cell Activation and Deactivation.

• The eNB can send Inter-System Cell Activation Request containing request information for inter-RAT Cell Activation.

• The NG-RAN node can send reply to reception of the Inter-System Cell Activation Request by signalling the Inter-System Cell Activation Response containing response information for inter-RAT Cell Activation.

# Text proposals for TS 38.413

============ Next change ==============

#### 9.3.3.35	SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >Failure Indication Information |  |  |  |  |
| >>Failure Indication | M |  | 9.3.3.37 |  |
| >HO Report Information |  |  |  |  |
| >>HO Report | M |  | 9.3.3.39 |  |
| >Successful HO Report Information |  |  |  |  |
| >>Successful HO report container FFS | M |  | OCTET STRING | Successful-HO-Report-r16 contained in the UEInformationResponse message (TS 38.331 [18]) FFS |

============ Next change ==============

#### 9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >NR Cell Activation Request |  |  |  |  |
| >>Inter-System Cell Activation Request | M |  | 9.3.3.x |  |
| >NR Cell Activation Response |  |  |  |  |
| >>Inter-System Cell Activation Response | M |  | 9.3.3.y |  |
| >Energy Savings Indication |  |  |  |  |
| >>Inter-System Cell State Indication | M |  | 9.3.3.z |  |

============ Next change ==============

## 9.3.3.x	Inter-System Cell Activation Request

This IE contains request information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-System Cells to Activate List |  | 1 .. <maxnoofCellingNB> |  |  |
| >Cell Identifier | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI |
| Minimum Activation Time | O |  | INTEGER (1..60) | Seconds |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellingNB | Maximum no. cells that can be served by an gNB. Value is 16384. |

============ Next change ==============

## 9.3.3.y	Inter-System Cell Activation Response

This IE contains response information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-System Activated Cells List |  | 0 .. <maxnoofCellingNB> |  |  |
| >Cell Identifier | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellingNB | Maximum no. cells that can be served by an gNB. Value is 16384. |

============ Next change ==============

## 9.3.3.z	Inter-System Cell State Indication

This IE contains notification information for inter-RAT Cell Activation and Deactivation

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-System Notification Cell List |  | 1 .. <maxnoofCellingNB> |  |  |
| >Cell Identifier | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI |
| >Notify Flag | M |  | ENUMERATED (Activated, Deactivated, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellingNB | Maximum no. cells that can be served by an gNB. Value is 16384. |

============ End of changes ==============